3GPP TSG-RAN WG4 Meeting #118                                                             draft R4-2602284

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			8.1

Source:	Feature lead (OPPO)

Title:	WF on [118][101-B] 6G system parameter (part II)

Document for:	Information

- Introduction

This document provides feature lead summary for 6GR system parameters including:

- Modulation

  - NR modulations

  - UL 1024QAM

  - DL 4096QAM

  - Constellation shaping

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Device types

  - Smallest max CBW

  - Device types

- Topic #1: Modulation

Open issues summary

Sub-topic 1-1: General

Issue 1-1-1 Evaluation Cases

Agreement:

- For the existing NR modulations (UL BPSK to 256QAM, and DL QPSK to 1024QAM):

  - No feasibility study required.

  - To be discussed in UE RF thread with new 6G assumptions like PA model and RF impairments.

- For higher-order modulation:

  - UL 1024QAM is considered as high priority in RAN4 evaluation

    - FWA UE at the beginning

    - Already agreed to start parallel study in last RAN4 meeting

  - DL 4096QAM:

    - FWA UE at the beginning.

    - Focusing on both BS and UE implementation feasibility.

    - Start the evaluation after UL 1024QAM has got enough progress.

- For constellation shaping:

  - Defer detailed evaluation until RAN1 made sufficient progress.

Issue 1-1-2 R1/R4 work split

Agreement:

- RAN4 works on EVM budget evaluation, power backoff and LLS/SLS as usual to complete the feasibility study and define applicable EVM requirements.

    - RAN4 strives to align with R1 on the LLS/SLS configurations if possible.

    - RAN4 target to stabilize the EVM budget range and the LLS assumption in RAN4#118bis.

Sub-topic 1-2: NR modulations

Issue 1-2-2 EVM for the NR modulations

Agreement:

- Discuss in UE RF thread.

Sub-topic 1-3: UL 1024QAM

Issue 1-3-1 Support UL 1024QAM

Agreement:

- Postpone until the evaluations results are clear.

Issue 1-3-2 Tx/Rx EVM budget

Agreement:

      - Targeting bands:

        - Focus on around 7GHz for feasibility study at this moment.

          - Other bands are not precluded.

      - The CBW used in the evaluation is 200MHz.

      - Regarding RF impairments in EVM budget evaluation:

        - It is only for high modulation feasibility study purpose. It doesn’t impact the discussion in UE RF thread for requirement definition.

      - Regarding PA models in the EVM evaluation:

        - Proper PA models from each company can be used.

      - FFS on below aspects

        - whether to consider DPD and/or DPoD in UL 1024QAM feasibility evaluation

        - unequal EVM split between BS Rx EVM and UE Tx EVM in supporting UL 1024QAM.

        - whether transients and RF chain noise floor impacts need to be considered in UL 1024QAM feasibility evaluation, and its related additional EVM margins

        - whether/how edge RB EVM is considered in addition to the average EVM for 200MHz bandwidth.

      - Below table is collecting of EVM budgets from each company in RAN4#118:

        - For around 7GHz:

| EVM contributor @ 7GHz |  | EVM (%) | SNR (dB) |
| --- | --- | --- | --- |
| UE Tx EVM | PA | 1.8 OPPO/ZTE 1.5 Skyworks | 34.9 OPPO/ZTE 36.5 Skyworks |
|  | Transmitter | 1.03 OPPO 1.2 ZTE 0.5 Skyworks | 39.7 OPPO 38.4 ZTE 46 Skyworks |
|  | LO Phase noise | 1.4 ZTE 1.0 Skyworks 1.0 OPPO/QC/MTK (FFS feasibility) | 37.1 ZTE 40 Skyworks 40 OPPO/QC/MTK (FFS feasibility) |
|  | IQ imbalance | 1.0 OPPO/QC/ZTE/Skyworks 1.5 OPPO | 40 OPPO/QC/ZTE/Skyworks 36.5 OPPO |
|  | Total Tx EVM | 1.8 LGE ~2 QC 2.12 Skyworks 2.5 OPPO 2.8 SONY/ZTE 3 OPPO Summary: [1.8~3] % Average: 2.51 % | 34.9 LGE 34 QC 33.5 Skyworks 32 OPPO 31.1 SONY/ZTE 30.6 OPPO Summary: [30.6~34.9] Average: 32 dB |
| BS Rx EVM budget |  | 2 Skyworks | 34 Skyworks |
| Total Tx+ Rx EVM |  | 2.92 Skyworks 2.88 + ∆ Samsung | 30.7 Skyworks 30.8 Samsung |

        - For below 4.2GHz

| EVM contributor < 4.2GHz |  | EVM (%) | SNR (dB) |
| --- | --- | --- | --- |
| Tx EVM | PA | 1.5 ZTE | 36.5 ZTE |
|  | Transmitter | 1.1 ZTE | 39.2 ZTE |
|  | LO Phase noise | 1.4 ZTE | 37.1 ZTE |
|  | IQ imbalance | 1.0 ZTE | 40 ZTE |
|  | Total Tx EVM | 2.5 SONY/ZTE | 32 SONY/ZTE |
| Rx EVM budget |  |  |  |
| Total Tx+ Rx EVM |  |  |  |

Issue 1-3-3 LLS assumptions

Agreement:

- The metric for LLS is the SINR vs Throughput, and purpose is to find out the SINR boundary at which the 1024QAM shows performance gain than UL 256QAM under the Tx and Rx EVM assumptions.

  - Other metrics can be considered based on company inputs.

For information:

- Below table is considered as starting point for UL 1KQAM LLS SINR evaluation.

- Companies are encouraged to check the configurations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 7GHz |
| CBW/SCS | 200MHz with 30kHz SCS |
| Waveform | CP-OFDM |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency AWGN |
| MCS | Based on NR MCS in Table 5.1.3.1-4 of TS 38.214: - UL 1KQAM: MCS 23, 24 - UL 256QAM: MCS 22 |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM + rxEVM | Example: - txEVM = rxEVM: 2.5%+2.5%, 3%+3% - txEVM > rxEVM: 3%+2%, 3%+2.5% |

Issue 1-3-4 SLS assumptions

Agreement:

- The metric for SLS is the SINR CDF in NW, and purpose is to find out the probability of targeting SINR that can be achieved.

  - Other metrics can be considered based on company inputs.

For information:

- Below table is considered as starting point for UL 1KQAM SLS SINR evaluation.

        - FFS on UE antenna configurations

        - FFS on BS antenna configurations

        - Companies are encouraged to check the configurations.

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Frequency band |  | 7GHz |  |
| System bandwidth |  | 200MHz with 30kHz SCS |  |
| UE max output power |  | PC2: 26 dBm |  |
| UE antenna configuration |  | CPE antenna: 2×4 array, 6.4 dBi gain per element, and 15.4 dBi max gain |  |
| Network layout |  | 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 500m | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Channel model |  | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Handover margin (dB) |  | 3 |  |
| Pathloss |  | 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873 | 3D-InH LOS and NLOS in Table 7.2-1 of 36.873 |
| BS antenna configuration (Align with R1 assumptions) |  | (M,N,P,Mg,Ng; Mp, Np)= (24, 16, 2, 1, 1; 4, 16) (dH, dv) = (0.5λ, 0.8λ) GE,max = 8 dBi | (M,N,P,Mg,Ng; Mp, Np)= (4, 8, 2, 1, 1; 2, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| Mechanic tilt |  | 10° | Boresight direction is perpendicular to the ceiling |
| BS noise figure |  | Around 7GHz: [5@R1, 6]dB | Around 7GHz: [5@R1, 14]dB |
| Target SNR at BS side |  | Get from link-level simulation |  |
| UE noise figure |  | [7@R1, 9]dB |  |
| Power control |  | TPC model specified in clause 9.1 TR 36.942 is applied with following parameters. - CLx-ile = 88 + 10*log10(200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure - γ = 1 |  |

Sub-topic 1-4: DL 4096QAM

Issue 1-4-1 Supporting DL 4096QAM

Agreement:

- Postpone until the evaluations results are clear.

Issue 1-4-5 Impacts to RF requirements

Agreement:

- BS RF impact can be discussed in BS RF thread.

Sub-topic 1-5: Constellation shaping

Issue 1-5-1 Impacts to RF requirements

Agreement:

- Hold on the discussion until sufficient progress are made in RAN1.

- Topic #2: Channel arrangement

Open issues summary

Sub-topic 2-1: Channel raster

Issue 2-1-1 Channel raster design

Agreement:

- Channel raster is defined in 6G specification.

- Only define one channel raster granularity (ΔFRaster) for each band or [sub-frequency range].

- When defining channel raster, the sync raster impact is considered.

- MRSS for the re-farming bands is considered.

- For re-farming bands with 100khz channel raster in NR, consider the below options

  - 10khz

  - 5khz

  - SCS based

    - shifting specific bands from the 100 kHz to an SCS-based raster, the decision must be based on comprehensive inputs from operators.

  - Other options are not precluded.

- For bands with SCS based channel raster in NR and for new bands, SCS based channel raster is adopted.

Issue 2-1-2 Relation b/w UE dedicated CBW and Channel raster

Agreement:

- channel bandwidth in UE dedicated CBW must be aligned with channel raster, whether to reflect them into RAN2 spec is up to RAN2.

Sub-topic 2-2: Sync raster

Issue 2-2-1 Sync raster design

Agreement:

- Following approaches can be considered with scheme details FFS:

  - Option 1: Two-level sync raster scheme, i.e., a coarser primary sync raster and a finer secondary sync raster

  - Option 2: Increase the channel raster step size in sync raster design, i.e., the reference channel raster used in sync raster calculation is N*channel raster (Basic Sync raster <= MinCBW - BWSSB + N*CHraster).

  - Option 3: Scalable sync raster step-size pending on SSB periodicity

  - Option 4: Consider PSS BW instead of SSB BW in 6G sync raster design (i.e., Sync raster = TBW – PSS BW + channel raster)

  - Option 5: CBW-dependent sync raster design

  - Option 6: valid channel raster grid is determined based on synchronization raster density and configuration

  - Option 7: the channel- and synchronization raster for 6GR shall be specified such that MRSS with PRB alignment to the NR carrier can be configured for all possible 100 kHz NR channel raster entries.

  - Other options are not precluded.

Sub-topic 2-3: Channel spacing

Issue 2-3-1 Channel spacing for CA and non-CA

Agreement:

- For FR1 and around 7GHz, mixed numerology scenarios are not considered in channel spacing definition for intra-band CA and non-CA

- FFS on the nominal channel spacing for CA and non-CA with single numerology

- Topic #3: Device types

Open issues summary

Feature Lead note: RAN#110 agreements in RP-253856.

Proposal 1: - 6GR supports the operation (but not required to be optimized for performance) in a minimum spectrum allocation of 3MHz with a 15kHz SCS Note: the following agreement made in RAN1#123 still holds, with the clarification that the bandwidth in Opt 1 below is assumed to be at least 5MHz with a 15kHz SCS. Agreement If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, - Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable - Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations - Regarding the smallest maximum UE bandwidth as discussed in the following RAN1 agreement, Opt 1 is excluded. Aim to conclude by RAN plenary no later than RAN#112 (June 2026). - RAN1 and RAN4 is tasked to continue providing more analysis accordingly. - Companies are encouraged to provide more analysis at RAN plenary particularly regarding the use cases, requirements, economy of scale, etc. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different Proposal 2: - Diverse device types as in 6G SIDs are expected to include supporting at least MBB (as highest priority), FWA, and Massive IoT services - Companies are encouraged to further contribute at RAN plenary regarding the analysis of necessary aspects to help progress RAN WG technical design and evaluations. - The detailed discussion for device types (how many, parameters, whether or not to define device types, whether or not to introduce minimum mandatory set of features, etc.) is on hold in RAN plenary and to resume from RAN#113 (September 2026). Discussions on device types are not to be discussed in WGs until further update from RAN plenary. Proposal 3: Capture the following aspects related to diverse device types into the TR: - Scalable and forward compatible design - Limited set of device types (based on the agreement in RAN#109) Detailed wording is to be drafted in RAN#111, along with possibly other aspects (e.g., scalability, differentiated services, etc.).

Sub-topic 3-1: Smallest max UE bandwidth

Issue 3-1-1 Smallest max UE bandwidth

Feature lead recommended WF:

- Smallest max CBW for DL

  - 20MHz for both RF and BB from RAN4 perspective

- FFS on below smallest max CBW options for UL

  - Option 1: 20MHz for both RF and BB

  - Option 2: 5MHz for both RF and BB

  - Option 3: 20MHz for RF and 5MHz for BB

  - FFS on the FDD band and TDD band differentiation.

  - FFS how to enable SAW-less design regardless of the smallest max CBW definition.

    - Whether requirements will be defined for SAW-less design UE belongs to normative work phase.

- Encourage companies to provide analysis on the smallest max CBW in the upcoming meeting.

Issue 3-1-2: How to enable SAW-Less design

Agreement:

- For 6G lowest tier device, study how to support the SAW-less design for HD-FDD/TDD, below solutions can be considered as starting point pertaining to different smallest max CBW:

  - Option 1: Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

  - Option 2: MPR/AMPR for certain bands

  - Option 3: Relaxation of the spurious emission requirement for UE cross-band coexistence.

  - Other options are not precluded

- FFS on the applicable bands.

- The agreement here doesn’t impact the 6GR smallest max CBW also min CBW discussion.

- FFS on the requirement impacts with the SAW-less design, e.g., Tx emission requirement, Rx blocking requirements.

Sub-topic 3-2: Device types

Issue 3-2-1: Device type related proposals

Feature lead recommended WF:

- Hold on the discussion of device type according to RAN plenary agreement until September 2026.