3GPP TSG-RAN WG4 Meeting #118                                                                             R4-2602140

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			8.1

Source:	Feature lead (OPPO)

Title:	Topic summary for [118][101-B] 6G system parameter (part II)

Document for:	Information

- Introduction

This document provides feature lead summary for 6GR system parameters including:

- Modulation

  - NR modulations

  - UL 1024QAM

  - DL 4096QAM

  - Constellation shaping

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Device types

  - Smallest max CBW

The running summary reflects the status of each system parameter and the previous agreements reached in RAN4, RAN1 and RAN can be found at: R4-2600884.

- Topic #1: Modulation

Companies’ contributions summary

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2600313 | Further discussion on modulation for 6GR | CATT |
| R4-2600386 | 6GR - Modulation | Nokia |
| R4-2600458 | View on 6GR modulation orders | Xiaomi |
| R4-2600575 | On 6G system parameters - Modulation | Apple |
| R4-2600668 | Discussion on 6G modulation | vivo |
| R4-2600698 | (6G system parameters) Modulation | LG Electronics |
| R4-2600737 | On EVM budget for 6G UL and DL higher order modulation | Skyworks Solutions Inc. |
| R4-2600813 | Discussion on 6GR modulation | CMCC |
| R4-2600886 | On system parameters for 6G —— Modulation | Huawei, HiSilicon |
| R4-2600909 | Views on 6G Modulation | MediaTek inc. |
| R4-2601053 | Views on 6G modulation | Spreadtrum,UNISOC |
| R4-2601123 | Discussion on modulation for 6GR | Samsung |
| R4-2601177 | Views on 6GR modulation | ZTE,Sanechips |
| R4-2601397 | views on modulation study of 6GR | Sony |
| R4-2601419 | Discussion on 6G modulation | OPPO |
| R4-2601466 | Views on 6G modulation | NTT DOCOMO, INC. |
| R4-2601955 | DL 4k-QAM & UL 1k-QAM achievability in a UMa FWA scenario | CableLabs, Charter Communications, Rogers |
| R4-2602059 | Qualcomm views on 6G UL 1024QAM | Qualcomm Incorporated |

Open issues summary

Background: WF in previous RAN4 meetings.

Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

Recommendation from FL in RAN4#117 - Evaluation Cases: - Case 1 (Baseline): Existing NR modulations (BPSK to 256QAM) with new 6G PA model(s). - Case 2 (Higher-order modulation): - UL 1024QAM: Primarily focusing on FWA UE implementation feasibility. - Already agreed to start parallel study in last RAN4 meeting - DL 4096QAM: Focusing on both BS and UE implementation feasibility. - Whether and when to consider it as an optional feature for study, pending on further RAN4 discussion and decision - Case 3 (Constellation shaping): Defer detailed evaluation until RAN1 conclusions are stable. - Evaluation assumptions: - PA Model: Depends on the discussion progress on 6G PA model. This is a foundational assumption for all modulation studies. - EVM budget: Define a clear EVM budget for higher order modulation (UL 1024QAM, DL 4096QAM), considering all impairment sources (PA non-linearity, I/Q imbalance, phase noise, CFR, etc.). - Scenarios and frequencies: Focus evaluations on agreed scenarios (TBD, like Urban Macro and indoor hotspot), across agreed frequencies (TBD, like ~700 MHz, 2 GHz, and 7 GHz). - Bandwidth: Consider wider channel bandwidths (TBD, e.g., 200 MHz)

Sub-topic 1-1: General

Issue 1-1-1 Evaluation Cases

Proposals:

- CATT R4-2600313

- Proposal 1: RAN4 could prioritize the evaluation of existing and new higher-order modulation schemes, without considering constellation shaping, under new 6G PA model(s) at 7 GHz.

- Xiaomi R4-2600458:

- For CAT 1 (NR modulation orders with uniform constellation):

      - RAN4 can further evaluate Tx EVM or MPR requirements improvement/relaxation by taking existing NR requirements as baseline (NO feasibility study required, this work not urgent and shall be handled in UE RF thread instead of system parameter thread)

- For CAT 2 (Higher modulation orders with uniform constellation including DL 4K QAM and UL 1K QAM): RAN4 focus on the evaluation of achievable Tx EVM/Rx EVM and MPR assumption in UL side

      - Taking as first priority as RAN1 required input from RAN4 on achievable Tx EVM/Rx EVM and MPR assumption

- For CAT 3 Constellation shaping schemes (PS and GS): RAN4 need to wait sufficient progress from RAN1 to evaluate potential impact on Tx EVM, MPR and other potential requirements impact

      - Wait for sufficient progress reached in RAN1 on detailed shaping algorithm

- Proposal 6: RAN4 needs to evaluate the limitation of achievable Tx EVM and MPR performance for DL 4KQAM and UL 1KQAM with associated device type assumption for high modulation (e.g. FWA only) and applicable operating frequency.

- Proposal 7: RAN4 also needs to evaluate the possibility of supporting 1K QAM for handheld UE in 6GR.

- Huawei R4-2600886

- Proposal 1: Identify clear evaluation cases for modulation in RAN4. The initial evaluation is based on uniform constellation as NR, whether to extend the evaluation to non-uniform constellation and/or 4096QAM pending on RAN1 final conclusion.

- Proposal 4: For existing NR modulations with uniform constellations, the evaluation should focus on assessing the MPR reduction under new transmitter impairment assumptions and with the new PA model.

- Proposal 5: For existing NR modulations that may adopt non-uniform constellations, When to start the evaluation pending on RAN1 conclusion.

- Spreadtrum R4-2601053

- Proposal 5: We can wait for the conclusion about the new PA model to evaluate existing supported modulation for 6GR.

- Samsung R4-2601123

- Proposal 1: RAN4 keeps the existing NR requirements as the baseline and studies higher-order uniform QAM feasibility concurrently with RAN1.

- CableLabs, Charter, Rogers R4-2601955

- Proposal 1: Based on the RAN4 and RAN1 October 2025 meeting agreements and our system-level simulation results, which indicate that UL 1024-QAM and DL 4096-QAM are widely achievable in a UMa FWA scenario, we propose that RAN4 continue to study the use of DL 4096-QAM, in addition to UL 1024-QAM, for 6GR FWA operation.

- Apple R4-2600575:

- Proposal 3: It is proposed to limit the support of UL 1024QAM to FWA UE and further discuss how the existing 2.5% or 2.8% EVM requirements can be met.

- Sony R4-2601397

- Proposal 2: RAN4 to study the modulation as part of a scalable design, to determine the proper order of modulation to be supported by each device type.

Feature Lead note:

- Case 1: Existing NR modulations (BPSK to 256QAM) - Case 2 (Higher-order modulation) - UL 1024QAM - DL 4096QAM - Case 3 (Constellation shaping)

- It is widely acknowledged that for different modulation cases, the RAN4 evaluations are different.

- For the existing NR modulations, there is no feasibility issue. What need to be done in RAN4 is to evaluate the applicable MPR based on new assumptions in RAN4 like PA model and RF impairments which should be handled in UE RF thread.

- For the higher order modulation, the evaluation focuses on the feasibility part and also the applicable EVM requirements.

  - The applicable device type is also one of the concerns which need to be clarified in the beginning.

- For the constellation shaping, it is premature for RAN4 to be involved for this moment. More progress is needed in RAN1.

Recommended WF:

- For the existing NR modulations (BPSK to 256QAM, and DL 1024QAM):

  - No feasibility study required.

  - To be discussed in UE RF thread with new 6G assumptions like PA model and RF impairments.

- For higher-order modulation:

  - UL 1024QAM is considered as high priority in RAN4 evaluation

    - Focus on FWA UE type.

    - Already agreed to start parallel study in last RAN4 meeting

  - DL 4096QAM:

    - Focus on FWA UE type.

    - Focusing on both BS and UE implementation feasibility.

    - Start the evaluation after UL 1024QAM has got enough progress.

- For constellation shaping:

  - Defer detailed evaluation until RAN1 conclusions are stable.

Issue 1-1-2 R1/R4 work split

- Xiaomi R4-2600458:

- Proposal 1: RAN4 work closely with RAN1 to avoid duplicated evaluation work across different WGs with clarification of responsibility of each WG

    - RAN4 focused on evaluation on Transmitter side for achievable Tx EVM and MPR reduction

    - RAN1 evaluation focused on the throughput gain/SNR gain in receiver side by taking RAN4 input into account including achievable Tx EVM, Rx EVM and MPR

    - The simulation assumption between RAN1 and RAN4 shall be well aligned e.g., BS/UE antenna modelling, fading channel profile, MIMO configuration, and operating frequency, CHBW.

- Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility.

- Proposal 3: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26

- CMCC R4-2600813

- Proposal 3: it’s suggested to further study the workload split between RAN1 and RAN4 is listed as below:

    - For the uniform higher order modulation without shaping, RAN4 can evaluate the MPR/A-MPR and EVM as RAN4 has done previously

    - For the constellation shaping case, RAN4 can be involved to provide more RF impairment analysis.

Feature Lead note:

- In previous RAN4 high modulation evaluations, RAN4 has done the EVM budget analysis, applicable SINR in LLS, and SINR distribution in SLS.

- Now in 6G, the parallel discussion happens in both R4 and R1. Some co-ordination would be useful especially in the LLS and SLS configurations. However, it is observed that the EVM requirement is key factor in supporting high modulations. The feasibility finally needs to be confirmed in RAN4.

Recommended WF:

- RAN4 works on Tx/Rx EVM budget evaluation and LLS/SLS as usual to complete the feasibility study and define applicable EVM requirement.

- RAN4 strives to align the LLS and SLS configurations with R1 if necessary.

Sub-topic 1-2: NR modulations

Issue 1-2-1 Supported modulations

- Nokia R4-2600386

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for data channel

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for CP-OFDM for data channel

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for DFT-s-OFDM for data channel

- FFS: Enhancements and other modulation schemes

- Xiaomi R4-2600458:

- Proposal 5: Taking NR uniform modulation orders as basis for 6GR

    - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel

    - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel

    - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel.

- Apple R4-2600575:

- Proposal 1: It is proposed to consider the following modulation schemes as a starting point for 6G:

    - UL: up to 256 QAM

    - DL: up to 1024QAM

Feature Lead note:

- RAN1 agreement is as below. It seems there is no much difference in “supported as basis” or “is supported” among companies. All the 5G modulations need to be supported.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Recommended WF:

- From RAN4 perspective, NR modulations up to 256QAM can be supported in UL and up to 1024QAM can be supported in DL.

Issue 1-2-2 EVM for the NR modulations

- OPPO R4-2601419

- Proposal 1:  The EVM values in 6G for the modulation (BPSK to 256QAM) can keep the same with the EVM values in NR.

Recommended WF:

- Discuss in UE RF thread.

Sub-topic 1-3: UL 1024QAM

Issue 1-3-1 Support UL 1024QAM

- R4-2600386 Nokia

- Proposal 2: Support uniform 1024QAM in Uplink.

- Spreadtrum R4-2601053

- Proposal 3: For above 4.2GHz especially around 7GHz, support UL 256QAM as the baseline. For below 4.2GHz, UL 1024QAM can be further discussed.

- Samsung R4-2601123

- Proposal 3: For uplink, in order to increase efficiency spectral in indoor and high‑SNR regions, it is proposed to support 1024K‑QAM and initiating study to define EVM requirements in RAN4.

Recommended WF:

- Postpone until the evaluations results are clear.

Issue 1-3-2 Tx/Rx EVM budget

- Apple R4-2600575:

- Proposal 2: The modulation schemes are studied with 200MHz CBW in ~7GHz to identify any issues or challenges, including the potential use of a new PA model.

- CATT R4-2600313

- Proposal 2: RAN4 to conduct evaluations of modulation schemes using channel bandwidths exceeding 100 MHz under new 6G PA model(s), in order to fully leverage the 6GR studies at a later normative stage.

- Spreadtrum R4-2601053

- Proposal 3: For above 4.2GHz especially around 7GHz, support UL 256QAM as the baseline. For below 4.2GHz, UL 1024QAM can be further discussed.

- Xiaomi R4-2600458:

- Proposal 8: For RAN4 evaluation for high modulation orders on achievable Tx EVM and Rx EVM, following table can be considered

- LGE R4-2600698

- Proposal 1: Consider EVM 1.8% for 1024QAM.

- Proposal 2: Study whether to consider DPD for 1024QAM.

- Skyworks R4-2600737

- Baseline 6G performance is UL256QAM at least up to 5GHz requiring IQ impairments at 34dB

- It is reasonable to assume >36dB in band SNR for an ET PA with DPD assuming:

    - Enough power backoff so that the intrinsic PA distortions are already good

    - A modulation BW no higher than 200MHz

    - RF frequency up to 10GHz, FFS up to 15GHz

    - Proper DPD training over a large set of waveforms and power range and a DPD BW of at least 3x the modulation BW.

- For equal UE/BS EVM split a <3% EVM link budget can be targeted. To improve this number, the BS EVM should be significantly better than the UE EVM

- Additional margin is needed in the system to account for transients and RF chain noise floor to enable UL1025 QAM over a n acceptable output power dynamic range.

- CMCC R4-2600813

- Proposal 2: RAN4 is suggested to use below table as the baseline for further EVM budged analysis.

Table 1 EVM budget for 256QAM and 64QAM

| Non-linearity source | 256QAM |  | 64QAM |  |
| --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N |
| PA | 1.50% | 36.5 | 4.00% | 28.0 |
| Transmitter nonlinearity | 1.34% | 37.4 | 3.37% | 29.5 |
| LO Phase noise | 1.78% | 35.0 | 2.24% | 33.0 |
| IQ Image | 2.24% | 33.0 | 5.62% | 25.0 |
| Transmitter total | 3.50% | 29.1 | 8.00% | 21.9 |

- Huawei R4-2600886

- Proposal 2: For the identified evaluation cases, defer the evaluation until concrete conclusions are reached regarding RF impairments and the PA modeling.

- Proposal 3: For UL 1024QAM, the EVM budget and implementation feasibility should be assessed and concluded in advance for all relevant Tx impairment factors, including at least PA non-linearity, I/Q imbalance, LO phase noise, and CFR-induced noise.

- MediaTek R4-2600909

- Proposal 2: For 1024QAM, RAN4 also study the feasibility for the expected requirements, i.e., Carrier Leakage and image rejection ratio (IRR) -40 dBc

- Samsung R4-2601123

- Proposal 1:

    - For initial link-level anchoring under AWGN, RAN4 uses 2.88% (1024-QAM) as reference target values for feasibility comparisons across companies and simulations.

    - RAN4 then evaluates how much additional EVM margin is required when RF impairments are included, and records the main drivers (e.g., phase noise, I/Q imbalance, PA nonlinearity, clipping/CFR if applied), without forcing a requirement change until there is consistent evidence.

- ZTE R4-2601177

- Proposal 1: For the UL 1024QAM study, it is proposed to use the same values as 5G NR gNB DL EVM values as starting point to evaluation performance.

Table 1. An example for 1024QAM EVM budget with -40dBc I/Q imbalance

| Non-linearity source | ≤4.2GHz |  |  | >4.2GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | % | C/N (dB) | EVMi2 | % | C/N (dB) | EVMi2 |
| PA | 1.50 | 36.5 | 2.3 | 1.80 | 34.9 | 3.2 |
| Transmitter nonlinearity | 1.10 | 39.2 | 1.2 | 1.20 | 38.4 | 1.4 |
| LO Phase noise | 1.40 | 37.1 | 2.0 | 1.40 | 37.1 | 2.0 |
| IQ imbalance | 1.00 | 40.0 | 1.0 | 1.0 | 40.0 | 1.0 |
| Tx EVM | 2.5 | 31.9 |  | 2.8 | 31.2 |  |

- Sony R4-2601397

- Proposal 1: RAN4 can assume the same EVM requirement on the UE side as the BS side, as a starting point to study the feasibility of UL 1024 QAM.

- Proposal 3: When determining the feasibility of high-order modulation in the uplink direction, it is also reasonable to consider DPoD on the BS.

- OPPO R4-2601419

- Proposal 2: RAN4 can evaluate EVM budget for UL 1024QAM from two perspectives based on the implementation:

    - Consider stringent EVM contributions from LO phase noise and IQ imbalance (i.e., 1% separately) to set 2.5% EVM for UL 1024QAM.

    - Consider a more relaxed EVM (i.e., 3%) for UL 1024QAM compared to that for DL 1024QAM.

| Non-linearity source | 256QAM |  | 1024QAM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N | % | C/N |
| PA | 2.0% | 34.0 | 1.8% | 34.9 | 1.8% | 34.9 |
| Transmitter nonlinearity | 1.03% | 39.7 | 1.03% | 39.7 | 1.03% | 39.7 |
| Carrier leakage | 1.78% | 35.0 | 1.0% | 40 | 1.5% | 36.5 |
| IQ Imbalance | 2.0% | 34.0 | 1.0% | 40 | 1.5% | 36.5 |
| Transmitter total | 3.5% | 29.1 | 2.5% | 32.0 | 3% | 30.6 |

- NTT DoCoMo R4-2601466

- Proposal 2: In addition to the average EVM, RAN4 considers the EVM distribution across frequency or edge RB EVM during the modulation evaluation for 200MHz bandwidth to ensure no localized degradation exceeds the system limits.

- Qualcomm R4-2602059

- Observation 1: UE 1024QAM EVM specification of ~2%, represents a good compromise across UE implementations

- Observation 2: Image (IQ Imbalance) and LO Leakage levels should be better than -40 dBc for 1024QAM

- Observation 3: PA designs currently used in commercial UEs can meet the 1024QAM EVM requirements

Feature Lead note:

      - In NR DL 1024QAM, the DL BS EVM is 2.5% (32dB) for below 4.2GHz, and 2.8% (31dB) for above 4.2GHz.

        - For below/above 4.2GHz, the EVM requirements were different. In the UL 1024QAM evaluation, targeting bands may need to be clarified.

      - Some companies in this meeting have shared some EVM budgets for UL 1024QAM, while other companies are still need more time.

      - There is proposal to use the NR DL 1024QAM EVM budget as starting point for 6G UL 1024QAM.

      - CBW is also one of the concerned points for some companies, and it is proposed to use at 200MHz in the evaluation to figure out the potential impacts of UL 1024QAM.

      - The RF impairments and PA models are still under discussion in other threads, there is proposal to wait for the outcome of these discussions.

        - However, in FL view, the EVM budget analysis doesn’t rely on certain PA, companies can use their own PA model as long as it is reasonable.

        - The RF impairments in the EVM budget is one of the assumptions to evaluate the feasibility of high modulations, proper value can be chosen from each company, and it doesn’t impact the UE RF discussion. If the RF impairment becomes the gating factor and not supported by UE RF requirements, then this means the high modulation is not feasible.

Recommended WF:

      - NR BS Tx EVM for 1024QAM is considered as starting point.

      - Targeting bands:

        - Focus on around 7GHz for feasibility study at this moment.

        - FFS whether some delta EVM values can be applied for low frequency bands (e.g., <4.2GHz) like BS 1kQAM EVM was defined to save some efforts

      - The CBW used in the evaluation is 200MHz.

      - Regarding RF impairments in EVM budget evaluation:

        - It is only for high modulation feasibility study purpose. It doesn’t preclude the discussion in UE RF thread for requirement definition.

      - Regarding PA models in the EVM evaluation:

        - Proper PA models from each company can be used.

      - FFS on below aspects

        - whether to consider DPD and/or DPoD in UL 1kQAM feasibility evaluation

        - unequal EVM split between BS Rx EVM and UE Tx EVM in supporting UL 1kQAM.

        - whether transients and RF chain noise floor impacts need to be considered in UL 1kQAM feasibility evaluation, and its related additional EVM margins

        - whether/how edge RB EVM is considered in addition to the average EVM for 200MHz bandwidth.

      - Below table is collecting of EVM budgets from each company, and can be used as starting point for further evaluation:

        - For around 7GHz:

| EVM contributor @ 7GHz |  | EVM (%) | SNR (dB) |
| --- | --- | --- | --- |
| UE Tx EVM | PA | 1.8 OPPO/ZTE 1.5 Skyworks | 34.9 OPPO/ZTE 36.5 Skyworks |
|  | Transmitter | 1.03 OPPO 1.2 ZTE 0.5 Skyworks | 39.7 OPPO 38.4 ZTE 46 Skyworks |
|  | LO Phase noise | 1.4 ZTE 1.0 Skyworks | 37.1 ZTE 40 Skyworks |
|  | IQ imbalance | 1.0 OPPO/QC/ZTE/Skyworks 1.5 OPPO | 40 OPPO/QC/ZTE/Skyworks 36.5 OPPO |
|  | Carrier leakage | 1.0 OPPO/QC/MTK (FFS feasibility) | 40 OPPO/QC/MTK (FFS feasibility) |
|  | Total Tx EVM | 1.8 LGE ~2 QC 2.12 Skyworks 2.5 OPPO 2.8 SONY/ZTE 2.88 + ∆ Samsung 3 OPPO Summary: [1.8~3] % Average: 2.51 % | 34.9 LGE 34 QC 33.5 Skyworks 32 OPPO 31.1 SONY/ZTE 30.8 Samsung 30.6 OPPO Summary: [30.6~34.9] Average: 32 dB |
| BS Rx EVM budget |  | 2 Skyworks | 34 Skyworks |
| Total Tx+ Rx EVM |  | 2.92 Skyworks | 30.7 Skyworks |

        - For below 4.2GHz

| EVM contributor < 4.2GHz |  | EVM (%) | SNR (dB) |
| --- | --- | --- | --- |
| Tx EVM | PA | 1.5 ZTE | 36.5 ZTE |
|  | Transmitter | 1.1 ZTE | 39.2 ZTE |
|  | LO Phase noise | 1.4 ZTE | 37.1 ZTE |
|  | IQ imbalance | 1.0 ZTE | 40 ZTE |
|  | Carrier leakage |  |  |
|  | Total Tx EVM | 2.5 SONY/ZTE | 32 SONY/ZTE |
| Rx EVM budget |  |  |  |
| Total Tx+ Rx EVM |  |  |  |

Issue 1-3-3 LLS assumptions

- Huawei R4-2600886

- Proposal 6: For uplink 1024QAM, the evaluation should focus on the implementation feasibility and performance gain, evaluated through both link-level and system-level simulations.

- NTT DoCoMo R4-2601466

- Proposal 1: For modulation evaluation with 200MHz channel bandwidth, RAN4 verifies the model's validity by comparing the performance trends (e.g., ACLR and EVM degradation) between 100MHz and 200MHz

- CMCC R4-2600813

- For link level, the metric should be the marginal performance loss at a relative throughput range (e.g. 70% -90%) compared to the performance with 0% EVM.

- MediaTek R4-2600909

- Proposal 1: RAN4 should set up a simulation campaign to check the feasibility of UL 1024QAM.

- OPPO R4-2601419

- Proposal 4: The phase noise model doesn’t need to introduce in 6G FR1 link-level simulation.

- Proposal 7: RAN4 could first evaluate higher-order modulation using RAN1 agreed BS modelling for 700MHz, 2GHz, and 7GHz.

- Proposal 5: The link-level simulation can focus on MCS 23 and MCS 24 for UL 1024QAM with Tx EVM between 2.5% and 3%.

- Proposal 6: The link-level simulation can use the link-level simulation assumption in Table 4 as the starting point.

Table 4 Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700 MHz, 2 GHz,7GHz |
| CBW/SCS | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN) |
| MCS | Based on 5G MCS for 1024QAM PDSCH in Table 5.1.3.1-4 of TS 38.214: CP-OFDM: MCS 23, 24 other MCSs are not precluded |
| Symbol type | CP-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM + rxEVM | txEVM = rxEVM: 2.5%+2.5%, 3%+3% txEVM > rxEVM: 3%+2%, 3%+2.5% |

Feature Lead note:

- The purpose of the LLS is to find out from which SINR the UL 1024QAM is better than UL 256QAM under specific Tx and Rx EVM conditions.

- RAN1 MCS table:

Recommended WF:

- The metric for LLS is the SINR vs Throughput, and purpose is to find out the SINR boundary at which the 1024QAM shows performance gain than UL 256QAM under the Tx and Rx EVM assumptions.

- Below table is considered as starting point for UL 1KQAM LLS SINR evaluation.

- Companies are encouraged to check the configurations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 7GHz |
| CBW/SCS | 200MHz with 30kHz SCS |
| Waveform | CP-OFDM |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency AWGN |
| MCS | Based on NR MCS in Table 5.1.3.1-4 of TS 38.214: - UL 1KQAM: MCS 23, 24 - UL 256QAM: MCS 22 |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM + rxEVM | txEVM = rxEVM: 2.5%+2.5%, 3%+3% txEVM > rxEVM: 3%+2%, 3%+2.5% |

Issue 1-3-4 SLS assumptions

- CMCC R4-2600813

- Proposal 1: For system level, the metric should be 5% throughput loss across all MCS

- CableLabs, Charter, Rogers R4-2601955

- Proposal 2: We encourage other companies to perform similar system-level simulations to evaluate the achievability of UL 1024-QAM in 6GR FWA scenarios.

- UMa scenario for FWA. - Network topology: 19 sites topology, 3 sectors per site, see Figure 1. - Both co-channel network and adjacent-channel network use TDD assuming they are TDD synced, with a grid shift of 100%, see Figure 1. - Frequency: around 4 GHz and around 7 GHz. - Inter-site distance (ISD) is 500 m around 4 GHz [6] and around 7 GHz [5]. - Channel bandwidth: 100 MHz [6] and [7]. - Max BS EIRP: - Around 4 GHz: 72 dBm around 4 GHz [6] (this value is conservative, the actual EIRP density limit in band n77 in the USA is 62 dBm/MHz, which is 10 dB higher than in a 100 MHz channel, so that the actual DL 4096-QAM achievability could be higher). - Around 7 GHz: 78.3 dBm [7] and [8]. - BS antenna: - Around 4 GHz: 8×8 array, 6.4 dBi gain per element, and 24.5 dBi max gain for the array [6]. - Around 7 GHz: 8×16 sub-arrays, 3×1 elements per sub-array 6.4 dBi gain per element, and 32.2 dBi max gain for the array [5]. Note that the RAN agreed BS antenna array size is 2304 [5], which provides much larger BS antenna gain. - BS antenna height: 25 m above ground. - BS noise figure: 5 dB at 4 GHz and 6 dB at 7 GHz [7]. - Max CPE EIRP: 31 dBm, UL power control disabled. - CPE antenna: 2×4 array, 6.4 dBi gain per element, and 15.4 dBi max gain for the array [7] and [5]. - CPE location: indoor (applying outdoor-to-indoor, O2I, loss based on TR 38.901) vs. outdoor (0 dB O2I loss). - CPE height: 4 m for outdoor CPE, 1.5 m for indoor CPE. - CPE distribution: uniformly distributed in a cell [6]. - CPE noise figure: 7 dB at 4 GHz and 9 dB at 7 GHz [9]. - Network load: 50% in DL and 50% in UL. - 30 dB SINR threshold is selected for 1024-QAM in UL, and 38 dB SINR threshold is chosen for 4096-QAM in DL.

- OPPO R4-2601419

- Proposal 3: For higher-order modulation, RAN4 can focus on the scenarios of Urban Macro and Indoor using the parameters in Table 2 as starting point:

- Table 2 The frequency parameters and scenarios for modulation evaluation

|  | Urban Macro | Indoor Hotspot |
| --- | --- | --- |
| Carrier frequency | Around 700 MHz Around 2 GHz Around 7 GHz | Around 2 GHz Around 7 GHz |
| Simulation BW | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |

- Proposal 8: System-level simulation can start using the system-level simulation assumption in Table 5

Table 5 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 500m | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873 | 3D-InH LOS and NLOS in Table 7.2-1 of 36.873 |
| Carrier frequency |  | 700MHz, 2GHz, 7GHz |  |
| BS antenna configuration |  | - 700MHz: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1; 1, 2) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi - 2GHz (M,N,P,Mg,Ng; Mp, Np)= (12, 8, 2, 1, 1; 4, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi - 7GHz (M,N,P,Mg,Ng; Mp, Np)= (64, 16, 2, 1, 1; 16, 16) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi | - 2GHz (M,N,P,Mg,Ng; Mp, Np)= (4, 4, 2, 1, 1; 1, 4) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi - 7GHz (M,N,P,Mg,Ng; Mp, Np)= (16, 16, 2, 1, 1; 8, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | FFS |  |
| System bandwidth |  | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |
| Target SNR at BS side |  | Get from link-level simulation |  |
| UE max output power |  | Around 700 MHz: PC3 23dBm Around 2 GHz: FDD 23dBm, TDD 26dBm Around 7 GHz: PC2: 26 dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

Feature Lead note:

- The purpose of the SLS is to find out the probability of targeting SINR that can be achieved in the NW.

- The SLS assumptions are derived from TR38.921 and taking R1 6G evaluation assumptions into account.

Recommended WF:

- The metric for SLS is the SINR CDF in NW, and purpose is to find out the probability of targeting SINR that can be achieved in the NW.

- Below table is considered as starting point for UL 1KQAM SLS SINR evaluation.

        - FFS on UE Type and its related antenna configurations

        - FFS on the BS antenna configurations

        - Companies are encouraged to check the configurations.

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Frequency band |  | 7GHz |  |
| System bandwidth |  | 200MHz with 30kHz SCS |  |
| UE max output power |  | PC2: 26 dBm |  |
| UE antenna configuration |  | FFS |  |
| Network layout |  | 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 500m | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Channel model |  | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Handover margin (dB) |  | 3 |  |
| Pathloss |  | 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873 | 3D-InH LOS and NLOS in Table 7.2-1 of 36.873 |
| BS antenna configuration (Align with R1 assumptions) |  | (M,N,P,Mg,Ng; Mp, Np)= (24, 16, 2, 1, 1; 4, 16) (dH, dv) = (0.5λ, 0.8λ) GE,max = 8 dBi | (M,N,P,Mg,Ng; Mp, Np)= (4, 8, 2, 1, 1; 2, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| Mechanic tilt |  | 10° | Boresight direction is perpendicular to the ceiling |
| BS noise figure |  | Around 7GHz: 6dB | Around 7GHz: 14dB |
| Target SNR at BS side |  | Get from link-level simulation |  |
| UE noise figure |  | 9dB |  |
| Power control |  | TPC model specified in clause 9.1 TR 36.942 is applied with following parameters. - CLx-ile = 88 + 10*log10(200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure - γ = 1 |  |

Issue 1-3-5 Impacts to RF requirements

- LGE R4-2600698

- Proposal 3: Study how to update a general in-band emission for 1024QAM.

- Proposal 4: Study how to update IQ image for 1024QAM.

- Proposal 5: Study how to update carrier leakage for 1024QAM.

- Spreadtrum R4-2601053

- Proposal 1: For uplink modulation evaluation, the affected RF requirements should include EVM and MPR/AMPR.

- Proposal 2: Define the EVM budget and PA model/PA calibration point to evaluate assumptions to simulate MPR/AMPR.

- ZTE R4-2601177

- Proposal 2: If LP-WUS signal is supported in 6G day1, it is proposed to further discuss the impacts on potential EVM degradation of 6GR other modulation orders due to the simultaneous LP-WUS signal transmission.

Recommended WF:

- The RF requirement impacts of UL 1024QAM can be discussed in UE RF thread.

Sub-topic 1-4: DL 4096QAM

Issue 1-4-1 Supporting DL 4096QAM

- Skyworks R4-2600737

- DL1024QAM is considered as baseline for 6G:

    - Assuming a link target of <2.5% and a contribution of the BS of 1.5%, a UE contribution of <2% EVM is achievable with the same LO phase noise and IQ impairment than in the UL.

- DL4096QAM is studied as an optional modulation for 6G:

    - Most contributors need to improve significantly (at least 2x versus 1024QAM) and, assuming that the most critical is the UE LO phase noise especially at higher RF frequencies, Sub 1% EVM will be needed from the BS Tx and most contributors. Based on this an EVM link budget <1.5% is achievable.

- Spreadtrum R4-2601053

- Proposal 4: Don’t consider DL 4096QAM and Keep DL 1024QAM in 6G day1.

- Samsung R4-2601123

- Proposal 2: For downlink, in order to increase spectral efficiency in indoor and high‑SNR regions, it is proposed to support 4096-QAM and initiating study to define EVM requirements in RAN4.

Recommended WF:

- Postpone until the evaluations results are clear.

Issue 1-4-2 EVM budget table

- Samsung R4-2601123

- For initial link-level anchoring under AWGN, RAN4 uses 1.52% (4096-QAM) as reference target values for feasibility comparisons across companies and simulations.

- RAN4 then evaluates how much additional EVM margin is required when RF impairments are included, and records the main drivers (e.g., phase noise, I/Q imbalance, PA nonlinearity, clipping/CFR if applied), without forcing a requirement change until there is consistent evidence.

Recommended WF:

- More input is needed based on below table.

| EVM contributor @ 7GHz |  | EVM (%) | SNR (dB) |
| --- | --- | --- | --- |
| BS Tx EVM | PA |  |  |
|  | Transmitter |  |  |
|  | LO Phase noise |  |  |
|  | IQ imbalance |  |  |
|  | Carrier leakage |  |  |
|  | Total Tx EVM |  |  |
| UE Rx EVM budget |  |  |  |
| Total Tx+ Rx EVM |  |  |  |

Issue 1-4-3 LLS assumptions

- MediaTek R4-2600909

- Proposal 3: RAN4 should set up a simulation campaign to check the feasibility of DL 4096QAM together with the target BS Tx EVM.

Recommended WF:

- More input is needed.

Issue 1-4-4 SLS assumptions

- CableLabs, Charter, Rogers R4-2601955

- Proposal 2: We encourage other companies to perform similar system-level simulations to evaluate the achievability of 4096-QAM in 6GR FWA scenarios.

- Apple R4-2600575:

- Proposal 4: It is proposed to understand the potential gain of supporting 4096QAM in real field.

Recommended WF:

- More input is needed.

Issue 1-4-5 Impacts to RF requirements

- Spreadtrum R4-2601053

- Proposal 1: For downlink modulation evaluation, from BS perspective, which should include EVM.

Recommended WF:

- BS RF impact can be discussed in BS RF thread.

Sub-topic 1-5: Constellation shaping

Issue 1-5-1 Impacts to RF requirements

- vivo R4-2600668

- Proposal 1: RAN4 needs to study the impact of GS and PS schemes on RF performance metrics, such as changes in the ideal constellation and possible adjustments to calculation rules for EVM calculation.

- MediaTek R4-2600909

- Proposal 4: RAN4 may need to check whether a different EVM requirement is needed for probabilistic shaping (pending on RAN1 discussion progress).

- Huawei R4-2600886

- Proposal 5: For existing NR modulations that may adopt non-uniform constellations, the evaluation should focus on determining whether new EVM requirements should be considered.

Recommended WF:

- Hold on the discussion until sufficient progress are made in RAN1.

- Topic #2: Channel arrangement

Companies’ contributions summary

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2600389 | 6GR - Channel arrengements | Nokia |
| R4-2601450 | on 6GR channel arrangement | OPPO |
| R4-2600315 | Further discussion on channel arrangement for 6GR | CATT |
| R4-2600460 | View on 6GR channel arrangement | Xiaomi |
| R4-2600577 | On 6G system parameters - Channel arrangement | Apple |
| R4-2600661 | Discussion on 6G channel arrangement | vivo |
| R4-2600701 | (6G system parameters) Channel arrangement | LG Electronics |
| R4-2600814 | Discussion on 6GR channel arrangement | CMCC |
| R4-2600889 | On system parameters for 6G —— Channel arrangement | Huawei, HiSilicon |
| R4-2601004 | Views on 6G Channel Arrangements | MediaTek inc. |
| R4-2601030 | Channel Arrangement for 6GR | Qualcomm Incorporated |
| R4-2601056 | Views on 6G channel arrangement | Spreadtrum,UNISOC |
| R4-2601070 | Views on 6G channel arrangement | ZTE Corporation, Sanechips |
| R4-2601126 | Discussion on channel arrangement for 6GR | Samsung |
| R4-2601382 | System parameters -- channel arrangement: on the channel- and synchronization raster | Ericsson |
| R4-2601400 | Further views on channel arrangement of 6GR | Sony |
| R4-2601468 | Views on 6G channel arrangement | NTT DOCOMO, INC. |

Open issues summary

Before Meeting, Feature Leads shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Background: WF in previous RAN4 meetings.

Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

Recommendation from FL in RAN4#117 - For the sub-3GHz bands, adopt smaller channel raster instead of 100kHz channel raster for 6GR - Further compare 5kHz vs. 10kHz channel raster for different scenarios. - E.g., evaluate the implementation and coexistence complexity for operators if 6G uses a different channel raster (e.g., 5kHz) in a band where 5G uses 100kHz/10kHz. - Further study the proposals on simplification and future migration

Sub-topic 2-1: Channel raster

Issue 2-1-1 Channel raster design

- Nokia R4-2600389

- Proposal 1: The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them.

- OPPO R4-2601450

- Proposal 1: For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted.

- CATT R4-2600315

- Proposal 1: RAN4 to defer the adoption of a 10kHz channel raster until the spectrum utilization study for 6GR is finalized.

- Proposal 2: If a 5kHz channel raster is to be introduced and the global frequency grid is also 5kHz, then RAN4 does not need to define a channel raster explicitly in the specification, as there is not any constraint on carrier placement in such a case.

- Xiaomi R4-2600460

- Proposal 1: Support 5kHz channel raster in day 1 for below 3GHz

- Proposal 2: RAN4 further study whether 100kHz channel raster still required for below 3GHz bands

- Proposal 3: Remove per band channel raster concept with following value per sub-frequency range basis

| Frequency range | Channel raster |
| --- | --- |
| <3GHz | 5kHz |
| 3GHz ~ 24.25kHz | 30kHz |
| 24.25GHz ~ 52GHz | 120kHz |

- Apple R4-2600577

- Proposal 1: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync raster points.

- Vivo R4-2600661

- Proposal 1: RAN4 to define 10 kHz channel raster for sub-3GHz frequency range and SCS based channel raster for above 3GHz

- CMCC R4-2600814

- Proposal 2: RAN4 needs to consider the joint design of the channel raster and sync raster, rather than discussing them in isolation and taken following aspects into consideration:

    - If RAN4 want to design the sync raster to cover all possible RF CBW and locations on 5/10kHz channel raster, sync raster must be designed much denser.

- Huawei R4-2600889

- Proposal 1: Enhance channel raster with granularity of 5kHz/10kHz could be adopted from the outset of 6G, replacing the 100kHz channel raster.

- Proposal 2: The channel raster entry structure for 6GR bands should remain similar to the current one for 5G. For any consideration of shifting specific bands from the 100 kHz to an SCS-based raster, the decision must be based on comprehensive inputs from operators.

- MediaTek R4-2601004

- Proposal 1: Define 6G channel raster based on the Greatest Common Factor (GCF) among the existing channel raster in a frequency range. Single channel raster per frequency range:

| Frequency range (GHz) | 5G Channel raster | 6G channel raster (based on GCF) |
| --- | --- | --- |
| 0-3 | 100kHz, 10kHz, 15kHz, 30kHz | 5kHz |
| 3 - 24.25 | 15kHz, 30kHz | 15kHz |
| 24.25 - 100 | 60kHz, 120kHz | 60kHz* |
| *For FR2 range we can also consider 120kHz channel raster instead of 60kHz to align with RAN1 latest agreement. |  |  |  |

- Proposal 2: Unify both global raster (ARFCN) and channel raster for simpler channel arrangement in 6G.

- Qualcomm R4-2601030

- Proposal 1: Adopt the SCS based raster as the baseline in all bands where it is currently specified and in all future 6G bands.

- Proposal 2: RAN4 should study optimization of the channel raster in bands which use the 100 kHz raster and/or possible addition of new raster points to enable future migration to SCS based raster when coexistence with NR is no longer necessary.

- Proposal 5. The impact of the channel raster on sync raster complexity should be considered in the channel raster study and in related design decisions.

- Spreadtrum R4-2601056

- Proposal 1: It is necessary to define channel raster, for re-farming bands with 10 kHz channel raster, using 10 kHz channel raster in 6GR. For other re-farming bands and new bands, SCS-based channel raster can be adopted in 6GR

- ZTE R4-2601070

- Proposal 1: For bands above 3GHz, SCS based channel raster should be applied.

- Proposal 2: For sub-3GHz bands, propose to define 10kHz channel raster in 6G day 1 instead of 100kHz.

- Samsung R4-2601126

- Proposal 1: It is proposed that the channel raster framework for 6GR Day-1 converges to:

    - Use SCS-based channel raster as the default for new 6G bands and for frequencies above around 3 GHz.

    - For refarmed FDD bands below around 3 GHz, where legacy 100 kHz-based planning already exists, either:

    - Continue to use a single 5 kHz channel raster as a common baseline across such bands; or

    - Migrate to an SCS-based raster while ensuring that the resulting centre frequencies remain compatible with existing deployments through appropriate band-specific migration rules.

    - Avoid defining multiple alternative rasters (e.g. 100 kHz, 10/5 kHz and SCS-based) per band for 6GR day-1, and instead treat any additional rasters as exceptional options that require clear coexistence justification.

- Ericsson R4-2601382

- Proposal 1: the channel- and synchronization raster for 6GR shall be specified such that MRSS with PRB alignment to the NR carrier can be configured for all possible 100 kHz NR channel raster entries

- Sony R4-2601400

- Proposal 2: It is proposed 6G considers an enhanced channel raster (finer than 100 kHz) from the beginning to ensure the spectrum usage of 6G can be more efficient than 5G.

- Proposal 3: Considering the MRSS and co-existence between 6GR and 5G NR, it is proposed 6GR to consider either 5kHz channel raster or the same channel raster as 5G NR for a given band.

- LG Electronics R4-2600701

- Proposal 1: Consider channel raster 5kHz for below 3GHz and SCS-based raster above 3GHz.

Feature Lead note:

- Regarding whether remove channel raster concept, most companies believe it is still needed in 6G in terms of conformance testing and also SCC ARFCN indication, etc.

- And it is desired to avoid defining multiple channel raster granularities, e.g., 15khz, 30khz, 100khz channel raster for one band.

- Some companies propose to consider the sync raster and MRSS impacts when define the channel raster.

- For the re-farming bands with 100khz channel raster in NR, all companies suggest to improve it by ether using smaller channel raster like 10khz/5khz or directly migrate to SCS based channel raster. However, there is also concern on the NBC with the NR carriers.

- For the new bands or SCS based channel raster, all companies agree to continue use SCS based channel raster.

- Some companies want to align the channel raster in a even larger scope like per frequency range or sub-frequency range based channel raster.

Recommended WF:

- Channel raster is defined in 6G specification.

- Only define one channel raster granularity (ΔFRaster) for each band.

- When defining channel raster, the sync raster impact is considered.

- MRSS with PRB alignment for the re-farming bands is considered.

- For re-farming bands with 100khz channel raster in NR, consider the below options

  - 10khz

    - FFS whether to defer the adoption of a 10kHz channel raster until the spectrum utilization study for 6GR is finalized.

  - 5khz

    - FFS whether RAN4 can avoid defining channel raster explicitly in the specification

  - SCS based

    - shifting specific bands from the 100 kHz to an SCS-based raster, the decision must be based on comprehensive inputs from operators.

- For bands with SCS based channel raster in NR and for new bands, SCS based channel raster is adopted.

- FFS whether per frequency range-based channel raster can be considered, e.g., the GCF (Greatest Common Factor) among the existing channel raster in NR.

Issue 2-1-2 Relation b/w UE dedicated CBW and Channel raster

- CMCC R4-2600814

- Proposal 1: RAN4 is suggested to study whether the carrier bandwidth in UE dedicated CBW must be aligned with channel raster, i.e., whether to reflect them into RAN2 spec.

Feature Lead note:

- This topic was discussed in NR for a long time with no conclusion.

Recommended WF:

- FFS whether the carrier bandwidth in UE dedicated CBW must be aligned with channel raster, i.e., whether to reflect them into RAN2 spec.

Sub-topic 2-2: Sync raster

Issue 2-2-1 Sync raster design

- Nokia R4-2600389

Observations:

- 5G NR principle for sync raster definition can be inherited for 6GR sync raster design.

- Limiting the sync raster locations in frequency domain can complicate the cell deployment.

- It should be up to the operators to suggest if some of the sync raster placements for a particular band can be removed.

- Aligning 6GR synchronization raster locations with NR, when feasible, can help to alleviate the UE initial search complexity when supporting both 5G and 6GR.

- OPPO R4-2601450

- Proposal 2: The channel raster and sync raster with SCS level alignment helps interference mitigation and needs to be kept.

- Proposal 3: The principle“For min CBW at each channel raster, there is at least one SSB can be used” can be improved as for multiple of the channel raster, there exists one SSB can be used.

- Proposal 4: It is proposed to use m*channel raster as reference channel raster to further develop 6G coarse sync raster. The value of m can be further investigated.

- CATT R4-2600315

- Proposal 3: RAN4 to consider a two-level sync raster in 6GR consisting of a coarser primary sync raster and a finer secondary sync raster to balance deployment flexibility and UE energy consumption.

- Xiaomi R4-2600460

- Proposal 4: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026

- Proposal 5: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area

    - Flexible step size per sub-frequency range/per band

    - Scalable step-size pending on SSB periodicity

    - SS raster design for MRSS

- Apple R4-2600577

- Proposal 2: It is proposed to study CBW-dependent sync raster design as an option to enable fast cell search and UE power saving.

- Vivo R4-2600661

- Proposal 2: RAN4 to adopt NR basic principle of sync raster design to define the basic sync raster as:

Min BWCHANNEL-BWSSB+ ΔFCH,Raster

where BWCHANNEL is the minimum CBW and BWSSB is the BW of 6G SSB which depend on the further progress of minimum CBW and SSB discussion.

- Proposal 3: Depending on whether SSB periodicity is extended in 6G, methods to mitigate impact on cell search time can be further investigated without impacting the basic sync raster definition

- Proposal 4: Sync raster to support 3MHz shall be separately discussed and defined, consistent with the approach in NR for specific bands.

- CMCC R4-2600814

- Proposal 3: RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operator’s wider spectrum holding with the benefits of shorter search time and lower power consumption.

- Proposal 4: A proposed solution is to implement a multi-tiered synchronization raster. For instance, one tier would correspond to a unified minimum channel bandwidth, while another would support larger bandwidths.

- Huawei R4-2600889

- Proposal 3: Retain the foundational design principles of the NR sync raster for 6GR as the guiding principles. On this basis, methods to optimize the SSB search time should be further investigated, under the condition that backward compatibility with the NR constraints is maintained.

- Proposal 4: Sparse sync raster should be studied for 6GR.

- MediaTek R4-2601004

- Proposal 3: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access.

- Proposal 4: RAN4 to assess different sync raster proposals considering the delay associated with sync raster and SSB periodicity. 6G sync raster design needs to ensure initial access delay is not worse than 5G.

- Proposal 5: Define 6G sync raster using

  - Coarse sync raster with higher priority: based on larger assumption of CBW

  - Normal sync raster with lower priority: based on smaller assumption of CBW

- Proposal 6: With coarse sync raster, operators to place their SSBs following the coarse sync raster entries if their spectrum allocation allows for that (e.g., allocation larger than min CBW). However, if such entries do not exist for some operators due to smaller CBW allocation, their SSBs can instead be placed following the normal sync raster entries.

- Proposal 7: Consider PSS BW instead of SSB BW in 6G sync raster design (i.e., Sync raster = TBW – PSS BW + channel raster) to achieve lower density sync raster.

- Qualcomm R4-2601030

- Proposal 3. RAN4 should study sync raster optimizations to minimize the number of sync raster entries while maintaining forward compatibility for flexible channel placement.

- Proposal 4. Maintain the same design principles for the 6GR sync raster design as for NR. The raster granularity depends on the channel raster, the SSB bandwidth and the minimum channel BW supported in a band

- Spreadtrum R4-2601056

- Proposal 2: Keep the position of sync raster is the subset of the position of channel raster as a starting point in 6GR.

- Proposal 3: Decouple the channel raster and sync raster to be more flexible deployment. Meanwhile, one cell is deployed in one channel raster, SSB can be deployed in one sync raster in this cell.

- Proposal 4: More sparse sync raster design can be considered in 6GR. The principle in NR for sync raster can be followed in 6GR.

- ZTE R4-2601070

- Proposal 3: To ensure deployment flexibility, it is proposed to retain the foundational design principle of 5G NR sync raster.

- Proposal 4: It is proposed to split frequency range into more sub-ranges and select different min channel bandwidth for each frequency range.

- Samsung R4-2601126

- Proposal 2: RAN4 continues the discussion on 6GR synchronization raster step design with the following study directions:

    - consider sparser and implementation-friendly raster step sizes by taking UE receiver sampling-rate and FFT-related aspects into account, for newly introduced higher bands such as around 7 GHz, in addition to minimum channel bandwidth and SSB size;

    - investigate the feasibility of a unified coarse raster step applicable across wide frequency ranges, potentially complemented by an on-demand finer search step to improve robustness; and

    - study potential simplification of GSCN definition and usage, including the feasibility of relative indexing per band or per range.

- SONY R4-2601400

- Proposal 1: RAN4 studies the approach to reducing the number of sync raster positions in 6GR.

- NTT DoCoMo R4-2601468

- Proposal 1: RAN4 to study the coupling mechanism between synchronization raster and channel raster, where the valid channel raster grid is determined based on synchronization raster density and configuration (e.g., frequency band, density, and position) through dynamic scaling or pre-defined mapping tables.

- Proposal 2: RAN4 to study dynamic raster control via RRC/SIB signalling to adapt to various network scenarios, including extended SSB periodicities for energy saving and SSB-less operations.

- Proposal 3: RAN4 to investigate raster derivation rules for SSB-less scenarios, specifically identifying the target channel frequency raster based on an anchor synchronization signal from another frequency range (e.g., FR1) and associated assistance information.

- LG Electronics R4-2600701

- Proposal 2: Consider design principle of 6G sync raster as follows.

  - Sync raster granularity = (NRB of minimum CBW – PSS_RB(or SSS_RB) + 1 RB)*SCS*12

- Proposal 3: Consider potential PSS/SSS bandwidth and potential 6G NRB for 6G sync raster.

- Potential 6G PSS/SSS bandwidth

  - 12/15                      for minimum CBW = 3MHz

  - 12/15/16/18/20 RB for minimum CBW > 3MHz

Potential 6G NRB

|  | Potential 6G NRB for minimum CBW [RB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz | 5MHz | 10MHz | 50MHz |
| SCS 15kHz | 15 | 25, 26 |  |  |
| SCS 30kHZ |  |  | 24, 25, 26 |  |
| SCS 120kHz |  |  |  | 32, 33 |

- Proposal 4: Decouple 6G sync raster design from channel raster.

- Proposal 5: Consider 2 step based 6G sync raster - 1st priority GSCN Group (Sparce) and 2nd priority GSCN Group (Dense)Proposal 1: Consider channel raster 5kHz for below 3GHz and SCS-based raster above 3GHz.

Feature Lead note:

- In NR, two principles have been applied in defining the sync raster:

    - SCS level alignment between sync raster and channel raster

    - For min CBW at each channel raster, there is at least one SSB can be used

- And below equation was used to calculate the basic sync raster (without considering the sync raster shifts):

  - Basic Sync raster <= MinCBW - BWSSB + CHraster

- Regarding whether to postpone the sync raster discussion until sufficient progress reached in RAN1 IA design, it is FL understanding that sync raster values maybe impacted by R1 SSB design, but the sync raster scheme discussion in R4 can be started.

- Majority companies would like to specify sparse sync raster to speed up IA, however, if no solution is agreed in the end, the NR sync raster can still be considered as baseline.

Recommended WF:

- Sync raster and channel raster are SCS level aligned.

- NR sync raster is considered as starting point, and RAN4 strives to define sparser sync raster if agreeable

- Following approaches can be considered as starting point to sparse the sync raster with scheme details FFS:

  - Option 1: Two-level sync raster scheme, i.e., a coarser primary sync raster and a finer secondary sync raster

  - Option 2: Increase the channel raster step size in sync raster design, i.e., the reference channel raster used in sync raster calculation is N*channel raster.

  - Option 3: Scalable sync raster step-size pending on SSB periodicity

  - Option 4: Consider PSS BW instead of SSB BW in 6G sync raster design (i.e., Sync raster = TBW – PSS BW + channel raster)

  - Option 5: CBW-dependent sync raster design

  - Option 6: valid channel raster grid is determined based on synchronization raster density and configuration

  - Other options are not precluded.

- FFS whether Sync raster to support 3MHz shall be separately discussed and defined

  - It depends on the min CBW definition in 6G

Sub-topic 2-3: Channel spacing

Issue 2-3-1 Channel spacing for CA and non-CA

- CATT R4-2600315

- Proposal 4: RAN4 to re-examine channel-spacing definitions for both single- and multiple-carrier operations by considering wider channel bandwidth, new operating frequencies and shared spectrum scenarios, and defer the discussion until the other core parameters have become more stable.

- Xiaomi R4-2600460

- Proposal 6:  Simplify Channel spacing definition without consideration of mixed numerology case in 6GR.

- Proposal 7: Further study “normal channel spacing” definition and the relation-ship between intra-band contiguous CA and NC CA.

  - Observation 8: “Normal channel spacing” concept was introduced for RAN4 requirements and conformance test. There is no restriction on real network deployment.

- MediaTek R4-2601004

- Proposal 8: Simplify channel spacing definition in 6G without considering mixed numerology scenarios.

- ZTE R4-2601070

- Proposal 5: It’s better to simplify the definition of nominal channel spacing without considering mixed numerologies.

- Proposal 6: It is critical to avoid such a misalignment between spec and practical deployment that spec supports 300kHz nominal channel spacing but this configuration cannot be used in practical deployment scenarios.

- LG Electronics R4-2600701

- Proposal 6: Consider 6G nominal channel spacing as follows.

  - Below 3GHz (assuming channel raster (∆FRaster) = 5kHz)

    - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

  - For above 3GHz, Normal Channel spacing can be rewriten as follows.

    - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{- (∆FRaster /3) kHz, 0 kHz, (∆FRaster /3) kHz}

Recommended WF:

- Mixed numerology scenarios are not considered in channel spacing definition for CA and non-CA

- FFS on the nominal channel spacing for CA and non-CA with single numerology

- Topic #3: Device types

Companies’ contributions summary

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2600054 | On RAN4 aspects of SAW/BAW-less half-duplex design in 6G | Nordic Semiconductor ASA |
| R4-2600388 | 6GR - Device types | Nokia |
| R4-2600457 | View on 6GR Device type: minimum maximum CHBW | Xiaomi |
| R4-2600501 | Smallest maximum CBW for low-tier devices | MediaTek Inc. |
| R4-2600578 | On 6G system parameters - Device type | Apple |
| R4-2600631 | On the Smallest Maximum Supported Bandwidth for Low-tier Device Types | AT&T |
| R4-2600674 | Discussion on 6G device type | vivo |
| R4-2600700 | (6G system parameters) Device types | LG Electronics |
| R4-2600812 | Discussion on 6GR device type | CMCC |
| R4-2600888 | Discussion on UE device type | Huawei, HiSilicon |
| R4-2601055 | Views on 6G device type | Spreadtrum, UNISOC |
| R4-2601125 | Discussion on device type for 6GR | Samsung |
| R4-2601179 | Views on 6GR device type | ZTE, Sanechips |
| R4-2601399 | Views on maximum channel bandwidth for low-tier devices | Sony |
| R4-2601451 | on 6GR device type | OPPO |
| R4-2600316 | Further discussion on device types for 6GR | CATT |
| R4-2601880 | On smallest maximum channel bandwidth for “low-tier” devices | Ericsson |

Open issues summary

Feature Lead note: RAN#110 agreements in RP-253856.

Proposal 1: - 6GR supports the operation (but not required to be optimized for performance) in a minimum spectrum allocation of 3MHz with a 15kHz SCS Note: the following agreement made in RAN1#123 still holds, with the clarification that the bandwidth in Opt 1 below is assumed to be at least 5MHz with a 15kHz SCS. Agreement If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, - Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable - Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations - Regarding the smallest maximum UE bandwidth as discussed in the following RAN1 agreement, Opt 1 is excluded. Aim to conclude by RAN plenary no later than RAN#112 (June 2026). - RAN1 and RAN4 is tasked to continue providing more analysis accordingly. - Companies are encouraged to provide more analysis at RAN plenary particularly regarding the use cases, requirements, economy of scale, etc. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different Proposal 2: - Diverse device types as in 6G SIDs are expected to include supporting at least MBB (as highest priority), FWA, and Massive IoT services - Companies are encouraged to further contribute at RAN plenary regarding the analysis of necessary aspects to help progress RAN WG technical design and evaluations. - The detailed discussion for device types (how many, parameters, whether or not to define device types, whether or not to introduce minimum mandatory set of features, etc.) is on hold in RAN plenary and to resume from RAN#113 (September 2026). Discussions on device types are not to be discussed in WGs until further update from RAN plenary. Proposal 3: Capture the following aspects related to diverse device types into the TR: - Scalable and forward compatible design - Limited set of device types (based on the agreement in RAN#109) Detailed wording is to be drafted in RAN#111, along with possibly other aspects (e.g., scalability, differentiated services, etc.).

Sub-topic 3-1: Smallest max UE bandwidth

Issue 3-1-1 Smallest max UE bandwidth

- Sony R4-2601399

- Proposal 1: The analysis and choice on the maximum bandwidth for the low tier device shall take into account the RF and BB complexity as the most important aspects, including the SAW-less aspect.

- Proposal 2: 6GR shall aim to support low-tier IoT module implementation without band-specific analogue filters (e.g. without SAW filters)

- Proposal 3: the maximum bandwidth of low tier device is up to 5MHz at least in the uplink.

- Nordic R4-2600054

- Proposal-1: Capture in RAN4 6G TR: If 6G maximum smallest DL/UL RF BW is 10 or 20MHz, RAN4 defines band-specific frequency regions where UL scheduling restrictions within 10 or 20MHz BWP apply after MSG4 for HD-FDD 6G IoT UE with reduced number of band-specific RF filter.

  - Note, it is assumed that initial access UL BB BW remains naturally limited to [4-5] MHz in UL as in NR in FR1.

- Proposal-3: Study how a 5MHz RF UE could operate within configured 10 or 20MHz UL BWP.

- Nokia R4-2600388

- Proposal 6: CBW choice for device type should primarily focus on minimum/baseline requirement definition.

- Xiaomi R4-2600457

- Proposal4: On smallest maximum CHBW for lower device type in 6GR, prefer to take 20MHz BW including RF and BB as mandatory BW in FR1.

- MediaTek R4-2600501

- Proposal 1: Economies of scale and ease of network integration to be considered as part of any analysis of the benefits of smallest maximum BW for low-tier devices.

- Proposal 2: The UE should support all specified CBWs up to the smallest maximum CBW defined for a given SCS.

- Proposal 3: HD-FDD should be supported by the lowest tier 6GR device for FDD bands.

- Proposal 4: Considering 20MHz CBW as the smallest maximum supported CBW for UL and DL for 15kHz and 30kHz SCS

- Apple R4-2600578

- Proposal 1: It is proposed to adopt 20MHz as the smallest maximum UE CBW which applies to both RF and BB, i.e., both RF and BB BW are 20MHz.

- Proposal 2: Unless there are significant cost/complexity saving, it is proposed to have the same UL and DL bandwidth to support broad use cases.

- vivo R4-2600674

- Proposal 1: Take 20MHz as maximum channel bandwidth for IoT device supported in 6GR for both FDD and TDD operation at least for FR1

- HW R4-2600888

- Proposal 1: The smallest maximum channel bandwidth is selected as 20MHz.

- Spreadtrum R4-2601055

- Proposal 1: The value of smallest maximum UE BW can be 20MHz for 6G low-tier devices.

- Proposal 2: Keep the same bandwidth for UL and DL in 6GR for smallest maximum UE BW.

- Proposal 3: Keep the same bandwidth value for different SCS, duplex modes and bands smallest maximum UE BW.

- Proposal 4: Keep the same bandwidth for RF and BB smallest maximum UE BW.

- Samsung R4-2601125

- Proposal 1: For 6GR, the smallest maximum UE-supported RF BW is 20MHz and BB BW is 5MHz.

- ZTE R4-2601179

- Proposal: 5MHz with 15kHz SCS as the smallest maximum UE bandwidth should be considered as the baseline for 6GR system design, which is scalable along with different SCS.

- OPPO R4-2601451

- Proposal 1: 	Study the two options for maximum UE bandwidth for lowest-tier 6G IoT devices.

  - Option 1: 5MHz. If taking this option, the wearable devices (i.e., 6G RedCap) can be treated as a separate device type (e.g., with maximum UE bandwidth of at least 20MHz).

  - Option 2: 20MHz. If taking this option, the wearable device (i.e., 6G RedCap) is combined into the lowest-tier 6G IoT device type.

- Ericsson R4-2601880

- Proposal-3: Limiting BB bandwidth may bring network benefit more than limiting RF bandwidth, other aspect could be discussed other than RF impact for SAW-less design, e.g., LO retuning time.

Feature Lead note:

- Based on the inputs, it seems majority companies prefer to define smallest max CBW as 20MHz. While several companies prefer 5MHz from enabling SAW-less design perspective.

  - For companies who support 20MHz:

    - UE complexity reduction achieved by BW reduction further reduction of RF BW from 20MHz to 5MHz or BB BW from 20MHz to 5MHz is small, i.e., around 5% or smaller.

    - On the other hand, there are benefits to enable 20MHz as the smallest maximum supported RF and BB UE BW:

      - Compared to 5MHz or 10MHz, a larger CBW has the potential to allow the low-tier devices to support more application scenarios. With more application scenarios, economy of scale can be achieved and can further reduce device cost.

      - 20MHz CBW will provide more options to support the 10Mbps data rate target for both DL and UL.

      - Having the same RF and BB BW will reduce the need of scheduling overhead/constraints.

  - For companies who support 5MHz:

    - 5MHz maximum bandwidth can provide about 5% reduction in cost/complexity and 9% reduction in power consumption although 5MHz may have some impacts on SIB design

- Regarding the relation b/w smallest max CBW and SAW-less design

  - Some companies think these two things should not be bundled together even these companies are also interested in enabling the SAW-less

    - enabling a partial or fully SAW-less UE design does not necessarily require the smallest maximum bandwidth to be limited

  - Some solutions in enabling SAW-less design within 20MHz CBW have been proposed in several papers like restriction of RB allocations, power restriction, emission relaxation, etc. Details are discussed in issue 3-1-2.

- Regarding RF/BB CBW

  - Most companies believe these two CBWs should be same, while some company think to enable RF bandwidth as 20MHz and BB BW as 5MHz.

- Regarding Tx/Rx CBW

  - Many companies think these two CBWs should be same, while some company think to enable Tx bandwidth as 5MHz and BB BW as 20MHz.

Recommended WF:

- Smallest max CBW is same for UL and DL

- Regarding UE RF and BB smallest max CBW, down select from RAN4 perspective:

  - Option 1: 20MHz for both RF and BB

  - Option 2: 5MHz for both RF and BB

  - Option 3: 20MHz for RF and 5MHz for BB

- FFS how to enable SAW-less design within the smallest max CBW.

  - Whether requirements will be defined for SAW-less design UE belongs to normative work phase.

Issue 3-1-2: How to enable SAW-Less design

- Nordic R4-2600054

- Proposal-1: Capture in RAN4 6G TR: If 6G maximum smallest DL/UL RF BW is 10 or 20MHz, RAN4 defines band-specific frequency regions where UL scheduling restrictions within 10 or 20MHz BWP apply after MSG4 for HD-FDD 6G IoT UE with reduced number of band-specific RF filter.

  - Note, it is assumed that initial access UL BB BW remains naturally limited to [4-5] MHz in UL as in NR in FR1.

  - FFS UL scheduling restrictions within 10 or 20Mhz UL BWP, UL <=5MHz, including dependency on transmit power/PSD and/or location from band edge.

  - FFS relaxation of spurious emission requirement(s).

- Proposal-2: Relax duplexing gap 2x compared to 5G to enable single PLL implementation of HD-FDD.

- Proposal-3: Study how a 5MHz RF UE could operate within configured 10 or 20MHz UL BWP.

- Sony R4-2601399

- Proposal 4: RAN4 further study additional approaches to enable a fully SAW-less implementation for a few extremely challenged scenario (e.g., band to band separation below 12MHz) with 5MHz uplink BW for the low tier device in 6GR.

- Proposal 5: If maximum DL bandwidth for low tier devices is larger than 5MHz, it is beneficial to limit the peak data rate to 10 Mbps to reduce the baseband complexity.

- Proposal 6: RAN4 further study the following additional solutions to enable a fully SAW-less implementation for the low tier device of 6GR:

  - Limit location of UL RBs

  - limited number/length of allocated UL RBs,

  - allowed power backoff for certain bands and/or certain UL RB allocations restrictions

  - possible relaxed band to band spurious emission level

- AT&T R4-2600631

- Proposal: RAN4 is requested to study the proposed approaches that enable SAW-less RF implementations for massive IoT devices in the 6G era

- MediaTek R4-2600501

- Proposal 4: study further the impact of partial or fully SAW-less designs in UE Tx side, with analysis on the following mitigation approaches:

  - Relaxation of the spurious emission requirement for UE cross-band coexistence.

  - Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

  - A-MPR for device in certain frequency locations that operates a wider CBW/RB allocation.

- Proposal 5: RAN4 to consider further Rx blocking and associated impacts to enabling fully or partially SAW-less Rx designs for the lowest tier 6GR device.

- vivo R4-2600674

- Proposal 2: RAN4 can further study whether and how to support SAW-less device based on 20MHz maximum channel bandwidth

- Ericsson R4-2601880

- Proposal-1:  Study the RF impact of SAW-less design with the smallest maximum bandwidth with below two bandwidth options:

  - Limit the maximum configurable UL bandwidth, with same RF and BB bandwidth

  - Limit the maximum contiguous RB allocation (BB bandwidth) within a wider configurable UL bandwidth (RF bandwidth), with different RF and BB bandwidth

- Proposal-2:  Revisit the coexisting requirement in 6G if necessary.

- Proposal-3: Limiting BB bandwidth may bring network benefit more than limiting RF bandwidth, other aspect could be discussed other than RF impact for SAW-less design, e.g., LO retuning time.

Feature Lead note:

- Several solutions have been proposed to enable SAW-less design

- The SAW-less design is not dependent on the smallest max CBW decision in previous issue. All the solutions can be further discussed.

Recommended WF:

- Regarding how to support the SAW-less design, below solutions can be considered as starting point:

  - Option 1: Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge.

  - Option 2: MPR/AMPR for certain bands and/or certain UL RB allocations restrictions

  - Option 3: Relaxation of the spurious emission requirement for UE cross-band coexistence.

  - Other options are not precluded

- FFS on the requirement impacts with the SAW-less design, e.g., Tx emission requirement, Rx blocking requirements.

Sub-topic 3-2: Device types

Feature Lead note: According to the agenda of this meeting, the discussion will focus on the smallest max CBW and no device type discussion.

Issue 3-2-1: Device type related proposals

- Nokia R4-2600388

- Proposal 1: Consider four receivers as mandatory baseline for bands re-farmed from 5G for eMBB UE for RAN4 requirement work.

- Proposal 2: Consider six receivers as mandatory baseline for new 6GR bands for eMBB UE for RAN4 requirement work.

- Proposal 3: Consider eight receivers as mandatory baseline for new 6GR bands for FWA UE for RAN4 requirement work.

- Proposal 4: Consider eMBB UE and FWA UE devices support two TX chains as mandatory baseline for RAN4 requirement work.

- Proposal 5: RAN4 to study how Maximum Output Power needs to differentiate for 6G eMBB, FWA, Wearable and massive IoT devices by candidate 6G frequency ranges.

- Proposal 7: RAN4 to study realistic feasibility boundaries for Modulation Order  for 6G eMBB, FWA, Wearable and massive IoT devices in UL and DL.

- Proposal 8: RAN4 does not differentiate device types by duplex mode, since duplex operation follows automatically from supported bands.

- Xiaomi R4-2600457

- Proposal 1: RAN4 focus on the evaulation of the mandatory RF/BB capabilities with different device assumption considering  implementation feasibility and constraints

  - Target to identify fundemental parameters/factors related to 6GR common system design and coverage (Tight cooperation with RAN1)

  - Provice necessary input to RAN-P to assist the discussion on device type defintion

  - Urgent to be concluded in intial stage on identified fundemental parameters for overall system design

- Proposal 2: RAN4 also can further discuss UE RF/BB capability sets and dynamic UE capability

  - This work have RAN-P and RAN2 dependency

  - No urgency to discuss in intial stage

- Proposal 3: RAN4 also needs to study how to handle and discriminate different device assumption/form factors  from RAN4 RF/BB requirements perspective

  - Target to have scalable RF/RRM/Demod requirement framework to support diverse device assumption especailly on form factor dependency requirements

  - Not necessary treated as device types/capabilities

  - Majorly RAN4 internal study, less cross WG dependency

  - Discuss under UE RF, RRM and demod agenda separately

- LGE R4-2600700

- Proposal 1: Consider RF/BB implementation feasibility and constraints in Table 2-2 for IoT, Wearable, Smartphone, and FWA.

- Proposal 2: Consider baseline and maximum values per device type in Table2-3 as starting point

- CMCC R4-2600812

- Proposal 1: for device type, it’s suggested to take above aspects into consideration.

- Proposal 2: it’s suggested to further discuss device types with above table as example.

- CATT R4-2600316

- Proposal 1: RAN4 to progress studies related to device types by focusing on identifying distinct RF requirement and implementation feasibility implications, while remaining independent of and not constraining device type categorization at the RAN level.

- Proposal 2: RAN4 to consider a scalable RF requirement framework, in which RF requirements are parameterized along key technical dimensions, and to further consider realistic baseline and maximum values for these parameters to represent widely deployable and energy-efficient implementations, as well as optional high-end capabilities respectively.

Recommended WF:

- Hold on the discussion of device type according to RAN plenary agreement until September 2026.

- Annex: Companies’ contribution proposals

Modulation

- Proposals from CATT R4-2600313

- Proposal 1: RAN4 could prioritize the evaluation of existing and new higher-order modulation schemes, without considering constellation shaping, under new 6G PA model(s) at 7 GHz. - Proposal 2: RAN4 to conduct evaluations of modulation schemes using channel bandwidths exceeding 100 MHz under new 6G PA model(s), in order to fully leverage the 6GR studies at a later normative stage.

- Proposals from Nokia R4-2600386

Proposal 1: - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for data channel - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for CP-OFDM for data channel - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes Proposal 2: Support uniform 1024QAM in Uplink. Observation 1: Uniform 4096QAM in DL drives computation complexity and the power consumption of R-ML MIMO detection at the UE side. Observation 2: Very low EVM is required for uniform 4096QAM in DL.

- Proposals from Xiaomi R4-2600458

Observation 1: RAN1 discussion on modulation orders can be categorized into three areas: - CAT 1: NR modulation orders with uniform constellation are supported in 6G as baseline - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - Note: Above NR modulation orders served as basis and reference for performance evaluation in RAN1 - CAT 2: Higher modulation orders with uniform constellation are considered pending on performance evaluation - For DL, 4K QAM is on evaluation - For UL, 1K QAM is on evaluation - For assumed TX/RX EVM, before receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL. - CAT 3: Constellation shaping schemes including geometric shaping (GS) and Probabilistic shaping (PS) Observation 2: RAN1 start on evaluation work for CAT 2(uniform high modulation orders) and CAT3 (constellation shaping schemes including geometric shaping (GS) and probabilistic shaping (PS). - Both link level evaluation and system level evaluation are considered in RAN1 - start with link-level evaluation first with throughput/BLER as target evaluation metric - RAN1 already agreed basis evaluation assumption including MIMO configuration in BS and UE side, and receiver processing assumption Observation 3: Tight cooperation with RAN4 is required for RAN1 evaluation especially for PA modelling, Tx EVM assumption and MPR. Proposal 1: RAN4 work closely with RAN1 to avoid duplicated evaluation work across different WGs with clarification of responsibility of each WG - RAN4 focused on evaluation on Transmitter side for achievable Tx EVM and MPR reduction - RAN1 evaluation focused on the throughput gain/SNR gain in receiver side by taking RAN4 input into account including achievable Tx EVM, Rx EVM and MPR - On high modulation orders, - The simulation assumption between RAN1 and RAN4 shall be well aligned e.g., BS/UE antenna modelling, fading channel profile, MIMO configuration, and operating frequency, CHBW. Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility. Proposal 3: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26 Proposal 4: RAN4 expected work for different cases can be considered separately - For CAT 1 (NR modulation orders with uniform constellation): RAN4 can further evaluate Tx EVM or MPR requirements improvement/relaxation by taking existing NR requirements as baseline (NO feasibility study required, this work not urgent and shall be handled in UE RF thread instead of system parameter thread) - For CAT 2 (Higher modulation orders with uniform constellation including DL 4K QAM and UL 1K QAM): RAN4 focus on the evaluation of achievable Tx EVM/Rx EVM and MPR assumption in UL side - Taking as first priority as RAN1 required input from RAN4 on achievable Tx EVM/Rx EVM and MPR assumption - For CAT 3 Constellation shaping schemes (PS and GS): RAN4 need to wait sufficient progress from RAN1 to evaluate potential impact on Tx EVM, MPR and other potential requirements impact - Wait for sufficient progress reached in RAN1 on detailed shaping algorithm Proposal 5: Taking NR uniform modulation orders as basis for 6GR - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel. Observation 4: The conclusion from gNB and WIFI for 1K QAM shall not applied to UE due to the difference from following aspects power consumption/power control, device form factor limitation, compliant RF requirements and applicable RF components. Observation 5: Based on initial survey, it’s extremely challenge to achieve desirable Tx EVM and MPR performance to get benefits for UL 1KQAM and DL 4K QAM for handheld UE (e.g. RAN1 assume 6dB tightened than existing UL 256QAM, and DL 1K QAM). Proposal 6: RAN4 needs to evaluate the limitation of achievable Tx EVM and MPR performance for DL 4KQAM and UL 1KQAM with associated device type assumption for high modulation (e.g. FWA only) and applicable operating frequency. Proposal 7: RAN4 also needs to evaluate the possibility of supporting 1K QAM for handheld UE in 6GR. Proposal 8: For RAN4 evaluation for high modulation orders on achievable Tx EVM and Rx EVM, following table can be considered: EVM contributor; MPR assumption 1; MPR assumption 2. EVM; SNR (dB); EVM; SNR (dB). Tx EVM; PA. Transmitter. Phase noise. IQ imbalance. Total Tx EVM. Rx EVM budget. Total Tx+ Rx EVM.

- Proposals from Apple R4-2600575

Proposal 1: It is proposed to consider the following modulation schemes as a starting point for 6G: - UL: up to 256 QAM - DL: up to 1024QAM Proposal 2: The modulation schemes are studied with 200MHz CBW in ~7GHz to identify any issues or challenges, including the potential use of a new PA model. Proposal 3: It is proposed to limit the support of UL 1024QAM to FWA UE and further discuss how the existing 2.5% or 2.8% EVM requirements can be met. Proposal 4: It is proposed to understand the potential gain of supporting 4096QAM in real field. Observation 1: It is very challenging to support UE RX EVM around 1.26%.

- Proposals from vivo R4-2600668

Proposal 1: RAN4 needs to study the impact of GS and PS schemes on RF performance metrics, such as changes in the ideal constellation and possible adjustments to calculation rules for EVM calculation.

- Proposals from LGE R4-2600698

Proposal 1: Consider EVM 1.8% for 1024QAM. Proposal 2: Study whether to consider DPD for 1024QAM. Proposal 3: Study how to update a general in-band emission for 1024QAM. Proposal 4: Study how to update IQ image for 1024QAM. Proposal 5: Study how to update carrier leakage for 1024QAM.

- Proposals from Skyworks R4-2600737

Proposal on EVM budget for UL1024QAM: - Baseline 6G performance is UL256QAM at least up to 5GHz requiring IQ impairments at 34dB - It is reasonable to assume >36dB in band SNR for an ET PA with DPD assuming: - Enough power backoff so that the intrinsic PA distortions are already good - A modulation BW no higher than 200MHz - RF frequency up to 10GHz, FFS up to 15GHz - Proper DPD training over a large set of waveforms and power range and a DPD BW of at least 3x the modulation BW. - For equal UE/BS EVM split a <3% EVM link budget can be targeted. To improve this number, the BS EVM should be significantly better than the UE EVM - Additional margin is needed in the system to account for transients and RF chain noise floor to enable UL1025 QAM over a n acceptable output power dynamic range. Proposal for DL1024QAM and DL4096QAM: - DL1024QAM is considered as baseline for 6G: - Assuming a link target of <2.5% and a contribution of the BS of 1.5%, a UE contribution of <2% EVM is achievable with the same LO phase noise and IQ impairment than in the UL. - DL4096QAM is studied as an optional modulation for 6G: - Most contributors need to improve significantly (at least 2x versus 1024QAM) and, assuming that the most critical is the UE LO phase noise especially at higher RF frequencies, Sub 1% EVM will be needed from the BS Tx and most contributors. Based on this an EVM link budget <1.5% is achievable.

- Proposals from CMCC R4-2600813

Proposal 1: it’s suggested to implement unified performance metric for EVM system level and link level simulation. - For system level, the metric should be 5% throughput loss across all MCS - For link level, the metric should be the marginal performance loss at a relative throughput range (e.g. 70% -90%) compared to the performance with 0% EVM. Table 1 EVM budget for 256QAM and 64QAM Non-linearity source; 256QAM; 64QAM. %; C/N; %; C/N. PA; 1.50%; 36.5; 4.00%; 28.0. Transmitter nonlinearity; 1.34%; 37.4; 3.37%; 29.5. LO Phase noise; 1.78%; 35.0; 2.24%; 33.0. IQ Image; 2.24%; 33.0; 5.62%; 25.0. Transmitter total; 3.50%; 29.1; 8.00%; 21.9. Proposal 2: RAN4 is suggested to use above table as the baseline for further EVM budged analysis. Observation 1: the main affected requirements for constellation shaping modulation evaluation include the EVM, MPR/A-MPR requirements. Proposal 3: it’s suggested to further study the workload split between RAN1 and RAN4 is listed as below: - For the uniform higher order modulation without shaping, RAN4 can evaluate the MPR/A-MPR and EVM as RAN4 has done previously - For the constellation shaping case, RAN4 can be involved to provide more RF impairment analysis.

- Proposals from Huawei R4-2600886

Evaluation cases Proposal 1: Identify clear evaluation cases for modulation in RAN4. The initial evaluation is based on uniform constellation as NR, whether to extend the evaluation to non-uniform constellation and/or 4096QAM pending on RAN1 final conclusion. Case 1: Existing modulations with new Tx assumptions including new PA model - UL: BPSK, QPSK, 16QAM, 256QAM - Uniform constellation as NR - Both existing bands and new spectrum should be considered Case 2: Existing modulations with new Tx assumptions including new PA model - UL: BPSK, QPSK, 16QAM, 256QAM - Potential non-uniform constellation (pending on RAN1 decision) - Both existing bands and new spectrum should be considered Case 3: New modulations with new Tx assumptions including new PA model - UL: 1024QAM - Uniform constellation as NR - Potential non-uniform constellation (pending on RAN1 decision) - Both existing bands and new spectrum should be considered Evaluation assumptions Proposal 2: For the identified evaluation cases, defer the evaluation until concrete conclusions are reached regarding RF impairments and the PA modeling. Proposal 3: For UL 1024QAM, the EVM budget and implementation feasibility should be assessed and concluded in advance for all relevant Tx impairment factors, including at least PA non-linearity, I/Q imbalance, LO phase noise, and CFR-induced noise. Evaluation Method Proposal 4: For existing NR modulations with uniform constellations, the evaluation should focus on assessing the MPR reduction under new transmitter impairment assumptions and with the new PA model. Proposal 5: For existing NR modulations that may adopt non-uniform constellations, the evaluation should focus on determining whether new EVM requirements should be considered. When to start the evaluation pending on RAN1 conclusion. Proposal 6: For uplink 1024QAM, the evaluation should focus on the implementation feasibility and performance gain, evaluated through both link-level and system-level simulations.

- Proposals from MediaTek R4-2600909

Observation 1: Based on our evaluation (limited in single layer transmission), the chance of using UL 1024QAM is low with optimistic assumptions (e.g., ideal channel estimation and synchronization, PC1 UE) Proposal 1: RAN4 should set up a simulation campaign to check the feasibility of UL 1024QAM. Observation 2: Based on our evaluation, supporting DL 4096QAM does not provide gain for SNR<40dB. Proposal 2: For 1024QAM, RAN4 also study the feasibility for the expected requirements. Proposal 3: RAN4 should set up a simulation campaign to check the feasibility of DL 4096QAM together with the target BS Tx EVM. Observation 3: Probabilistic shaping has a higher PAPR for DFT-s-OFDM. Proposal 4: RAN4 may need to check whether a different EVM requirement is needed for probabilistic shaping (pending on RAN1 discussion progress).

- Proposals from Spreadtrum R4-2601053

Observation1: Based on the WIFI 7 spec, the required RCE (~EVM) is -35dB (1.7%) for 1024QAM. Proposal 1: For uplink modulation evaluation, the affected RF requirements should include EVM and MPR/AMPR. For downlink modulation evaluation, from BS perspective, which should include EVM. Proposal 2: Define the EVM budget and PA model/PA calibration point to evaluate assumptions to simulate MPR/AMPR. Proposal 3: For above 4.2GHz especially around 7GHz, support UL 256QAM as the baseline. For below 4.2GHz, UL 1024QAM can be further discussed. Proposal 4: Don’t consider DL 4096QAM and Keep DL 1024QAM in 6G day1. Proposal 5: We can wait for the conclusion about the new PA model to evaluate existing supported modulation for 6GR.

- Proposals from Samsung R4-2601123

Evaluation methodology Observation 1: The main practical question for RAN4 is whether these modulation orders are feasible under realistic RF constraints, and what EVM levels should be assumed and later standardized. Observation 2: RAN1 agreed a simple anchor assumption (e.g., a 6 dB tightening per ×4 constellation-size increase) before the concrete numbers from RAN4 for the higher-order modulation feasibility studies. Observation 3: For the link EVM target, a simple and useful starting anchor could be considered from link-level analysis, which are about 2.88% for 1024-QAM and 1.52% for 4096-QAM. Proposal 1: The following approach can be captured as a practical approach for the higher order modulation EVM study in RAN4: - RAN4 keeps the existing NR requirements as the baseline and studies higher-order uniform QAM feasibility concurrently with RAN1. - For initial link-level anchoring under AWGN, RAN4 uses 2.88% (1024-QAM) and 1.52% (4096-QAM) as reference target values for feasibility comparisons across companies and simulations. - RAN4 then evaluates how much additional EVM margin is required when RF impairments are included, and records the main drivers (e.g., phase noise, I/Q imbalance, PA nonlinearity, clipping/CFR if applied), without forcing a requirement change until there is consistent evidence. Higher-order modulations Proposal 2: For downlink, in order to increase spectral efficiency in indoor and high‑SNR regions, it is proposed to support 4096-QAM and initiating study to define EVM requirements in RAN4. Proposal 3: For uplink, in order to increase efficiency spectral in indoor and high‑SNR regions, it is proposed to support 1024K‑QAM and initiating study to define EVM requirements in RAN4.

- Proposals from ZTE R4-2601177

UL 1024QAM: Observation 1. For 6GR, the existing 5G NR modulations are supported as basis, other modulations are not precluded and are under discussion in RAN1. Proposal 1: For the UL 1024QAM study, it is proposed to use the same values as 5G NR gNB DL EVM values as starting point to evaluation performance. DL 4096QAM: Observation 2. The EVM for DL 4096QAM is about 0.9%, corresponding to about 41dB target SNR for high code rates. Observation 3. It is quite challenging to support DL 4096QAM while meeting stringent EVM requirements without a power backoff solution, which comes at the cost of reduced network coverage and degraded overall system performance. - Others: Proposal 2: If LP-WUS signal is supported in 6G day1, it is proposed to further discuss the impacts on potential EVM degradation of 6GR other modulation orders due to the simultaneous LP-WUS signal transmission.

- Proposals from Sony R4-2601397

Observation 1: The EVM requirements on DL 1024 QAM are specified with 2.5% and 2.8%, respectively, for frequency bands below and above 4.2 GHz. Observation 2: in 5G NR, DL EVM and UL EVM are the same for modulation orders up to 256QAM. Observation 3: In general, we consider supporting 1024QAM is possible, with a focus on improving PA linearity and IQ imbalance, while also considering mild improvements in other EVM contributors in a transmitter. Observation 4: The actual EVM improvement can depend heavily on the device type and the corresponding implementations. Observation 5: it is possible to relax the EVM requirements with DPoD on BS side. Proposal 1: RAN4 can assume the same EVM requirement on the UE side as the BS side, as a starting point to study the feasibility of UL 1024 QAM. Proposal 2: RAN4 to study the modulation as part of a scalable design, to determine the proper order of modulation to be supported by each device type. Proposal 3: When determining the feasibility of high-order modulation in the uplink direction, it is also reasonable to consider DPoD on the BS.

- Proposals from OPPO R4-2601419

Existing NR modulations Observation 1: MPR, AMPR and IBE impacted by modulation are through the EVM requirements, if the nonlinearity of components don’t change, the EVMs don’t change in 6G for existing NR UL modulation. Proposal 1: The EVM values in 6G for the modulation (BPSK to 256QAM) can keep the same with the EVM values in NR. UL 1024QAM Observation 2: It is hard to achieve lower EVM for LO phase noise and IQ imbalance than the values for 256QAM. Observation 3: RAN1 has given some configurations and scenarios for system-level simulations. Observation 4: The required SNR for UL 1024QAM are less than 30dB only for MCS23 with some Tx+Rx EVM values. Observation 5: The required SNR for UL 1024QAM will be larger than 30dB for MCS23 with 3%+3% Tx+Rx EVM. Observation 6: RAN1 has agreed BS modelling for 700MHz, 2GHz and 7GHz. Proposal 2: RAN4 can evaluate EVM budget for UL 1024QAM from two perspectives based on the implementation: • Consider stringent EVM contributions from LO phase noise and IQ imbalance (i.e., 1% separately) to set 2.5% EVM for UL 1024QAM. • Consider a more relaxed EVM (i.e., 3%) for UL 1024QAM compared to that for DL 1024QAM. Proposal 3: For higher-order modulation, RAN4 can focus on the scenarios of Urban Macro and Indoor using the parameters in Table 2 as starting point: - Table 2 The frequency parameters and scenarios for modulation evaluation Urban Macro; Indoor Hotspot. Carrier frequency; Around 700 MHz Around 2 GHz Around 7 GHz; Around 2 GHz Around 7 GHz. Simulation BW; Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS. Proposal 4: The phase noise model doesn’t need to introduce in 6G FR1 link-level simulation. Proposal 5: The link-level simulation can focus on MCS 23 and MCS 24 for UL 1024QAM with Tx EVM between 2.5% and 3%. Proposal 6: The link-level simulation can use the link-level simulation assumption in Table 4 as the starting point. Table 4 Link level simulation assumptions Parameter; Value. Carrier frequency; 700 MHz, 2 GHz,7GHz. CBW/SCS; Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS. Allocated RBs; Full allocation. Propagation; TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN). MCS; Based on 5G MCS for 1024QAM PDSCH in Table 5.1.3.1-4 of TS 38.214: CP-OFDM: MCS 23, 24 other MCSs are not precluded. Symbol type; CP-OFDM. HARQ; 4, None. Antenna configuration; Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix). Channel estimation; Practical. Receiver type; MMSE. PUSCH configuration; Type A mapping, Start symbol 0, Duration 14. DMRS configuration; Type 1, Single symbol, 1 additional DMRS. txEVM + rxEVM; txEVM = rxEVM: 2.5%+2.5%, 3%+3% txEVM > rxEVM: 3%+2%, 3%+2.5%. Proposal 7: RAN4 could first evaluate higher-order modulation using RAN1 agreed BS modelling for 700MHz, 2GHz, and 7GHz. Proposal 8: System-level simulation can start using the system-level simulation assumption in Table 5 - Table 5 System level simulation assumptions Parameters; Urban macro; Indoor. Network layout; hexagonal grid, 19 macro sites, 3 sectors per site with wrap around; 50m x 120m, 12BSs. Inter-site distance; 500m; 20m. BS antenna height; 25 m; 3 m. UE location; Outdoor/indoor; Outdoor and indoor; Indoor. Indoor UE ratio; 20%. Low/high Penetration loss ratio; 50% low loss, 50% high loss. LOS/NLOS; LOS and NLOS; LOS and NLOS. UE antenna height; Same as 3D-Uma in TR 36.873; 1.5 m. UE distribution (horizontal); Uniform. Minimum BS – UE distance (2D); 35 m; 0 m. Shadowing correlation; Between cells: 1.0 Between sites: 0.5. Pathloss; 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873; 3D-InH LOS and NLOS in Table 7.2-1 of 36.873. Carrier frequency; 700MHz, 2GHz, 7GHz. BS antenna configuration; 700MHz: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1; 1, 2) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 2GHz (M,N,P,Mg,Ng; Mp, Np)= (12, 8, 2, 1, 1; 4, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 7GHz (M,N,P,Mg,Ng; Mp, Np)= (64, 16, 2, 1, 1; 16, 16) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi; 2GHz (M,N,P,Mg,Ng; Mp, Np)= (4, 4, 2, 1, 1; 1, 4) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi (M,N,P,Mg,Ng; Mp, Np)= (16, 16, 2, 1, 1; 8, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi. UE antenna configuration; Omnidirectional, 0dBi. System bandwidth; Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS. Target SNR at BS side; Get from link-level simulation. UE max output power; Around 700 MHz: PC3 23dBm Around 2 GHz: FDD 23dBm, TDD 26dBm Around 7 GHz: PC2: 26 dBm. Power control; Power control parameters set to achieve the target SNR at BS side..

- Proposals from NTT DoCoMo R4-2601466

Observation 1: PA memory effects and video bandwidth limitations impact linearity in wider channel bandwidths (e.g., 200MHz) at 7GHz. A PA model validated only at narrower bandwidths may not fully capture the non-linear degradation slope when the bandwidth is doubled. Observation 2: In wideband scenarios (200MHz), frequency-dependent distortion (e.g., ripple, band-edge roll-off) can vary across the channel. A single average EVM metric may not be sufficient to guarantee link performance for all allocated RBs. Proposal 1: For modulation evaluation with 200MHz channel bandwidth, RAN4 verifies the model's validity by comparing the performance trends (e.g., ACLR and EVM degradation) between 100MHz and 200MHz. Proposal 2: In addition to the average EVM, RAN4 considers the EVM distribution across frequency or edge RB EVM during the modulation evaluation for 200MHz bandwidth to ensure no localized degradation exceeds the system limits.

- Proposals from CableLabs, Charter, Rogers R4-2601955

Observation 1: RAN4#116bis agreed to study UL 1024-QAM in Rel-20 6GR. RAN1#122bis agreed to study both DL 4096-QAM and UL 1024-QAM in Rel-20 6GR. RAN4#117 discussed both UL 1024-QAM and DL 4096-QAM. Proposal 1: Based on the RAN4 and RAN1 October 2025 meeting agreements and our system-level simulation results, which indicate that UL 1024-QAM and DL 4096-QAM are widely achievable in a UMa FWA scenario, we propose that RAN4 continue to study the use of DL 4096-QAM, in addition to UL 1024-QAM, for 6GR FWA operation. Observation 2: The simulation results indicate that there is a significant probability that the SINR can support UL 1024-QAM and DL4096-QAM: (a) at around 4 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 42.7% and DL 4096-QAM can be supported is 72.3%; (b) at around 4 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 15.1% and DL 4096-QAM can be supported is 46%; (c) at around 7 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 70.9% and DL 4096-QAM can be supported is 97.7%; and (d) at around 7 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 18% and DL 4096-QAM can be supported is 51.2%. Proposal 2: We encourage other companies to perform similar system-level simulations to evaluate the achievability of UL 1024-QAM and 4096-QAM in 6GR FWA scenarios.

- Proposals from Qualcomm R4-2602059

Observation 1: UE 1024QAM EVM specification of ~2%, represents a good compromise across UE implementations Observation 2: Image (IQ Imbalance) and LO Leakage levels should be better than -40 dBc for 1024QAM Observation 3: PA designs currently used in commercial UEs can meet the 1024QAM EVM requirements

Channel arrangement

- Proposals from Nokia R4-2600389

Proposal 1: The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them. Observation 1: 5G NR principle for sync raster definition can be inherited for 6GR sync raster design. Observation 2: Limiting the sync raster locations in frequency domain can complicate the cell deployment. Observation 3: It should be up to the operators to suggest if some of the sync raster placements for a particular band can be removed. Observation 4: Aligning 6GR synchronization raster locations with NR, when feasible, can help to alleviate the UE initial search complexity when supporting both 5G and 6GR.

- Proposals from OPPO R4-2601450

Channel raster Observation 1: To consider the 5G and 6G channel raster as SCS level aligned helps to reduce the interference of adjacent carrier. Observation 2: The 5kHz channel raster of 6G helps SCS level alignment of 5G and 6G channel raster. Observation 3: The 10kHz channel raster is introduced in late release 18 and not mandatory to all bands. Observation 4: 5kHz channel raster can co-exist better to 15kHz channel raster compared to 10kHz Observation 5: 5kHz is the GCD of 10kHz and 15kHz and used as global frequency raster in NR-ARFCN. Proposal 1: For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted. Sync raster Observation 6: Kssb is introduced to find the RB edge based on Rule 1 as SCS level alignment of sync raster and channel raster. Proposal 2: The channel raster and sync raster with SCS level alignment helps interference mitigation and needs to be kept. Observation 7: Rule 2 gives full flexibility of NW deployment to put SSB in minCBW through the spectrum while limiting the largest granularity of sync raster. Proposal 3: The Rule 2 can be improved as for multiple of the minCBW, there exists one SSB to be covered. Observation 8: The equation 1 Sync raster separation ≤ minCBW + Channel raster – BWSSB is used to calculate the sync raster separation. Observation 9: With introduction of offset, the sync raster for channel raster of 100kHz as well as 5kHz and 10kHz are three points for each granularity step which brings 3 times sync raster compared to channel raster of SCS which is only one point for each granularity step. Observation 10: With the SSB default periodicity is likely to be increased, much sparser sync raster is needed. Proposal 4: It is proposed to use m*channel raster as reference channel raster to further develop 6G coarse sync raster. The value of m can be further investigated.

- Proposals from CATT R4-2600315

Proposal 1: RAN4 to defer the adoption of a 10kHz channel raster until the spectrum utilization study for 6GR is finalized. Proposal 2: If a 5kHz channel raster is to be introduced and the global frequency grid is also 5kHz, then RAN4 does not need to define a channel raster explicitly in the specification, as there is not any constraint on carrier placement in such a case. Proposal 3: RAN4 to consider a two-level sync raster in 6GR consisting of a coarser primary sync raster and a finer secondary sync raster to balance deployment flexibility and UE energy consumption. Proposal 4: RAN4 to re-examine channel-spacing definitions for both single- and multiple-carrier operations by considering wider channel bandwidth, new operating frequencies and shared spectrum scenarios, and defer the discussion until the other core parameters have become more stable.

- Proposals from Xiaomi R4-2600460

Channel raster Observation 1: Channel raster concept majorly existed in RAN4 for channel position indication and conformance test Observation 2: Global channel raster (basic granularity as per sub-frequency range basis) plus per band channel raster with multiple step-size was introduced in NR Figure 1: Global channel raster with Per band step-size for channel raster in NR Observation 3: From RAN1/RAN2 perspective, RB mapping/allocation was designed based on SSB position and reference point A Observation 4: The concept of per band wide channel raster especially large channel raster i.e., 100kHz channel raster bring restriction on efficient spectrum usage and channel placement especially for irregular BW supporting and enhanced 10kHz channel raster was introduced in later stage for specific bands which originally have 100kHz channel raster. Observation 5: Supporting 100kHz channel raster on LTE refarming bands in NR which bring sync raster shift issue with 3 candidate locations Proposal 1: Support 5kHz channel raster in day 1 for below 3GHz Proposal 2: RAN4 further study whether 100kHz channel raster still required for below 3GHz bands Proposal 3: Remove per band channel raster concept with following value per sub-frequency range basis Frequency range; Channel raster. <3GHz; 5kHz. 3GHz ~ 24.25kHz; 30kHz. 24.25GHz ~ 52GHz; 120kHz. Sync raster Observation 6: Global sync raster (basic granularity per sub-frequency range) plus band specific step-size was introduced NR. Figure 2: Global sync raster + band specific step-size in NR Observation 7: The initial access channel design is still on discussion in RAN1 including SSB BW and SSB periodicity. Proposal 4: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026 Proposal 5: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area - Flexible step size per sub-frequency range/per band - Scalable step-size pending on SSB periodicity - SS raster design for MRSS Channel spacing Observation 8: “Normal channel spacing” concept was introduced for RAN4 requirements and conformance test. There is no restriction on real network deployment. Observation 9: Mixed numerologies case were considered in NR on the same band which bring specification over complicated. Proposal 6: Simplify Channel spacing definition without consideration of mixed numerology case in 6GR. Proposal 7: Further study “normal channel spacing” definition and the relation-ship between intra-band contiguous CA and NC CA.

- Proposals from Apple R4-2600577

Observation 1: From initial access point of view, channel raster is not needed in 5G. Observation 2: In 5G, the number of bits needed to signal the absolute frequency in RRC depends on the granularity of the global frequency raster. Observation 3: In 5G, because of the adoption of 100kHz, three raster points are specified to ensure that each channel has a corresponding sync raster that lies on the same SCS grid for bands of 100kHz channel raster. However, if the channel raster is specified with a granularity that is the common factor of both 100kHz and SCS (15/30/60/120kHz), such complication can be avoided. Observation 4: From MRSS perspective, 6G channel raster design at least needs to include the channel raster points of 5G channels, especially those located on the 100kHz raster grid. Proposal 1: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync raster points. Proposal 2: It is proposed to study CBW-dependent sync raster design as an option to enable fast cell search and UE power saving.

- Proposals from Vivo R4-2600661

Proposal 1: RAN4 to define 10 kHz channel raster for sub-3GHz frequency range and SCS based channel raster for above 3GHz. Observation 1: NR basic principle of sync raster design shall be adopted to guarantee that at least one “SSB” can be cover by the minimum transmission bandwidth. Observation 2: Sync raster to support 3MHz shall be separately discussed, pending the progress on RAN1 initial access discussion. Proposal 2: RAN4 to adopt NR basic principle of sync raster design to define the basic sync raster as: Min BWCHANNEL-BWSSB+ ΔFCH,Raster where BWCHANNEL is the minimum CBW and BWSSB is the BW of 6G SSB which depend on the further progress of minimum CBW and SSB discussion. Observation 3: Any solutions to mitigate the impact of potential SSB periodicity extension should be considered based on the basic sync raster without impacting the basic sync raster definition. Proposal 3: Depending on whether SSB periodicity is extended in 6G, methods to mitigate impact on cell search time can be further investigated without impacting the basic sync raster definition Proposal 4: Sync raster to support 3MHz shall be separately discussed and defined, consistent with the approach in NR for specific bands. Observation 5: It is unnecessary to avoid sync raster overlap between 5G and 6G for 6G sync raster design which can be handled by implementation.

- Proposals from CMCC R4-2600814

Observation 1: channel raster is mainly used for RAN4 conformance testing and to show whether the carrier center is. Proposal 1: RAN4 is suggested to study whether the carrier bandwidth in UE dedicated CBW must be aligned with channel raster, i.e. whether to reflect them into RAN2 spec. Observation 2: current sync raster is based on 100kHz channel raster rather than 10kHz which lead to the SSB doesn’t cover all RF CBW and locations. If RAN4 want to design the sync raster to cover all possible RF CBW and locations on 5/10kHz channel raster, sync raster must be designed much denser. Observation 3: the main drawbacks of SCS basis channel raster is inflexibility when co-current with 10kHz channel raster network especially when migration from 5G to 6G. Proposal 2: RAN4 needs to consider the joint design of the channel raster and sync raster, rather than discussing them in isolation and taken following aspects into consideration: - If RAN4 want to design the sync raster to cover all possible RF CBW and locations on 5/10kHz channel raster, sync raster must be designed much denser. Proposal 3: RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operator’s wider spectrum holding with the benefits of shorter search time and lower power consumption. Proposal 4: A proposed solution is to implement a multi-tiered synchronization raster. For instance, one tier would correspond to a unified minimum channel bandwidth, while another would support larger bandwidths.

- Proposals from Huawei R4-2600889

Channel raster Proposal 1: Enhance channel raster with granularity of 5kHz/10kHz could be adopted from the outset of 6G, replacing the 100kHz channel raster. Proposal 2: The channel raster entry structure for 6GR bands should remain similar to the current one for 5G. For any consideration of shifting specific bands from the 100 kHz to an SCS-based raster, the decision must be based on comprehensive inputs from operators. Sync raster Proposal 3: Retain the foundational design principles of the NR sync raster for 6GR as the guiding principles. On this basis, methods to optimize the SSB search time should be further investigated, under the condition that backward compatibility with the NR constraints is maintained. Proposal 4: Sparse sync raster should be studied for 6GR.

- Proposals from MediaTek R4-2601004

<Channel raster> Observation 1: In NR, two types of raster granularity were defined for channel arrangement, global raster (ARFCN) and channel raster. Observation 2: Multiple channel raster granularities existed in NR for different operating bands. Proposal 1: Define 6G channel raster based on the Greatest Common Factor (GCF) among the existing channel raster in a frequency range. Single channel raster per frequency range: Frequency range (GHz); 5G Channel raster; 6G channel raster (based on GCF). 0-3; 100kHz, 10kHz, 15kHz, 30kHz; 5kHz. 3 - 24.25; 15kHz, 30kHz; 15kHz. 24.25 - 100; 60kHz, 120kHz; 60kHz*. *For FR2 range we can also consider 120kHz channel raster instead of 60kHz to align with RAN1 latest agreement.. Proposal 2: Unify both global raster (ARFCN) and channel raster for simpler channel arrangement in 6G. <Sync raster> Observation 3: NR sync raster is over designed and only about 10% of sync raster entries are utilized in real-field environment. Proposal 3: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access. Observation 4: Increasing SSB periodicity larger than 20ms would potentially increase the delay associated with sync raster. Proposal 4: RAN4 to assess different sync raster proposals considering the delay associated with sync raster and SSB periodicity. 6G sync raster design needs to ensure initial access delay is not worse than 5G. Observation 5: A minimum CBW of 5MHz for sub 3GHz frequency range is assumed in NR, which results in a highly dense sync raster design. Proposal 5: Define 6G sync raster using - Coarse sync raster with higher priority: based on larger assumption of CBW - Normal sync raster with lower priority: based on smaller assumption of CBW Proposal 6: With coarse sync raster, operators to place their SSBs following the coarse sync raster entries if their spectrum allocation allows for that (e.g., allocation larger than min CBW). However, if such entries do not exist for some operators due to smaller CBW allocation, their SSBs can instead be placed following the normal sync raster entries. Observation 6: SSB bandwidth is one of the main contributors for having a highly dense sync raster in NR. Observation 7: Narrow SSB BW design is an optimal solution to achieve a sync raster based on PSS. Proposal 7: Consider PSS BW instead of SSB BW in 6G sync raster design (i.e., Sync raster = TBW – PSS BW + channel raster) to achieve lower density sync raster. <Channel spacing> Observation 8: In NR, nominal channel spacing specifications were over-complicated due to the consideration of mixed numerologies for carriers within the same band. Proposal 8: Simplify channel spacing definition in 6G without considering mixed numerology scenarios.

- Proposals from Qualcomm R4-2601030

Proposal 1: Adopt the SCS based raster as the baseline in all bands where it is currently specified and in all future 6G bands. Proposal 2: RAN4 should study optimization of the channel raster in bands which use the 100 kHz raster and/or possible addition of new raster points to enable future migration to SCS based raster when coexistence with NR is no longer necessary. Observation 1. The sync raster design has to account for the channel raster design such that any possible channel position is covered by at least a single sync signal entry. Proposal 3. RAN4 should study sync raster optimizations to minimize the number of sync raster entries while maintaining forward compatibility for flexible channel placement. Proposal 4. Maintain the same design principles for the 6GR sync raster design as for NR. The raster granularity depends on the channel raster, the SSB bandwidth and the minimum channel BW supported in a band. Proposal 5. The impact of the channel raster on sync raster complexity should be considered in the channel raster study and in related design decisions.

- Proposals from Spreadtrum R4-2601056

Proposal 1: It is necessary to define channel raster, for re-farming bands with 10 kHz channel raster, using 10 kHz channel raster in 6GR. For other re-farming bands and new bands, SCS-based channel raster can be adopted in 6GR. Observation 1: The position of sync raster is the subset of the position of channel raster, the sync raster is the integral multiple of channel raster in 5G. Proposal 2: Keep the position of sync raster is the subset of the position of channel raster as a starting point in 6GR. Proposal 3: Decouple the channel raster and sync raster to be more flexible deployment. Meanwhile, one cell is deployed in one channel raster, SSB can be deployed in one sync raster in this cell. Proposal 4: More sparse sync raster design can be considered in 6GR. The principle in NR for sync raster can be followed in 6GR.

- Proposals from ZTE R4-2601070

Channel raster: Proposal 1: For bands above 3GHz, SCS based channel raster should be applied. Observation 1: FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster, the frequency range of band itself is limited and don’t see the strong motivation/necessity to deploy the intra-band contiguous CA. Proposal 2: For sub-3GHz bands, propose to define 10kHz channel raster in 6G day 1 instead of 100kHz. Sync raster: Proposal 3: To ensure deployment flexibility, it is proposed to retain the foundational design principle of 5G NR sync raster. Proposal 4: It is proposed to split frequency range into more sub-ranges and select different min channel bandwidth for each frequency range. Observation 2: Compared with only SCS based channel raster, 5kHz and 10kHz channel rasters may necessitate multiple sync rasters. Channel spacing: Proposal 5: It’s better to simplify the definition of nominal channel spacing without considering mixed numerologies. Observation 3: Nominal channel spacing is defined as integer multiples of 300kHz or SCS in current spec, while this 300kHz nominal channel spacing is not implemented in commercial UEs. Proposal 6: It is critical to avoid such a misalignment between spec and practical deployment that spec supports 300kHz nominal channel spacing but this configuration cannot be used in practical deployment scenarios.

- Proposals from Samsung R4-2601126

Channel raster Observation 1: RAN4 discussed the channel raster for 6GR mainly in relation to the legacy FR1 bands below around 3 GHz that have been using a 100 kHz raster in NR. Observation 2: It is considered reasonable to maintain a channel raster framework for 6GR at least as a reference for test specifications and verification, and to avoid ambiguity in defining representative carrier centre frequencies. Observation 3: An SCS-based raster can naturally align the carrier centre frequencies to the OFDM grid and provide a consistent framework across frequency ranges, which is in line with the overall direction of simplified system parameter design. Observation 4: Adopting 5 kHz may improve flexibility for spectrum planning and migration without introducing additional functional complexity in UE operation, since the channel raster is typically used as a deployment reference grid rather than a list of candidates that the UE must exhaustively search. Proposal 1: It is proposed that the channel raster framework for 6GR Day-1 converges to: - Use SCS-based channel raster as the default for new 6G bands and for frequencies above around 3 GHz. - For refarmed FDD bands below around 3 GHz, where legacy 100 kHz-based planning already exists, either: - Continue to use a single 5 kHz channel raster as a common baseline across such bands; or - Migrate to an SCS-based raster while ensuring that the resulting centre frequencies remain compatible with existing deployments through appropriate band-specific migration rules. - Avoid defining multiple alternative rasters (e.g. 100 kHz, 10/5 kHz and SCS-based) per band for 6GR day-1, and instead treat any additional rasters as exceptional options that require clear coexistence justification. Synchronization raster Observation 5: RAN4 discussed the design of synchronization raster for 6GR with the general objective of reducing unnecessary raster entries and simplifying initial cell search. Observation 6: The raster step could be selected in a way that is more aligned with practical sampling-rate and FFT-size structures in UE receivers in addition to minimum channel bandwidth and SSB occupancy when defining the raster step. Observation 7: A two-step concept can be considered as a potential study direction, and further investigation is needed for a unified sync raster framework across a wide range of 6G bands. Observation 8: A relative GSCN per band, or per frequency rage, could be considered for the sake of simplification based on the practical usage of the legacy GSCN. Proposal 2: RAN4 continues the discussion on 6GR synchronization raster step design with the following study directions: - consider sparser and implementation-friendly raster step sizes by taking UE receiver sampling-rate and FFT-related aspects into account, for newly introduced higher bands such as around 7 GHz, in addition to minimum channel bandwidth and SSB size; - investigate the feasibility of a unified coarse raster step applicable across wide frequency ranges, potentially complemented by an on-demand finer search step to improve robustness; and - study potential simplification of GSCN definition and usage, including the feasibility of relative indexing per band or per range.

- Proposals from Ericsson R4-2601382

Observation 1: MRSS will be crucial for 6GR rollout based on the NR network. 6GR network will have to coexist with 4G IoT for a long time. and therefore propose that Proposal 1: the channel- and synchronization raster for 6GR shall be specified such that MRSS with PRB alignment to the NR carrier can be configured for all possible 100 kHz NR channel raster entries. Observation 2: less of a problem with SCS-based 6GR raster in bands above 3 GHz (including FR2-1) the synchronization raster permitting. Observation 3: 6GR PRB alignment with NR carriers on the 100 kHz channel raster also allows coexistence with 4G IoT with sub-carrier alignment in case the 6GR carrier also supports the 7.5 kHz shift in the UL. We also observe that Observation 4: the sub-carrier offset kSSB and the triplets M enable assignment of an NR carrier on any arbitrary 100 kHz channel raster entry below 3 GHz. Observation 5: the granularity of the 6GR synchronization raster must be sufficiently fine such that the SSB can be flexibly configured within BWPs located within the carrier. Observation 6: the main purpose of the NR enhanced channel raster with its 10 kHz is to be able to locate a smaller UE CHBW with PRB granularity within a wider BS carrier centered on the 100 kHz channel raster. There is still hope: Observation 7: improvements of the NR synchronization raster for 6GR with a view to reducing cell-search time and/or increasing the SSB periodicity for energy saving are possible while still maintaining coexistence with legacy networks and enabling efficient MRSS.

- Proposals from SONY R4-2601400

Observation 1: A large number of sync raster defined in 5G NR are not used in the field, which causes longer delay and power consumption for cell searching. Observation 2: The delay due to scanning through a large number of sync raster positions is expected to be more prominent in the 6GR design as the SSB periodicity may be even further reduced, and thus a sparse sync raster design would be beneficial in 6GR. Observation 3: RAN4 can consider a larger configured DL BW, a narrower SSB bandwidth or reduced reference channel raster to reduce the granularity of sync raster. Proposal 1: RAN4 studies the approach to reducing the number of sync raster positions in 6GR. Proposal 2: It is proposed 6G considers an enhanced channel raster (finer than 100 kHz) from the beginning to ensure the spectrum usage of 6G can be more efficient than 5G. Proposal 3: Considering the MRSS and co-existence between 6GR and 5G NR, it is proposed 6GR to consider either 5kHz channel raster or the same channel raster as 5G NR for a given band.

- Proposals from NTT DoCoMo R4-2601468

Observation 1: Defining synchronization and channel raster independently leads to excessive UE scanning complexity and power consumption, particularly in wideband FR3 operations and scenarios with extended SSB periodicities. Proposal 1: RAN4 to study the coupling mechanism between synchronization raster and channel raster, where the valid channel raster grid is determined based on synchronization raster density and configuration (e.g., frequency band, density, and position) through dynamic scaling or pre-defined mapping tables. Proposal 2: RAN4 to study dynamic raster control via RRC/SIB signalling to adapt to various network scenarios, including extended SSB periodicities for energy saving and SSB-less operations. Proposal 3: RAN4 to investigate raster derivation rules for SSB-less scenarios, specifically identifying the target channel frequency raster based on an anchor synchronization signal from another frequency range (e.g., FR1) and associated assistance information.

- Proposals from LG Electronics R4-2600701

Proposal 1: Consider channel raster 5kHz for below 3GHz and SCS-based raster above 3GHz. Proposal 2: Consider design principle of 6G sync raster as follows. Sync raster granularity = (NRB of minimum CBW – PSS_RB(or SSS_RB) + 1 RB)*SCS*12 Proposal 3: Consider potential PSS/SSS bandwidth and potential 6G NRB for 6G sync raster. Potential 6G PSS/SSS bandwidth - 12/15 for minimum CBW = 3MHz - 12/15/16/18/20 RB for minimum CBW > 3MHz Potential 6G NRB Potential 6G NRB for minimum CBW [RB]. 3MHz; 5MHz; 10MHz; 50MHz. SCS 15kHz; 15; 25, 26. SCS 30kHZ; 24, 25, 26. SCS 120kHz; 32, 33. Proposal 4: Decouple 6G sync raster design from channel raster. Proposal 5: Consider 2 step based 6G sync raster - 1st priority GSCN Group (Sparce) and 2nd priority GSCN Group (Dense). Proposal 6: Consider 6G nominal channel spacing as follows. - Below 3GHz (assuming channel raster (∆FRaster) = 5kHz) - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 - For above 3GHz, Normal Channel spacing can be rewriten as follows. - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{- (∆FRaster /3) kHz, 0 kHz, (∆FRaster /3) kHz}

Device type

- Proposals from Nordic R4-2600054

Observation-1: Without band-specific RF filter, for 20RF20BB TX case it is not possible to achieve -50dBm/1MHz=-110dBm/Hz spurious emissions for coexistence requirements, unless victim DL band edge is ~40-50MHz away from TX band edge. For 20RF5BB and 5RF5BB TX cases requirement is feasible if the victim DL band (own band DL or other band DL) edge is ~10MHz from TX band edge. Proposal-1: Capture in RAN4 6G TR: If 6G maximum smallest DL/UL RF BW is 10 or 20MHz, RAN4 defines band-specific frequency regions where UL scheduling restrictions within 10 or 20MHz BWP apply after MSG4 for HD-FDD 6G IoT UE with reduced number of band-specific RF filter. - Note, it is assumed that initial access UL BB BW remains naturally limited to [4-5] MHz in UL as in NR in FR1. - FFS UL scheduling restrictions within 10 or 20Mhz UL BWP, UL <=5MHz, including dependency on transmit power/PSD and/or location from band edge. - FFS relaxation of spurious emission requirement(s). Observation-2: If 6G maximum smallest DL/UL RF BW is 10 or 20MHz, few problematic bands will require DL RF-band specific filters. Proposal-2: Relax duplexing gap 2x compared to 5G to enable single PLL implementation of HD-FDD. Proposal-3: Study how a 5MHz RF UE could operate within configured 10 or 20MHz UL BWP.

- Proposals from Nokia R4-2600388

Proposal 1: Consider four receivers as mandatory baseline for bands re-farmed from 5G for eMBB UE for RAN4 requirement work. Proposal 2: Consider six receivers as mandatory baseline for new 6GR bands for eMBB UE for RAN4 requirement work. Proposal 3: Consider eight receivers as mandatory baseline for new 6GR bands for FWA UE for RAN4 requirement work. Proposal 4: Consider eMBB UE and FWA UE devices support two TX chains as mandatory baseline for RAN4 requirement work. Proposal 5: RAN4 to study how Maximum Output Power needs to differentiate for 6G eMBB, FWA, Wearable and massive IoT devices by candidate 6G frequency ranges. Proposal 6: CBW choice for device type should primarily focus on minimum/baseline requirement definition. Proposal 7: RAN4 to study realistic feasibility boundaries for Modulation Order for 6G eMBB, FWA, Wearable and massive IoT devices in UL and DL. Proposal 8: RAN4 does not differentiate device types by duplex mode, since duplex operation follows automatically from supported bands.

- Proposals from Xiaomi R4-2600457

Observation 1: According to 6G SID, 6GR target to have ”scalable and forward compatible design for diverse device types” which involved both RAN4 and other WGs on this objective. Observation 2: According to 6G SID, RAN4 also needs to study UE RF capabilities considering different device types and implementations. Observation 3: The standalization and defintion of device types belong to RAN-P decision including possible parameters/factors and the discussion will be resumed by RAN#113 (September 2026) . Obseravtion 4: 6GR design target to support diverse device types in the same cell. Figure 1: 6GR harmonic system design for diverse device types Proposal 1: RAN4 focus on the evaulation of the mandatory RF/BB capabilities with different device assumption considering implementation feasibility and constraints - Target to identify fundemental parameters/factors related to 6GR common system design and coverage (Tight cooperation with RAN1) - Provice necessary input to RAN-P to assist the discussion on device type defintion - Urgent to be concluded in intial stage on identified fundemental parameters for overall system design Proposal 2: RAN4 also can further discuss UE RF/BB capability sets and dynamic UE capability - This work have RAN-P and RAN2 dependency - No urgency to discuss in intial stage Proposal 3: RAN4 also needs to study how to handle and discriminate different device assumption/form factors from RAN4 RF/BB requirements perspective - Target to have scalable RF/RRM/Demod requirement framework to support diverse device assumption especailly on form factor dependency requirements - Not necessary treated as device types/capabilities - Majorly RAN4 internal study, less cross WG dependency - Discuss under UE RF, RRM and demod agenda separately Figure 3: Workflow on RAN4 device type related and relationship with RAN1/RAN2 and RAN-P Observation 5: Regarding minimum CHBW, two aspects need to be considered in RAN4 separately - “Minimum available spectrum from 6GR deployment” – Minimum CHBW per numerology basis (RAN4 scope) - Lowest device capability that 6GR design can be applied with smallest maximum CHBW (RAN1/RAN4 joint effort) Figure 4: Maximum BW on different device types Observation 6: RAN task RAN1 and RAN4 to continue providing more analysis for Smallest maximum CHBW for low tier device among options between 5MHz and 20MHz. Observation 7: Smallest maximum CHBW for low tier device shall be larger than minimum system bandwidth for FR1 including both FDD bands (15kHz SCS/5MHz), and TDD bands (30kHz SCS/10MHz) to avoid market fragmentation on lower -tier device type. Use case and NW scheduling perspective Observation 8: Relative larger CHBW for low tier UE can be helpful to minimize number of device types and provide scheduling flexibility. Cost/complexity perspective Observation 9: The further reduction of complexity with reduced BW from 20MHz to 5MHz is limited (taking the state of art technology in commercial device, the actual value is less than TR 38.865 calculation). - Compared to Rel-15 baseline 100MHz BW, 20MHz BW can significantly reduce complexity by 40% ~ 60% - Compared to 20MHz BW, further reduced BW to 5MHz, the additional complexity reduction is limited, 5% ~8% Figure 5: Average UE complexity with different RF/BB BW Peak data rate/Coverage perspective Observation 8: 20MHz CHBW can bring benefits to extend coverage and/or increased data rate compared to 5MHz CHBW. Proposal4: On smallest maximum CHBW for lower device type in 6GR, prefer to take 20MHz BW including RF and BB as mandatory BW in FR1.

- Proposals from MediaTek R4-2600501

Observation 1: Economies of scale and broad applicability both in terms of use cases and deployments/spectrum are key for the success of 6G low-tier devices, as a trade-off with BOM reduction. Proposal 1: Economies of scale and ease of network integration to be considered as part of any analysis of the benefits of smallest maximum BW for low-tier devices. Proposal 2: The UE should support all specified CBWs up to the smallest maximum CBW defined for a given SCS. Observation 2: BW reduction from 20MHz to 5MHz has small benefit on overall device complexity. Proposal 3: HD-FDD should be supported by the lowest tier 6GR device for FDD bands. Observation 3: There are different approaches to allowing SAW-less UE Tx operation that do not require constraining the maximum supported CBW of the UE. Proposal 4: Considering 20MHz CBW as the smallest maximum supported CBW for UL and DL for 15kHz and 30kHz SCS, study further the impact of partial or fully SAW-less designs in UE Tx side, with analysis on the following mitigation approaches: - Relaxation of the spurious emission requirement for UE cross-band coexistence. - Restricting the operated UL bandwidth by the UE or UL RB allocation in certain strict coexistence scenarios when the UE is located close to the band edge. - A-MPR for device in certain frequency locations that operates a wider CBW/RB allocation. Proposal 5: RAN4 to consider further Rx blocking and associated impacts to enabling fully or partially SAW-less Rx designs for the lowest tier 6GR device.

- Proposals from Apple R4-2600578

Proposal 1: It is proposed to adopt 20MHz as the smallest maximum UE CBW which applies to both RF and BB, i.e., both RF and BB BW are 20MHz. Proposal 2: Unless there are significant cost/complexity saving, it is proposed to have the same UL and DL bandwidth to support broad use cases.

- Proposals from AT&T R4-2600631

Proposal: RAN4 is requested to study the proposed approaches that enable SAW-less RF implementations for massive IoT devices in the 6G era

- Proposals from vivo R4-2600674

Observation 1: 20MHz UE RF&BB BW is already small enough for substantial complexity reduction and device cost saving. Observation 2: 20MHz UE RF&BB BW facilitates common signals/channels shared by different device types without degrading eMBB performance. Observation 3: 20 MHz UE RF & BB BW is already sufficiently narrow to ensure low UE peak power consumption, and will yield an overall lower energy consumption for both the UE and gNB owing to shorter transmission durations Proposal 1: Take 20MHz as maximum channel bandwidth for IoT device supported in 6GR for both FDD and TDD operation at least for FR1 Proposal 2: RAN4 can further study whether and how to support SAW-less device based on 20MHz maximum channel bandwidth

- Proposals from LGE R4-2600700

Proposal 1: Consider RF/BB implementation feasibility and constraints in Table 2-2 for IoT, Wearable, Smartphone, and FWA. Table 2-2 : RF/BB implementation feasibility and constraints by Device Type Feature; IoT (Narrowband); Wearable (Smartwatch); Smartphone (High-end); FWA. Size/Form Factor; Very Small; Small; Medium; Large. Use Case; Low data rate, long battery life, wide coverage; Medium data rate, calls, notifications, health monitoring; High data rate, mobility, multi-media; Very high data rate, stable connection, replace wireline broadband. Frequency Range; U1GHz; U6GHz; U6GHz, Around 7GHz, Around 15GHz. Antennas (TX/RX); 1T1R, 1T2R for diversity; 2T2R; 4T4R (U6GHz, Around 7GHz), 8T8R (Around 15GHz). CBW; Narrow (200kHz); Up to 20 MHz; Up to 90 MHz (U6GHz(FDD)), Up to 200 MHz (U6GHz(TDD), Around 7GHz), Up to 400 MHz (Around 15GHz). Power Class (PC); PC3; PC3/PC2; PC3/PC2/PC1.5/PC1. Modulation (DL); Up to 64QAM; Up to 256QAM; Up to [1024]QAM; Up to [4096]QAM. Modulation (UL); Up to 64QAM; Up to 64QAM; Up to [1024]QAM; Up to [1024]QAM. Proposal 2: Consider baseline and maximum values per device type in Table2-3 as starting point. Table 2-3 : Baseline and Maximum Values per Device Type Baseline(Mandatory) / Maximum(Optional). IoT (Narrowband); Wearable (Smartwatch); Smartphone (High-end); FWA. CBW; 200kHz / 200kHz; 20MHz / 20MHz; ~100MHz / 200MHz; ~100MHz / 200MHz. Modulation; BPSK,QPSK / 64QAM; 64QAM / 256QAM; 64QAM / 1024QAM; 64QAM / 1024QAM. Antenna; 1T1R / 1T2R; 1T1R / 2T2R; 2T4R / 4T4R; 4T4R /8T8R. Power Class; PC3 / PC3; PC3 / PC2; PC3 or PC2, / PC1.5; PC2 / PC1.

- Proposals from CMCC R4-2600812

Proposal 1: for device type, it’s suggested to take above aspects into consideration. Following list our preliminary suggestion for different characteristics of device types Max UE bandwidth; Tx/Rx antenna number; Duplex mode; Max modulation order; coverage; [power class]. [FWA]; 400M; [16T, 16R]; TDD FDD; [1024] QAM; normal; PC1. eMBB; [400M]; 4T, 8R; TDD FDD; [256QAM]; normal; PC1.5. [low end eMBB]; 100M; 4T, 8R; TDD FDD; 256QAM; normal; PC1.5. High end IoT; 20M; 1T, 1/2R; HD-FDD TDD FDD; 64QAM; normal; TBD. Lowest tier IoT; 5M; 1T,1R; HD-FDD TDD; 16QAM; Normal+10dB; [20dBm]. Proposal 2: it’s suggested to further discuss device types with above table as example.

- Proposals from HW R4-2600888

Observation 1: 5MHz gets little complexity reduction and almost no cost reduction, on the price of large coverage performance gap. Observation 2: The feasibility SAW-less architecture requires further analysis with regards to restriction of applicable scenario, cost of low-pass filter and risk of co-existence spurious emission. Observation 3: The potential solution for SAW-less architecture to meet the co-existence spurious emission could apply to both 20MHz and 5MHz, therefore there is no need to restrict the smallest maximum channel bandwidth to 5MHz. Proposal 1: The smallest maximum channel bandwidth is selected as 20MHz.

- Proposals from Spreadtrum R4-2601055

Observation: Based on the RAN#110 agreement, the detailed discussion of device type is on hold and to resume from RAN#113. Proposal 1: Support option4. The value of smallest maximum UE BW can be 20MHz for 6G low-tier devices. Proposal 2: Keep the same bandwidth for UL and DL in 6GR for smallest maximum UE BW. Proposal 3: Keep the same bandwidth value for different SCS, duplex modes and bands smallest maximum UE BW. Proposal 4: Keep the same bandwidth for RF and BB smallest maximum UE BW.

- Proposals from Samsung R4-2601125

Observation 1: 5 MHz would be good enough for 7 Mbps ~ 10 Mbps as the peak data rate for low-tier UEs. Observation 2: Restricting the RF bandwidth to smaller than 20MHz may lose some flexibility while increasing complexity. Proposal 1: For 6GR, the smallest maximum UE-supported RF BW is 20MHz and BB BW is 5MHz.

- Proposals from ZTE R4-2601179

Observation: Compared to 10MHz, 5MHz maximum bandwidth can provide about 5% reduction in cost/complexity and 9% reduction in power consumption although 5MHz may have some impacts on SIB design Proposal: 5MHz with 15kHz SCS as the smallest maximum UE bandwidth should be considered as the baseline for 6GR system design, which is scalable along with different SCS.

- Proposals from Sony R4-2601399

Observation 1: the required peak data rate for low tier devices in 6GR is 10Mbps in both UL and DL. Observation 2: The achievable data rate with 5MHz, 10 MHz and 20 Mhz BW can all fulfill the required peak data rate for the low tier devices from RAN, and it’s not the major consideration when determining the maximum bandwidth for the low tier device. Observation 3: the low tier device, e.g., massive IoT device, is very sensitive to the implementation complexity and power consumption. Observation 4: a wider bandwidth requires more advanced BB processing capability, larger memory footprint in the digital domain, while also requiring more complicated design in RF including the implementation of filter/duplexer, switch and power amplifier. Observation 5: with 5MHz transmission bandwidth, in most cases, only IMD9 or the tail part of IMD7 would fall into the protected band which is usually below the spurious emission limit without any band specific SAW/BAW filter. Observation 6: with 10MHz or even larger transmission bandwidth, in many cases, IMD5 or even IMD 3 production would fall into the protected band, and emission suppression via band specific SAW/BAW filters would be necessary. Observation 7: A SAW-less implementation is possible for most bands with 5MHz BW for low tier devices with HD-FDD. Observation 8: a HD-FDD device with SAW-filters would also require additional switches and higher output power PA and more complicated RFIC design, while the device complexity and module size will be increased as well. Observation 9: regional stock-keeping units (SKUs) can partially address the module size and complexity, but these would jeopardize the economies of scale of the device and complicates the design to support new bands for future. Observation 10: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, along with - improved power efficiency. - significantly reduced module size. - with lower overall BoM associated with the module. - with greater longevity of the module. Observation 11: from RF perspective, wider bandwidth than 5MHz can be considered but additional attention should be paid to the baseband implementation. Observation 12: a higher data rate requires a wider bandwidth ADC/DAC, stronger baseband processing capability and larger memory footprint size. Observation 13: with 5MHz uplink BW, it is possible to be SAW-less in most of frequency bands, but there are still a handful number of band combinations are challenging due to the closely allocation of the spectrum between aggressor band and the protected band. Proposal 1: The analysis and choice on the maximum bandwidth for the low tier device shall take into account the RF and BB complexity as the most important aspects, including the SAW-less aspect. Proposal 2: 6GR shall aim to support low-tier IoT module implementation without band-specific analogue filters (e.g. without SAW filters) Proposal 3: the maximum bandwidth of low tier device is up to 5MHz at least in the uplink. Proposal 4: RAN4 further study additional approaches to enable a fully SAW-less implementation for a few extremely challenged scenario (e.g., band to band separation below 12MHz) with 5MHz uplink BW for the low tier device in 6GR. Proposal 5: If maximum DL bandwidth for low tier devices is larger than 5MHz, it is beneficial to limit the peak data rate to 10 Mbps to reduce the baseband complexity. Proposal 6: RAN4 further study the following additional solutions to enable a fully SAW-less implementation for the low tier device of 6GR: - Limit location of UL RBs - limited number/length of allocated UL RBs, - allowed power backoff for certain bands and/or certain UL RB allocations restrictions - possible relaxed band to band spurious emission level

- Proposals from OPPO R4-2601451

Proposal 1: Study the two options for maximum UE bandwidth for lowest-tier 6G IoT devices. Option 1: 5MHz. If taking this option, the wearable devices (i.e., 6G RedCap) can be treated as a separate device type (e.g., with maximum UE bandwidth of at least 20MHz). Option 2: 20MHz. If taking this option, the wearable device (i.e., 6G RedCap) is combined into the lowest-tier 6G IoT device type.

- Proposals from CATT R4-2600316

Proposal 1: RAN4 to progress studies related to device types by focusing on identifying distinct RF requirement and implementation feasibility implications, while remaining independent of and not constraining device type categorization at the RAN level. Proposal 2: RAN4 to consider a scalable RF requirement framework, in which RF requirements are parameterized along key technical dimensions, and to further consider realistic baseline and maximum values for these parameters to represent widely deployable and energy-efficient implementations, as well as optional high-end capabilities respectively.

- Proposals from Ericsson R4-2601880

Observations: Observation-1: From cost perspective, RF bandwidth from option 2 of 5MHz to option 4 20MHz is not significantly a deciding factor according to TR38.865, without considering the SAW-less design[5]. Proposals: Proposal-1: Study the RF impact of SAW-less design with the smallest maximum bandwidth with below two bandwidth options: - Limit the maximum configurable UL bandwidth, with same RF and BB bandwidth - Limit the maximum contiguous RB allocation (BB bandwidth) within a wider configurable UL bandwidth (RF bandwidth), with different RF and BB bandwidth Proposal-2: Revisit the coexisting requirement in 6G if necessary. Proposal-3: Limiting BB bandwidth may bring network benefit more than limiting RF bandwidth, other aspect could be discussed other than RF impact for SAW-less design, e.g. LO retuning time.