3GPP TSG-RAN4 Meeting #118	R4-26xxxxx

# Gothenburg, Sweden, 9-13 February, 2026

Agenda Item:	8.1

Source: 	Feature lead (Xiaomi)

Title: 						WF for [118] [102-B] 6G general RF and UE RF (part II)

Document for:	Approval

- Topic #1: Frequency range between FR1 and FR2-1

    - Sub Topic 1-1: Antenna architecture assumption (Conductive vs Radiated RF characteritics)

- Online Agreement:

  - On around 7GHz, “FR1 like” conductive requirements shall be applied with conductive test

- Recommended WF from FL for agreement

  - On around 10GHz and 15GHz, further evaluate feasible antenna architecture considering following aspects

    - Antenna performance on coverage, antenna size limitation with  associated device type/form factor assumption, feasibility with antenna connector

    - Other aspects not precluded

  - Tight cooperation with 6G spectrum thread on 6G new spectrum regulation update

    - Sub Topic 1-2: NF assumption and implementation margin

- Recommended WF from FL for agreement

  - On around 7GHz, taking TR 38.822 conclusion as starting point with NF as 13 dB

  - On around 10GHz, 15GHz, FFS on NF level pending on further study

  - Further discuss implementation margin

- Topic #2: Spectrum aggregation

- Ad-hoc Agreement:

  - On necessity of supporting intra-band NC CA and intra-band CCA with non-contiguous RB allocation on UL part

    - Further collect the demand and operating scenarios from operator and infra-vendor

    - Study possible way to optimize requirements for MPR reduction e.g., restriction of single carrier scheduled/configured/activated via switching for intra-band NC CA

- Updated FL recommendation for Bandwidth class for agreement

  - On the needs of BW class narrower than or equal to widest single carrier BW

    - Further study the feasibility of only introducing BW class larger than widest single carrier BW at least considering following aspects

      - The impact on UE with different maximum BW capabilities

      - Operators’ spectrum which is less than widest single carrier BW and not aligned with RAN4 specified CHBW sets

    - Sub-topic 2-3 Tx/Rx swicthing

- Ad-hoc agreement:

  - On target scenarios

    - At least considering following existing scenarios from NR for initial study purpose

      - UL Tx Swicthing including SUL, inter-band CA for up to 4 bands with 1T-1T, 1T-2T, 2T-2T, 1T/2T-2T/1T

      - LB CA via swicthing

      - Note: FFS which scenarios shall be supported in 6G day1

    - FFS whether considering addtional scenarios including

      - Adpative BWP swicthing

      - UE-specific CBW change

      - DL-UL swicthing in same carrier on TDD band

      - SRS Tx swicthing

      - Addtional new scenarios which not identied in NR not precluded

  - On swicthing and interruption time

    - Further discuss the benefits and implmentation constraints of shortening swicthing time/intteruption time on identified scenarios

    - Consider interruptions, UL-DL switching, carrier switching and transient times separately

    - For interruptions, focus on need for interruptions and RF re-tuning time impact on interruption duration

    - Final total interruption duration including potential different granularity for interruption durations is not discussed in UE RF

    - Tight cooperation with other WGs if cross WG impact identified

  - Strive to align the RF switching/interruption time assumption on relevant RRM requirements per request from 6G RRM thread

  - Strive to unify Tx/Rx switching framework on idenfied sceanrios

    - Tight cooperation with other WGs (RAN1, RAN2)

    - Sub-topic 2-3 Simultaneous Tx-Rx operations between different operating bands

- Ad-hoc agreement

  - Study which band combinations should be specified for concurrent Tx/Rx operations in the specifications in consistent manner

    - Strive to identify the technical criteria on identify band combinations which can support concurrent operations as default and/or which band combinations can not support concurrent operations

      - It’s not precluded to discuss other band combinations in case-by-case manner on supporting concurrent operations per operators’ request

      - It’s encouraged operators can bring the demand of band combinations with co-current operations

    - Strive to identify cause for requirement complexity in 5G and how to alleviate the situation for 6G

    - Tight cooperation with 6G spectrum thread on potential 6G band combinations

    - Sub-topic 2-4 Flexcible Spectrum aggregation

- Ad-hoc agreement:

  - On fragmented spectrum allocation

    - Collect the demand on fragmented spectrum allocation scenarios especially from operators

  - Tight cooperation with other WGs to identify relevant RAN4 work scope on advanced spectrum aggregation schemes e.g., single cell multi-carrier (Elastic cell with multiple carriers), DL/UL decoupling/paring

    - Postpone the discussion until sufficient progress from other WGs with the clarity on the concept is clear or triggered by incomming LS

- Topic #3: Joint BS and UE RF

    - Sub-topic 3-1 Tx EVM relaxtion with DPoD

- Ad-hoc agreement

  - On performance evaluation metric: Net gain adopted as target metric

    - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER)

  - On Tx EVM non-linearity modelling/budget

    - For initial evaluation purpose: Take existing NR UL 64QAM, 256QAM Tx EVM budget as starting point with PA non-linearity, phase noise, IQ imbalance and transmitter as major contributor

  - Encourage further input on following issues

    - Additional aspects which have impact on Tx EVM e.g., DPD, CFR and other aspects from Tx DFE, Tx AFE and whether/how to take them into consideration for simulation for 5G and 6G study

    - DoPD performance robustness and [testability] considering UE implementation diversity on Tx nonlinearity especially PA non-linearity

- Recommendation from FL for agreement

  - On PA model

    - For initial evaluation purpose: Companies can use their own PA model with aligned calibration points from legacy MPR simulation

      - Using multiple PA models not precluded considering UE implementation and power class diversity

      - When provide evaluation results, companies shall clarify the PA model used for their evaluation

    - Tight cooperation with 6G system parameter on suitable PA model for 6G consideration

  - Further discuss following detailed Tx EVM non-linearity modelling

| Factors | Model |
| --- | --- |
| For phase noise | Option 1: Phase only |
|  | Option 2: Model as Gaussian white noise |
| For PA non-linearity | GMP model (refer to TR 38.803) |
| For IQ imbalance | Both Phase and magnitude unified distribution |
| Transmitter | Model as Gaussian white noise |

- Other aspects not list above table for Tx non-linearity modelling are not precluded

    - Sub-topic 3-2 Tx/Rx EVM split assumption

- Recommended WF from FL for agreement

  - Further discus Tx/Rx EVM split between BS and UE

- Topic #4: Others

- Recommended WF from FL for agreement

- Encourage input on following areas and strive to identify the study scope in Q2

- Energy efficiency

- UL coverage

- Testability

- TN and NTN integration

- FR2 scope

Reference

[1] R4-2602142, Topic Summary for [118] [102-B] 6G general RF and UE RF (part II), Feature lead (Xiaomi)

[2] R4-xxxx, Ad-hoc