3GPP TSG-RAN4 Meeting #118	R4-260xxxx

Gothenburg, 9-13 February, 2026

Title: 	WF on 6G BS RF and co-existence

Agenda item:	8.1

Source:	Feature Lead (Ericsson)

Document for:	Approval

# Agreements and Way Forward

## BS RF requirements

### Issue 2-1-1: Coexistence study for ~7 GHz

Agreement:

- In the 6G SI scope, RAN4 will do coexistence study for ~7 GHz:

  - List of parameters that might be changed:

    - PC1 or PC2/PC1.5 UEs

    - vertical steering range

    - UL power control

    - BS antenna parameters

    - SNR range

    - Channel BW

    - UE antenna pattern

    - Others are not precluded

  - Scenario: Urban macro (prioritized scenario)

    - Other scenarios are not precluded

  - Assumptions and methodology updates: to be finalized by RAN4#119 meeting

  - Outcomes shall be consistent with the outcomes from the former studies for PC3 UE

## NTN aspects

### Issue 3-1-1: NTN coexistence studies – priority settings

Agreement:

At least MEO will be studied. The example bands can include, but not limited to, Ku Ka and/or L-band.

Agreement:

The following priorities have been set for the NTN coexistence studies:

- Priority #1: NTN-NTN coexistence – S-band (agreed in RAN4#117)

  - The DL-UL scenarios are put on hold, waiting for RAN clarification on the same topic.

  - The DL-DL scenarios will be studied from next RAN4#118bis meeting.

- Priority #2: satellite orbit MEO for L-/Ku-/Ka-band

- Priority #3: Q/V band

- Priority #4: C-band

- Priority #5: vLEO

Way Forward:

Study on NTN and TN coverage overlap – adjacent channel – ~2 GHz :

- Proponents of this study are encouraged to:

  - Further detailed which aspects and which parameters they would like to further study.

  - Early evidence (e.g. preliminary results) of the expected gains that would motivate studying again this scenario.

## AAS BS below 1 GHz

### Issue 4-1-2: Antenna model

Agreement:

For an AAS BS operating within 694 to 960 MHz, the AAS BS array antenna model described in TR 38.922 shall be used.

### Issue 4-1-3: Element parameters

Agreement:

  - Front-to-back ratio:  Am = 30dB

  - Side-lobe suppression: SLAv = 30 dB

  - Horizontal half power beamwidth: j3dB = 90 deg

  - Vertical half power beamwidth: q3dB = 65 deg

### Issue 4-1-4: Element separation parameters

Agreement:

  - Vertical element separation in sub-array: dv,sub=0.7l

  - Horizontal sub-array separation: dh=0.5l

  - Vertical sub-array separation: dv=Msub * dv,sub

### Issue 4-1-4: Element separation parameters

Way forward:

2 options to be further discussed in next RAN4#118bis meeting:

  - Option 1: Msub=4

  - Option 2: Msub=5

### Issue 4-1-6: Array parameter values

Agreement:

  - Number of sub-array rows in array: M = 2

  - Number of sub-array columns in array N=4