3GPP TSG-RAN WG4 Meeting # 118	R4-260xxxx

# Gothenburg, Sweden, 09th - 13th, Feb 2026

Agenda item:			8.1

Source:	Feature Lead (China Telecom)

Title:	Draft WF for [118][104-B] 6G spectrum (part II)

Document for:	Approval

Introduction

Back ground: Agreement in WF of R4-2522463 in RAN4 #117

- RAN4 to study the Band Group Concept to structure the RF requirement improvement - Concept of band group includes RF requirements simplifications and FFS on whether include RF architecture assumptions. - Note that the Band Group Concept is not a replacement to normal CA. - No restriction on operators’ request on band combination

Topic #1: Band group concept study

Open issues summary

Sub-topic 1-1 Scopes for band group study

Issue 1-1-1: Scopes for band group study in 6G

Agreements:

RAN4 to study the Band Group Concept to structure the RF requirement improvement, which includes RF requirements simplifications.

- Study the benefits and feasibility for below options for band group definition.

  - Option 1: requirements structure is based on operators’ requests

  - Option 2: requirements structure is based on region/country’s spectrum allocation.

Example for Option 1 and Option 2 band group definition

| Band groups | Supported bands | CA with in band group | Descriptions (Reason for supported bands in one band group) |
| --- | --- | --- | --- |
| [BG1] | n5, n8 | CA_n5-n8 | Adjacent bands |
| [BG2] | n1, n3, | CA_n1-n3 | Adjacent bands |
| [BG3] | n78 |  |  |
| Note: CAs across band groups are composed with one band/CA per band group |  |  |  |

  - Option 3: requirements structure is based on operating frequency range considering companies’ input.

Example for Option 3 band group definition

| Band group | Operating frequency | Supported bands |
| --- | --- | --- |
| [BG1] | TBD | TBD |
| … | … | … |

  - Option 4: Other options are not precluded.

Note: Define the boundary of the band group by considering the similar RF characteristic.

- Study the benefits and feasibility for impacted requirements such as delta TIB, delta RIB and MSD etc.

- Study the benefits and feasibility for band group related UE capabilities.

- FFS on  band switching operation mode for inter-band combination within the band group which is based on UE capability on top of the current CA operation.

Oppo: what’s the purpose of band group?

ZTE: What is the target of the band group? To simply the RF requirement could be based on traditional BC.

Samsung: band group is to replace CA operating band whose requirement could be simplified, interested operator could share view for which approach. Without group, the requirement could be also simplified. Capability is RAN2 scope should be removed. Requirement structure is confused.

QC: it should be only based on frequency range. Option1 and 2 are too open, we should focus on option3.

Murata: support based on frequency range. Which thread/agenda is for TIB, RIB and MSD requirements?

Huawei: for frequency range which could share the same RF requirement. Streamline the requirement for certain frequency range. How capture into spec is open. For all the CA with in band group, switch could be supported on top of current CA. UE capability is based on RAN1 and 4 input. If have RAN1 impact, RAN1 could be involved.

Skyworks: share the same antenna could define the same band group

CATT: we should do step by step, first band group concept, study how to consolidate bands to band group, then the impacts to define band group requirements.

LGE: frequency range based band group is the main rule. Switching is just optional.

Rogers：we support the frequency range based on band group definition

CHTTL：What’s the meaning of the requirements enhancement?

CMCC: support the study for band group. It could simplify the BC. For the bands have similar RF requirement we can do simplify work.

MTK: for RF similarity to do the band group work.

APPLE: We agree that band group concept is not new from LTE. To do CA switching, we should do based on specific CA rather than band group.

T-Mobile: separate the two issues of band group and switching. We have concern to use band group to limit some function.

ATT: Support rogers and T-mobile.

Issue 1-1-2: Requirements structure for band group

Proposals

- Option 1:

| Band group | Operating frequency | Supported bands |
| --- | --- | --- |
| FR1-BG1 | <1GHz | Band 8, 20,28 |
| FR1-BG2 | 1GHz ~ 2.3GHz | Band 1, 3, 39 |
| FR1-BG3 | 2.3 GHz ~3GHz | Band 7, 40, 41 |
| FR1-BG4 | 3GHz ~5GHz | Band 77, 79 |

- Option 2:

| Frequency group numbering | Frequency range | Definition | SCS (kHz) |
| --- | --- | --- | --- |
| 1 | Around 400MHz ~ 1695MHz | Low band | 15 |
| 2 | 1695MHz ~ 2690MHz | Mid band | 15 |
| 3 | 2690MHz ~ 3300MHz | High band | 30 |
| … | … | … | … |

- Option 3:

| Band Group | Frequency Range | Representative NR Bands (Example) | Description |
| --- | --- | --- | --- |
| BG-L | sub-1 GHz | n5, n8, n12, n20, n28 | Coverage-oriented, strong harmonic impact |
| BG-M1 | 1.4 – 2.3 GHz | n1, n3, n66 | Low/Mid boundary |
| BG-M2 | 2.3 – 3.0 GHz | n7, n38, n40, n41 | Upper mid-band |
| … | … | … | … |

- Option 4:

| Frequency ranges | Band group No |
| --- | --- |
| [410MHz]-1GHz | 0 |
| 1.3GHz-1.695GHz | 1 |
| 1.695GHz-2.4GHz | 2 |
| … | … |

- Option 5:

| Band group | Frequency range | Corresponding bands |
| --- | --- | --- |
| BG1 | <1 GHz | n5, n8, n20, n26, n28, n105,… |
| BG2 | 1.4 GHz – 2.2 GHz | n1, n2, n3, n34, n39, n66,… |
| BG3 | 2.3 GHz – 2.7 GHz | n7, n38, n40, n41,… |
| BG4 | 3.3 GHz – 5 GHz | n48, n77, n78, n79 |
| BG5 | 5 GHz – 7 GHz | n96, n102, n104 |
| BG6 | 7 GHz – 15 GHz | TBD |
| BG7 | 24.25 GHz – 52.6 GHz | n257, n258, n259, n260, n261, n262 |

Recommended WF

- For the reference of issue 1-1-1 discussion

Issue 1-1-3: Operation mode for inter-band combination

Proposals

- Option 1: Only legacy CA operation for larger aggregated channel bandwidth

- Option 2: Both Band switching for higher transmission power with reduced insertion loss and legacy CA operation for larger aggregated channel bandwidth based on UE capability

Recommended WF

- TBA

Issue 1-1-4: The applicable inter-band combinations for band switching operation mode

Proposals

- Option 1: Per specific inter-band combination

- Option 2: Inter-band combinations within certain frequency range

- Option 3: All inter-band combinations to be defined

Recommended WF

- TBA

Topic #2: Spectrum related regulatory survey

Open issues summary

Sub-topic 2-1 Potential 6G operating bands survey

Many companies provided their observations and servey at different regions on potential 6G bands. Most views think we should discuss bands around 7GHz at first and keep following guidance from WRC in 6G.

Issue 2-1-1: Candidate 6G operating bands survey

Proposals

- Proposal 1: RAN4 can consider the candidate 6G operating bands in these refarming FR1 bands from 5G NR and 6G new spectrums (7.125~15.35GHz) as following priority

  - 1st Priority is around 7GHz and these 5G NR refarming bands in FR1.

  - 2nd priority is around 15GHz.

Recommended WF

- Encourage companies to give input to the TR38.760-4 based on above proposal