3GPP TSG-RAN WG4 Meeting # 118	R4-260xxxx

# Gothenburg, Sweden, 09th - 13th, Feb 2026

Agenda item:			8.1

Source:	Feature Lead (China Telecom)

Title:	Draft WF for [118][104-B] 6G spectrum (part II)

Document for:	Approval

Introduction

Back ground: Agreement in WF of R4-2522463 in RAN4 #117

- RAN4 to study the Band Group Concept to structure the RF requirement improvement - Concept of band group includes RF requirements simplifications and FFS on whether include RF architecture assumptions. - Note that the Band Group Concept is not a replacement to normal CA. - No restriction on operators’ request on band combination

Topic #1: Band group concept study

Open issues summary

Sub-topic 1-1 Scopes for band group study

Issue 1-1-1: Scopes for band group study in 6G

Proposals:

RAN4 to study the Band Group Concept to structure the RF requirement improvement, which includes RF requirements simplifications and FFS on whether include RF architecture assumptions.

- Study the benefits and feasibility for below options for band group definition.

  - Option 1: requirements structure is based on operators’ requests

  - Option 2: requirements structure is based on region/country’s spectrum allocation.

| Band groups of category 1 | Supported bands | CA with in band group |
| --- | --- | --- |
| [BG1] | n5, n8 | CA_n5-n8 |
| [BG2] | n1, n3, | CA_n1-n3 |
| [BG3] | n78 |  |
| Note: CAs across band groups are composed with one band/CA per band group |  |  |

  - Option 3: requirements structure is based on operating frequency range considering companies’ input.

| Band group | Operating frequency | Supported bands |
| --- | --- | --- |
| [BG1] | TBD | TBD |
| … | … | … |

- Study the benefits and feasibility for impacted requirements such as delta TIB, delta RIB and MSD etc.

- Study the benefits and feasibility for band group related UE capabilities.

Recommended WF

- Recommend to discuss in ad-hoc

Issue 1-1-2: Requirements structure for band group

Proposals

- Option 1:

| Band group | Operating frequency | Supported bands |
| --- | --- | --- |
| FR1-BG1 | <1GHz | Band 8, 20,28 |
| FR1-BG2 | 1GHz ~ 2.3GHz | Band 1, 3, 39 |
| FR1-BG3 | 2.3 GHz ~3GHz | Band 7, 40, 41 |
| FR1-BG4 | 3GHz ~5GHz | Band 77, 79 |

- Option 2:

| Frequency group numbering | Frequency range | Definition | SCS (kHz) |
| --- | --- | --- | --- |
| 1 | Around 400MHz ~ 1695MHz | Low band | 15 |
| 2 | 1695MHz ~ 2690MHz | Mid band | 15 |
| 3 | 2690MHz ~ 3300MHz | High band | 30 |
| … | … | … | … |

- Option 3:

| Band Group | Frequency Range | Representative NR Bands (Example) | Description |
| --- | --- | --- | --- |
| BG-L | sub-1 GHz | n5, n8, n12, n20, n28 | Coverage-oriented, strong harmonic impact |
| BG-M1 | 1.4 – 2.3 GHz | n1, n3, n66 | Low/Mid boundary |
| BG-M2 | 2.3 – 3.0 GHz | n7, n38, n40, n41 | Upper mid-band |
| … | … | … | … |

- Option 4:

| Frequency ranges | Band group No |
| --- | --- |
| [410MHz]-1GHz | 0 |
| 1.3GHz-1.695GHz | 1 |
| 1.695GHz-2.4GHz | 2 |
| … | … |

Recommended WF

- For the reference of issue 1-1-1 discussion

Topic #2: Spectrum related regulatory survey

Open issues summary

Sub-topic 2-1 Potential 6G operating bands survey

Many companies provided their observations and servey at different regions on potential 6G bands. Most views think we should discuss bands around 7GHz at first and keep following guidance from WRC in 6G.

Issue 2-1-1: Candidate 6G operating bands survey

Proposals

- Proposal 1: RAN4 can consider the candidate 6G operating bands in these refarming FR1 bands from 5G NR and 6G new spectrums (7.125~15.35GHz) as following priority

  - 1st Priority is around 7GHz and these 5G NR refarming bands in FR1.

  - 2nd priority is around 15GHz.

Recommended WF

- Encourage companies to give input to the TR38.760-4 based on above proposal