3GPP TSG RAN WG4 Meeting #118		R4-2602311

Gothenburg, Sweden, 09 - 13, Feb 2026

Title:	LS on RAN4 down-selected AI/ML use cases

Response to:	-

Release:	Rel-20

Work Item:	FS_6G_Radio

Source:	RAN WG4

To:	RAN

Cc:	RAN WG1, RAN WG2

Contact person:	He (Jackson) Wang

h0809.wang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 had collected observations on RAN4-related AI/ML use cases reported by proponent companies, and RAN4 has studied the prioritization of relevant use cases during Rel-20 6G study item.

From RAN4 perspective, the following AI/ML use cases are discussed as potential RAN4-led AI/ML use cases

  - Use Case 1: AI-based non-linearity compensation (AI-NC)

    - AI-NC Sub-Case 1: AI-based DPoD (Digital Post-Distortion) at NW

    - AI-NC Sub-Case 2: AI-based DPD (Digital Pre-Distortion) at UE

  - Use Case 2: AI-based SRS power imbalance compensation

  - Use Case 3 AI-RRM use case, including the following 5 sub-cases:

    - AI-RRM Sub-Case 1: FR2-1 L3 spatial domain beam-level prediction for Tx (intra-cell)

    - AI-RRM Sub-Case 2: FR1 L3 frequency domain (within the same FR) cell-level prediction (inter-cell, non-collocated)

    - AI-RRM Sub-Case 3: Spatial domain Rx prediction for intra-cell

    - AI-RRM Sub-Case 4: AI/ML-based prioritization for MOs

    - AI-RRM Sub-Case 5: Non-collocated frequency domain prediction in beam level

Among these use cases, RAN4 concluded the following prioritization by considering the RAN4 meeting arrangement and limited bandwidth in RAN4.

  - RAN4 has agreed to introduce AI-NC Sub-Case 1 (AI-based DPoD at NW) as 6G RAN4-led AI use case to study. Once the AI-based DPoD study is completed, AI-NC Sub-Case 2 (AI-based DPD at UE) will be studied in Rel-20 by taking the performance KPI of AI-based DPoD at NW as the benchmark to identify the potential gain from the aspects of performance, complexity and power efficiency.

  - RAN4 has agreed to introduce the Use Case-2 (AI-based SRS power imbalance compensation) as 6G RAN4-led AI use case.

  - For AI-RRM including 5 sub-use cases, RAN4 has agreed to begin the study on AI-RRM Sub-Case 1 and 2. Upon the completion of these two sub-cases, RAN4 will do the following:

    - Upon the completion of Sub-Case 1 and 2 study, it can be further discussed if and how to generalize the conclusion of Sub-Case 1 and 2 study to other FR.

    - Sub-Case 3 will be started.

    - Decide whether to study Sub-Case 4 and 5.

    - Note: It may require cross-WG study on channel model for AI-RRM use cases involving non-collocated prediction

Figure. Illustrative prioritization for RAN4 down-selected AI/ML use cases

Detailed corresponding use case descriptions, observations and other related information can be found in the corresponding tables in Appendix.

# 2	Actions

To RAN

ACTION: 	RAN4 respectfully asks RAN to take the above conclusion into consideration.

# 3	Dates of next TSG RAN WG4 meetings

RAN4#118bis	13th – 17th April 2026	Malta, MT

RAN4#119	18th – 22nd May 2026	Dalian, China

# 4	Appendix

## 4.1 AI-NC Sub-Case 1: AI-DPoD at NW

Table 4.1. AI-NC Sub-Case 1: AI-DPoD at NW

| RAN4-identified Use case |  | AI-NC Sub-Case 1: AI-DPoD at NW |
| --- | --- | --- |
| Use case description | AI model input | The received signal after channel estimation and equalization |
|  | AI model output | The compensated signal |
|  | Label | Known signal e.g., Data/DMRS |
|  | Training type | - Online training - Offline training with or without finetuning - Hybrid training of online and offline |
|  | Model location for inference | NW-sided model |
|  | Collaboration/interaction between UE and NW | For inference: UE capability and NW indication to UE to adjust PA back-off and other network-controlled behavior are not precluded. For training: data collection procedure with UE operation in compressed/non-linear PA region and associated RBs |
| Observations on benefits and/or gain |  | With the same TX EVM level, receiver performance improvement With relaxed TX EVM level, PA power backoff is reduced |
| Observations on complexity |  | Some companies provided evaluation results based on the following complexity: - Lightweight NN (~3M FLOPs) - ~100M FLOPs Other options are not precluded. Implementation complexity other than AI/ML model complexity will be further studied. |
| Standardization effort required |  | Link level simulation, RF evaluation for MPR, FFS the necessity to conduct system level simulation or field test data to demonstrate the performance gain |
| Potential RAN4 spec impact |  | RAN4 requirements, e.g, EVM, MPR, and BS demodulation performance |
| Impacted other working groups |  | Potentially including: RAN1: DMRS/Sequence design/selection, Tx power determination RAN2: Signaling/ procedure related to LCM for NW-sided model Note: The eventual impact on RAN1 and RAN2, whether/what it is needed, depends on the study outcome. Key factors include whether online, offline or hybrid training is supported, what kind of label data is required, and whether the solution is transparent to the UE. |

## 4.2 AI-NC Sub-Case 2: AI-DPD at UE

Table 4.2. AI-NC Sub-Case 2: AI-DPD at UE

| RAN4-identified Use case |  | AI-NC Sub-Case 2: AI-DPD at UE |
| --- | --- | --- |
| Use case description | AI model input | - Time domain samples after PA distortion - Time domain samples before PA distortion |
|  | AI model output | The compensated signal |
|  | Label | Time domain samples w/o distortion |
|  | Training type | - Online training - Offline training with or without finetuning - Hybrid training of online and offline |
|  | Model location for inference | UE-sided model |
|  | Collaboration/interaction between UE and NW | Similar to UE-sided model as NR |
| Observations on benefits and/or gain |  | PA power backoff is reduced, UL budget improvement |
| Observations on complexity |  | Some companies provided evaluation results based on the following complexity: - FLOPs: 34.8 k, MACs: 17.4 k Other options are not precluded. Implementation complexity other than AI/ML model complexity will be further studied. |
| Standardization effort required |  | RF evaluation |
| Potential RAN4 spec impact |  | RAN4 UE RF requirements, e.g, MPR, and FFS other requirements are not precluded. |
| Impacted other working groups |  | Potentially including: RAN1: SRS and/or PUSCH enhancement/Tx power determination for data collection RAN2: Signaling/ procedure related to LCM for UE-sided model Note: The eventual impact on RAN1 and RAN2, whether/what it is needed, depends on the study outcome. Key factors include whether online, offline or hybrid training is supported, what kind of label data is required. |

## 4.3 Use Case 2: AI-based SRS power imbalance compensation

Table 4.3. Use Case 2: AI-based SRS power imbalance compensation

| RAN4-identified Use case |  | Use Case 2: AI-based SRS power imbalance compensation |
| --- | --- | --- |
| Use case description | AI model input | Measured CSI from SRS with power imbalance, by taking UE-side compensation into account |
|  | AI model output | CSI after compensation of SRS power imbalance |
|  | Label | CSI with high quality, candidate solutions could be: - CSI feedback based on CSI-RS by considering different noise levels - Reliable SRS measurements, e.g., with time filtering or with high SNR - SRS measurements with pre-compensation at UE |
|  | Training type | Offline training Online training/finetuning is not precluded |
|  | Model location for inference | NW-sided model |
|  | Collaboration/interaction between UE and NW | 1. Inference: UE assistance information/capability reporting related to SRS power imbalance for NW to decide whether and how to compensate, e.g., model selection 2. Data collection procedure for model training/monitoring |
| Observations on benefits and/or gain |  | With SRS power imbalance compensation, CSI with higher accuracy is obtained to potentially improve throughput |
| Observations on complexity |  | Some companies provided evaluation results based on the following complexity: - 73 M FLOPs Other options are not precluded. Implementation complexity other than AI/ML model complexity will be further studied. |
| Standardization effort required |  | Link-level simulation |
| Potential RAN4 spec impact |  | Potential UE RF related requirements |
| Impacted other working groups |  | Potential UE assistance information/capability reporting related to SRS power imbalance |

## 4.4 AI-RRM Sub-Case 1: FR2-1 L3 spatial domain beam-level prediction for Tx (intra-cell)

Table 4.4. AI-RRM Sub-Case 1: FR2-1 L3 spatial domain beam-level prediction for Tx (intra-cell)

| RAN4-identified Use case |  | AI-RRM Sub-Case 1: FR2-1 L3 spatial domain beam-level prediction for Tx (intra-cell) |
| --- | --- | --- |
| Use case description | AI model input | L3 beam-level measurement results |
|  | AI model output | L3 beam-level predication results |
|  | Label | Measurement results at certain target beam |
|  | Training type | offline training |
|  | Model location for inference | UE-sided model or NW-sided model |
|  | Collaboration/interaction between UE and NW | UE capability reporting, and other mechanisms similar as NR UE/NW-sided model, NW configuration with reduced RS resources |
| Observations on potential benefits and/or gain |  | Measurement reduction, and/or Measurement overhead reduction |
| Observations on complexity |  | To be studied |
| Standardization effort required |  | System-level simulation, and link-level simulation could be potentially needed for specifying requirement |
| Potential RAN4 spec impact |  | Prediction accuracy/delay requirement |
| Impacted other working groups |  | FFS the necessity of RAN2 signalling support (including LCM procedure) |

## 4.5 AI-RRM Sub-Case 2: FR1 L3 frequency domain (within the same FR) cell-level prediction (inter-cell, non-collocated)

Table 4.5. AI-RRM Sub-Case 2: FR1 L3 frequency domain (within the same FR) cell-level prediction 
(inter-cell, non-collocated)

| RAN4-identified Use case |  | AI-RRM Sub-Case 2: FR1 L3 frequency domain (within the same FR) cell-level prediction (inter-cell, non-collocated) |
| --- | --- | --- |
| Use case description | AI model input | L3-cell level measurement results |
|  | AI model output | L3-cell level prediction results |
|  | Label | Measurement results at certain target frequency layer |
|  | Training type | offline training |
|  | Model location for inference | UE-sided model or NW sided model |
|  | Collaboration/interaction between UE and NW | UE capability reporting, and other mechanisms similar as NR UE/NW-sided model |
| Observations on potential benefits and/or gain |  | Measurement RS reduction, and/or MG reduction, and/or Measurement delay reduction |
| Observations on complexity |  | To be studied |
| Standardization effort required |  | System-level simulation, and link-level simulation could be potentially needed for specifying requirement |
| Potential RAN4 spec impact |  | Prediction accuracy/delay requirement |
| Impacted other working groups |  | FFS the necessity of RAN2 signalling support (including LCM procedure) |

## 4.6 AI-RRM Sub-Case 3: Spatial domain Rx prediction for intra-cell

Table 4.6. AI-RRM Sub-Case 3: Spatial domain Rx prediction for intra-cell

| RAN4-identified Use case |  | AI-RRM Sub-Case 3: Spatial domain Rx prediction for intra-cell |
| --- | --- | --- |
| Use case description | AI model input | L1/L3 beam measurement results on selected UE RX beams |
|  | AI model output | L1/L3 Rx beam prediction results (e.g., L1/L3 beam-level RSRP) for reporting |
|  | Label | Measurement results from other RX beams |
|  | Training type | offline training |
|  | Model location for inference | UE-sided model |
|  | Collaboration/interaction between UE and NW | UE capability reporting, and other mechanisms similar as NR UE-sided model |
| Observations on potential benefits and/or gain |  | Reduced measurement delay, and measurement overhead reduction |
| Observations on complexity |  | To be studied |
| Standardization effort required |  | System-level simulation, and link-level simulation could be potentially needed for specifying requirement |
| Potential RAN4 spec impact |  | Reduced measurement delay requirement |
| Impacted other working groups |  | FFS the necessity of RAN2 signalling support (including LCM procedure) |

## 4.7 AI-RRM Sub-Case 4: AI/ML-based prioritization for MOs

Table 4.7. RRM Sub-Case 4: AI/ML-based prioritization for MOs

| RAN4-identified Use case |  | AI-RRM Sub-Case 4: AI/ML-based prioritization for MOs |
| --- | --- | --- |
| Use case description | AI model input | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells, and other informaiton. |
|  | AI model output | - Information about if, when and the priority to measure MOs, - Classification of measurement result level information |
|  | Label | Depending on AI model output, label can be constructed based on measurement result and other information. |
|  | Training type | Offline training |
|  | Model location for inference | UE-sided model or NW-side model |
|  | Collaboration/interaction between UE and NW | UE capability reporting, and other mechanisms similar as NR UE/NW-sided model |
| Observations on potential benefits and/or gain |  | Measurement delay optimization |
| Observations on complexity |  | To be studied |
| Standardization effort required |  | System-level simulation, and link-level simulation could be potentially needed for specifying requirement |
| Potential RAN4 spec impact |  | Measurement delay requirements |
| Impacted other working groups |  | FFS the necessity of RAN2 signalling support (including LCM procedure) |

## 4.8 AI-RRM Sub-Case 5: Non-collocated frequency domain prediction in beam level

Table 4.8. AI-RRM Sub-Case 5: Non-collocated frequency domain prediction in beam level

| RAN4-identified Use case |  | AI-RRM Sub-Case 5: Non-collocated frequency domain prediction in beam level |
| --- | --- | --- |
| Use case description | AI model input | L3 beam-level measurement results |
|  | AI model output | L3 beam-level predication results on different frequencies |
|  | Label | L3 Beam-level measurement results at certain target frequencies |
|  | Training type | Offline training |
|  | Model location for inference | UE-sided model or NW sided model |
|  | Collaboration/interaction between UE and NW | UE capability reporting, and other mechanisms similar as NR UE/NW-sided model |
| Observations on potential benefits and/or gain |  | Measurement RS reduction, and/or Measurement delay reduction |
| Observations on complexity |  | To be studied |
| Standardization effort required |  | System-level simulation, and link-level simulation could be potentially needed for specifying requirement |
| Potential RAN4 spec impact |  | Prediction accuracy/delay requirement |
| Impacted other working groups |  | FFS the necessity of RAN2 signalling support (including LCM procedure) |