3GPP TSG-RAN WG4 Meeting #118                                                              R4-260xxxx
Gothenburg, Sweden, 09th – 13st Feb., 2025

Agenda item:			8.1

Source:	Samsung

Title:	WF on [118][107] 6G AI

Document for:	Approval

# Topic #1: General aspects

### Issue 1-1: Expected RAN4 AI/ML outcomes to RAN#111

Agreement:

- Use the following template to summarize the identified RAN4-led 6G AI use cases for study.

| RAN4-identified Use case |  | Name, and identification of sub-cases as well |
| --- | --- | --- |
| Supported companies |  | Company-A, B, … |
| Use case description | AI model input |  |
|  | AI model output |  |
|  | Label |  |
|  | Training type | e.g., offline training, online training, offline training with online refining |
|  | Model location for inference | e.g., NW-sided, UE-sided, two-sided model |
|  | Collaboration/interaction between UE and NW | e.g., network indication, UE capability reporting, etc. |
| Observations on benefits and/or gain |  |  |
| Observations on complexity |  |  |
| Standardization effort required |  | e.g., System-level simulation, etc. |
| Potential RAN4 spec impact |  |  |
| Impacted other working groups |  | If yes, elaborate work required by other WGs to complete the study |

[Drafted RAN4 input to RAN#111]

- RAN4 had collected observations on RAN4-related AI/ML use cases reported by proponent companies, and RAN4 has studied the prioritization of relevant use cases during Rel-20 6G study item.

- From RAN4 perspective, the following AI/ML use cases are identified as RAN4-led AI/ML use cases

  - AI-based DPoD (Digital Post-Distortion) at NW

  - AI-based DPD (Digital Pre-Distortion) at UE

  - AI-based SRS power imbalance compensation

  - [AI-RRM use case TBA]

- RAN4 has agreed to introduce (1) AI-based DPoD at NW as 6G RAN4-led AI use case to study. Once the AI-based DPoD study is completed, (2) AI-based DPD at UE will be studied in Rel-20 by taking the performance KPI of AI-based DPoD at NW as the benchmark to identify the potential gain from the aspects of performance, complexity and power efficiency.

- RAN4 has not reached the consensus on whether or not to study (3) AI-based SRS power imbalance compensation as 6G RAN4-led AI use case, and the proponent companies are planning to prepare more justifications.

- [Prioritization for AI-RRM to be added]

- Detailed corresponding use case descriptions, observations and other related information can be found in the attachment.

# Topic #2: AI-based non-linearity compensation

### Issue 2-1: Overall view towards AI-DPoD and AI-DPD

Agreement:

- Introduce AI-DPoD at NW as 6G RAN4-led AI use case to study.

  - It is assumed no AI-DPD. UE with and without non-AI DPD will be considered.

- Once the AI-DPoD study is completed, AI-DPD at UE will be studied in Rel-20 by taking the performance KPI of AI-DPoD at NW as the benchmark to identify the potential gain from the aspects of performance, complexity and power efficiency.

  - It is assumed no AI-DPoD and no non-AI DPoD at NW

    - It can be revisited depending on the 5GA work in R20.

For information:

| Use case | Compensation for UL or DL | Model location and training | Be positive | Be neutral or more study is required | Be negative (including deprioritize) |
| --- | --- | --- | --- | --- | --- |
| AI-DPoD only in gNB | UL | NW-sided model | *(4) Samsung, Huawei, vivo, OPPO ‡(2) Tejas Networks, CMCC | *(2) Nokia, ZTE ‡(5) CATT, Apple, Xiaomi, Ericsson, Qualcomm | ‡ (0) |
| AI-DPD only in UE | UL | UE-sided model | *(2) vivo, Nokia ‡(3) Ericsson, CMCC, ZTE | *(0) ‡(2) MediaTek, CATT | *(1) OPPO, ‡(2) Tejas Networks, Qualcomm |
| Note: for companies show supporting, with and without numerical results in the Tdocs are separated counted. * with numerical results in Tdocs ‡ without numerical results in Tdocs (x) in which x is the number of companies for that category |  |  |  |  |  |

## 2.1 Use Case: AI-DPoD in gNB

### Issue 2-x: Use case description for AI-DPoD in gNB

[Drafted Use Case information for discussion]

| RAN4-identified Use case |  | AI-DPoD at NW |
| --- | --- | --- |
| Supported companies |  | (6) Positive: Samsung*, Huawei*, vivo*, OPPO*, Tejas Networks, CMCC (7) Neutral: Nokia*, ZTE*, CATT, Apple, Xiaomi, Ericsson, Qualcomm (0) Negative: Note: Company with * provides numerical results in Tdocs |
| Use case description | AI model input | The received signal after channel estimation and equalization |
|  | AI model output | The compensated signal |
|  | Label | Known RS signal e.g., DMRS |
|  | Training type | - Online training - Offline training with or without finetuning |
|  | Model location for inference | NW-sided model |
|  | Collaboration/interaction between UE and NW | For inference: UE capability and NW indication to UE to adjust PA back-off For training: data collection procedure |
| Observations on benefits and/or gain |  | With the same TX EVM level, receiver performance improvement With relaxed TX EVM level, PA power backoff is reduced |
| Observations on complexity |  | - Lightweight NN (~3M FLOPs) - ~100M FLOPs |
| Standardization effort required |  | Link level simulation, RF evaluation for MPR |
| Potential RAN4 spec impact |  | RAN4 requirements, e.g, EVM, MPR |
| Impacted other working groups |  | RAN1: DMRS/Sequence design/selection, Tx power determination RAN2: Signaling/ procedure related to LCM for NW-sided model |

## 2.2 Use Case: AI-DPD in UE

### Issue 2-y: Use case description for AI-DPD in UE

[Drafted Use Case information for discussion]

| RAN4-identified Use case |  | AI-DPD at UE |
| --- | --- | --- |
| Supported companies |  | (5) Positive: vivo*, Nokia*, Ericsson, CMCC, ZTE (7) Neutral: MediaTek, CATT (3) Negative: OPPO*, Tejas Networks, Qualcomm Note: Company with * provides numerical results in Tdocs |
| Use case description | AI model input | - Time domain samples after PA distortion - Time domain samples before PA distortion |
|  | AI model output | The compensated signal |
|  | Label | Time domain samples w/o distortion |
|  | Training type | - Online training - Offline training with or without finetuning |
|  | Model location for inference | UE-sided model |
|  | Collaboration/interaction between UE and NW | Similar to UE-sided model as NR |
| Observations on benefits and/or gain |  | PA power backoff is reduced |
| Observations on complexity |  | TBA |
| Standardization effort required |  | RF evaluation |
| Potential RAN4 spec impact |  | RAN4 requirements, e.g, EVM, MPR |
| Impacted other working groups |  | RAN1: SRS and/or PUSCH enhancement/Tx power determination for data collection RAN2: Signaling/ procedure related to LCM for UE-sided model |

# Topic #3: AI-based SRS power imbalance compensation

### Issue 3-1: Overall view towards AI-based SRS power imbalance compensation

For information:

| Use case | Model location and training | Be positive | Be neutral or more study is required | Be negative (including deprioritize) |
| --- | --- | --- | --- | --- |
| AI-based SRS residual power imbalance compensation | NW-sided model | (1) Huawei, ‡(3) OPPO, vivo, Ericsson | *(1) Samsung ‡ (2) CMCC, Apple | ‡(3) ZTE, Nokia, Qualcomm |
| Note: for companies show supporting, with and without numerical results in the Tdocs are separated counted. * with numerical results in Tdocs ‡ without numerical results in Tdocs (x) in which x is the number of companies for that category |  |  |  |  |

[Tentative Agreement based on Tuesday Offline]

- For AI-based SRS IL compensation use case:

  - Some companies see the value for at least studying this use case

  - Other companies propose concerns on the following aspects:

    - (1) The target scenario of SRS IL compensation at gNB side is questionable to some companies. At least UE side SRS IL compensation should be considered as benchmark to be compared with in the evaluation.

      - FFS the mid-range SNR is targeted scenario.

      - FFS if some of UE compensate the IL partially depending on available power headroom.

    - (2) The potential specification impact, including the necessity of IL or IL-related information reporting.

### Issue 3-z: Use case description for AI-based SRS power imbalance compensation

[Drafted Use Case information for discussion]

| RAN4-identified Use case |  | AI-based SRS power imbalance compensation |
| --- | --- | --- |
| Supported companies |  | (4) Positive: Huawei*, OPPO, vivo, Ericsson (3) Neutral: Samsung, CMCC, Apple (3) Negative: ZTE, Nokia, Qualcomm Note: Company with * provides numerical results in Tdocs |
| Use case description | AI model input | UL measured channel matrix from SRS with residual power imbalance |
|  | AI model output | Ideal DL channel matrix |
|  | Label | CSI measurement without power imbalance, candidate solutions could be: - CSI feedback based on CSI-RS - SRS measurements that are processed via time domain filtering to achieve high enough SNR - SRS measurements with perfect pre-compensation at UE |
|  | Training type | Offline training |
|  | Model location for inference | NW-sided AI |
|  | Collaboration/interaction between UE and NW | 1. Inference: UE reporting on the range of SRS residual power imbalance for model training and model selection 2. Data collection/monitoring: UE reporting/assistance information for constructing labels, e.g., CSI feedback based on CSI-RS or SRS measurements with perfect pre-compensation |
| Observations on benefits and/or gain |  | TBA |
| Observations on complexity |  | TBA |
| Standardization effort required |  | Link-level simulation |
| Potential RAN4 spec impact |  | 1) UE assistance information/reporting related to SRS residual power imbalance 2) Potential UE RF related testing |
| Impacted other working groups |  | TBA |

# Topic #4: AI/ML use case for RRM

TBA

## Reference for RAN4#118 (Feb. 2026, Gothenburg)

R4-2601166	Running Summary of 6G AI										Samsung

R4-2602149	Topic Summary for [118][107] 6G AI								Moderator (Samsung)

R4-2600249	Discussion on general parts of RAN4 driven AI use cases		CATT

R4-2600526	Discussion on 6G AI/ML Framework and General Issues		Apple

R4-2600653	Discussion on General aspects of 6G AI							CAICT

R4-2600845	Discussion on general aspects for 6G AI/ML						CMCC

R4-2600902	Discussion on general aspects for RAN4 6G AI					Huawei, HiSilicon

R4-2601167	Discussion on AI/ML general aspects for 6GR					Samsung

R4-2601216	General issues for AI/ML in RAN4									OPPO

R4-2601353	Discussion on 6G AI general part									ZTECorporation,Sanechips

R4-2601417	Discussion on general aspects related to AIML study for 6G		Ericsson

R4-2601475	General aspect on 6G AI											vivo

R4-2601685	6G AI/ML General Aspects											Nokia

R4-2601699	PDV Considerations for Selected 6G AI/ML Candidate Use Cases	Korea Testing Laboratory

R4-2601899	Views on 6G AI study RAN4 general aspects						China Unicom

R4-2602130	6G AI/ML Framework												Qualcomm Incorporated

R4-2600168	Discussion on AI-based non-linearity compensation				MediaTek Inc.

R4-2600250	Discussion on RAN4 driven AI use cases - AI-based non-linearity compensation	CATT

R4-2600408	Views on AI-based non-linearity compensation					Tejas Network Limited

R4-2600441	Discussion on AI-based non-linearity compensation				Xiaomi

R4-2600527	Discussion on AI-based non-linearity compensation				Apple

R4-2600847	Discussion on use cases for AI-based non-linearity compensation	CMCC

R4-2600903	Discussion on AI-based non-linearity compensation				Huawei, HiSilicon

R4-2600925	Discussion on AI-based Non-linearity Compensation				vivo

R4-2601186	Discussions on AI-based non-linearity compensation	ZTE 		Corporation,Sanechips

R4-2601217	AI-based non-linearity compensation								OPPO

R4-2601508	Discussion on AI-based non-linearity compensation				Ericsson

R4-2601639	View on AI based nonlinearity compensation for 6G AI/ML		Samsung

R4-2601735	AI-based non-linearity compensation								Nokia

R4-2601998	6G AI : Comments on AI based non-linearity compensation		Qualcomm Incorporated

R4-2602021	Views on 6G AI-based non-linearity compensation				NTT DOCOMO, INC.

R4-2600251	Discussion on RAN4 driven AI use cases - AI-based SRS power imbalance compensation	CATT

R4-2600528	Discussion on AI-based SRS power imbalance compensation	Apple

R4-2600846	Discussion on use cases for AI-based SRS power imbalance compensation	CMCC

R4-2600904	Discussion on AI-based SRS power imbalance compensation	Huawei, HiSilicon

R4-2601168	Discussion on AI-based SRS power imbalance compensation	Samsung

R4-2601218	AI-based SRS power imbalance compensation					OPPO

R4-2601246	Views on AI-based SRS power imbalance compensation			ZTE Corporation, Sanechips

R4-2601705	Discussion on use case of AI-SRS channel reconstruction		Ericsson

R4-2601752	Discussion on AI-assisted SRS Power Imbalance Compensation	Nokia

R4-2602042	AI-ML based SRS Power Imbalance Compensation				Qualcomm Incorporated

R4-2600169	Discussion on AI/ML use cases for RRM							MediaTek Inc.

R4-2600252	Discussion on RAN4 driven AI use cases - AI/ML use cases for RRM	CATT

R4-2600446	Discussion on 6G RRM AI usecase									Xiaomi

R4-2600529	Discussion on AI/ML use cases for RRM							Apple

R4-2600848	Discussion on use cases for AI/ML use cases for RRM			CMCC

R4-2600905	Discussion on AI/ML use cases for RRM							Huawei, HiSilicon

R4-2600953	6G AI/ML use cases for RRM										OPPO

R4-2601093	Discussion of AI use cases for RRM								LG Electronics Inc.

R4-2601169	Discussion on AI/ML use cases for RRM							Samsung

R4-2601295	Views on AI study in 6G RAN4-driven use cases for RRM		ZTE Corporation, Sanechips

R4-2601418	Discussion on RRM use cases for AIML study for 6G			Ericsson

R4-2601476	Discussion on AI/ML use cases for RRM							vivo

R4-2601686	AI/ML use cases for RRM											Nokia

R4-2602043	AI-ML use cases for RRM											Qualcomm Incorporated

R4-2601510	Discussion on AI-enabled PRACH receiver						Ericsson

[FL Recommended Discussion Point]

- AAA

  - BBB

    - CCC

      - DDD

        - EEE