3GPP TSG-RAN WG4 Meeting # 118	                                R4-26x

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			6.11.1

Source:	vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, Keysight

Title:	WF on updated simulation assumption for AI based beam management

Document for:	Approval

# Introduction

In this contribution, the simulation assumption for AI/ML based BM case-1 is further updated based on [1] and [2]. Companies are encouraged to provide simulation results to evaluate the impact of the measurement error and channel model based on the performance metrics in section 5.

# Simulation assumptions for baseband error

Link level simulation (LLS) assumptions and System level simulation (SLS) assumptionsfor evaluation of measurement error on AI/ML based BM case-1 performance are defined in Table 1-7.

Table 1 Baseline Link Level Simulation assumptions for AI/ML in beam management evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Channel model | Case 1: NLOS channel - Baseline: - TDL-C, DS=100ns - Optional: - CDL-C, DS=100ns |
|  | Case 2: AWGN channel |
| BS antenna configurations | - Option 1: - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 32 - Option 2: - Number of TX beams: 1 - Baseband errors are independent for each TX beam |
| BS antenna element radiation pattern | Option1: Table 2 Option 2: N/A |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | Option1: 25m, 110° Option 2: N/A |
| Beamforming characteristic of the BS pattern | Use same assumption for System-level simulation assumption defined Table 6 |
| UE antenna configurations | Option 1: Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional Number of Rx beams is up to UE Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 1: Table 3 Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | SSB or CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | Baseline: - 1 sample Optional: - 3 samples - 5 samples |
| Note 1: TXRU weights mapping for BS and UE is up to company. |  |

Table 2 BS antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 3 UE antenna radiation pattern model

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: are in local coordinate system. |  |

Table 4 Cell-specific parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 20 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 120 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 |
| SNR | dB | -3 Note: Simulation with other SNR levels are not precluded, e.g., for determining the SNR for ideal measurement. |
| Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed. |  |  |

Table 5 CSI-RS configuration parameters

| Parameter | Value |
| --- | --- |
| CSI-RS bandwidth | 48 PRBs; |
| CSI-RS SCS | 120 kHz |
| Periodicity | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot | <32,1,4,FD-CDM2> |

Table 6 Beamforming characteristic of the BS pattern for System-Level simulation assumption in narrow-to-narrow scenario

| Parameter | Value |
| --- | --- |
| Beam pattern legend for Set A and Set B |  |
| Azimuth and elevation angle ranges | 32 beams with grid of 4 elevation angles from (100°to 150°) and 8 azimuth angles from (-60°to 60°) Note: here the 0 degree in Elevation refers to +Z axis |
| AoDs of Set A and Set B (°) | - Set A: - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] - Set B [(azimuth, elevation)]: - [(-52.5, 143.75), (-37.5, 131.25), (-22.5, 118.75), (-7.5, 106.25), (7.5, 143.75), (22.5, 131.25), (37.5, 118.75), (52.5, 106.25) |

Table 7 Beamforming characteristic of the BS pattern for System-Level simulation assumption in wide -to-narrow scenario

| Parameter | Value |
| --- | --- |
| AoDs of Set A and Set B (°) | Set A (CSI-RS): - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] Set B (SSB): - 2 elevation DFT beams: [112.5, 137.5] - 4 azimuth DFT beams: [-45, -15, 15, 45] Companies can use a subset of antenna elements to generate wider DFT beams for SSB. One of the options to generate setB beams is shown below. Other options to generate setB beams are not precluded. |
| BS Antenna Configuration and number of beams | For setA determination: Use the setup from 38.843 Table 6.3.1-1 Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of CSI-RS beams: 32 For setB determination: Use a subset of antenna elements, e.g., below. Antenna setup and port layouts at gNB: (2, 4, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of SSB beams: 8 Other options to generate setB beams are not precluded. |

For the case of wide-to-narrow spatial beam prediction (SSB to CSI-RS),  the simulation assumption in Table 7 can be the reference for the simulation assumption.

# Simulation assumptions for RF error

The impact of RF errors should be considered. Following assumptions can be used to generate RF errors.

- Truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used

  - Each TX beam are Independent

  - 2Rx chains with independent RF error per chain (same distribution for each chain)

# Link Level Simulation assumption for Simplified channel model (mimic test in TE chamber)

This is an independent and different procedure from the system simulation in Chapter 7. This product will only use the CDL channel models (Table 8/9), which is different from the channels generated in the system-level simulation. The data generated also follows the format of that generated based on system-level simulation (Chapter 8 Dataset). Therefore, the simulation results can also be used to define requirements.

The overall simulation flow, as an example, is represented as:

- Tx 32 SSB → CDL C/D cluster AoD → CDL C/D cluster AoA → PAS → best Rx beam selection → AIML prediction

- High-Level Procedure (for one iteration):

The gNB transmits each of the 32 SSB beams individually through the CDL-C/D channel model.

Each SSB beam pattern is used to illuminate the CDL-C/D clusters.

The CDL-C/D channel is treated as a “black box”: the internal propagation and scattering processes are unknown.

For each transmitted SSB, the channel input corresponds to the clusters’ angles of departure (AoD) defined in the CDL-C/D model.

The channel output for each SSB corresponds to the clusters’ angles of arrival (AoA) from the CDL-C/D model.

The resulting power angular spectrum (PAS) for each SSB is formed by the power contributions of the illuminated clusters at their respective AoA directions.

The UE performs Rx beam sweeping (if needed) and selects the best Rx beam to capture the PAS, producing one SSB specific RSRP measurement.

This procedure is repeated for all 32 SSBs. The resulting 32-dimensional RSRP input vector is then fed into the AIML model for inference.

- Step 1-8 are only for one iteration. For increase the variety, multiple iteration with UE rotation should be simulated, following the assumptions below.

------------------

Simulation assumptions for Simplified channel model are defined as follows.

- Channel model parameters

  - Use table 9 as prioritized option for evaluation

  - Table 8 is optional, companies are invited to evaluate

Table 8 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 9 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

- UE rotation:

  - 3D Rotation – multiple orientations should de be simulated,

  - Each evaluation is at a static position (UE not rotated during a test iteration)

- gNB and UE radiation pattern to reuse the parameters from the SLS

- UE baseband SNR > -3dB

  - Only used in simulation, actual setting in the test will be discussed for the test setup

  - Inference results for instances not meeting the side condition are discarded from the performance statistic

- Companies are invited to study if UE location affects performance

# Reference model

For better alignment of the performance between companies, it would be better to perform simulations based on a reference model. The model description in Fig. 1 is an example of reference model for BM-case 1 performance evaluation, while it is not the intention to preclude other reference models.

Fig. 1 Reference model description

# Performance metrics

At least the following performance characteristics are to be provided

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1, X= 0, 1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the 1st, 2nd,3rd, 4th,5th strongest predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 defined in clause 6)

- Measurement result with consideration of measurement error (for Case 3 defined in clause 6)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

# System-level Simulation procedures (mimic UE in the field)

Performance evaluation simulation procedure

We provide the following procedures for companies to perform simulations to evaluate the impact of measurement error in both of following scenarios of BM prediction:

- Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.

- Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B

The simulated cases include:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

For Case 1 (performance evaluation excluding error), the simulation procedure includes:

- Companies to generate ideal L1-RSRP dataset from the SLS with following assumptions:

  - Table 6 (for scenario 1),

  - Table 7 (for scenario 2) and

  - A combination of:

    - TR 38.843 Table 6.3.1-1 replacing channel model by TR 38.901 Table 7.5-6 part I (UMi) with the assumption of Tx height 10m, ISD 200m and minimum gNB-UE distance for studying different locations 10m (mandatory)

    - TR 38.843 Table 6.3.1-1 with the assumption of Tx height 25m, ISD 500m and minimum gNB-UE distance for studying different locations 35m can be added to the dataset.(optional)

    - A combination of both above (optional)

- Training dataset generation: Use subset samples of the ideal dataset generated from the 1st step for training models.

- Use the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) for inference and evaluate the performance metric assuming that the ground truth is without the measurement error

Note: UE distribution is 100% outdoor

For Case 3 (performance evaluation including error) , the simulation procedure includes:

- Companies to generate ideal L1-RSRP dataset from the SLS with following assumptions:

  - Table 6 (for scenario 1),

  - Table 7 (for scenario 2) and

  - A combination of:

    - TR 38.843 Table 6.3.1-1 replacing channel model by TR 38.901 Table 7.5-6 part I (UMi) with the assumption of Tx height 10m, ISD 200m and minimum gNB-UE distance 10m (mandatory)

    - TR 38.843 Table 6.3.1-1 with the assumption of Tx height 25m, ISD 500m and minimum gNB-UE distance 35m can be added to the dataset (optional)

    - A combination of both above (optional)

- Add errors to ideal SLS as shown below:

- Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

- Use the Simulation assumptions for RF error defined in Clause 3 to generate RF errors.

- Add the measurement errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be:

  - Alt 1: Only baseband errors are included

  - Alt 2: Only RF errors are included

  - Alt 3: Both baseband errors and RF errors are included

- Use subset samples of the dataset generated from the 2nd step including added measurement error for training models (The same Alt of error as selected in Step 2).

- Filter UEs in the other subset samples of the dataset from 2nd step (none overlapping samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference.

Note: UE distribution is 100% outdoor

Use the subset of dataset derived in 4th step for inference and evaluate the performance metric assuming that the ground truth is with measurement errors.

Channel simplification evaluation simulation procedure

In order to evaluate channel simplification proposed, the simulation procedure includes:

- Steps 1, 2 and 3 from performance evaluation simulation procedure for Case 1.

- Repeat inference in step 3 in performance evaluation simulation without error using LLS with LLS simulation assumptions defined in Tables 1-5 and channel model defined in TR 38.827 Table 7.2.2-3.

- Repeat inference in step 3 in performance evaluation simulation without error using LLS with LLS simulation assumptions defined in Tables 1-5 and channel model defined in Table 9.

- Optionally, repeat inference in step 4 using LLS with LLS simulation assumptions defined in Tables 1-5 and channel model defined in Table 8.

- The prediction difference between the “results” of:

- Steps 2 and 3 in channel model evaluation simulation and step 3 in performance evaluation simulation without error

- Optionally steps 2 and 4 in channel model evaluation simulation and step 3 in performance evaluation simulation without error

will be evaluated to validate channel model simplification.

NOTE: Interesting companies can also evaluate channel simplification considering baseband and RF errors.

# Dataset

## 8.1 Dataset sharing format

Companies are encouraged to share their own simulation dataset based on the following format:

Dataset format

Use NumPy for dataset sharing

Use npy – single array in each file

Data file format

- Dataset file format: N (samples) * Beam RSRPs per Rx beam (Set A Tx beams) * M (Rx beams).

Dataset should content following info

  - 1st dimension: Number of samples

  - 2nd dimension: Number of Set A Tx beams per Rx beam

  - 3rd dimension: Number of Rx beams

Note: Each element of the dataset will be a float32 real number

- Using the aligned beam pattern specified in Table 6 and Table 7, RSRPs of each beam in Set B can be obtained from RSRPs of each beam in Set A.

Note 1: Dataset files can be split into multiple sub-files to enable easier upload.

- Use the 2 digits for split files starting from 00, increment for each additional file.

- Split files and then archive each sub-file

Note 2: The full Dataset files from each company follows the ratio of [training samples, validation samples, testing samples] = [90%, 5%, 5%]

File naming scheme

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”

- Subfolder created for each meeting

- File naming scheme (dataset file)

  - a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary, same as the identifier used in CSI compression)

  - meeting number

  - dataset can be split in multiple files – 2 digits

  - files to be compressed to zips and uploaded

Example for folder:

Folders to be created under https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/

Folder for datasets: /Data_sharing/ NR_AIML_air/Beam/Datasets/R4_XXX

Example for dataset file:

VIV0R4_115_00.npy

## 8.2 Dataset parameter

Dataset size

The dataset size shall be greater than [100,000] if company uses its own dataset for simulation

# References

- R4-2511797, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- R4-2522430, WF on [117][112] R19 AI for air interface, Qualcomm Incorporated

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)