Issue 1: Channel model parameters

Background: RAN4 agreed to consider the Channel model parameters for UMi CDL-C in following Table 2 and Table 3

Agreement in RAN4#117 Issue 2-7: Channel model simplification Agreement: Take option 2 below as the baseline channel model simplification for Umi CDL-C at 28GHz and if no significant issue is identified on option 2, the final agreement will be confirmed in RAN4#118. - The value of CASA in table 3 is FFS - Interested companies are encouraged to evaluate the option 2, including based on the methodology described in option 1. Option 2: Simplification based on weak clusters - Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM. - Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4318; -30.4353; -134.4434; 98.9242; 74.51134. 2; 12.594; -1.2500; -20.9269; 129.1633; 99.1915; 74.51134. 5; 13.056; -5.5318; -28.0782; -152.8206; 99.5732; 74.51134. 6; 38.196; 0.0000; -11.6982; 164.1145; 99.306; 74.51134. 13; 73.71; -8.1318; -33.911; 93.1719; 100.165; 74.51134. 14; 78.498; -9.8318; -37.5066; -112.0441; 100.2604; 74.51134. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; 10.4021; 0.5726; 0; 7. - Table 3 – Channel model parameters for UMi CDL-C at 28 GHz with AoA aligned to probe layout and no intra-cluster angle spread Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4318; -30.4353; -114.436; 98.9242; 74.51134. 2; 12.594; -1.2500; -20.9269; 125.5639; 99.1915; 74.51134. 5; 13.056; -5.5318; -28.0782; -174.436; 99.5732; 74.51134. 6; 38.196; 0.0000; -11.6982; -174.436; 99.306; 74.51134. 13; 73.71; -8.1318; -33.911; 95.5639; 100.165; 74.51134. 14; 78.498; -9.8318; -37.5066; -114.436; 100.2604; 74.51134. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; FFS; 0.5726; 0; 7.

- Recommended Option: Use the Channel model parameters for UMi CDL-C in Table 2 for simulation

Issue 2: how to simulate the simplified CDL channel

- Option 1: Modify the system-level simulation platform in the existing simulations by changing the parameters, e.g., delay and cluster power, to the agreed CDL-C channel parameters

- Option 2: Develop the agreed channel model on the link-level simulation platform

  - Tx beam power:

    - Fixed

    - Random in a certain range

Issue 3: Cases to be simulated

RAN4 to conduct the simulation for the following cases:

- Case 1a: SLS based on Uma

  - With Tx height 25m and ISD 500m to align with the description in TR 38.901 clause 6.2

- Case 1b: SLS based on Umi (to be determined)

  - With Tx height 10m and ISD 200m to align with the description in TR 38.901 clause 6.2

- Case 2: Based on simplified Channel model

Issue 4: Training and inference dataset generation

For training:

RAN4 to decide the training dataset generation

- for Case 1a and Case 2

  - Option 1: The training dataset is generated based on the SLS assumption defined in TR38.843 Table 6.3.1-1 (existing results)

  - Option 2: Mixed dataset by Option 1 and simplified CDL channel

- for Case 1b

  - Further discuss the training dataset if this case is agreed

For inference:

For Case 1a and Case 1b, the inference dataset is generated based on generated based on the SLS assumption defined in TR38.843 Table 6.3.1-1

For Case 2, The inference dataset is generated based on simplified CDL channel