3GPP TSG-RAN4 Meeting #118	R4-2601499

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3196 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 for n28 3MHz UL/5MHz DL Asymmetric BW change from Optional to Mandatory |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rakuten Mobile, Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Core |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | To make n28 3MHz UL/5MHz DL Asymmetric BW change from Optional to Mandatory in Rel-19 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change BCS from 1 to 0 in Table 5.3.6-1: for Band n28 and reflect same change in NOTE 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators for specific Bands will need to work with each device vendor to ensure support for 3MHz Uplink and 5MHz Asymmetric Downlink channel bandwidth. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in Table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in Table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
|  | 10, 15, 20 | 25, 35 | 1 |
|  | 10, 15, 20 | 25, 30, 35 | 2 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n26 | 20 | 25, 30 | 0 |
| n283 | 3 | 5 | 14 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
|  | 5, 10, 15 | 20, 25, 30, 35, 40 | 2 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
|  | 5, 15 | 10 | 1 |
|  | 10, 15 | 5 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 20 | 25, 30, 35 | 2 |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n105 | 20 | 25, 30, 35 | 0 |
| n1091 | 5 | 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | 10 | 15, 20, 25, 30, 40, 50 |  |
|  | 15 | 20, 25, 30, 40, 50 |  |
|  | 20 | 25, 30, 40, 50 |  |
|  | 25 | 30, 40, 50 |  |
|  | 30 | 40, 50 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated for asymmetricBandwidthCombinationSet in TS 38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. NOTE 3: This BCS1 is limited to uplink 715-718 MHz and downlink 768-773 MHz. NOTE 4: For n28 band, support of BCS1 and support of UE capability support3MHz-ChannelBW-Asymmetric-r18 are mandatory in Rel-19 of this specification. This requirement is applicable only in Japan. |  |  |  |  |

Table 5.3.6-2: TDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n50 | 60 | 80 | 0 |
| NOTE 1: Both centre frequency and BWP-ID shall match between DL and UL carriers as defined in TS 38.331 [7] cl. 6.3.2 and TS 38.213 [8] clause 12. NOTE 2: In a case a UE is configured with a full width of BWP within both UL/ DL channels, the centre frequency of UL/ DL channels shall be same. NOTE 3: A position of Point A is common between UL and DL carriers as defined in TS 38.331 [7] cl. 6.3.2. NOTE 4: The maximum transmission bandwidth configuration NRB for 60 MHz UL channel BW is reduced from the value defined in table 5.3.2-1 to 161 for 30kHz SCS for Asymmetric channel bandwidth combination set 0 for n50. |  |  |  |

==============End of change==============