3GPP TSG-RAN WG4 Meeting #118		R4-2602155
Gothenburg, Sweden, Feb. 9-13, 2026

Agenda item:			4.1.1

Source:	Moderator (Spreadtrum)

Title:	Topic summary for [118][113] R19_UERF_maintenance

Document for:	Information

Introduction

This is the summary for R19_UERF_maintenance under agenda 4.2 and except 4.2.7.

List of topics below:

- Discussion papers and corresponding CRs (26)

  - Draft CRs for 38.101-1 (7)

  - Draft CRs for 38.101-3 (2)

  - Draft CRs for 38.101-5 (2)

  - Draft CRs for 36.101 (2)

  - Draft CRs for 38.104 (1)

  - Draft CRs for 38.106 (1)

  - Draft CRs for 38.115-2 (1)

  - Draft CRs for 38.176-2 (1)

  - Draft CRs for 38.307 (1)

  - Draft CRs for 38.719-03-01 (1)

  - Draft CRs for 38.863 (2)

  - Others (5)

Topic #1: R19 spectrum related WI maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2600126 | Apple | CR for TR 38.863 corrections for the NTN NR band n252 NS flag references |
| R4-2600127 | Apple, Qualcomm | CR for TS 38.101-5 correction for the NTN NR L-band emission limits and A-MPR values |
| R4-2600128 | Apple | CR for TR 38.863 corrections for the NTN NR L-band NS flag references |
| R4-2600139 | Apple | CR for TS 36.101 bug fix: introduce uplink configurations for downlink fallbacks |
| R4-2600140 | Apple | CR for TS 38.101-1 bug fix: Correcting notation bugs and adding fallbacks (DL and/or UL) of combinations that have been introduced without the required fallbacks DL fallbacks : - CA_n40A-n41B (Parent: CA_n40A-n41B-n77A) Some ULs of these DL fallbacks are missing: - CA_n7A-n78A-n79A (Parent: CA_n1A-n7A-n78A-n79A, CA_n7A-n40A-n78A-n79A) - CA_n1A-n7A-n79A (Parent: CA_n1A-n7A-n78A-n79A) - CA_n7A-n40A-n79A (Parent: CA_n7A-n8A-n40A-n79A, CA_n7A-n28A-n40A-n79A, CA_n7A-n40A-n78A-n79A) - CA_n7A-n25A-n71A (Parent: CA_n7A-n25A-n66A-n71A, CA_n7A-n25A-n71A-n77A) - CA_n1A-n7A-n40A-n79A (Parent: CA_n1A-n7A-n40A-n78A-n79A) |
| R4-2600141 | Apple | CR for TS 38.101-3 bug fix: Correcting notation bugs and adding fallbacks or UL fallbacks of combinations that have been introduced without the required fallbacks DL fallbacks : - DC_1A-28A_n41A (Parent: DC_1A-28A-32A_n41A) - DC_1A-32A_n40A (Parent: DC_1A-32A_n40A-n41A) - DC_28A-32A_n40A (Parent: DC_28A-32A_n40A-n41A) - DC_1A-28A-32A_n40A (Parent: DC_1A-28A-32A_n40A-n41A) - DC_8A-38A_n1A-n78A (Parent: DC_3A-8A-38A_n1A-n78A) - DC_3A-38A_n1A-n78A (Parent: DC_3A-8A-38A_n1A-n78A) - DC_20A-38A_n1A-n78A (Parent: DC_8A-20A-38A_n1A-n78A) - DC_8A-20A-38A_n1A-n28A (Parent: DC_3A-8A-20A-38A_n1A-n28A) - DC_3A-8A-38A_n1A-n28A (Parent: DC_3A-8A-20A-38A_n1A-n28A) - DC_n77(2A)-n261A/G/H (Parent: DC_n25A-n77(2A)-n261A/G/H) Some ULs of these DL fallbacks are missing: - CA_n1A-n78A-n257H (Parent: CA_n1A-n78A-n257I) - CA_n40A-n77A-n257L (Parent: CA_n40A-n77A-n257M, CA_n40A-n77C-n257L, CA_n40B-n77A-n257L) - CA_n1A-n78A-n79A-n257H (Parent: CA_n1A-n78A-n257H) - CA_n28A-n41A-n79A-n257A (Parent: CA_n28A-n41A-n79A-n257G) - CA_n1A-n3A-n77A-n257I (Parent: CA_n1A-n3A-n28A-n77A-n257I, CA_n1A-n3A-n41A-n77A-n257I, CA_n1A-n3A-n77A-n79A-n257I) - CA_n28A-n41A-n79A-n257A (Parent: CA_n1A-n28A-n41A-n79A-n257A, CA_n3A-n28A-n41A-n79A-n257A) - CA_n3A-n28A-n77(2A)-n257A/G/H (Parent: CA_n3A-n28A-n77(2A)-n79A-n257A/G/H) - CA_n3A-n28A-n77A-n79A-n257I (Parent: CA_n3A-n28A-n77(2A)-n79A-n257I) - CA_n3A-n28A-n77(2A)-n257I (Parent: CA_n3A-n28A-n77(2A)-n79A-n257I) - CA_n28A-n41A-n79A-n257A (Parent: CA_n28A-n41A-n77A-n79A-n257A) |
| R4-2600142 | Apple | Proposal 1: In Table 7.3A.6-1 and for CA_n29-n71, consider adopting the Band n29 PC3 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 39.56,9; ACLR1. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. Proposal 2: In Table 7.3A.6-1a-1 and for CA_n29-n71, consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 42.56,8 45.57,8; ACLR1. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. |
| R4-2600291 | NTT DOCOMO, INC. | N.A |
| R4-2600305 | NTT DOCOMO, INC. | N.A |
| R4-2600310 | NTT DOCOMO, INC. | N.A |
| R4-2600336 | CATT | CR for TS 38.101-1 to void the Note 6 in Table 5.3.5-1 According to the RP-250958, NOTE 6 was replaced by NOTE 12 |
| R4-2600337 | CATT | CR for TS 38.104 to void the NOTE 1 in table 5.3.5-1 According to the RP-250958, NOTE 6 was replaced by NOTE 12 |
| R4-2600339 | CATT | CR for TS 38.307 to introduce NR bands n110 CR R4-2502824 was agreed at RAN4#114 meeting and approved in CR pack RP-250627 at RAN#107. But the changes in R4-2502824 failed to be implemented into TS 38.307. This CR is the resubmission of R4-2502824. |
| R4-2600582 | Qualcomm Technologies | Main message of this draft LS: RAN4 kindly requests RAN2 to take the following actions: - Define signaling to enable UE Power Class 1 for inter-band UL CA for both NR and NR-DC configurations. - Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17 |
| R4-2600583 | Qualcomm Technologies | Proposal 1: Send LS to RAN2 informing them that PC1 for inter-band CA should be enabled for both NR and NR-DC band combinations. Proposal 2: Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17. |
| R4-2600584 | Viasat | CR for TS 38.101-5 updates related to combined L-band |
| R4-2600597 | ViaSat Satellite Holdings Ltd | Observation 1: EN 301 681 recommends use of Table 3a for compliance for newer MESs that support n251 and n255. Observation 2: EN 301 681 requires use of Table 3a (and not Table 3) when the UE supports both sub-bands, i.e., any combination of the following combinations: (n250/b250, n251/n251), (n250/b250, n255/b255), (n251/b251, n253/b253) or (n255/b255, n253/b253). Observation 3: For combined L-Band, i.e., n250 and n251, Table 3a should have been used to specify the potential A-MPR. Observation 4: ETSI TC SES LS requests 3GPP to consider Table 3a for UE supporting in operations in 1626.5 – 1660.5 MHz as well as 1668 – 1675 MHz. Observation 5: Table 3 has more stringent limits compared to Table 3a for all channel bandwidths compared to the general 3GPP SEM. Observation 6: Use of stringent limits in Table 3 requires significant additional power back-off for all channel bandwidths. Observation 7: Continued use of Table 3, against the recommendation of EN 301 681, will result in higher additional power back-off. Proposal 1: To align with EN 301 681 recommendation, update NS_10N emission table in TS 38.101-5 (Table 6.3.3.3.9-1) with changes (highlighted in blue) as shown in Table below and update the associated A-MPR values as part of Rel-19 maintenance work related to n251 at the RAN4 next meeting. Frequency range (MHz); Spectrum emission limit1 (dBm); Measurement bandwidth; NOTE (measurement method). 30 to 1000; -36; 100 kHz; Peak hold. 1000 to 1559; -31; 1 MHz; Average. 1559 to 1605; -40; 1 MHz; Average. 1605 to 1612.5; -40 to -28.5; 1 MHz; Average. 1612.5 to 1616.5; -25 to -20; 1 MHz; Average. 1616.5 to 1621.5; -20 to -16; 1 MHz; Average. 1621.5 to 1624.5; -30 to -27.5; 30 kHz; Average. 1624.5 to 1625.125; -27.5 to -27.2; 30 kHz; Average. 1625.125 to 1625.8; -27.2 to -20; 30 kHz; Average. 1625.8 to 1626; -20 to -17; 30 kHz; Average. 1626 to 1626.2; -17 to -10; 30 kHz; Average. 1626.2 to 1626.5; -10; 30 kHz; Average. 1660.5 to 1660.525; 30 to 15; 3 kHz; Average. 1660.525 to 1660.625; 15 to - 20; 3 kHz; Average. 1660.625 to 1660.925; -20; 3 kHz; Average. 1660.925 to 1662; -20 to -35; 3 kHz; Average. 1662 to 1677; -25; 30 kHz; Average. 1677 to 1680; -30; 30 kHz; Average. 1680 to 1685; -30; 100 kHz; Average. 1685 to 1695; -30; 300 kHz; Average. 1695 to 1705; -30; 1 MHz; Average. 1705 to 2250; -30; 3 MHz; Average. 2250 to 12750 (Note 4); -30; 3 MHz; Peak hold. NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz NOTE 4: In the bands 3 253.0 MHz to 3 321.0 MHz and 3 336.0 MHz to 3 350.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -8 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 4 879.5 MHz to 4 981.5 MHz and 5 004.0 MHz to 5 025.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 6 506.0 MHz to 6 642.0 MHz and 6 672.0 MHz to 6 700.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 8 132.5 MHz to 8 302.5 MHz and 8 340.0 MHz to 8 375.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied.. |
| R4-2600647 | NTT DOCOMO, INC. | CR for TS 38.101-1 Correction of the DL Fc on MSD table for PC1.5 CA_n1-n78 |
| R4-2601045 | ZTE Corporation, Sanechips | CR for TS 38.101-3 to correct the test frequency for DC_8_n40-n71 and notes for DC_2_n77 |
| R4-2601046 | ZTE Corporation, Sanechips | CR for TR 38.719-03-01 to correct UL transmission bandwidth symbol LCRB in MSD tables |
| R4-2601129 | Ericsson | CR for TS 36.101 correcting LTE CA configuration: Correcting so that CA_4A-4A-7A refers to CA_4A-4A Moving 71 to correct column for CA_40A-71A Correcting rows between CA_1A-1A-3C-38A and CA_1A-3A-3A-38A Correcting rows between CA_7A-32A-46E and CA_7A-38C-66A Correcting rows between CA_3A-3A-7A-8B and CA_3A-3A-7A-7A-8A |
| R4-2601499 | Rakuten Mobile, Inc | CR for TS 38.101-1 for n28 3MHz UL/5MHz DL Asymmetric BW change from Optional to Mandatory |
| R4-2601670 | Nokia, BT Plc | CR to TS 38.101-1 Corrections to BT band combinations with band n1, n3, n7, n28, n75 and n78 |
| R4-2601863 | Rakuten Mobile, Inc | Proposal 1: Rakuten Mobile request RAN4 to approve CR R4-2601499 to define the n28 asymmetric 3 MHz UL / 5 MHz DL case under BCS0 in TS 38.101-1 Table 5.3.6-1, so that UE support becomes mandatory in Release-19 through the existing asymmetric BCS0 rule in TS 38.306. |
| R4-2602028 | T-Mobile USA | CR for TS 38.101-1 Corrections for HPUE notes Adds the superscript notes 3,5 for n41 and n77 for downlink CA_n25(2A)-n41A-n66A-n71A-n77A, and corrects 3,4 to 3,5 in several other places |
| R4-2602029 | T-Mobile USA | CR for TS 38.101-1 Corrections for HPUE MOP table Adds 29 dBm for CA_n25A-n41A to Table 6.2A.1.3-1 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: PC1 in inter-band CA

Issue 1-1-1: Signaling support for PC1 in inter-band CA and Release independence

- Proposals

  - Proposal 1: Send LS to RAN2 informing them that PC1 for inter-band CA should be enabled for both NR and NR-DC (Qualcomn).

  - Proposal 2: Enable PC1 support for inter-band CA for both NR and NR-DC from Release-17(Qualcomn).

- Recommended WF

  - Check whether proposals 1 and 2 are agreeable

Issue 1-1-2: LS to RAN2 on the signalling(Qualcomn)

ACTION: RAN4 kindly requests RAN2 to take the following actions: - Define signaling to enable UE Power Class 1 for inter-band UL CA for both NR and NR-DC configurations. - Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17

- Recommended WF

  - Check whether send and revise the LS R4-2600582 based on agreements in issue 1-1-1.

Sub-topic 1-2: Correcting emission requirements for NS_10N for NR NTN band n251

Issue 1-2-1: Correcting emission requirements for NS_10N for NR NTN band n251

- Proposals

  - Proposal 1: To align with EN 301 681 recommendation, update NS_10N emission table in TS 38.101-5 (Table 6.5.3.3.9-1) with changes (highlighted in blue) as shown in Table below and update the associated A-MPR values as part of Rel-19 maintenance work related to n251 at the RAN4 next meeting (Viasat).

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 (Note 4) | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz NOTE 4: In the bands 3 253.0 MHz to 3 321.0 MHz and 3 336.0 MHz to 3 350.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -8 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 4 879.5 MHz to 4 981.5 MHz and 5 004.0 MHz to 5 025.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 6 506.0 MHz to 6 642.0 MHz and 6 672.0 MHz to 6 700.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 8 132.5 MHz to 8 302.5 MHz and 8 340.0 MHz to 8 375.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. |  |  |  |

- Recommended WF

  - Firstly, check proposal 1 for TS 38.101-5 (Table 6.5.3.3.9-1) with changes (highlighted in blue) can be agreeable, then discuss whether to update AMPR in RAN4#118bis meeting.

Sub-topic 1-3: Change the n28 3 MHz UL/5MHz DL Asymmetric BW from optional to mandatory in Release 19

Sub-topic description

In RAN4 #117 meeting, CR (R4-2522167) to 38.101 for 3MHz BW change from optional to Mandatory is postpone due to the flags by other companies. There is no consensus and agreement for this issue. Comments were shown as below for reference.

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2522167 (R19 38.101-1) | Rakuten Mobile, Inc, T-Mobile,Anterix,Southern Linc | Huawei (Mohammad): Flag R4-2522167: Requires chip redesign and has very high impact and risk of UEs not supporting these bands at all. 3MHz shall remain optional. Skyworks: Note 7 of Table 5.3.5-1 should be updated in case RAN4 agrees to mandate the support of 3MHz CBW for Band n28 UL/DL operation. MediaTek (Suhwan): Flag R4-2522167: The change from optional to mandatory for 3MHz CBW for (e)RedCap UE is required big changes in Baseband. So it is difficult to change with mandatory for 3MHz CBW. we prefer to keep the legacy agreements just support asymmetric CBW between UL and DL in n28. T-Mobile USA (Bill) 3 MHz for n85 is not for Redcap only, but for MBB. We have 6 MHz of n85 and need to keep 3 MHz for Cat-M LTE on B12 for another decade. We want to use the other 3 MHz for NR. We're not aware of any other operators using n85. Maybe the Canadians? If we deploy 3 MHz of n85 for coverage and devices don't support 3 MHz for n85, users may experience coverage holes that could have been avoided. | Revised |

Issue 1-3-1: Change the n28 3 MHz UL/5MHz DL Asymmetric BW from optional (BCS1) to mandatory (BCS0) in Release 19

- Proposals

  - Proposal 1: Rakuten Mobile request RAN4 to approve CR R4-2601499 to define the n28 asymmetric 3 MHz UL / 5 MHz DL case under BCS0 in TS 38.101-1 Table 5.3.6-1, so that UE support becomes mandatory in Release-19 through the existing asymmetric BCS0 rule in TS 38.306 (Rakuten Mobile).

- Recommended WF

  - Firstly, to check whether companies can accept 3MHz for n28 from optional to mandatory in Release 19.

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2601499 (R19 38.101-1) | Rakuten Mobile, Inc |  |  |

Sub-topic 1-4: n71 with UL CBW 25MHz

Sub-topic description

In RAN4 #117 meeting, CR was agreed (R4-2522388) for PC3 and PC2 MSD CA_n29-n71 with UL 25MHz CBW at n71. Therefore, there is no need to discuss Apple (R4-2600142) in this meeting.

CRs for TS 38.101-1 (6)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600140 | Apple |  |  |
| R4-2600336 | CATT |  |  |
| R4-2600647 | NTT DOCOMO, INC |  |  |
| R4-2601670 | Nokia, BT Plc |  |  |
| R4-2602028 | T-Mobile USA |  |  |
| R4-2602029 | T-Mobile USA |  |  |

CRs for TS 38.101-3 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600141 | Apple |  |  |
| R4-2601045 | ZTE Corporation, Sanechips |  |  |

CRs for TS 38.101-5 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600127 | Apple,Qualcomm |  |  |
| R4-2600584 | Viasat |  |  |

CRs for TS 36.101 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600139 | Apple |  |  |
| R4-2601129 | Ericsson |  |  |

CRs for TS 38.104 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600337 | CATT |  |  |

CRs for TS 38.307 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600339 | CATT |  |  |

CRs for TR 38.719-03-01 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2601046 | ZTE Corporation, Sanechips |  |  |

CRs for TR 38.863 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600126 | Apple |  |  |
| R4-2600128 | Apple |  |  |