3GPP TSG-RAN WG4 Meeting # 118					Rev_R4-2601677

Gothenburg Meeting, Feb. 9th – Feb. 13th, 2026

Title:	TP to TR 38.7xx-02-01 CA_n5-n13

Source:	Nokia, Verizon

Agenda item:	5.6.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.7xx-02-01 to add CA_n5B-n13A with ULCA CA_n5B. The contiguous intra-band uplink CA is not increasing the uplink transmission bandwidth beyond the already achievable bandwidths of 25MHz, so requirements are kept as included in the following analysis.

************************************* Start of TP*****************************************

## 5.x	CA_n5-n13

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n5+n13

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz - 849 MHz | 869 MHz - 894 MHz | FDD |
| n13 | 777 MHz - 787 MHz | 746 MHz - 756 MHz | FDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? - N/A

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n5+n13

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5B-n13A | CA_n5B | n5 | CA_n5B_BSC4 and 5 | 4 and 5 |
|  |  | n13 | See n13 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n13.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n2 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n13 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 746 | 756 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1492 | 1512 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2238 | 2268 | - | - | N/A | - | N/A |  |
| DL4 | 2984 | 3024 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3730 | 3780 | - | D | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL5 |  |  |  |  |  |
| UL/DL harmonics |  | n13 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 777 | 1554 | 2331 | 3108 | 3885 |  |
| n2 | fLow | fHigh | 787 | 1574 | 2361 | 3148 | 3935 |  |
| DL1 | 1930 | 1990 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3860 | 3980 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5790 | 5970 | - | - | N/A | - | N/A |  |
| DL4 | 7720 | 7960 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9650 | 9950 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n5-n13.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n5 |  | n13 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 824 | 849 | 777 | 787 |
| fDL (MHz) | 869 | 894 | 746 | 756 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 25 | 5 | 10 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 799 | 874 | 767 | 797 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 774 | 899 | 757 | 807 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 749 | 924 | 747 | 817 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 724 | 949 | 737 | 827 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 699 | 974 | 727 | 837 |
| Analysis | There is an issue of >ACLR2. affects n13 |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.x.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for headline

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n5 fUL | 824 | 849 | Order | flow | fhigh |
| n13 fDL | 746 | 756 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 25 |  | 10 | 25 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 50 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 799 | 874 |  | 30 | 75 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 774 | 899 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 749 | 924 |  | 1623 | 1723 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 724 | 949 |  | 1598 | 1748 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 699 | 974 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 674 | 999 |  | 2447 | 2572 |
| Analysis | There is an issue of IMD5 into n13 |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n13, the TIB,c and RIB,c values have already been specified.

#### 5.x.1.5	REFSENS requirements

There are no new requirements to the Refsens defined under this clause. This is treated in next clause.

### 5.x.2	Specific for UL CA

#### 5.x.2.1	Maximum output power for intra-band CA

Table 5.x.2.1-1: UE Power Class for uplink intra-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5B | 23 | +2/-3 |

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n5 + Band n13 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n2 and Band n13 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1063 - 1133 |  | 2627 - 2697 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 - 3043 |  | 276 - 356 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 - 4607 |  | 3404 - 3484 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 - 4953 |  | 421 - 511 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2126 - 2266 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6327 - 6517 |  | 4181 - 4271 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5254 - 5394 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1198 - 1298 |  | 6613 - 6863 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1339 - 1489 |  | 3976 - 4176 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4958 - 5058 |  | 8177 - 8427 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 - 6181 |  | 7104 - 7304 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, 3rd order IMD may also fall into Rx frequencies of bands n5 and band n13.

#### 5.x.2.3	REFSENS requirements

MSD for n5B UL is already defined within TS 38.101-1 Table 7.3A.2.1-1 but not for the specific DL CA configuration This is added below. Note MSD for IMD3 is already included in the specification.

Table 5.x.2.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n13 | n5 | 826.5 | 5 | 4 (RBSTART=0) | 871.5 | N/A18 | FDD | N/A |
|  |  | 839 | 20 | 16 (RBSTART=64) | 884 |  | FDD |  |
|  | n13 | N/A | 5 | N/A | 753.5 | 26.2 | FDD | IMD9 |
| NOTE 18: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified. |  |  |  |  |  |  |  |  |

************************************* End of TP******************************************