3GPP TSG-RAN WG4 Meeting # 118					Rev_R4-2601676

Gothenburg Meeting, Feb. 9th – Feb. 13th, 2026

Title:	TP to TR 38.7xx-02-01 CA_n2-n13

Source:	Nokia, Verizon

Agenda item:	5.6.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.7xx-02-01 to add CA_n2-n13 including ULCA.

************************************* Start of TP*****************************************

## 5.x	CA_n2-n13

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n2+n13

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850 MHz - 1910 MHz | 1930 MHz - 1990 MHz | FDD |
| n13 | 777 MHz - 787 MHz | 746 MHz - 756 MHz | FDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? - N/A

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n2+n13

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n13A | CA_n2A-n13A | n2 | See n2 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n13 | See n13 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n2(2A)-n13A | CA_n2A-n13A | n2 | CA_n2(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n13 | See n13 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n13.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n2 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n13 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 746 | 756 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1492 | 1512 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2238 | 2268 | - | - | N/A | - | N/A |  |
| DL4 | 2984 | 3024 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3730 | 3780 | - | D | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL5 |  |  |  |  |  |
| UL/DL harmonics |  | n13 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 777 | 1554 | 2331 | 3108 | 3885 |  |
| n2 | fLow | fHigh | 787 | 1574 | 2361 | 3148 | 3935 |  |
| DL1 | 1930 | 1990 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3860 | 3980 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5790 | 5970 | - | - | N/A | - | N/A |  |
| DL4 | 7720 | 7960 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9650 | 9950 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

The analysis shows harmonic mixing between 2nd harmonic of n2 uplink into 5th harmonic of n13 downlink.

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n2-n13.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n13 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1850 | 1910 | 777 | 787 |
| fDL (MHz) | 1930 | 1990 | 746 | 756 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 40 | 5 | 10 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 767 | 797 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 757 | 807 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 747 | 817 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 737 | 827 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 727 | 837 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n13, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n2-n13 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n2-n13 | - | 0.2 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENS requirements

Based on the co-existence analysis, harmonic mixing requirements must be defined.

Table 5.x.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n2 | n13 | 5 | 15 | 12 | 5 | TBD | Note 14 | UL2/DL5 |
| NOTE 14: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.250.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n13A | 23 | +2/-3 |

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n2 + Band n13 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n2 and Band n13 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1063 - 1133 |  | 2627 - 2697 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 - 3043 |  | 276 - 356 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 - 4607 |  | 3404 - 3484 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 - 4953 |  | 421 - 511 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2126 - 2266 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6327 - 6517 |  | 4181 - 4271 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5254 - 5394 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1198 - 1298 |  | 6613 - 6863 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1339 - 1489 |  | 3976 - 4176 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4958 - 5058 |  | 8177 - 8427 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 - 6181 |  | 7104 - 7304 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, there is no IMD issues of this combination.

Protected bands are re-used from the EN-DC combination DC_13_n2.

Table 5.x.2.2-3 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2.2-3: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n13 | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.x.2.3	REFSENS requirements

There are no requirements for Refsens, based on the IMD analysis.

************************************* End of TP******************************************