3GPP TSG RAN WG4 Meeting #118	R4-2601194

Gothenburg, SE., 9th – 13th, Feb. 2026

# Source:	ZTE Corporation

# Title:	TP for TR38.719-02-01_CA_n39A-n104A

# Agenda Item:	5.6.3

# Document for:	Approval

- Introduction

CA_n39A-n104A_BCS0 and BCS 4 and 5 were included in the R20 basket WID. In this contribution, we provide a TP to the TR38.720-02-01[1] to introduce CA_n39A-n104A.

- Reference

- [1] RP-253849, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei

- [2] TR38.720-02-01,Rel-20 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n39-n104

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n39+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n39 | 1880 | - | 1920 MHz | 1880 MHz | - | 1920 MHz | TDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n39+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n104A | CA_n39A-n104A | n39 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n39A-n104A | CA_n39A-n104A | n39 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | n104 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n39-n104.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n39 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1880 | 3760 | 5640 | 7520 | 9400 | MSD type |
| n104 | fLow | fHigh | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There are no Harmonic/Harmonic mixing issue of n39 ULx / n104 DLy |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n39 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 1880 | 1920 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3760 | 3840 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5640 | 5760 | - | - | N/A | - | N/A |  |
| DL4 | 7520 | 7680 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9400 | 9600 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There are no Harmonic/Harmonic mixing issue of n104 ULx / n39 DLy |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n39-n104.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n39 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1880 | 1920 | 6425 | 7125 |
| fDL (MHz) | 1880 | 1920 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 100 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1780 | 2020 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1680 | 2120 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1580 | 2220 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1480 | 2320 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1380 | 2420 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n39 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n39 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n39 DL up to ACLR 5 according to the calculation. Moreover, n39 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

      - 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n39-n104, the TIB,c and RIB,c requirements are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n39-n104 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n41 the band order from left to right is n1 and n41. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n39-n104 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

      - 5.x.1.5	REFSENS requirements

There are no harmonic/cross band isolation co-existence problem according to UE co-existence study in section 5.x.1.3, so there is need to defined additional REFSEN requirements(MSD).

      - 5.x.1.6	OOB blocking exception requirements

No OOB blocking issues for CA_n39-n104.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n104A | 23 | +2/-3 |

      - 5.x.2.2	UE co-existence studies for 2 bands UL

Since both n39 and n104 are TDD band, so there is no need to check the 2UL IMD for the UE-to-UE coexistence analysis.

Also there is no need to list the protected band of n39 and n104 since it can refer to the intersection of the requirements for the individual bands specified

      - 5.x.2.3	REFSENS requirements

There are no IMD co-existence problem according to UE co-existence study in section 5.x.2.2, so there is no need to defined additional REFSEN requirements(MSD).

----- End of TP -----