3GPP TSG-RAN WG4 Meeting #118	R4-2600926

# Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6365 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on test case for 5MHz Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The unit of Io (dBm/xxxMHz) is not aligned with the configured bandwidth. - Some Io values are wrong. And for the test cases with PSCell (A.6.6.1.14 and A.6.6.1.15), 10MHz bandwidth is configured for PCell and 3MHz bandwidth is configured for PSCell. Therefore, separate Io values should be set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To correct the above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test configuration will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.11.2 A.6.5.12.3 A.6.5.12.4 A.6.6.1.14 A.6.6.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

A.6.5.11.2	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz

A.6.5.11.2.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delay under NR-DC with a bandwidth of less than 5 MHz meets the requirements defined in Clauses 8.9.2 and 8.9.3, where the PSCell is unknown to the UE at the time of addition.

Supported test configurations are shown in Table A.6.5.11.2.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 and radio channel 2 in FR1, respectively. Test parameters are given in Table A.6.5.11.1.1-2. The cell-specific parameters as specified in Table A.6.5.11.1.1-3 with config 1 apply except those specified in Table A.6.5.11.2.1-2.

The test procedure specified in A.6.5.11.1 applies to this test.

Table A.6.5.11.2.1-1: Supported test configurations for FR1 PSCell Addition and Release with less than 5 MHz

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 3 MHz (12 PRB SSB), FDD |

Table A.6.5.11.2.1-2: Cell Specific Parameters for FR1 PSCell Addition and Release with less than 5 MHz

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 | T4 |
| BWchannel | MHz | 1 |  | 3: NPRB,c = 15 |  |  |  |  |
| Data PRBs allocated |  | 1 |  | 12 |  |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 |  | SR.1.3 FDD |  |  |  |  |
| RMSI CORESET parameters |  | 1 |  | CR.1.2 FDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 |  | CCR.1.6 FDD |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 |  | 15 |  |  |  |  |
| SSB configuration |  | 1 |  | SSB.13 FR1 |  |  |  |  |
| IoNote | dBm/2.7MHz | 1 |  | N/A |  | –59.4 dBm |  |  |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

< Non-changed part is omitted >

------------ END OF CHANGE 1--------------

------------ START OF CHANGE 2--------------

A.6.5.12.3	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

A.6.5.12.3.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell change with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.11E.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.2 and  intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.3 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.4 and handover with PSCell delay defined in clause A.6.3.1.19.

A.6.5.12.3.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.3.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell1 is on radio channel 1 in FR1. Cell 2 and 3 are on radio channel 2 in FR1. General test parameters as specified in table A.6.5.12.1.2-2 with config 1 apply except those specified in table A.6.5.12.3.2-2. Cell specific test parameters as specified in table A.6.5.12.1.2-3 with config 1 apply except those specified in table A.6.5.12.3.2-3.

The test procedure specified in clause A.6.5.12.1.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.3.2-1: Supported test configurations for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode. |

Table A.6.5.12.3.2-2: General Test Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2, Cell 3 | Neighbour cells with 12 PRB SSB bandwidth on RF channel number 2. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 2. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |

Table A.6.5.12.3.2-3: Cell Specific Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.6 FDD |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |
| IoNote1 | dBm/2.7 MHz | Config 1 | -75.45 | -66.81 | -62.46 | -75.45 | -66.81 | -62.46 |
| Propagation Condition |  | Config 1 | AWGN |  |  | AWGN |  |  |
| NOTE 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

< Non-changed part is omitted >

------------ END OF CHANGE 2--------------

------------ START OF CHANGE 3--------------

A.6.5.12.4	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

A.6.5.12.4.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell addition with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.9C.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.2 and intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth in clause A.6.5.12.3 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.4 and handover with PSCell delay defined in clause A.6.3.1.19.

A.6.5.12.4.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.4.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Cell 3 is on radio channel 3 in FR1.

General test parameters as specified in table A.6.5.12.2.2-2 with config 1 apply except those specified in table A.6.5.12.4.2-2. Cell specific test parameters as specified in table A.6.5.12.2.2-3 with config 1 apply except those specified in table A.6.5.12.4.2-3.

The test procedure specified in clause A.6.5.12.2.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.4.2-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode. |

Table A.6.5.12.4.2-2: General Test Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |
|  |  |  | Cell 3 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 3. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 3. |
|  | Neighbour Cell |  | Cell 2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2 |

Table A.6.5.12.4.2-3: Cell Specific Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter | Unit | Test configuration | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.6 FDD |  |  |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |  |  |
| IoNote1 | dBm/2.7 MHz | Config 1 | -62.4 | -59.4 | -62.4 | -62.4 | -62.4 | -62.4 | -62.4 | -59.4 |
| Propagation Condition |  | Config 1 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

< Non-changed part is omitted >

------------ END OF CHANGE 3--------------

------------ START OF CHANGE 4--------------

A.6.6.1.14	Deactivated PSCell measurement test with 12 PRB SSB bandwidth in FR1

A.6.6.1.14.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of deactivated PSCell measurement results when the PSCell is configured with 12 PRB SSB bandwidth. This test verifies the deactivated PSCell measurement period requirements specified in clause 9.2.5.1.

A.6.6.1.14.2	Test Parameters

The supported test configurations are given in table A.6.6.1.14.2-1. Only 15kHz FDD cases are considered. The test scenario comprises 2 NR cells – PCell (Cell 1) and target PSCell (Cell 2).

Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Test parameters are given in Tables A.6.6.1.14.2-2, A.6.6.1.14.2-3 below. Note that for Cell 2 the SSB configuration refers to SSB pattern 13 in FR1: SSB allocation for SSB SCS=15kHz in 3 MHz. In the test, the SSB is configured with 12PRB bandwidth.

The test consists of two successive time periods, with time duration of T1, and T2 respectively.

At the start of T1, the UE is in connected mode to Cell 1. During time duration T1, the UE is configured to add Cell 2 as PSCell so that Cell 1 and Cell 2 serve UE under NR-DC operation. Meanwhile the UE is configured to measure on both Cell 1 and Cell 2 serving carrier frequencies. The UE is configured to read the SSB index for measurement of PSCC.

At the end of T1, the UE deactivates Cell 2. T2 starts exactly when the UE receives deactivation command for Cell 2. During T2, Cell 2 remains detectable to the UE.

Measurement gap or DRX is not configured.

The UE is required to make correct and timely reporting during T2. This test case verifies that the UE reporting is within the delay requirement and measurement accuracy is tested in other test cases.

Table A.6.6.1.14.2-1: Supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, PCell with 10 MHz bandwidth and PSCell with 3 MHz bandwidth, both FDD duplex mode |

Table A.6.6.1.14.2-2: General test parameters for deactivated PSCell measurements when PSCell is configured with 12 PRB SSB bandwidth

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR1 NR carrier frequencies are used in this test. |
| PCell |  | Config 1 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Deactivated PSCell |  | Config 1 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| measCyclePSCell | ms | Config 1 | 160 | Same as SMTC and SSB periodicity |
| Gap Pattern Id |  | Config 1 | Not configured |  |
| Measurement gap offset |  | Config 1 | Not configured |  |
| SMTC parameters |  | Config 1 | SMTC.1 for PCC SMTC.2 for PSCC | As specified in clause A.3.10.1 |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 3 |  |

Table A.6.6.1.14.2-3: Cell specific test parameters for deactivated PSCell measurement with 12 PRB SSB bandwidth

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP BW |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | DLBWP.0.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | SR.1.2FDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | CR.1.2 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | CCR1.6 FDD |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.13 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1 | -98 |  | -98 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -91 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
| IoNote3 |  | dBm/9.36 MHz | Config 1 | -64.60 |  | NA |  |
|  |  | dBm/2.7MHz | Config 1 | NA |  | -67.66 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

A.6.6.1.14.3	Test Requirements

The UE shall send one measurement report for the deactivated PSCell, with a measurement reporting delay less than 2720 ms from the beginning of time period T2.

The UE is required to successfully detect PSS/SSS for Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

The UE is required successfully read the SSB index for Cell 2 within

Ceil(7 x Kp) x measCyclePSCell x CSSFintra;

The UE is required to successfully measure on Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

Where

-	measCyclePSCell = 160 ms,

-	Kp = 1,

-	CSSFintra = 1.

This sums up as 2720 ms in total, consisting of 800 ms of PSS/SSS detection, 1120 ms of SSB index detection and 800 ms of measurement time.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

< Non-changed part is omitted >

------------ END OF CHANGE 4--------------

------------ START OF CHANGE 5--------------

A.6.6.1.15	SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell

A.6.6.1.15.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 makes correct reporting of an event on an intra-frequency layer measurement performed without measurement gaps when the UE is also configured deactivated SCell. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

A.6.6.1.15.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on Carrier 1, FR1 Neighbouring cell (Cell 2) and a FR1 SCell (Cell 3) on Carrier 2. Carrier 2 is on a different frequency than the PCell.

The test parameters for PCell, SCell and neighbour cell are given in table A.6.6.1.15.2-1, A.6.6.1.15.2-2 and A.6.6.1.15.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and the SCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.15.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; Neighbouring cell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode Scell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.15.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | PCell |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| Deactivated SCell |  |  | Cell 3 | Deactivated throughout the test. |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 and Cell 3 | 1: Cell 1 is PCell 2: Cell 2 is neighbouring cell to be detected |
| SSB configuration |  |  | SSB.1 FR1 | Cell 1 |
|  |  |  | SSB.13 FR1 | 12 PRB Cell 2 and Cell 3 |
| SMTC configuration |  |  | SMTC.2 | Cell 1 |
|  |  |  | SMTC.1 | Cell 2 and Cell 3 |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  |  | Normal |  |
| Hysteresis | dB |  | 0 |  |
| Time To Trigger | s |  | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| measCycleScell | ms |  | 160ms |  |
| Time offset between serving and neighbour cells |  |  | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  |  | 3 s | Synchronous cells |
| T1 | s |  | 5 |  |
| T2 | s |  | 5 |  |

Table A.6.6.1.15.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| PDSCH RMC configuration |  |  | SR.1.1 FDD |  | SR.1.3 FDD |  |  |  |
| RMSI CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 FDD |  |  |  |
| IInitial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Active DL BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |  |  |  |  |
| RLM-RS |  |  | SSB |  |  |  |  |  |
| Note 2 | dBm/SCS |  | -98 |  |  |  |  |  |
| Note 2 | dBm/15 kHz |  | -98 |  |  |  |  |  |
|  | dB |  | 4 | 4 | -Infinity | -1.46 | 4 | -1.46 |
|  | dB |  | 4 | 4 | -Infinity | 4 | 4 | 4 |
| SS-RSRP Note 3 | dBm/SCS |  | -94 | -94 | -Infinity | -94 | -94 | -94 |
| Io | dBm/9.36 MHz |  | -64.60 | -64.60 | NA | NA | NA | NA |
|  | dBm/2.7 MHz |  | NA | NA | -70.00 | -67.65 | -70.00 | -67.65 |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.6.1.15.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2720 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

< Non-changed part is omitted >

------------ END OF CHANGE 5--------------