# 3GPP TSG-RAN WG4 Meeting #118	R4-2601942

# Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Intra-frequency L3 for a UE configured with LB CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements are agreed to be supported LB CA via switching. The TC shoukld be defined to test the feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC on Intra-frequency L3 measurement for activated SDL SCell and FDD PCell for a UE configured with LB CA via switching |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------- Start of changes ---------------------------

#### A.6.6.1.X	SA event triggered reporting test for a UE configured with LB CA via switching

##### A.6.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that a UE supporting LBCA-SwitchingPattern performs correct event triggered reporting for intra-frequency measurements on the FDD PCell and SDL SCell. This test will partly verify the intra-frequency measurement period requirements as specified in Clauses 9.2.5.1 and 9.2.5.2. In this test configuration, DRX and measurement gaps are not configured.

##### A.6.6.1.X.2	Test parameters

In this test setup, three FR1 cells are deployed. Cell 1 is the PCell, operating on FDD Carrier 1. Cell 2 is an intra-frequency neighbouring cell, also on FDD Carrier 1. Cell 3 is an SCell, operating on Carrier 2, which is on a different frequency than the PCell and is configured as a supplementary downlink (SDL) SCell.

In the measurement control information, a measurement object (MO) is configured for both the PCell frequency and the SCell frequency. The UE is instructed to perform event-triggered reporting using:

- Event A3 for the MO of the PCell

- Event A6 for the MO of the SCell

The test consists of two successive time periods, with durations T1 and T2, respectively. During T1, the UE shall not have any timing information for Cell 2. Before start of the T1, SCell is configured and activated.

Table A.6.6.1.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; Neighbouring cell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: 15 kHz SSB SCS, 10 MHz bandwidth, SDL mode |

In the test,

- the switching from PCell to SCell shall end at the end of the last slot from a sequence of consecutive slots with bit value ‘0’ and the switching to SCell shall not start N1 symbols before the last symbol of the last slot in that sequence.

- the switching from SCell to PCell shall end at the end of the last slot from a sequence of consecutive slots with bit value ‘1’ and the switching to PCell shall not start N2 symbols before the last symbol of the last slot in that sequence.

Table A.6.6.1.X.2-2: Test parameters for LB CA switching

| Parameter | Values | Comments |
| --- | --- | --- |
| SwitchingPattern | 0000001111111110000 000011100000111111000 | Switching pattern is bitmap with periodicity of 40ms, and hence the pattern is 40 bits long. In the bitmap, a bit value of ‘0’ for a first slot indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot indicates that the UE can receive on the SCell in the second slot |
| SwitchingGap-Duration for PCell to SCell (N1) | 1 | For a UE supporting 35 µs RF switching delay |
|  | 3 | For a UE supporting 140 µs RF switching delay |
| SwitchingGap-Duration for SCell to PCell (N2) | 2 | For a UE supporting 35 µs RF switching delay |
|  | 4 | For a UE supporting 140 µs RF switching delay |

Table A.6.6.1.X.2-2: General test parameters for SA intra-frequency event triggered reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 | PCell |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Activated SCell |  | Cell 3 | Activated throughout the test. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 2: Cell 3 | 1: Cell 1 is PCell and neighbour cell to be detected |
| SSB configuration |  | SSB.1 FR1 | Cell 1 and Cell 2 |
|  |  | SSB.14 FR1 | Cell 3 |
| SMTC configuration |  | SMTC.2 | Cell 1 and Cell 2 |
|  |  | SMTC.13 | Cell 3 |
| A3-Offset | dB | -4.5 |  |
| A6-Offset | dB | -4.5 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 | For both A3 and A6 events |
| Time To Trigger | s | 0 | For both A3 and A6 events |
| Filter coefficient |  | 0 | L3 filtering is not used for both A3 and A6 events |
| DRX |  |  | OFF |
| measCycleSCell | ms | 160ms |  |
| Time offset between cells |  | 3 s | MRTD between Cell 1 and Cell 3 |
|  |  | 33µs | MRTD between cell 1 and 2 |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| THARQ | ms | 2 | 2k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case |

Table A.6.6.1.X.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| PDSCH RMC configuration |  |  | SR.1.1 FDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | CR.1.1 FDD |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | CCR.1.1 FDD |  |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 FDD |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Active DL BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |  |  |  |  |
| RLM-RS |  |  | SSB |  |  |  |  |  |
| Note 2 | dBm/SCS |  | -98 |  |  |  |  |  |
| Note 2 | dBm/15 kHz |  | -98 |  |  |  |  |  |
|  | dB |  | 4 | -1.46 | -Infinity | -1.46 | 4 | -1.46 |
|  | dB |  | 4 | 4 | -Infinity | 4 | 4 | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz |  | -94 | -94 | -Infinity | -94 | -94 | -94 |
| Io | dBm/9.36 MHz |  | -64.60 | -62.25 | --64.60 | -62.25 | -64.60 | -62.25 |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

--------------------END OF CHANGES--------------------------