3GPP TSG-RAN4 Meeting #118	R4-2601989

# Gothenburg, Sweden, February 09th – 13th, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test-cases on L1-SRS-RSRP measurement in FR1/FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2026-1-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2522639, TC1-2: L1-SRS-RSRP measurement in FR1/FR2 need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test-cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC on L1-SRS-RSRP measurement in FR1/FR2 are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.x.1 (new), A.7.6.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change 1>

#### A.6.6.x.1	L1-SRS-RSRP measurement with DRX with SBFD

##### A.6.6.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SRS-RSRP measurement when configured with SBFD in DU operation. This test will verify the L1-SRS-RSRP measurement requirements in clause 9.18.2 with the testing configurations for NR cells in table A.6.6.x.1.1-1.

Table A.6.6.x.1.1-1: Applicable NR configurations for FR1 L1-SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.6.6.x.1.2	Test Parameters

One cell is deployed in the test, which is FR1 PCell (Cell 1). The test parameters for PCell is given in table A.6.6.x.1.2-1 and A.6.6.x.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SRS-RSRP measurement on SRS-RSRP-MeasurementResourceSet configured in a CSI-ResourceConfig and report aperiodically. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During the test, the test system transmits SRS resource for measurement in the SBFD slot according to the SRS configuration in table A.6.6.x.1.2-4 and the test parameters for the (virtual) neighbour cell UE in table A. 6.6.x.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

At 640ms after the beginning of time period T2, the DCI triggering L1-SRS-RSRP reporting is sent to UE and UE provides the report back based on the reporting configuration.

Table A.6.6.x.1.2-1: General test parameters for L1-SRS-RSRP reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.14 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.x.1.2-4 |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 17.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 2 |  |

Table A.6.6.x.1.2-2: NR Cell specific test parameters for L1-SRS-RSRP reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.2.4 |  |
| SBFD configuration |  | 1 | SBFD.1 FR1 |  |
| PDSCH RMC configuration |  | 1 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Note 2, 3 | dBm/15 kHz | 1 | -94.65 |  |
| Note 2, 3 | dBm/SCS | 1 | -91.65 |  |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: In this test Noc is transmitted in PRBs in UL subband in addition to PRBs in DL subbands. |  |  |  |  |

Table A.6.6.X.1.2-3: FR1 test parameters for aggressor UE

| Parameter | Unit | Test configuration | Aggressor UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
| Note 2 | dBm/SCS | 1 | -95 |  |
|  | dB | 1 | -infinity | 3 |
|  | dB | 1 | -infinity | 3 |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94 |
| Io | dBm/98.28 MHz | 1 | -59.84 | -54.39 |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.7.X.1.2-4: SRS configuration parameters

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | Sl640, 4 |
|  | sequenceId | 0 |

##### A.6.6.x.1.3	Test Requirements

Within 650 ms from the beginning of time period T2, the UE shall send L1-SRS-RSRP report.

The rate of correct events observed during repeated tests shall be at least 90%.

## <End of Change 1>

## <Start of Change 2>

#### A.7.6.x.1	L1-SRS-RSRP measurement with DRX with SBFD

##### A.7.6.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SRS-RSRP measurement when configured with SBFD in DU operation. This test will verify the L1-SRS-RSRP measurement requirements in clause 9.18.2 with the testing configurations for NR cells in table A.7.6.x.1.1-1.

Table A.7.6.x.1.1-1: Applicable NR configurations for FR1 L1-SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.6.x.1.2	Test Parameters

One cell is deployed in the test, which is FR2 PCell (Cell 1). The test parameters for PCell is given in table A.7.6.x.1.2-1 and A.7.6.x.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SRS-RSRP measurement on SRS-RSRP-MeasurementResourceSet configured in a CSI-ResourceConfig and report aperiodically. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During the test, the test system transmits SRS resource for measurement in the SBFD slot according to the SRS configuration in table A.7.6.x.1.2-4 and the test parameters for the (virtual) neighbour cell UE in table A. 6.6.x.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

At 640ms after the beginning of time period T2, the DCI triggering L1-SRS-RSRP reporting is sent to UE and UE provides the report back based on the reporting configuration.

Table A.7.6.x.1.2-1: General test parameters for L1-SRS-RSRP reporting for PCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.7.6.6.1.2-4 |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 7.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

Table A.7.6.x.1.2-2: NR Cell specific test parameters for L1-SRS-RSRP reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| SBFD configuration |  | 1 | SBFD.1 FR2 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |
| Antenna configuration |  | 1 | 1x2 |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: OCNG is not transmitted in the SRS-RSRP measurement resources. |  |  |  |  |

Table A.7.6.X.1.2-3: FR1 test parameters for aggressor UE

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15 kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak) |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/46.08 MHz Note3 | -61.03 | (Table B.2.7-2 Rx Beam Peak +[50.79] dB) |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2 dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.6.7.X.1.2-4: SRS configuration parameters

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | Sl160, 26 |
|  | sequenceId | 0 |

##### A.7.6.x.1.3	Test Requirements

Within 7 ms from the beginning of time period T2, the UE shall send L1-SRS-RSRP report.

The rate of correct events observed during repeated tests shall be at least 90%.

## <End of Change 2>