3GPP TSG-RAN WG4 Meeting # 118	R4-2600447

# Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6310 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_LPWUS-Perf) CR on TC of UE exit Case 3 to legacy where only LP-SS signal is used for OOK based LR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Technical updates for LR-WUR test cases based on endorsed draft Big CR R4-2522848. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test case on UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The tese case for LP-WUR will not be accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.XX.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### A.XX.1.3	UE exits relaxed measurement mode to legacy mode with LR using LP-SS signal

##### A.XX.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly exists the relaxed measurement mode to legacy mode with LR using LP-SS signal. This test will verify the evaluation requirement for the exit condition of RRM relaxation based on LP-SS for UEs configured with relaxed measurement criterion specified in clause 4.8.2.2.3.

##### A.XX.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.XX.1.3.2-1, A.XX.1.3.2-2 and A.XX.1.3.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. The LP-SS configuration 2 in A.3.X.2 will be used for LP-SS configuration.

Table A.XX.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.XX.1.3.2-2: General test parameters for FR1 intra-frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active Cell |  | 1, 2, 3 | Cell 1 | The UE camps on Cell 1 in the initial phase and enters into relaxed measurement mode before the end of T1 period |
|  | Neighbour Cells |  | 1, 2, 3 | Cell 2 |  |
| T2 end condition | Active Cell |  | 1, 2, 3 | Cell 1 | The UE shall fulfil the relaxed measurement criterion based on measurement and evaluation by MR during T1 period |
|  | Neighbour Cells |  | 1, 2, 3 | Cell 2 |  |
| T3 end condition | Active Cell |  | 1, 2, 3 | Cell 1 | During T2 period, the UE shall exist the relaxed measurement mode based on measurement and evaluation by LP-WUR using LP-SS, and reselect to Cell 2 based on measurement and evaluation by MR |
|  | Neighbour Cells |  | 1, 2, 3 | Cell 2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between Cells |  |  | 1 | 3 ms | Asynchronous Cells |
|  |  |  | 2 | 3 s | Synchronous Cells |
|  |  |  | 3 | 3 s | Synchronous Cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| LP-SS configuration |  |  | 1, 2, 3 | LP-SS.2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all Cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, the intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2. Before the end of T1, the UE enters relaxed measurement mode. |
| T2 |  | s | 1, 2, 3 | 15 | T2 is based on the LR evaluation period based on LP-SS plus MR wake up period. During T2, only LP-SS signal is transmitted and the SSB signal from Cell 1 will not be transmitted |
| T3 |  | s | 1, 2, 3 | 11 | T3 needs to be defined so that cell re-selection time is taken into account. |

Table A.XX.1.3.2-3: Cell specific test parameters for FR1 intra-frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in clause A.3.2.1 |  | OP.1 defined in clause A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
| for MR | dB | 1 | 2.79 | -3.11 | -3.11 | 2.79 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for LR |  | 1 | 8.21 | -9.11 | -9.11 | 8.21 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for MR | dB | 1 | 16 | 13 | 13 | 16 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for LR |  | 1 | 16 | 7 | 7 | 16 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -85 | -82 |
|  |  | 2 | -82 | -85 | -85 | -82 |
|  |  | 3 | -79 | -82 | -82 | -79 |
| LP-RSRP Note3 | dBm/SCS | 1 | -82 | -91 | -91 | -82 |
|  |  | 2 | -82 | -91 | -91 | -82 |
|  |  | 3 | -78.99 | -87.99 | -87.99 | -78.99 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 | -46.12 | -46.12 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1,2,3 | 60 | 60 | 60 | 60 |
| SLP_WUS_RelaxThresholdP_MR | dB | 1, 2, 3 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both Cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other Cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and LP-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.XX.1.3.3	Test Requirements

The delay for a UE existing the relaxed measurement mode to the legacy mode and performing cell re-selection to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The delay for a UE existing the relaxed measurement mode to the legacy mode and performing cell re-selection to an already detected cell shall be less than 10 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to an already detected cell for UE fulfilling with relaxed measurement criterion can be expressed as: 800ms + Tevaluate-LP-WUR-LP-SS + Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate-LP-WUR-LP-SS	See table 4.8.2.3-1 in clause 4.8.2.2.3

Tevaluate,NR_Intra	See table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a Cell; 1280 ms is assumed in this test case.

This gives a total of 9.6 s, allow 10 s for the cell re-selection delay to an already detected cell for UE fulfilling not-at-cell edge criterion in the test case.

==============End of change==============