3GPP TSG-RAN WG2 Meeting #133	draft R2-2601132

Gothenburg, Sweden, Feb. 09th – 13th

Source: 	RAN2 Vice Chairman (CATT)

Title:	Report from session on Rel-18/19 MIMO, Rel-19 LPWUS, SBFD, NR Others, Rel-19/20 AIPHY

Agenda item:	11.2

## Organizational email discussion

[AT133][200] Organizational – Rel-18/19 MIMO, Rel-19 LPWUS, SBFD, NR Others, Rel-19/20 AIPHY (RAN2 VC)

Scope:

a) Share plans for online/offline discussions during the meeting, and

b) Share draft session notes and agreements for review

#### 7.0.2.13 NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

# 8	Rel-19

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-252930)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.1.1	Organizational

LS, Rapporteur input,etc.

LS

R2-2600014	LS on definition of “ground truth label” in AI/ML-based Positioning Case 3a (R3-258796; contact: Huawei)	RAN3	LS in	Rel-19	NR_AIML_air-Core	To:RAN1	Cc:RAN2, SA2

Noted

R2-2600017	Reply LS on Continuous MDT (R3-258848; contact: Huawei)	RAN3	LS in	Rel-19	NR_AIML_NGRAN_enh-Core	To:SA5	Cc:RAN2

Noted

R2-2600045	Reply LS on OAM-centric solution for NW-side data collection (S5-255699; contact: Nokia)	SA5	LS in	Rel-19	TraceQoE_OAM	To:RAN2	Cc:RAN3

Noted

R2-2600048	LS on completion of Study on AI/ML consistency alignment (SP-251699; contact: Magenta)	SA	LS in	Rel-19	FS_AIML_CAL	To:RAN, CT, SA1, SA2, SA3, SA4, SA5, SA6, RAN1, RAN2, RAN3, RAN4, RAN5, CT1, CT3, CT4, CT6

Noted

On 38.331

R2-2601076	RRC Rapporteur Corrections to AIML for NR air interface	Ericsson	CR	Rel-19	38.331	19.1.0	5678	-	F	NR_AIML_air-Core

- ?? [AT133][214][AIPHY] CR for TS 38.331 (Ericsson)

Intended outcome: Agree the CR for TS 38.331 in R2-2601184

Deadline:  before the end of the meeting

On 37.355

R2-2600420	Addition of RAN4 DL AI/ML Positioning Capabilities	Qualcomm Incorporated	CR	Rel-19	37.355	19.1.0	0570	-	F	NR_AIML_air-Core

Agreed

R2-2600823	Discussion on corrections to AIML for positioning and beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

P1, P2

Proposal 1	RAN2 dummifies capability signaling multipleActivatedPRS-ProcessingWindows in LPP specification and adds the capability in RRC specification and TS 38.306 and sends an LS to RAN1 to notify the change.

Proposal 2	Remove sentence “A cell may contain only a single TRP” from field description nr-AIML-AssociatedID.

DISCUSSION

P1

-	Nokia think this belongs to R1, and if we change we should inform.

-	ZTE think it is reasonable to let gNB know this capability, but not sure why we remove from LPP. Qualcomm share this view.

-	Xiaomi think this was copy-paste issue by R1, so ok to change.

Add capability signaling multipleActivatedPRS-ProcessingWindows in TS 38.331 and 38.306. Send LS to RAN1 asking whether this capability is needed in LMF or only gNB

P2

-	Nokia think this part of the sentence is added based on R1 agreement.

- [AT133][215][AIPHY] LS to R1 on  capability multipleActivatedPRS-ProcessingWindows  (Ericsson)

Intended outcome: draft LS in R2-2601185

Deadline:  before CB

### 8.1.2	Corrections

Remaining issues for all related specs

On handling of inference configuration for mode B

R2-2600273	Corrections for L1 measurement configuration delivery for Option B	OPPO	discussion	Rel-19

Noted

Proposal 1: RAN2 confirms the following agreement is also applicable for Option B:

RAN2 confirms upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplet.

Proposal 2: RAN2 to agree the TP in section 5.

R2-2600465	Discussion on how to support periodic CSI inference when only option B is configured	Apple	discussion	Rel-19	NR_AIML_air-Core

Noted

Proposal 1: When option A is disabled (i.e. disableApplicabilityCSI-ReportConfig is present), the UE regards each CSI-ReportConfig included in the RRCReconfiguration message, whose reportConfigType is set to periodic and configurationForBM-PredictionAndDataCollection with reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, is applicable and submit the configuration in CSI-ReportConfig to the lower layers.

Proposal 2: Agree the CR in Appendix.

R2-2600559	On Left issues on R19 AIML For PHY	ZTE Corporation, Sanechips	discussion	Rel-19	NR_AIML_air-Core

Noted

Proposal 1: Modify the current text procedure regarding the flag disableApplicabilityCSI-ReportConfig for reflecting the below aspects:

-	it only can be configured when option B is configured

-	if configured ,Disable option A applicability reporting in either RRCReconfigurationComplete or UAI;

-	it is only available for AIML Beam management use case.

Proposal 2: There is no need for UE to determine whether to submit the CSI-ReportConfig where the ReportType is set to periodic to the lower layer if disableApplicabilityCSI-ReportConfig is configured. No spec change is needed.

R2-2601139	Correction for applicability and inference configuration procedures	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Noted

Observation 1: The submission of the inference configuration for periodic CSI to the lower layer is still needed if the inference configuration for periodic CSI is deemed applicable from Option B applicability point of view, even if Option A applicability reporting is disabled.

Proposal 1: Adopt the TP provided in section 4 so that UE actions of submitting applicable CSI-ReportConfig to lower layers are not dependent on whether disableApplicabilityCSI-ReportConfig is configured or not.

R2-2600823	Discussion on corrections to AIML for positioning and beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Noted

Proposal 3	Remove “Editor's Note: The details of how to consider the disableApplicabilityCSI-ReportConfig flag in the procedures still require further clarifications and it is still to be determined how and where the UE action of submitting the applicable periodic CSI configuration to the lower layers should be captured”.

Proposal 4	In the procedural text for applicability reporting in clause 5.3.5.3 adopt the changes as shown above, i.e.:

a.	replace “submit the configuration in CSI-ReportConfig to the lower layers” with “perform CSI reporting according to the configuration”; and

b.	remove the text referring to the action that the UE does not do, i.e. “do not submit...”, and add “and only if” to the preceding if clause.

DISCUSSION

-	LG do not think we need to change the spec, and think the periodic CSI report works well, think we just remove the editor notes. Samsung agree with LG, and think some improvement to the wording may be useful. Qualcomm agree.

-	Qualcomm think this is a corner case, so no need to enhance.

-	Nokia think there is an issue and think we can go with Apple proposal.

-	ZTE think majority’s view is option B only + periodic CSI works, and think nothing is broken without the changes.

-	Apple still think some clarification is needed, and think companies have different understanding about the UE behaviour. Xiaomi agree with Apple.

-	Ericsson think we should not violate the previous agreement. Qualcomm share this view.

[CB]

Other issues

R2-2600559	On Left issues on R19 AIML For PHY	ZTE Corporation, Sanechips	discussion	Rel-19	NR_AIML_air-Core

Noted

NW side data collection

Proposal 3: For UE to perform the logging of measurements, the reference logged measurement configuration is referred to the CSI-LoggedMeasurementConfig in CSI-MeasConfig rather than the  refCSI-LoggedMeasurementConfigId in VarCSI-LogMeasReport, and the reference cell is referred to the cell where the CSI-LoggedMeasurementConfig is configured rather than the cellId in VarCSI-LogMeasReport.

DISCUSSION

-	Ericsson think this is OK and think ZTE proposal can be dicussed togheter with Sharp’s similar proposal.

For UE to perform the logging of measurements, the reference logged measurement configuration is referred to the CSI-LoggedMeasurementConfig in CSI-MeasConfig rather than the  refCSI-LoggedMeasurementConfigId in VarCSI-LogMeasReport, and the reference cell is referred to the cell where the CSI-LoggedMeasurementConfig is configured rather than the cellId in VarCSI-LogMeasReport. Detailed wording can be further checking.

UE side data collection

Proposal 4: Change the information elements type of dataCollectionCandidateIdList-r19 and dataCollectionIdList-r19 from ‘optional’ to mandatory.

DISCUSSION

-	Samsung understand the intention but think no change is needed. Apple agree, and think this is NBC. LG, vivo Agree.

-	Ericsson think if we want to make a change then maybe we should change in the field description.

-	ZTE fine with only change the FD.

Capture in the field description that dataCollectionCandidateIdList and dataCollectionIdList are mandatory in this release of the specification.

R2-2600331	Corrections on applicability reporting in TS38.331	CATT	discussion	Rel-19	NR_AIML_air-Core

Noted

Correction on enable applicability reporting via UAI

Proposal 1: Remove the description about applicability reporting in section 5.3.5.9.

Change field description of applicabilityReportConfig

Proposal 2: Replace applicabilityReportConfig with applicabilityConfigToAddModList in the field description, and it’s “Indicates for serving cells the list of sets of prediction related parameters configured for UE applicability reporting.”.

P1

-	Ericsson, Qualcomm ok with this.

P2

-	Ericsson think we should not drop, but we can add more FD if needed. Qualcomm agree.

P1 and P2 will be taken into account in offline #214

P3, P4

-	Ericsson think it should be optional, because we may extend in the future releases. OPPO agree.

Optional or mandatory configuration id in Step 4

Observation 1: In current latest TS38.331, the configuration id is optional reported in applicability reporting, the network cannot get to know which configuration the reported applicability state refers to if the configuration id is not included, this applicability reporting doesn’t work and also affect the whole applicability reporting function.

Proposal 3: RAN2 confirm that the configuration id (i.e., applicabilityInfoReportId) in applicability reporting should be mandatory present.

Proposal 4: If P3 is agreed, RAN2 agree to add a restriction “This field is mandatory present.” in the field description of the configuration id (i.e., applicabilityInfoReportId) in applicability reporting.

R2-2600556	Remaining issues on AI/ML for NR air interface	Sharp	discussion	Rel-19

=> Revised in R2-2601140

R2-2601140	Remaining issues on AI/ML for NR air interface	Sharp	discussion	Rel-19

Noted

Proposal 1: Upon receiving csi-LoggedMeasurementConfigToAddModList, reset TTT for CSI logged measurement configuration with csi-LoggedMeasurementEventTriggerConfig.

Proposal 2: Adopt the Text Proposal 1 in Annex in RRC spec.

P1, P2

-	Ericsson think these are fine.

Upon receiving csi-LoggedMeasurementConfigToAddModList, reset TTT for CSI logged measurement configuration with csi-LoggedMeasurementEventTriggerConfig.

Proposal 5: Further clarify in the field description of CSI-SemiPersistentOnPUSCH-TriggerStateList that associatedReportConfigInfo refers to the reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig.

Proposal 6: Adopt the Text Proposal 3 in Annex in RRC spec.

P5, P6

-	Ericsson not sure whether we will create other issues because this is about adding FD to legacy fields.

-	Nokia think this comes from R1 so no need to change. LG agree. Sharp think R1 does not capture so we should capture in R2 spec.

R2-2600788	Correction on data collection	Nokia	discussion	Rel-19	NR_AIML_air-Core

Noted

Proposal 1: Align 37.320 with SA5 specifications and the change of title of clause 5.4.1.5 based on the draft CR in Annex A.

P1

-	Huawei not Ok with P1, do not think the section title should be changed. Xiaomi agree.

Align 37.320 with SA5 specifications, no changes to the current section title.

[AT133][216][AIPHY] CR for TS 37.320 (Nokia)

Intended outcome: Agree the CR for TS 37.320 in R2-2601186

Deadline: before CB

Proposal 2: Remove the redundant conditions from the procedure and update the procedural text as in Annex B.

P2

-	Ericsson think this change brings new behaviour

Proposal 3: Add clarification text in Clause 5.5c.3.2 of TS 38.331 as suggested in the Annex C.

R2-2601301	Correction on AI/ML Air	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2600358	Correction on AI/ML Air	Samsung	CR	Rel-19	38.331	19.1.0	5641	-	F	NR_AIML_air-Core

=> Withdrawn

R2-2600415	Correction for applicability and prediction configuration procedures	Huawei, HiSilicon	draftCR	Rel-19	38.331	19.1.0	NR_AIML_air-Core

=> Withdrawn

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-251200)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, etc.

LS

R2-2600016	Reply LS on paging capability loss issue (R3-258831; contact: Huawei)	RAN3	LS in	Rel-19	NR_LPWUS-Core	To:SA2	Cc:RAN2, CT1

Noted

R2-2600018	Reply LS on CN assigned subgroup ID for LP-WUS (R3-258851; contact: Ericsson)	RAN3	LS in	Rel-19	NR_LPWUS-Core	To:SA2, RAN2	Cc:RAN1

Noted

R2-2600035	Reply LS on enabling/disabling LP-WUS per UE with NAS signalling (S2-2511295; contact: Huawei)	SA2	LS in	Rel-19	NR_LPWUS-Core	To:RAN2, CT1, RAN3, CT4

Noted

On 38.331

R2-2600290	Miscellaneous corrections on RRC for Rel-19 LP-WUS WUR	vivo (Rapporteur)	CR	Rel-19	38.331	19.1.0	5636	-	F	NR_LPWUS-Core

Content agreeable, will be updated based on more discussions on other proposals during the meeting

?? [AT133][206][LPWUS] RRC CR (vivo)

Intended outcome: Update and agree the CR in R2-2601177

Deadline:  before the end of the meeting

On 38.304

R2-2600392	Miscellaneous Corrections on LP-WUS in TS 38.304	CATT	CR	Rel-19	38.304	19.1.0	0453	-	F	NR_LPWUS-Core

can further discuss detailed change on ‘Np’

Content agreeable, will be updated based on more discussions on other proposals during the meeting

?? [AT133][207][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: Update and agree the CR in R2-2601178

Deadline:  before the end of the meeting

On 38.306

R2-2600412	Correction to R19 LP-WUS UE Capabilities	Huawei, HiSilicon (Rapporteur)	draftCR	Rel-19	38.306	19.1.0	F	NR_LPWUS-Core

will be updated based on more discussions on other proposals during the meeting, if any

- ?? [AT133][210][LPWUS] draft CR for UE capability (Huawei)

Intended outcome: Endorse the CR(s) for UE capability

Deadline:  before the end of the meeting

On 38.300

R2-2601092	Miscellaneous corrections for LP-WUS	Ericsson 	CR	Rel-19	38.300	19.1.0	1102	-	F	NR_LPWUS-Core

Content agreeable, will be updated based on more discussions on other proposals during the meeting

- ?? [AT133][209][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: Update and agree the CR in R2-2601180

Deadline:  before the end of the meeting

### 8.4.2	Corrections

Remaining issues for all related specs

On low mobility criterion

R2-2600291	Discussion on low mobility criteria for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: RAN2 to discuss whether/how to support low mobility criterion for Rel-19 RRM relaxation/offloading, including:

Direction 1: Do not introduce low mobility criterion;

Direction 2: Introduce low mobility criterion, and the capability for UE supporting low mobility criterion is optional if the UE supports Rel-19 RRM relaxation/offloading;

Direction 3: Introduce low mobility criterion, and the capability for UE supporting low mobility criterion is mandatory if the UE supports Rel-19 RRM relaxation/offloading.

Proposal 2: RAN2 confirms there is no impact on RAN4 for low mobility criterion, i.e., no new requirements or no new test cases.

DISCUSSION

-	Ericsson think there is no need to have separate thresholds for SSB/LP-SS. Ericsson agree that there is no impact to R4.

-	Qualcomm think if we introduce it should be optional. ZTE ok to keep it optional.

-	ZTE think if UE does not support low mob criteria but NW configures this, then UE should not relax the measurement. Ericsson agree. Apple and OPPO think UE can just use the no-cell-edge criteria in this case.

-	Nokia think if it is optional we should clarifiy what is the UE behaviour is UE does not support the criteria. InterDigitial agree.

-	Apple think the previous agreement is a compromise, therefore it should be optional.

Introduce low mobility criterion, and the capability for UE supporting low mobility criterion is optional if the UE supports Rel-19 RRM relaxation/offloading

RAN2 understand that there is no RAN4 impact if we introduce the low mobility criterion.

R2-2600711	LP-WUS and low mobility criterion	Ericsson Nokia, ZTE Corporation, Sanechips, Vodafone, Interdigital, T-Mobile USA, BT Plc, Deutsche Telekom, NTT DOCOMO INC., Verizon, Sony, Nordic Semiconductor ASA	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	RAN2 to discuss the UE capability aspect and select either option 1 or 2.

R2-2600701	Discussion on the remaining issues on low mobility criterion in LP-WUS	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	Introduce an optional UE capability of supporting LR based low mobility criterion for serving cell and neighbour cell measurement relaxation and serving cell measurement offloading.

Proposal 2	To support the LR based low mobility criterion, LR specific delta threshold, SSearchDeltaP and time period, TSearchDeltaP should be introduced separately for different reference signal, i.e., LP-SS and SSB.

Proposal 3	Adopt the TPs to TS38.304, TS38.306 and TS38.331 in Annex.

Proposal 4	Send the LS to RAN4 to indicate supporting the LR based mobility for serving cell and neighbour cell measurement relaxation and serving cell measurement offloading

R2-2600549	Discussion on remaining issues in LP-WUS	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Section 2.2

Proposal 2: When low mobility criterion is configured, only both low mobility criterion and cell edge criterion are met, UE can enter Rel-19 RRM measurement relaxation or offloading state.

[AT133][201][LPWUS] On low mobility criteria (vivo)

Intended outcome: Summary with Proposals/TPs in R2-2601171

Deadline: before CB

Other issues

[CB]

R2-2600393	Correction on the number of PO associated with LO in RRC	CATT, Samsung, vivo, Huawei, HiSilicon, OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

-	ZTE understand the intention but think we can instead say that if the filed is absent we can use default value.

-	Ericsson think it is obvious NW should include this filed, and think there may be other similar cases in the spec that we are not changing. Ericsson suggest we have more offline, so that we can check those cases.

-	Xiaomi think the paper propose a good way, and think ZTE’s suggestion is actually a new solution.

R2-2600214	Remaining issues on avoiding missing PO and proposed TP 38.304	Xiaomi Communications, Ericsson	discussion

Noted

R2-2600877	Remaining issues on LP-WUS and PO monitoring	Nokia	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

-	Qualcomm think no need, since the current spec is already clear. Vivo, CATT, OPPO, Huawei agree.

R2-2600411	Relation between RRM measurement relaxation and LP-WUS monitoring	Huawei, HiSilicon, Vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

-	Ericsson think there is no issue if LPWUS provides full coverage.

-	InterDigital do not think we should associate different criteria and think NW can handle by implementation.

R2-2600549	Discussion on remaining issues in LP-WUS	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Section 2.1

Proposal 1: Update TS 38.321 to clarify that the lpwus-PDCCH-MonitoringTimer is started from the beginning of the slot, as in the TP  in the Annex.

DISCUSSION

P1

-	vivo think this is correct. Xiaomi agree.

-	LG think this changes the UE behaviour.

Update TS 38.321 to clarify that the lpwus-PDCCH-MonitoringTimer is started from the beginning of the slot, will be included in the Rapp’s CR.

?? [AT133][208][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: Update and agree the CR in R2-2601179

Deadline:  before the end of the meeting

R2-2601110	LP-WUS corrections	Ericsson	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	RAN2 to confirm that the reason why the LP-WUS time offset is included in the Active Timer evaluation to make clear when the UE can send SRS/CSI and the network can expect to receive SRS/CSI.

Proposal 2	RAN2 to discuss whether there is a need to specify that the UE shall not sent SRS/CSI when the UE is in the process to wake-up the MR, i.e. during the time period when the UE is not able to send SRS/CSI.

Proposal 3	RAN2 to discuss whether the wording of Active Time for LP-WUS should be improved, e.g. and after receiving LP-WUS indication for the duration of LP-WUS time offset prior to symbol n.

Proposal 4	RAN2 to remove “PO/” from “reference PO/PF”.

Proposal 5	The UE is not allowed to use RRM relaxation outside LP-WUS coverage.

DISCUSSION

-	CATT think P4 is not needed, since the current wording is aligned with R1.

-	APPLE think the MAC changes are not new and have been discussed.

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP-251874)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, etc.

LS

R2-2600013	Reply LS on SBFD and CA (R1-2509608; contact: Xiaomi)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN4

Noted

On 38.331

R2-2600159	Corrections to WI SBFD	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.1.0	5630	-	F	NR_duplex_evo-Core

Content agreeable, will update if there are other agreements during the meeting

- ?? [AT133][211][SBFD] RRC CR (Huawei)

Intended outcome: Agree the RRC CR in R2-2601181

Deadline:  before the end of the meeting

On 38.300

R2-2600323	Supporting SBFD with CA	CATT, Xiaomi	CR	Rel-19	38.300	19.1.0	1093	-	F	NR_duplex_evo-Core

Content agreeable, will update if there are other agreements during the meeting

- ?? [AT133][212][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Agree the CR for TS 38.300 in R2-2601182

Deadline:  before the end of the meeting

- ?? [AT133][213][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Agree the CR for TS 38.321 in R2-2601183

Deadline:  before the end of the meeting

### 8.11.2	Corrections

Remaining issues for all related specs

R2-2600203	Corrections for SBFD operation	OPPO	discussion	Rel-19	NR_duplex_evo-Core

Noted

DISCUSSION

-	Qualcomm do not think this is needed, because for each RO type UE will calculate separately. Ericsson, Samsung, Sony, ZTE agree.

-	Samsung and LG Think the PRACH occasion depends on physical resource.

-	Nokia support OPPO proposal.

-	OPPO suggest that if the TP is not agreed, we can capture ‘RAN understand the index pools for the 1st and 2nd PRACH occasions are separate. No need to change the spec.’ in the Chair notes. Huawei think the intention is only to clarify, but there is no confusion, so no need to capture anything.

R2-2600600	SBFD – Remaining Open Issues	Nokia 	discussion	Rel-19	NR_duplex_evo-Core

Noted

DISCUSSION

-	ZTE think we should wait for R4. InterDigital, Qualcomm agree.

-	InterDigital think if we do this the change is only stage 2 so it is ok to wait until R4 indicate us. Qualcomm share this understanding.

-	Ericsson agree with P1 and think we should conclude considering it is very late stage. OPPO agree.

R2-2600642	Discussion on remaining SBFD issues	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

DISCUSSION

P1

-	Huawei think intention may be OK, but the stage 2 change is not necessary.

-	Qualcomm think we could capture in chair notes.

-	LG support P1.

?? RAN2 understand that SBFD RO for RACH associated with additional PCIs in one serving cell is not supported in Rel-19.

P2

-	Huawei think there is no confusion so no need to change. Nokia agree.

-	Qualcomm OK with P2.

-	LG E think there is no need to clarify for every feature combination.

P3

-	CATT think P3 is correct. OPPO agree.

-	Huawei think the behaviour is correct, but there is no need to change, think NW will configure in right way. Nokia agree. LG also do not support.

-	Qualcomm OK with P3, think it is useful to make it clear.

-	Samsung think this is already clear from the R1 spec.

-	LG wonders if we change in MAC, will this also impact R1 spec in the definition of the DCI bits. ZTE think this does not impact R1 spec.

?? RAN2 to add ‘If S/U field is set to 1, this field should not be set to 1’ in the RO field in (Enhanced) LTM Cell Switch MAC CE. Adopt the TP in TS38.321.

R2-2600825	Correction on RO type switch	Ericsson	CR	Rel-19	38.300	19.1.0	1099	-	F	NR_duplex_evo-Core

Noted

The texts ‘as specified in TS 38.213 [38]’ are removed from the paragraph.

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, etc.

### 8.12.2	Corrections

Remaining issues for all related specs

R2-2600338	Correction to the field description of tag2	CATT, OPPO, Ofinno, Ericsson, Huawei, HiSilicon, ZTE, CMCC, Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

For non mDCI mTRP two-TA, the tag2 is mandatory configured if twoTA-Without-MultiDCI-MultiTRP is configured for a BWP.

R2-2600204	Corrections for MIMO	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	Huawei think this is not critical, and if we change it may be mis-leading, since we do no have two UL only TRPs. CATT agree with Huawei, and think UL TRP means UL only TRP.

-	CATT think we can just add reference to R1 spec. Ericsson and LG  agree.

-	Ericsson think another way is to simply the sentence by removing ‘one for.. and the other for…’.

To remove ‘one for the DL TRP and the other for UL TRP’, and add reference to RAN1 spec.

R2-2600572	Remaining issue on CSI report configuration for mode B	ASUSTeK	discussion	Rel-19	38.331	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	CATT think there is no such restriction as proposed by the paper, and it is not aligned with previous R2 discussions.

-	LG think UE may multiplex two report in one PUSCH, or can do prioritization based on its capability, so no issue. LG think R1 spec is already sufficient to handle this case.

-	ZTE think this was already discussed in R1 and it was considered as an error case, no need to change. OPPO, Huawei agree this is error case.

-	Qualcomm do not see an issue since NW can handle the configuration properly, and think R2 do not need to discuss. Ericsson agree it is R1 discussion.

-	CATT think this case may happen and it is up to UE implementation which report is included.

-	Huawei think we can confirm O3, but do not think we need to change anything.

-	Ericsson think no need to capture anything. LG agree.

-	AsusTek suggest to reword O3 and capture as an understanding in the chair notes, without changes to the spec. LG and Ericsson see no need to capture in the chair notes.

R2-2600598	MIMO – Remaining Open issues	Nokia 	discussion	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	Xiaomi think this is enhancement, and if we change like this it will bring more UE complexity.

-	Huawei wonders what if the PUCCH and PUSCH are in different TAT groups.

-	LG think we do not need to change since for the case of two TAT groups UE can still transmit PUSCH, and for the case of the same TAT group UE just monitors PDCCH in a rather short time period.

-	Ericsson and Offino think this change might cause other issues, so no need.

-	CATT also do not think this is needed.

R2-2600822	RRC correction for mode-A UE-initiated CSI report	Samsung	discussion	NR_MIMO_Ph5-Core

Noted

For mode-A UE-initiated CSI report, add the field description of reportSlotOffsetList-r19.

DISCUSSION

-	OPPO think the added FD is OK.

R2-2600835	Discussion of startingBitOfFormat2-3-r19	Ericsson	discussion

Noted

R2-2600728	Corrections on MIMO	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Specify in TS 38.331 that the network does not set extendedStartBitDCI-2-3-r19 to a value from 1 to 31.

[CB]

?? Clarify in TS 38.306 that extendedStartBitDCI-2-3-r19 indicates whether the UE supports values greater than or equal to 32.

R2-2601030	Miscellaneous corrections to UEIBM Configuration Parameters	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	Huawei think TP1 and 2 are good, and TP3 and TP4 we do not need to report R1 spec.

-	Nokia also think TP3 not needed. Nokia think TP4 is useful.

- [AT133][202][MIMOevo] RRC CR for MIMO (Ericsson)

Intended outcome: Agreeable RRC CR in R2-2601172, taking into account the agreements, can also discuss the proposed changes in R2-2601030

Deadline:  before CB

- [AT133][203][MIMOevo] Stage 2 CR for MIMO (CMCC)

Intended outcome: Agreeable Stage 2 CR in R2-2601173, taking into account the agreements

Deadline:  before CB

## 8.20	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

### 8.20.1	RAN4

On MPR enhancement

R2-2600022	LS on further signaling for MPR enhancement (R4-2522409; contact: Apple)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN2

Noted

R2-2600133	Discussion on R4-2522409	OPPO	discussion	Rel-19	NR_ENDC_RF_Ph4-Core

Noted

Proposal 1	R2 define the capability to indicate UE is capable to support co-configuration of mprReductionExtensionRatio-r19 and powerBoostPi2BPSK-r18/powerBoostQPSK-r18. If this capability bit is absent, the UE does not support such co-configuration.

Proposal 2	R2 define the capability on a per-band basis, consistent with the approach taken for mpr-SingleCC-Single/MultipleValue-r19.

Proposal 3	R2 sends LS to R4 to ask if any concern on the definition or the granularity of the newly defined capability.

R2-2601031	Consideration on the MPR Signaling Enhancement	ZTE Corporation	discussion	Rel-19	NR_ENDC_RF_Ph4-Core

Noted

Proposal 1: Introduce a new capability to indicate the UE can support power boosting on the newly Inner RB allocation. If this capability is absent, it means the UE can only support power boosting on the old Inner RB allocation.

Proposal 1a: If the proposal 1 is agreed, send LS to RAN4 to indicate RAN2 conclusion.

DISCUSSION

On the capability

-	Apple fine with capturing the new capability as a ‘positive’ capability.

-	Qualcomm, Ericsson support the P1 from OPPO.

RAN2 define the capability to indicate UE is capable to apply Rel-19 requirement related to mprReductionExtensionRatio-r19 and powerBoostPi2BPSK-r18/powerBoostQPSK-r18. If this capability bit is absent, the UE would only be able to meet the requirements per Rel-18 power boosting.

On granularity of the capability

-	Apple think for DL, CA can be configured, and think UE may report different capability for a band in different BC. Qualcomm agree.

Proposal 2	R2 define the capability on a per-band basis, consistent with the approach taken for mpr-SingleCC-Single/MultipleValue-r19.

Proposal 3	R2 sends LS to R4 to ask if any concern on the definition or the granularity of the newly defined capability.

[AT133][204][NR_Others] On MPR Signaling (Apple)

Intended outcome: a) draftCRs for endorsement for 38.331 and 38.306 in R2-2601174 and R2-2601175, based on the agreement, can also discuss granularity based on P2 in R2-2600133, b) draft LS in R2-2601176

Deadline: before CB

R2-2600473	Corrections on Rel-19 MPR enhancement with power boosting	Apple	draftCR	Rel-19	38.331	19.1.0	F	NR_ENDC_RF_Ph4-Core

R2-2600474	Corrections on Rel-19 MPR enhancement with power boosting	Apple	draftCR	Rel-19	38.306	19.1.0	F	NR_ENDC_RF_Ph4-Core

On Increasing the number of NR inter-frequency carriers for UE monitoring

R2-2600998	Motivation on Increasing the number of NR inter-frequency carriers for UE monitoring	KDDI Corporation (TTC), Ericsson	discussion

Noted

-	KDDI explains that this is submitted to R2 for info.

R2-2601000	New WID on Increasing the number of NR inter-frequency carriers for UE monitoring	KDDI Corporation (TTC), Ericsson	discussion

Not treated

R2-2600996	Motivation on Increasing the number of NR inter-frequency carriers for UE monitoring	KDDI Corporation (TTC)	discussion	Withdrawn

R2-2600997	New WID on Increasing the number of NR inter-frequency carriers for UE monitoring	KDDI Corporation (TTC)	discussion	Withdrawn

Chair: the following documents will be treated in other sessions.

R2-2600180	Clarification of FR1-NTN band in TS 38.306 and TS 38.331	Sharp, Huawei, HiSilicon, Ericsson	discussion	Rel-19	NR_NTN_Ku_bands

R2-2600635	Introduction of FR1-NTN terminology	Huawei, HiSilicon, Sharp, Ericsson	CR	Rel-19	38.331	19.1.0	5652	-	B	NR_NTN_Ku_bands

R2-2600636	Introduction of FR1-NTN terminology	Huawei, HiSilicon, Sharp, Ericsson	CR	Rel-19	38.306	19.1.0	1417	-	B	NR_NTN_Ku_bands

### 8.20.2	Other WGs

MINT related

R2-2600004	Reply LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition (C1-257574; contact: Google)	CT1	LS in	Rel-19	MINT_Ph2	To:RAN2	Cc:SA1, SA2

Noted

R2-2601095	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-17	36.331	17.15.0	5194	-	F	TEI17

R2-2601096	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-18	36.331	18.8.0	5195	-	A	TEI17

R2-2601097	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-19	36.331	19.1.0	5196	-	A	TEI17

R2-2601098	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-17	38.331	17.15.0	5683	-	F	TEI17

R2-2601099	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-18	38.331	18.8.0	5684	-	A	TEI17

R2-2601100	Correction on disaster roaming access barring check for emergency call [MINT]	Huawei, HiSilicon	CR	Rel-19	38.331	19.1.0	5685	-	A	TEI17

DISCUSION

-	Qualcomm think for NR there is no need to change, and for LTE we need further checking.

[AT133][205][NR_Others] on disaster roaming access barring check for emergency call (Huawei)

Intended outcome: Agreeable CRs

Deadline:  before CB

On temporary suspension of trace production

R2-2600044	Reply to LS on temporary suspension of trace production (S5-255659; contact: Ericsson)	SA5	LS in	Rel-20	PM_KPI_Trace_MDT_QoE-OAM	To:RAN2, RAN3

Noted

R2-2601094	Discussion on disaster roaming access barring check for emergency call	Huawei, HiSilicon	discussion	Rel-17	TEI17	Withdrawn

# 9	NR Rel-20

## 9.1	AI/ML for PHY Ph2

(NR_AIML_air_Ph2, leading WG: RAN1; REL-20; WID: RP-253340)

Time budget: 1 TU

Tdoc Limitation: 2 tdoc

### 9.1.1	Organizational

LS, Rapporteur input, including workplan.

LS

R2-2600026	Reply LS on specification of dataset and model parameters exchange (RP-253846; contact: InterDigital)	RAN	LS in	Rel-20	FS_AIML_MGT_Ph3, NR_AIML_air_Ph2, FS_AIML_CN_Ph2	To:SA, SA5	Cc:RAN2, SA2

Noted

R2-2600027	Reply LS on AI/ML UE sided data collection (RP-253847; contact: InterDigital)	RAN	LS in	Rel-20	NR_AIML_air_Ph2, FS_AIML_CN_Ph2, FS_AIML_MGT_Ph3	To:SA, SA2	Cc:RAN2, SA3, SA5

Noted

R2-2600031	LS on AI/ML UE sided data collection (S2-2510958; contact: Samsung & vivo)	SA2	LS in	Rel-20	NR_AIML_air, FS_AIML_CN_Ph2	To:RAN2, RAN	Cc:SA, SA3, SA5, RAN1

Noted

R2-2600043	Reply LS on specification of dataset and model parameters exchange (S5-254846; contact: NEC)	SA5	LS in	Rel-20	FS_AIML_MGT_Ph3, NR_AIML_air-Core	To:SA, RAN, RAN2	Cc:SA2, SA3, RAN1

Noted

R2-2600046	Reply LS on AI/ML UE sided data collection (SP-251687; contact: Apple)	SA	LS in	Rel-20	NR_AIML_air_Ph2, FS_AIML_CN_Ph2, FS_AIML_MGT_Ph3	To:RAN, SA2, SA3, SA5	Cc:RAN2]

Noted

R2-2600049	Reply LS on specification of dataset and model parameters exchange (SP-251707; contact: LGE)	SA	LS in	Rel-20	FS_AIML_MGT_Ph3, NR_AIML_air_Ph2, FS_AIML_CN_Ph2	To:RAN, SA2, SA5	Cc:RAN2

Noted

Workplan

R2-2601077	RAN2 Work Plan for Rel-20 WI on AI/ML for NR air interface Ph2	Ericsson	discussion

Noted

DISCUSSION

-	For P1, Nokia think for NW data collection we may also need input from R1. For P2, Nokia not sure at this stage what is the R4 impact.

### 9.1.2	LCM for two-sided model

On general understanding of LCM procedure for Two-sided model, including applicability reporting with model pairing, whether to support both option A and option B, dependencies with RAN1 and what is needed from RAN1 to progress RAN2 work, etc.

General

R2-2600610	Discussions on LCM for the two-sided model	NTT DOCOMO, INC.	discussion

Noted

Proposal 1

For AI/ML CSI compression with the two-sided model, use the Rel-19 LCM procedure for CSI-related use cases (AI/ML beam management and CSI prediction) as a starting point.

Study the enhancements related to model pairing and the pairing ID.

R2-2600225	Discussion on AI/ML based CSI compression LCM	Xiaomi	discussion	Rel-20	NR_AIML_air_Ph2

Noted

Proposal 1: Two-sided model AI/ML based CSI compression LCM (for both Direction A and Direction C) includes the following components:

-	UE capability;

-	NW-side training data collection;

-	UE-side training data collection;

-	Dataset/model parameter sharing (for Direction A);

-	Model training;

-	Applicability reporting;

-	Model inference operation;

-	Functionality activation, deactivation, switching, and fallback operation;

-	Functionality monitoring.

DISCUSSION BASED ON BOTH PROPOALS

-	Samsung ok with the general proposal, but think it is too early to conclude on paring ID since we need R1 input.

-	Samsung think there is no R2 impact from Direction A. LG agree, think the bullet should be removed.

-	ZTE think Direction C is based on fully specified model, so not sure what need to be discussed for LCM. ZTE not sure why should we discuss fallback, as it is not part of Rel-19 discussion.

-	Apple think first part of DCM proposal is sufficient. Apple think the detailed bullets, e.g., modelling training in Xiaomi proposal are not needed. Ericsson share this view.

-	Ericsson think the part of UE side data collection is only related to radio configuration, and think it is too detailed in this stage.

-	Qualcomm, Nokia have similar view as Apple and Ericsson.

-	InterDigital suggest to take Rel-19 as baseline

-	vivo agree with Docomo proposal, and regarding Xiaomi proposal we should talk about UE data collection configuration.

-	Huawei also do not think we should have those 2nd level details. OPPO agree.

-	LG fine with Docomo proposal.

-	MediaTek think we should focus on what impacts R2.

?? For AI/ML CSI compression with the two-sided model, use the Rel-19 LCM procedure for CSI-related use cases as baseline. FFS on the following aspects (if needed)

-	UE capability;

-	NW-side training data collection;

-	UE-side training data collection (for configuration only);

-	Dataset/model parameter sharing (for Direction A);

-	Model training (no specification impact);

-	Applicability reporting / model paring procedure;

-	Model inference operation;

-	Functionality activation, deactivation, switching, and fallback operation;

-	Functionality monitoring.

For AI/ML CSI compression with the two-sided model, use the Rel-19 LCM procedure for CSI-related use cases as baseline.

Applicability reporting - general aspects

R2-2600955	Discussion on LCM for two-sided model	CMCC	discussion	Rel-20	NR_AIML_air_Ph2

Noted

Proposal 1: The applicability reporting procedure for BM and CSI prediction in R19 can be reused with some essential enhancements for two-sided model.

DISCUSSION

-	Apple agree with the intention, but not sure what are the enhancements.

-	Lenovo think P1 is already covered by the previous agreement.

-	Qualcomm Ok with P1.

-	Nokia think baseline is too strong wording and we can use starting point instead.

-	Mediatek think a separate agreement is useful.

The applicability reporting procedure defined in Rel-19 can be reused as starting point with necessary enhancements for two-sided model.

Applicability reporting - Option A, Option B, other potential options

R2-2600841	LCM for Two-Sided Models	Qualcomm Incorporated 	discussion	Rel-20

Noted

Proposal 1: For two-sided CSI feedback use case applicability reporting, only option A is supported.

R2-2600561	Considerations On LCM  For Two-side Model	ZTE Corporation, Sanechips	discussion	Rel-20	NR_AIML_air_Ph2

Noted

Proposal 5:	Only option B is supported for applicability reporting procedure for CSI feedback enhancement.

R2-2601016	LCM for two-sided model	InterDigital	discussion	Rel-20	NR_AIML_air_Ph2

Noted

Proposal 4:	From RAN2 perspective, both Option A and Option B can be supported for CSI-compression use case (i.e., the current LCM framework can support signalling both).

DISCUSSION

-	Samsung think this should be decided by R1, and think it is now FFS in R1. Samsung think we can discuss from Signaling point of view and wait for R1 for decision.

-	Apple think there is no urgency and we can further discuss. Apple think with the agreement in R1 that pCSI is not supported then the gap btw Option A and B is not too large. InterDigital understand that the final decision is up to R1, and OK to postpone. Nokia share the view of InterDigital.

-	vivo think we can say from signalling point of view both can be supported. LG agree. Xiaomi ok with vivo suggestion, and think we can add some FFS points.

From RAN2 signalling point of view, both Option A and Option B can be supported for Rel-20 CSI compression use case. RAN2 understand that RAN1 will discuss and decide.

Chair: we will continue in the 2nd session

R2-2600225	Discussion on AI/ML based CSI compression LCM	Xiaomi	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 5: For two-sided model, support UE-initiated applicability reporting procedure with the following steps:

-	UE reports its supported pairing ID of two-sided CSI compression prior to Step 3 (RRCReconfiguration).

-	NW provides full inference configuration in CSI-ReportConfig to one of UE supported pairing ID in Step 3.

-	UE reports RRCReconfigurationComplete without applicability report list in Step 4.

Applicability reporting – General aspects on paring ID

R2-2600532	Discussion on Functionality based LCM	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 4: RAN2 assumes that pairing ID (if confirmed by RAN1) is used during applicability determination.

Applicability reporting – Further discussions on pairing ID, including how it works, potential questions to other WG(s) in order to progress R2 discussions, etc.

R2-2600325	Discussion on LCM for two-sided model	LG Electronics	discussion	NR_AIML_air_Ph2-Core

Proposal 1. RAN2 to discuss whether the pairing ID should be designed differently or distinctly per inter-vendor collaboration option

R2-2600550	Applicability Report with Model Pairing for Two-sided AI/ML Model	NEC	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 2:	RAN2 waits for further RAN1 progress on whether and how to link the pairing ID with the inter-vendor collaboration before discussing detailed signalling of model pairing procedure provided in Step3.

R2-2600306	LCM for two-sided model	vivo	discussion	NR_AIML_air_Ph2

Proposal 4.	RAN2 to discuss the relationship between associated ID and pairing ID for CSI compression use case, including the following options:

-	Alt 1: pairing ID is used, no associated ID is needed;

-	Alt 2: both pairing ID and associated ID are used;

-	Alt 3: using an unified term “associated ID” to align with Rel-19, which assumes the function of pairing ID;

Send LS to RAN1 to ask for more information or informate RAN2’s conclusion if needed.

R2-2601078	Discussion on LCM for two-sided model	Ericsson	discussion

Proposal 2	RAN2 to discuss the following aspects related to the assignment of the pairing IDs and involve SA5 in the discussion if needed:

a.	How to ensure that the pairing IDs are unique within the PLMN

b.	Whether it is the gNB and/or the OAM that assigns a pairing ID to the dataset/model parameters.

UE data collection

R2-2600383	Discussion on AI/ML LCM for CSI Compression	MediaTek Inc.	discussion

Proposal 9: For the two-sided model of CSI compression, UE-side data collection applies to Direction A sub-option 3a-1 and Direction C.

R2-2600360	LCM for CSI compression	Samsung	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 1: RAN2 assume that there is no need to introduce new procedures for UE side data collection and inference procedure for CSI compression use case.

R2-2600561	Considerations On LCM  For Two-side Model	ZTE Corporation, Sanechips	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 7:	To reuse the framework adopted in Rel-19 AI/ML to enable UE to request UE side data collection configuration in CSI feedback enhancement. The parameters that are included in candidate configuration and in the configuration for UE side data collection should be determined by RAN1.

Chair: the following will be treated if time allows

Other aspects, e.g., inference configuration, performance monitoring, and UE capability

R2-2600789	Discussion on LCM for two-sided models	Nokia	discussion	Rel-20	NR_AIML_air_Ph2-Core

Proposal 4: RAN2 can wait for RAN1 to define further additional information needed for inference configuration.

R2-2600463	Initial discussion on LCM framework for two-sided model	Apple	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 10: On NW-side performance monitoring in CSI compression, RAN2 evaluate the feasibility of reusing logged L1 measurement framework to report multiple time samples of target CSI and send conclusion to RAN1.

Proposal 11: On UE-side performance monitoring in CSI compression, no RAN2 specification impacts are foreseen.

R2-2600196	Discussion on LCM for two-sided model in the CSI compression use case	CATT	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 3: In the Rel-20 CSI compression use case, the following UE capability signaling can be used:

-	UECapabilityEnquiry: network sends UECapabilityEnquiry message to UE for its reporting of the supported AI/ML based CSI compression capability;

-	UECapabilityInformation: UE sends UECapabilityInformation message to network to indicate the supported AI/ML based CSI compression capability.

R2-2600063	LCM for two-sided model	OPPO	discussion	Rel-20

R2-2600196	Discussion on LCM for two-sided model in the CSI compression use case	CATT	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600225	Discussion on AI/ML based CSI compression LCM	Xiaomi	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600306	LCM for two-sided model	vivo	discussion	NR_AIML_air_Ph2

R2-2600307	Discussion on Network side data collection	vivo	discussion	NR_AIML_air_Ph2

R2-2600325	Discussion on LCM for two-sided model	LG Electronics	discussion	NR_AIML_air_Ph2-Core

R2-2600341	Discussion on LCM for two-sided model	Fujitsu	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600360	LCM for CSI compression	Samsung	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600383	Discussion on AI/ML LCM for CSI Compression	MediaTek Inc.	discussion

R2-2600463	Initial discussion on LCM framework for two-sided model	Apple	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600514	Discussion on LCM for two-sided model	HONOR	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600532	Discussion on Functionality based LCM	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600550	Applicability Report with Model Pairing for Two-sided AI/ML Model	NEC	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600561	Considerations On LCM  For Two-side Model	ZTE Corporation, Sanechips	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600573	Discussion on LCM for two-sided model	ASUSTeK	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600610	Discussions on LCM for the two-sided model	NTT DOCOMO, INC.	discussion

R2-2600660	Discussion on LCM for two-sided model	Spreadtrum, UNISOC	discussion	Rel-20

R2-2600754	LCM for AIML based CSI compression	Lenovo	discussion	Rel-20

R2-2600789	Discussion on LCM for two-sided models	Nokia	discussion	Rel-20	NR_AIML_air_Ph2-Core

R2-2600841	LCM for Two-Sided Models	Qualcomm Incorporated 	discussion	Rel-20

R2-2600872	Discussion on LCM for two-sided model	Ofinno	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600955	Discussion on LCM for two-sided model	CMCC	discussion	Rel-20	NR_AIML_air_Ph2

R2-2601016	LCM for two-sided model	InterDigital	discussion	Rel-20	NR_AIML_air_Ph2

R2-2601078	Discussion on LCM for two-sided model	Ericsson	discussion

### 9.1.3	NW side data collection

On NW side data collection for CSI compression use case, including signaling design for NW sided model training, potential questions to RAN1 if any to progress RAN2 work, etc.

General

R2-2600533	Discussion on NW side data collection	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 1: For network-side data collection for two-sided model, Rel-19 network-side data collection solution can be used as a starting point. The measurements to be collected for CSI compression use case should be decided by RAN1.

R2-2600842	Network Side Data Collection for Two-Sided Models	Qualcomm Incorporated 	discussion	Rel-20

Proposal 1: Support both gNB-centric and OAM-centric data collections, where the RRC is used to configure measurement and logging configurations.

R2-2600790	Discussion on network-side data collection	Nokia	discussion	Rel-20	NR_AIML_air_Ph2-Core

Proposal 2: RAN2 to wait for further refinement on the procedure and signalling details on the network-side data collection for two-sided models until RAN1 evaluates the payload requirement.

Proposal 3: RAN2 to wait for RAN1 on any additional requirements (e.g., data quality) for network-side data collection for two-sided models.

Configuration of NW data collection

R2-2601079	Discussion on network side data collection	Ericsson	discussion

Proposal 1	For NW-side data collection configuration for CSI compression, CSI-LoggedMeasurementConfig is reused. FFS any enhancements, based on RAN1 input.

Proposal 2	The pairing ID is not signalled to the UE for the purpose of NW-side data collection.

Proposal 3	Use the UEInformationRequest/UEInformationResponse mechanism to retrieve data from UE for NW-side data collection.

Proposal 4	Enhance UEInformationRequest to distinguish between requesting logged data for beam management versus CSI compression.

R2-2600842	Network Side Data Collection for Two-Sided Models	Qualcomm Incorporated 	discussion	Rel-20

Proposal 2: The network optionally indicates pairing ID in the RRC configuration for network-side model training.

Proposal 6: The UE Information Request and Response messages are used for reporting logged measurements over SRB6.

Event triggered, periodic report

R2-2600064	NW side data collection for two-sided model	OPPO	discussion	Rel-20

Proposal 5: For NW-side data collection for two-sided model training, event-triggered data logging method is not supported, i.e. only support periodic data logging method for CSI compression use case in R20.

Proposal 7: Only periodic CSI resources are used for NW sided data collection.

R2-2600956	Discussion on NW side data collection	CMCC	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 1: NW side data collection procedure for BM in R19 can be reused for CSI compression, including at least supporting L1 measurements logging, data reporting via UEInformationRequest and UEInformationResponse message, supporting periodic and event-triggered data logging, low power state and AS buffer indication, etc..

On handling of the logged data

R2-2600755	NW side data collection for AIML based CSI compression	Lenovo	discussion	Rel-20

Proposal 1	The same NW side data collection framework/IE specified in Rel19 can be the baseline for CSI compression use case.

a.	CSI-LoggedMeasurmentConfig can be enhanced to configure the data logging for CSI compression use case

b.	UE variable VarCSI-LogMeasReport can be enhanced to log the CSI payload as configured

c.	UE indicates the availability of logged CSI payload to RAN via UAI upon buffer threshold/full or low power state

d.	During handover the UE can be configured to retain or not retain any logged CSI payload.

e.	UE will release any logged CSI payload upon entering RRC Inactive/Idle.

R2-2601017	NW-side data collection for CSI-compression use case	InterDigital	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 4a:	For CSI compression use case, when the AS buffer for storing logged data becomes full, the UE stops measurement and logging for data collection.

Proposal 4b:	When the AS buffer reserved for storing logged data becomes full or reaches an absolute threshold (if configured), the UE indicates data availability to the network. It is up to UE implementation how to determine the AS buffer threshold is reached.

Observation 8:	If the UE indicates low power to the network, the network can deconfigure/release the data collection configuration and enable the UE preserve battery life for other UP/CP data reception/transmission.

Proposal 6:	For CSI compression use case, a UE configured for network side data collection can send a low power indication. The low power determination is left to UE implementation.

On data size and content

R2-2600464	Initial discussion on data collection in two-sided model	Apple	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 3: RAN2 conclude that it is technically feasible to use logged L1 measurement framework to support target CSI payload size greater than single RRC message in CSI compression. FFS detailed solutions.

R2-2600197	Consideration on NW side data collection in the CSI compression use case	CATT, CBN, China Broadnet	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 4: A target CSI payload size greater than a single RRC message is supported.

Proposal 5: To support a target CSI payload size greater than a single RRC message, the following two options can be considered:

­	Option1: RRC segmentation;

­	Option2: 1-bit indicator for additional logged data available for transmission similar to logMeasAvailable-r16 in logged MDT.

R2-2600342	Discussion on NW side data collection for CSI compression	Fujitsu	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 2	For NW side data collection with higher layer reporting in AI/ML based CSI compression, RAN2 confirms that the size limit of one RRC message is unchanged.

Proposal 3	To meet the size limit of one RRC message, layer number of target CSI in data collection report can be configured by NW.

R2-2600384	Discussion on Network-side Data Collection for CSI Compression	MediaTek Inc.	discussion

Proposal 1: Send LS to RAN1 requiring the inputs for the following points:

1.	How large is the data sample?

2.	How many Target-CSI samples (per UE / per cell / per scenario) are foreseen for an “offline model-training campaign”? How large the overall dataset size would be?

3.	Over what time window (e.g., hour, day, week, or month) should these samples be collected?

4.	To what extent the model training can tolerate missing or partially delivered samples

Chair: the following will be treated if time allows

Other aspects

R2-2600384	Discussion on Network-side Data Collection for CSI Compression	MediaTek Inc.	discussion

Proposal 2: Maintain the UE buffer size at 64 kB, to be shared among multiple AI-enabled features (e.g., AI-BM, AI-CSI prediction, AI mobility, AI CSI compression).

R2-2600560	Considerations On NW Side Data Collection For CSI-feedback Enhancement	ZTE Corporation, Sanechips	discussion	Rel-20	NR_AIML_air_Ph2

Proposal 4: The AS buffer for NW side data collection is shared across all the AIML use cases, including AIML beam management, AIML mobility, AIML CSI feedback enhancement.

Proposal 5: the minimum size of AS buffer is extended to the MB level for supporting target CSI logging , the detail number is FFS.

Proposal 6: Introduce a UE capability to indicate the maximum AS buffer size that the UE supports, the supported maximum AS buffer size shall be at least larger than the minimum AS buffer size.

R2-2600064	NW side data collection for two-sided model	OPPO	discussion	Rel-20

R2-2600197	Consideration on NW side data collection in the CSI compression use case	CATT, CBN, China Broadnet	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600226	Discussion on CSI compression NW-side data collection	Xiaomi	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600324	Discussion on NW-side Data Collection	LG Electronics Inc.	discussion	Rel-20	NR_AIML_air_Ph2-Core

R2-2600342	Discussion on NW side data collection for CSI compression	Fujitsu	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600361	NW side data collection for CSI compression	Samsung	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600384	Discussion on Network-side Data Collection for CSI Compression	MediaTek Inc.	discussion

R2-2600464	Initial discussion on data collection in two-sided model	Apple	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600533	Discussion on NW side data collection	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600551	Discussion on NW Side Data Collection	NEC	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600560	Considerations On NW Side Data Collection For CSI-feedback Enhancement	ZTE Corporation, Sanechips	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600611	Discussions on the NW side data collection	NTT DOCOMO, INC.	discussion

R2-2600661	Discussion on NW side data collection	Spreadtrum, UNISOC	discussion	Rel-20

R2-2600755	NW side data collection for AIML based CSI compression	Lenovo	discussion	Rel-20

R2-2600790	Discussion on network-side data collection	Nokia	discussion	Rel-20	NR_AIML_air_Ph2-Core

R2-2600811	NW Side Data Collection	SHARP Corporation	discussion

R2-2600842	Network Side Data Collection for Two-Sided Models	Qualcomm Incorporated 	discussion	Rel-20

R2-2600873	Discussion on NW side data collection	Ofinno	discussion	Rel-20	NR_AIML_air_Ph2

R2-2600956	Discussion on NW side data collection	CMCC	discussion	Rel-20	NR_AIML_air_Ph2

R2-2601017	NW-side data collection for CSI-compression use case	InterDigital	discussion	Rel-20	NR_AIML_air_Ph2

R2-2601079	Discussion on network side data collection	Ericsson	discussion

## List of post meeting email discussions

Short

To be added.

Long

To be added.