3GPP TSG-RAN WG4 Meeting #118	R4-2600711

Gothenburg, Sweden, Feb. 09-13, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0855 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BigCR on Rel-19 NR MIMO Phase 5 UE Demod |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2026-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Introduce UE PMI reporting requirements for MIMO phase 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - R4-2522686: Add Applicability rules for 64 CSI-RS ports PMI reporting - R4-2602568: Add PMI reporting requirements for TypeI (2Rx, FDD) - R4-2602572: Add PMI reporting requirements for eTypeII (2Rx, FDD) - R4-2602569: Add PMI reporting requirements for TypeI (2Rx, TDD) - R4-2602573: Add PMI reporting requirements for eTypeII (2Rx and 4Rx, TDD) - R4-2602570: Add PMI reporting requirements for TypeI (4Rx, FDD) - R4-2602574: Add PMI reporting requirements for eTypeII (4Rx, FDD) - R4-2602571: Add PMI reporting requirements for TypeI (4Rx, TDD) - R4-2521038: Add FRC for FDD 64 CSI-RS ports PMI reporting - R4-2522694: Add FRC for TDD 64 CSI-RS ports PMI reporting - R4-2522695: Add MIMO Channel Correlation Matrices |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE PMI reporting requirements for TypeI-SinglePanel-r19 and eTypeII-r19 will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.3 6.3.2.1.X1, 6.3.2.1.X2, 6.3.2.2.X1, 6.3.2.2.X2 6.3.3.1.Y1, 6.3.3.1.Y2, 6.3.3.1.Z1, 6.3.3.1.Z2 A.3.2.1.1, A.3.2.2.2, B.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2521039 |  |  |  |  |  |  |  |  |

=============First change (R4-2522686)==============

6.1	General

This clause includes conducted requirements for the reporting of channel state information (CSI).

6.1.1	Applicability of requirements

<Unchanged sections omitted>

6.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 6.1.1.3-1 shall apply for UEs which support optional UE features with capability signalling only.

Table 6.1.1.3-1: Requirements applicability for optional features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | CQI | Clause 6.2.2.1.1.2 Clause 6.2.3.1.1.2 |  |
|  | FR1 TDD | CQI | Clause 6.2.2.2.1.2 Clause 6.2.3.2.1.2 |  |
| Alternative 64QAM MCS table for PDSCH New 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | CQI | Clause 6.2.2.1.1.2 Clause 6.2.3.1.1.2 |  |
|  | FR1 TDD | CQI | Clause 6.2.2.2.1.2 Clause 6.2.3.2.1.2 |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | CQI | Clause 6.2.2.2.1.3 Clause 6.2.3.2.1.3 Clause 6.2A.3.1.2 Clause 6.2A.4.1.1 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16 ) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | CQI | Clause 6.2.2.2.1.3 Clause 6.2.3.2.1.3 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |
| 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17) | FR1 FDD | CQI | Clause 6.2.2.1.1.3 (Test 1) Clause 6.2.3.1.1.3 (Test 1) |  |
|  | FR1 TDD | CQI | Clause 6.2.2.2.1.4 (Test 1) Clause 6.2.3.2.1.4 (Test 1) |  |
| Support of Type II codebook with at least 16 ports per CSI-RS resource (CodebookParameters contains type2, supportedCSI-RS-ResourceList, parameterLx, amplitudeScalingType, amplitudeSubsetRestriction, maxNumberTxPortsPerResource) | FR1 FDD | PMI | Clause 6.3.2.1.5 Clause 6.3.3.1.5 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.5 Clause 6.3.3.2.5 |  |
| Support of Enhanced Type II codebook with at least 16 ports per CSI-RS resource(codebookParametersAddition-r16 contains etype2R1-r16, supportedCSI-RS-ResourceListAdd-r16, maxNumberTxPortsPerResource) | FR1 FDD | PMI | Clause 6.3.2.1.6 Clause 6.3.3.1.6 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.6 Clause 6.3.3.2.6 |  |
| Support of Enhanced Type II Codebook (eType-II) based on doppler CSI (codebookParametersetype2DopplerCSI-r18 contains eType2Doppler-r18, supportedCSI-RS-ResourceList-r18, maxNumberTxPortsPerResource) | FR1 FDD | PMI | Clause 6.3.2.1.8 Clause 6.3.3.1.8 | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports. |
|  | FR1 TDD | PMI | Clause 6.3.2.2.9 Clause 6.3.3.2.8 |  |
| Support of Enhanced Type II Codebook (eType-II) with refinement for multi-TRP CJT (codebookParametersetype2CJT-r18 contains eType2CJT-r18, supportedCSI-RS-ResourceList-r18, maxNumberTxPortsPerResource) | FR1 FDD | PMI | Clause 6.3.2.1.9 Clause 6.3.3.1.9 | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports. |
| Support of CSI enhancements for multi-TRP including support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis with N=1 (mTRP-CSI-EnhancementPerBC-r17) | FR1 FDD | PMI | Clause 6.3.2.1.7 Clause 6.3.3.1.7 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.8 Clause 6.3.3.2.7 |  |
| Support of Baseline 8Rx Receiver or Simplified 8Rx receiver | FR1 FDD | CQI | Clause 6.2.4.1.2 |  |
|  | FR1 TDD | CQI | Clause 6.2.4.2.2 |  |
| Support of enhanced Type-I SP codebook for 64 ports Scheme-A by aggregating multiple NZP CSI-RS resources within 1 slot (codebookParametersType1SP-SchemeA-r19 contains enhType1SP64PortsSchemeA-r19, maxRank-r19, maxNumberResource-r19, processingCapability-r19) | FR1 FDD | PMI | Clause 6.3.2.1.X1 Clause 6.3.3.1.Y1 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.X1 Clause 6.3.3.2.Z1 |  |
| Support of extended eType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot (codebookParameterseType2Ext-r19 contains eType2-64PortExt-r19, maxNumberResource-r19, processingCapability-r19) | FR1 FDD | PMI | Clause 6.3.2.1.X2 Clause 6.3.3.1.Y2 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.X2 Clause 6.3.3.2.Z2 |  |

============Second change (R4-2602568)==============

6.3	Reporting of Precoding Matrix Indicator (PMI)

6.3.1	1RX requirements

6.3.2	2RX requirements

6.3.2.1	FDD

6.3.2.1.X1	Single PMI with 64Tx TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.X1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.X1-2.

Table 6.3.2.1.X1-1: Test parameters (dual layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| portMappingMethod |  |  | method1 |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | modeA |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  | Not configured |  |
|  | typeI-CBSR-r19 |  | Not configured |  |
|  | (valueOfX1-typeI-powerScaling-r19, valueOfX2-typeI-powerScaling-r19) |  | Not configured |  |
|  | Power scaling factor configurations (typeI-softScalingRanks1-2-r19) |  | Not configured |  |
|  | RI Restriction |  | 00000010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.6 FDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 6.3.2.1.X1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 7.0 |

============Third change (R4-2602572)==============

6.3.2.1.X2	Multiple PMI with 64TX Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.1.X2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.X2-2.

Table 6.3.2.1.X2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | etypeII-r19 |  |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  | 1 |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |  |
|  | typeII-CBSR-r19 |  | Not configured |  |
|  | RI Restriction (typeII-RI-Restriction-r19) |  | 0010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.6 FDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in Table 6.3.2.1.X1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.2.1.X2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 15.0 |

============Fourth change (R4-2602569)==============

6.3.2.2	TDD

6.3.2.2.X1	Single PMI with 64TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.X1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.X1-2.

Table 6.3.2.2.X1-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 10/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | modeA |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  | Not configured |  |
|  | typeI-CBSR-r19 |  | Not configured |  |
|  | (valueOfX1-typeI-powerScaling-r19, valueOfX2-typeI-powerScaling-r19) |  | Not configured |  |
|  | typeI-softScalingRanks1-2-r19 |  | Not configured |  |
|  | RI Restriction |  | 00000010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-43.1 TDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 6.3.2.2.X1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 7.0 |

=========Fifth change (R4-2602573 change 1)===========

6.3.2.2.X2	Multiple PMI with 64Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.2.X2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.X2-2.

Table 6.3.2.2.X2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 10/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (2, 9) | Row 17, (5, 12) |
|  | CSI-RS interval and offset |  | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5, 7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Not configured |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 11111111111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | etypeII-r19 |  |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  | 1 |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |  |
|  | typeII-CBSR-r19 |  | Not configured |  |
|  | RI Restriction (typeII-RI-Restriction-r19) |  | 0010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-43.1 TDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in Table 6.3.2.2.X1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.2.2.X2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 15.0 |

============Sixth change (R4-2602570)=============

6.3.3	4RX requirements

6.3.3.1	FDD

6.3.3.1.Y1	Single PMI with 64TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.Y1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.Y1-2.

Table 6.3.3.1.Y1-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | modeA |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  | Not configured |  |
|  | typeI-CBSR-r19 |  | Not configured |  |
|  | (valueOfX1-typeI-powerScaling-r19, valueOfX2-typeI-powerScaling-r19) |  | Not configured |  |
|  | typeI-softScalingRanks1-2-r19 |  | Not configured |  |
|  | RI Restriction |  | 00000010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.6 FDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 6.3.3.1.Y1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 13.0 |

============Seventh change (R4-2602574)=============

##### 6.3.3.1.Y2	Multiple PMI with 64Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.3.1.Y2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.Y2-2.

Table 6.3.3.1.Y2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Not configured |  |
| Sub-band Size |  | RB | 4 |  |
| csi-ReportingBand |  |  | 1111111111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | etypeII-r19 |  |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |  |
|  | typeII-CBSR-r19 |  | Not configured |  |
|  | RI Restriction (typeII-RI-Restriction-r19) |  | 0010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.6 FDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in table 6.3.3.1.Y1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.3.1.Y2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 15.0 |

============Eighth change (R4-2602571)=============

6.3.3.2	TDD

6.3.3.2.Z1	Single PMI with 64TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.Z1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.Z1-2.

Table 6.3.3.2.Z1-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 10/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | modeA |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeI-CBSR-r19, valueOfX2-typeI-CBSR-r19) |  | Not configured |  |
|  | typeI-CBSR-r19 |  | Not configured |  |
|  | (valueOfX1-typeI-powerScaling-r19, valueOfX2-typeI-powerScaling-r19) |  | Not configured |  |
|  | typeI-softScalingRanks1-2-r19 |  | Not configured |  |
|  | RI Restriction |  | 00000010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-43.1 TDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 6.3.2.2.Z1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 13.0 |

=========Ninth change (R4-2602573 change 2)===========

6.3.3.2.Z2	Multiple PMI with 64Tx Enhanced Type II Codebook

For the parameters specified in Table 6.3.3.2.Z2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.Z2-2.

Table 6.3.3.2.Z2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 4 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 10/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17, (2, 9) | Row 17, (5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5, 7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Not configured |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 11111111111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | etypeII-r19 |  |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2 ) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  | 1 |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |  |
|  | typeII-CBSR-r19 |  | Not configured |  |
|  | RI Restriction (typeII-RI-Restriction-r19) |  | 0010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-43.1 TDD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel-r19' codebook configuration as specified in Table 6.3.3.2.Z1-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.3.2.Z2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 15.0 |

============Tenth change (R4-2521038)=============

A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

A.3.2.1	FDD

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<Unchanged sections omitted>

Table A.3.2.1.1-6: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-6.1 FDD | R.PDSCH.1-6.2 FDD | R.PDSCH.1-6.3 FDD | R.PDSCH.1-6.4 FDD | R.PDSCH.1-6.5 FDD | R.PDSCH.1-6.6 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 | 25 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 | 15 | 15 | 15 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 20 | 13 | 13 | 22 |
| Modulation |  | 16QAM | 16QAM | 64QAM | 16QAM | 16QAM | 64QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 | 0.48 | 0.48 | 0.65 |
| Number of MIMO layer |  | 1 | 2 | 2 | 2 | 1 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 12040 | 24072 | 40976 | 24072 | 5760 | 49176 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..,19} | CBs | 2 | 3 | 5 | 3 | 1 | 6 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10 | Bits | 23712 | 47424 | 71136 | 44928 | 11400 | 71136 |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..9,11,…,19} | Bits | 24960 | 49920 | 74880 | 49920 | 12000 | 74880 |
| Max. Throughput averaged over 2 frames | Mbps | 9.030 | 18.054 | 30.732 | 18.054 | 4.320 | 36.882 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 4: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition. No change to PDSCH scheduling for other type of CSI-RS slots. |  |  |  |  |  |  |  |

============Eleventh change (R4-2522694)=============

A.3.2.2	TDD

<Unchanged sections omitted>

A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<Unchanged sections omitted>

Table A.3.2.2.2-43: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-43.1 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |  |
| Allocated slots per 2 frames |  | 23 |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |
| MCS index |  | 22 |  |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |  |
| Target Coding Rate |  | 0.65 |  |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =6 for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For Slot i = 20 | Bits | 98376 |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5} for i from {1,…,19,22,…,39} | Bits | 98376 |  |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =6 for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For Slot i = 20 | Bits | 24 |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5} for i from {1,…,19,22,…,39} | Bits | 24 |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =6 for i from {0,…,39} | CBs | N/A |  |  |  |  |  |
| For Slot i = 20 | CBs | 12 |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5} for i from {1,…,19,22,…,39} | CBs | 12 |  |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =6 for i from {0,…,39} | Bits | N/A |  |  |  |  |  |
| For Slot i = 20 | Bits | 145008 |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5} for i from {1,…,19,22,…,39} | Bits | 152640 |  |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 113.132 |  |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 4: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition. No change to PDSCH scheduling for other type of CSI-RS slots. |  |  |  |  |  |  |  |

============Twelfth change (R4-2522695)=============

B.2.3	MIMO Channel Correlation Matrices

<Unchanged sections omitted>

B.2.3.2	MIMO Correlation Matrices using Cross Polarized Antennas (X-pol)

The MIMO channel correlation matrices defined in B.2.3.2 apply for the antenna configuration using cross polarized (XP/X-pol) antennas at both gNB and UE. The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB and cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at UE.

For the 2D cross-polarized antenna array at eNodeB, the N antennas are indexed by , and total number of antennas is , where

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization, and

-	 is the number of polarization groups.

For the 2D cross-polarized antennas at gNB, the N antennas are labelled such that antennas shall be in increasing order of the second dimension firstly, then the first dimension, and finally the polarization group. For a specific antenna element at p-th polarization, n1-th row, and n2-th column within the 2D antenna array, the following index number is used for antenna labelling:

where N is the number of transmit antennas, p is the polarization group index, n1 is the row index, and n2 is the column index of the antenna element.

For the linear (single dimension, 1D) cross-polarized antenna, the N antennas are labelled following the above equations with N2=1.

B.2.3.2.1	Definition of MIMO Correlation Matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

where

-	 is the spatial correlation matrix at the UE with same polarization,

-	 is the spatial correlation matrix at the gNB with same polarization,

-	is a polarization correlation matrix, and

-	denotes transpose.

The matrix is defined as

A permutation matrix P elements are defined as

.

where Nt and Nr is the number of transmitter and receiver respectively. This is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in B.2.3.2.

For the 2D cross-polarized antenna array at gNB, the spatial correlation matrix at the gNB is further expressed as following for 2D cross-polarized antenna array at gNB:

where

-	-	 is the correlation matrix of antenna elements in first dimension with same polarization and can be calculated as follows:

For the first dimension (N1), define the vector r1:

r₁(k) = α1k - 1N1- 12, for k = 1, 2, ..., N₁

RgNB_Dim1 = Toeplitz(r₁)

where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1.

, and

-	-	 is the correlation matrix of antenna elements in second dimension with same polarization and can be calculated as follows:

For the second dimension (N2), define the vector r2:

r₂(k) = α2k - 1N2- 12, for k = 1, 2, ..., N₂

RgNB_Dim2 = Toeplitzr2

If N2=1, RgNB_Dim2=1.

For the 2D cross polarized antenna array at gNB side, the spatial correlation matrices in one direction of antenna array are as follows for selected examples based on previous definition:

-	For 1 antenna element with the same polarization in one direction,

.

-	For 2 antenna elements with the same polarization in one direction,

.

-	For 3 antenna elements with the same polarization in one direction,

.

-	For 4 antenna elements with the same polarization in one direction,

.

where the index i = 1,2 stands for first dimension and second dimension respectively.

For the 1D cross-polarized antenna array at gNB, the matrix ofis determined by follow the equations for 2D cross-polarized antenna array and letting, i.e.,

The spatial correlation matrices at UE side are as follows:

rue(k) = βk - 1N- 12, for k = 1, 2, ..., N

RUE = Toeplitz(rue)

-	For 1 antenna element with the same polarization,

.

-	For 2 antenna elements with the same polarization,

.

B.2.3.2.2	MIMO Correlation Matrices using cross polarized antennas

The values for parameters α1, α2, β and γ for the cross polarized antenna models are given in Table B.2.3.2.2-1.

Table B.2.3.2.2-1: The α and β parameters for cross-polarized MIMO correlation matrices

| Correlation Model | 1 | 2 |  |  |
| --- | --- | --- | --- | --- |
| Medium Correlation | 0.3 | 0.3 | 0.6 | 0.2 |
| High Correlation | 0.9 | 0.9 | 0.9 | 0.3 |
| Note 1: Value of α1 applies when more than one pair of cross-polarized antenna elements in first dimension at gNB side. Note 2: Value of α2 applies when more than one pair of cross-polarized antenna elements in second dimension at gNB side. Note 3: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |  |  |

For the 1D cross polarized antenna array at gNB side, the correlation matrices for high spatial correlation and medium correlation are defined in Table B.2.3.2.2-2 and Table B.2.3.2.2-3 as below.

For the 2D cross polarized antenna array at gNB side, the correlation matrices for high spatial correlation are defined in Table B.2.3.2.2-4 as below.

Round-off and regularization are optional. In case round-off to 4-digit precision is used, the values in Table B.2.3.2.2-2, and Table B.2.3.2.2-4 have been adjusted to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

or

Where the value "a" is a regularization factor such that the smallest value is used to obtain a positive semi-definite result.

Table B.2.3.2.2-1a show the suggested values of “a” for different number of ports and correlation level in case round-off to 4 digit precision is used.

Table B.2.3.2.2-1a: Suggested regularization factors.

| Antenna Configuration | Correlation Level | Regularization factor (a) |
| --- | --- | --- |
| 8(4,1,2)x2 | High | 0.00010 |
| 16(4,2,2)x2 | High | 0.00012 |
| 16(4,2,2)x4 | High | 0.00012 |
| 32(4,4,2)x2 | High | 0.00022 |
| 32(4,4,2)x4 | High | 0.00022 |
| 64(8,4,2)×2 | Medium | 0.00065 |
| 64(8,4,2)×4 | Medium | 0.00092 |

Table B.2.3.2.2-2: MIMO correlation matrices for high spatial correlation (1D cross polarized antenna array at gNB side)

| 4(2,1,2)x2 case |  |
| --- | --- |
| 2(1,1,2)x4 case |  |
| 4(2,1,2)x4 case |  |
| 8(4,1,2)x2 case |  |

<Unchanged sections omitted>

==============End of change==============