3GPP TSG-RAN WG4 Meeting #118                                                      R4-2602519

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda item:			7.5.1

Source:	Moderator (China Telecom)

Title:	WF for NR_demod_Ph6

Document for:	Approval

- Introduction

This Way Forward captures the agreements, open issues and candidate options for Rel-20 Demod performance requirements WI. Previously approved WFs are listed as below:

R4-2514823	WF on NR_demod_Ph6_Ph5, China Telecom

R4-2522662	WF on NR_demod_Ph6, Nokia, China Telecom

- Topic #1: UE requirements for FR1 SU-MIMO with spatial channel model

- rCDL-C2 channel model

Issue 1-1-1: rCDL-C2 parameter table

Agreement

  - Use the following rCDL-C2 parameter table for requirement definition:

| Cluster # | Delay [ns] |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -7.4 |  | -38.4 |  | -117.2 |  | 95.4 |  | 101.1 |
| 2 | 76 |  | -1.2 |  | -22.8 |  | 123.5 |  | 98.0 |  | 70.9 |
| 3 | 80 |  | -5.5 |  | -34.5 |  | -140.0 |  | 101.6 |  | 66.9 |
| 4 | 232 |  | 0 |  | -7.8 |  | 166.7 |  | 99.1 |  | 77.1 |
| 5 | 240 |  | -10.4 |  | 39.6 |  | 68.0 |  | 110.0 |  | 61.7 |
| 6 | 290 |  | -10.1 |  | -50.0 |  | 77.5 |  | 92.0 |  | 54.7 |
| 7 | 300 |  | -13.7 |  | 44.2 |  | -71.8 |  | 111.7 |  | 69.5 |
| 8 | 340 |  | -14.1 |  | -71.2 |  | 60.7 |  | 88.7 |  | 48.2 |
| 9 | 448 |  | -8.1 |  | -44.0 |  | 78.9 |  | 107.3 |  | 106.9 |
| 10 | 478 |  | -9.8 |  | -49.9 |  | -89.5 |  | 108.2 |  | 47.4 |
| 11 | 792 |  | -11.7 |  | -59.1 |  | 90.4 |  | 87.8 |  | 49.2 |
| 12 | 990 |  | -16.2 |  | 58.9 |  | 46.8 |  | 108.2 |  | 126.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

- Test scope and test parameters for SCM SU-MIMO requirements

- General

Issue 1-2-1-1: Antenna Array Virtualization (AAV) assumptions

Agreement

  - Only consider AAV (1,1), i.e., Ms=Ns=1.

- PDSCH demodulation requirements with SCM

Issue 1-2-2-1: MCS and test metric for 8T8R 8L test case

<Agreement>

  - As start point, the SNR requirement is defined based on the SNR @ 70% Max TP of CW0 that use MCS13.

- Companies are also encouraged to provide simulation results SNR @ 70% Max total TP

  - Select MCS for CW1 should have lower SNR @ 70% Max TP than that of CW0.

  - Candidate options for MCS for CW1:

- Option 1: MCS 8

- Option 2: MCS 10

Issue 1-2-2-2: Other test parameters for PDSCH demodulation requirements with SCM

Agreement:

Common simulation assumptions for PDSCH Parameter; Value. Duplex mode; TDD and FDD. TDD Slot Configuration Pattern; 7D1S2U (PDSCH is not scheduled in S slot for rank 8). Channel Bandwidth/SCS; 40MHz/30kHz SCS for TDD 10MHz/15kHz SCS for FDD. Antenna configuration; Rank4: 4T4R Rank8: 8T8R. MCS; 13 (64 QAM table) for rank 4 For rank 8, according to Issue 1-2-2-1. Codebook configuration for PDSCH and DMRS; CodebookType; Random Single Panel Type I and randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook. Codebook configuration; 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1). PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. Resource allocation type; Type 0. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 2 for rank 8 1 for rank 4. Number of HARQ Processes; 8 for TDD 4 for FDD. Maximum HARQ transmissions; 4. UE receiver type; MMSE-IRC. Test metric; Rank 4: SNR(dB)@ 70% of maximum throughput Rank 8: According to Issue 1-2-2-1. Simulation assumptions for CDL channel Parameter; Value. Rank4; Rank8. FR; FR1. UE movement direction; (65°,90°). AAV assumptions; (M,N,P,Ms,Ns) = (1,2,2,1,1); (M,N,P,Ms,Ns) = (1,4,2,1,1). Channel Geometry; LCS UE; α = 180°, β=0°, γ = 0°. LCS gNodeB; α = 0°, β=10°, γ = 0°. GCS UE; Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m. GCS gNodeB; Height = 25 m; Azimuth = 0; X Coordinate = 0 m. BS Antenna Polarisation; Cross Polarized antenna elements with +/-45 degrees polarization slant angles. BS Radiation Pattern; Defined Table 7.3-1 in TS 38.901. UE Antenna Polarisation; cross-polarized antenna elements with +90/0 degrees polarization slant angles. UE Antenna Radiation Pattern; Omnidirectional. Antenna Panel Placement; YZ Plane.

- PMI reporting requirements with SCM

Issue 1-2-3-1: PMI bias study

Agreement

RAN4 to prioritize the study on the following approaches to address PMI bias:

  - Option 1: Beam steering approach

- Study the feasibility to apply the existing beam steering approach specified in TS 38.101-4 B.2.3.2.3 and B.2.3.2.3A for PMI testing with SCM as a start point.

  - Option 2: Time varying BS bearing angle rotation, with BS omni-directional antenans

α=α0+∆α*k

  - Example:

  - α0 is the random start value with the uniform distribution, ∆α is the step of phase variation, k is the linear increment.

  - To ensure the relative even PMI distribution, it’s proposed to limit the α to [-76°, 14°]. So the α could be described as following:

α=modα0+∆α*k,90°-76°

  - Check the PMI distribution

  - Study the TE feasibility

  - Companies to provide the candidate parameters for the initial phase, step phase of variation, CSI-RS periodicity and simulation duration regarding to CDL model characters, which are applicable for both option 1 and 2.

Note: It is not precluded to not consider time-wise evolution of the spatial characteristics of SCM, depending on the further discussion in RAN4.

Issue 1-2-3-2: Scope for eType II PMI reporting requirements (if introduced) / PMI bias study

Agreement:

On antenna number and rank number:

  - For PMI bias study: 8T4R with rank 2

  - For requirement definition: further discuss

On codebook:

  - For PMI bias study: Type I and eTypeII

  - For requirement definition: eType II following the WID

Issue 1-2-3-5: Simulation result alignment for eType II PMI reporting / PMI bias study

Agreement:

For the PMI bais study and simulation result alignment, companies to provide simulatin results for:

  - SNR @ 90% Max TP

  - PMI distribution, with the index of i_1,1, under SNR @ 90% Max TP

Issue 1-2-3-6: Other parameters for eType II PMI reporting requirements (if introduced) / PMI bias study

Agreement:

Common simulation assumptions for PMI Parameter; Unit; Value. Duplex Mode; TDD and FDD. Bandwidth and SCS; 40MHz/30kHz SCS for TDD 10MHz/15kHz SCS for FDD. TDD DL-UL configurations; 7D1S2U S=6D+4G+4U. Antenna configuration; 8T4R (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1). NZP CSI-RS for CSI acquisition; CSI-RS resource Type; Periodic. Number of CSI-RS ports (X); 8. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 8, (4,6). First OFDM symbol in the PRB used for CSI-RS (l0); Row 8, (5). CSI-RS interval and offset; slot; 10, 1 for TDD 5, 1 for FDD. cqi-FormatIndicator; Wideband. pmi-FormatIndicator; Not configured for eType II Wideband for Type I. Sub-band Size; RB; 8 for TDD 4 for FDD. csi-ReportingBand; 11111111111111 for TDD 1111111111111 for FDD. Codebook configuration; Codebook Type; typeII-r16. eType II CB config; paramCombination-r16; 6 (L =4, pν =1/2, β=1/2 ). R(numberOfPMISubbandsPerCQISubband-r16); 1. (CodebookConfig-N1,CodebookConfig-N2); (4,1). (CodebookConfig-O1,CodebookConfig-O2); (4,1). CodebookSubsetRestriction; 0x FFFF. RI Restriction (typeII-RI-Restriction-r16); Rank 2: 0010 Rank 4: 1000. Physical channel for CSI report; PUSCH. CQI/RI/PMI delay; ms; 7 for TDD 9 for FDD. Maximum number of HARQ transmission; 4. Simulation assumptions for CDL channel Parameter; Value. Rank4; Rank8. FR; FR1. UE movement direction; (65°,90°). AAV assumptions; (M,N,P,Ms,Ns) = (1,2,2,1,1); (M,N,P,Ms,Ns) = (1,4,2,1,1). Channel Geometry; LCS UE; α = 180°, β=0°, γ = 0°. LCS gNodeB; α = 0°, β=10°, γ = 0°. GCS UE; Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m. GCS gNodeB; Height = 25 m; Azimuth = 0; X Coordinate = 0 m. BS Antenna Polarisation; Cross Polarized antenna elements with +/-45 degrees polarization slant angles. BS Radiation Pattern; TBD. UE Antenna Polarisation; cross-polarized antenna elements with +90/0 degrees polarization slant angles. UE Antenna Radiation Pattern; Omnidirectional. Antenna Panel Placement; YZ Plane.

Issue 1-2-3-7: Test metric for eType II PMI reporting (if introduced)

- Proposals:

  - Proposal 1: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

  - Proposal 2: Define PMI reporting performance requirements with the metric of SNR@70% follow PMI throughput

  - Proposal 3: Take SNR @ 90 % throughput as the eType-II PMI requirement metric

- Test applicability for SCM SU-MIMO requirements

Issue 1-3-1: Test applicability for SCM SU-MIMO requirements

- Proposals:

  - Proposal 1: Introduce new declaration/capability for the SCM testing.

- CTC: RAN4 needs to discuss whether to define single or multiple declarations on SCM requirements for each individual SCM test cases.

- Nokia: Define optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules.

- Apple: Requirements with rCDL shall be introduced as optional and based on UE declaration (capability without signaling)

- Samsung, ZTE, Huawei, Ericsson, Qualcomm: Introduce one single new declaration for all rCDL based demodulation performance tests

  - Ericsson: Consider “SCM tested MIMO receiver” as common declaration.

  - Proposal 1A: The associated feature and applicability of requirements can be discussed during CR drafting stage towards the end of the WI

  - Proposal 2: Discuss the UE declaration and applicability rules if requirements with rCDL are introduced

Issue 1-3-2: Release independent for SCM SU-MIMO requirements

- Proposals:

  - Proposal 1: Discuss whether to define release independent rules for the new SCM requirements.

  - Proposal 2: No release independent from previous release

- Topic #2: UE requirements for 6Rx UE with interference

- General

Issue 2-1-1: Antenna correlation

Agreements:

ULA correlation channel with α = 0 and β = 0.07179 as baseline for all new 6Rx requirements.

- PDCSH requirements with inter cell interference

Issue 2-2-1: Propagation condition for PDCSH requirements with inter cell interference

- Same status as in the previous WF R4-2522662:

Agreement: For initial simulation purpose - For 2 interfering cells (HomNet): - Option 1: TDLA30-10 - Option 2: TDLC300-100 (prioritized if companies can only provide simulation for one propagation channel) - For 1 interfering cell (HetNet): - TDLA30-10

Issue 2-2-2: MCS for PDCSH requirements with inter cell interference

- Way forward:

  - Candidate options for initial simulation evaluation:

- Rank 1: MCS 13, MCS 17

- Rank 2: MCS 13, MCS 17

- Rank 4: MCS 11, Other MCS indexes are not precluded for rank 4

- PDCSH requirements with intra cell inter user interference

Issue 2-3-1: Propagation condition for PDCSH requirements with intra cell inter user interference

- Same status as in the previous WF R4-2522662:

- Option 1: TDLA30-10 - Option 2: Cover both TDLC300-100 and TDLA30-10 for initial simulation - Option 2a: TDLC300-100 for rank 1+1 and TDLA30-10 for rank 2+2

Issue 2-3-2: Precoding method for the co-scheduled UE for PDCSH requirements with intra cell inter user interference

Agreement

  - Random precoding with rank 1+1 and orthogonal precoding for rank 2+2

Issue 2-3-3: MCS for the target UE for PDCSH requirements with intra cell inter user interference

- Way forward:

  - Candidate options for initial simulation evaluation:

- Rank 1+1: MCS 13, MCS 17, MCS 19

- Rank 2+2: MCS 13, Other MCS indexes are not precluded for rank 2+2

- CQI reporting requirements with inter-cell interference

Issue 2-4-1: SINR range for simulation for CQI reporting requirements with inter-cell interference

  - Proposal 1: For simulation alignment purpose, companies to provide simulation results for SINR from -4dB to 10 dB

- Applicability and release independent

Issue 2-5-1: Test applicability for 6Rx requirements with interference

- Proposals:

  - Proposal 1: Define the following applicability rules for 6Rx requirements

    - For UEs supporting 6Rx and other antenna ports (2/4/8Rx), if the UE passes tests with interference with more UE antennas, it can skip requirements with interference with less UE antennas, no matter the same test configurations are defined or not.

    - No additional test applicability is needed.

  - Proposal 2: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule

  - Proposal 3

    - Define a test applicability rule similar as 8Rx defined in sub-section 5.1.1.2 to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

    - for UEs could support both 6Rx and 8Rx bands, the test cases to be conducted need to be clarified.

  - Proposal 4: follow the same applicability rules defined in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’

  - Proposal 5: UE that meets 8Rx performance requirements can skip 6Rx requirements if and only if the test settings are identical

  - Proposal 5: Further discuss

Issue 2-5-2: Release independent for 6Rx requirements with interference

Further discuss:

  - Option 1: Release independent from Rel-19

  - Option 2: No release independent should be introduced