3GPP TSG-RAN WG4 Meeting #118	R4-2600xxx

# Gothenburg, Sweden, 9th – 13th February, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.161 | CR | 0030 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.161 on Rel-19 FR1 TRP TRS requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Apple, Samsung, Xiaomi, Huawei, HiSilicon, OPPO, Vodafone, Telecom Italia, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The agreed Rel-19 FR1 TRP TRS requirements should be added in TS to complete the WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the Rel-19 FR1 TRP TRS requirements into TS 38.161. The following requirements have been added: - n3/5/7/8 TRP for both talk and browsing mode - n3/5/7/8 TRS for both talk and browsing mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1, 6.2.1.2.1, 7.2.1.1.1, 7.2.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR will be revised and updated to accomodate the final requirements. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.2	Minimum requirement

6.2.1	Minimum requirement for handheld UE

The average measured total radiated power (TRP) of low, mid and high channel for handheld UE shall be higher than the average TRP requirement specified in subclauses 6.2.1.1 and 6.2.1.2. The averaging shall be done in linear scale for the TRP results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRP results of both right and left hand phantom measurements.

TRPaverage=10log10Pleft_low/10+10Pleft_mid/10+10Pleft_high/10+10Pright_low/10+10Pright_mid/10+10Pright_high/106

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of this specification.

6.2.1.1	Hand phantom browsing mode

Hand phantom browsing mode positions are defined in Clause B.3.1.

6.2.1.1.1	NR FR1

Handheld UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode are defined in Tables 6.2.1.1.1-1 and 6.2.1.1.1-2.

Table 6.2.1.1.1-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR | 12.1 | 11.6 |
| n3 | 20 | HL and HR | 11 | 11 |
| n5 | 15 | HL and HR | 10 | 10 |
| n7 | 15 | HL and HR | 11 | 11 |
| n8 | 15 | HL and HR | 10 | 10 |
| n28 | 20 | HL and HR | 10.5 | 10.5 |
| n41 | 100 | HL and HR | 9.5 | 9.5 |
| n78 | 100 | HL and HR | 10 | 10 |
| n79 | 100 | HL and HR |  |  |
| NOTE 1: Void |  |  |  |  |

Table 6.2.1.1-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  |  |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR | 12.5 | 12.5 |
| n78 | 100 | HL and HR | 13 | 13 |
| n79 | 100 | HL and HR |  |  |

6.2.1.1.2	NR FR1 in EN-DC mode

Handheld UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the hand phantom browsing position and the primary mechanical mode are defined in Tables 6.2.1.1.2-1 and 6.2.1.1.2-2.

Table 6.2.1.1.2-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  |  |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR |  |  |
| n78 | 100 | HL and HR |  |  |
| n79 | 100 | HL and HR |  |  |

Table 6.2.1.1.2-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  |  |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR |  |  |
| n78 | 100 | HL and HR |  |  |
| n79 | 100 | HL and HR |  |  |

6.2.1.2	Beside the head and hand phantom talk mode

Beside the head and hand phantom mode positions are defined in Clause B.3.2 of this specification.

6.2.1.2.1	NR FR1

Handheld UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode are defined in Tables 6.2.1.2.1-1 and 6.2.1.2.1-2.

Table 6.2.1.2.1-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR | 11.1 | 10.6 |
| n3 | 20 | BHHL and BHHR | 9.63 | 9.63 |
| n5 | 15 | BHHL and BHHR | 7.6 | 7.6 |
| n7 | 15 | BHHL and BHHR | 9.64 | 9.64 |
| n8 | 15 | BHHL and BHHR | 7.6 | 7.6 |
| n28 | 20 | BHHL and BHHR | 7.6 | 7.6 |
| n41 | 100 | BHHL and BHHR | 7 | 7 |
| n78 | 100 | BHHL and BHHR | 9 | 9 |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: 0.6dB higher value will be adopted in September 2027 (RAN#117). NOTE 4: 0.8dB higher value will be adopted in September 2027 (RAN#117). |  |  |  |  |

Table 6.2.1.2.1-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  |  |
| n28 | 20 | BHHL and BHHR |  |  |
| n41 | 100 | BHHL and BHHR | 10 | 10 |
| n78 | 100 | BHHL and BHHR | 12 | 12 |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: Void. |  |  |  |  |

6.2.1.2.2	NR FR1 in EN-DC mode

Handheld UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the beside head and hand phantom talk position and the primary mechanical mode are defined in Tables 6.2.1.2.2-1 and 6.2.1.2.2-2.

Table 6.2.1.2.2-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  |  |
| n28 | 20 | BHHL and BHHR |  |  |
| n41 | 100 | BHHL and BHHR |  |  |
| n78 | 100 | BHHL and BHHR |  |  |
| n79 | 100 | BHHL and BHHR |  |  |

Table 6.2.1.2.2-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands (in EN-DC mode) in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  |  |
| n28 | 20 | BHHL and BHHR |  |  |
| n41 | 100 | BHHL and BHHR |  |  |
| n78 | 100 | BHHL and BHHR |  |  |
| n79 | 100 | BHHL and BHHR |  |  |

==============Next change==============

7.2	Minimum requirement

7.2.1	Minimum requirement for handheld UE

The average measured total radiated sensitivity (TRS) of low, mid and high channel for handheld UE shall be lower than the average TRS requirement specified in subclauses 7.2.1.1 and 7.2.1.2. The averaging shall be done in linear scale for the TRS results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRP results of both right and left hand phantom measurements.

TRSaverage=10log6/(110Pleft_low10+110Pleft_mid10+110Pleft_high10+110Pright_low10+110Pright_mid10+110Pright_high10)

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of this specification.

7.2.1.1	Hand phantom browsing mode

Hand phantom browsing mode positions are defined in Clause B.3.1.

7.2.1.1.1	NR FR1

Handheld UE TRS minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode are defined in Tables 7.2.1.1.1-1 and 7.2.1.1.1-2.

Table 7.2.1.1.1-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR | -89 | -88.5 |
| n3 | 20 | HL and HR | -86.5 | -86.5 |
| n5 | 15 | HL and HR | -84.5 | -84.5 |
| n7 | 15 | HL and HR | -87.5 | -87.5 |
| n8 | 15 | HL and HR | -84.5 | -84.5 |
| n28 | 20 | HL and HR | -83.5 | -83.5 |
| n41 | 100 | HL and HR | -80 | -80 |
| n78 | 100 | HL and HR | -81.2 | -81.2 |
| n79 | 100 | HL and HR |  |  |

7.2.1.1.2	NR FR1 in EN-DC mode

Handheld UE TRS minimum performance requirement for NR FR1 bands (in EN-DC mode) in the hand phantom browsing position and the primary mechanical mode are defined in Tables 6.2.1.1.2-1 and 6.2.1.1.2-2.

Table 7.2.1.1.2-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands (in EN-DC mode) in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  |  |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR |  |  |
| n78 | 100 | HL and HR |  |  |
| n79 | 100 | HL and HR |  |  |

7.2.1.2	Beside the head and hand phantom position

Beside the head and hand phantom mode positions are defined in Clause B.3.2.

7.2.1.2.1	NR FR1

Handheld UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom position and the primary mechanical mode are defined in Tables 7.2.1.2.1-1 and 7.2.1.2.1-2.

Table 7.2.1.2.1-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands in the beside head and hand phantom position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR | -87.5 | -87 |
| n3 | 20 | BHHL and BHHR | -84 | -84 |
| n5 | 15 | BHHL and BHHR | -80.6 | -80.6 |
| n7 | 15 | BHHL and BHHR | -85 | -85 |
| n8 | 15 | BHHL and BHHR | -80.6 | -80.6 |
| n28 | 20 | BHHL and BHHR | -80 | -80 |
| n41 | 100 | BHHL and BHHR | -79 | -79 |
| n78 | 100 | BHHL and BHHR | -80 | -80 |
| n79 | 100 | BHHL and BHHR |  |  |

==============End of change==============