3GPP TSG-RAN4 Meeting #118                                                                        R4-2602079

# Gothenburgh, Sweden, 9th Nov 2025 – 13th Feb 2026

Agenda item:			4.1.3

Source:	Moderator (Eutelsat Group)

Title:	[118][303] Ku_band_Maintenance

Document for:	Information

# Introduction

This is the topic summary for Ku Band maintenance.

:

- Introduction of Ku Band for NR NTN [NR_NTN_Ku_bands-Core] - System parameters [NR_NTN_Ku_bands-Core] - UE RF requirements [NR_NTN_Ku_bands-Core] - SAN RF core requirements - RRM core requirements

.

# Topic #1: System parameters (agenda 4.2.7.1)

## Companies’ contributions summary

All Tdocs related to the following topics (agenda 4.2.7.1) are listed here:

| Tdoc | Company | Proposals / Observations |
| --- | --- | --- |
| Moved to 11.2 [324] |  |  |

# Topic #2: UE RF requirements (agenda 4.2.7.2)

## Companies’ contributions summary

All Tdocs related to the following topics (agenda 4.2.7.2) are listed here:

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2600777 | MediaTek (Chengdu) Inc. | Observation 1:When the VSAT transmit signal bandwidth is greater than or equal to 4.32 MHz (SCS = 15 kHz, 24 RBs), the A‑MPR is 0 dB, and when the VSAT transmit signal bandwidth is smaller than 4.32 MHz (SCS = 15 kHz, 24 RBs), A MPR is needed to reduce the EIRP PSD level to meet the off-axis EIRP requirements. Proposal 1: The A-MPR values can be calculated based on the formula Max( 0, -10log10(NRB SCS 12 / factor)), where the factor is 24 RBs 15 kHz SCS 12, i.e., 4.32 MHz. Observation 2: When the VSAT transmit signal bandwidth is greater than or equal to 1.08 MHz (SCS = 15 kHz, 6 RBs), the A‑MPR is 0 dB, and when the VSAT transmit signal bandwidth is smaller than 1.08 MHz (SCS = 15 kHz, 6 RBs), A‑MPR is needed to reduce the EIRP PSD level to meet the off‑axis EIRP requirements. Proposal 2: The A-MPR values can be calculated based on the formula Max( 0, -10log10(NRB SCS 12 / factor)), where the factor is 6 RBs 15 kHz SCS 12, i.e., 1.08 MHz. Observation 3: When the VSAT transmit signal bandwidth is greater than or equal to 9.36 MHz (SCS = 15 kHz, 52 RBs), the A MPR is 0 dB, and when the VSAT transmit signal bandwidth is smaller than 9.36MHz (SCS = 15 kHz, 52RBs), A MPR is needed to reduce the EIRP PSD level to meet the off axis EIRP requirements. Proposal 3: The A-MPR values can be calculated based on the formula Max( 0, -10log10(NRB SCS 12 / factor)), where the factor is 52 RBs 15 kHz SCS 12, i.e. 9.36 MHz. Observation 4: No A-MPR is needed for the mechanically steered VSAT Proposal 4: RAN4 to consider adding A‑MPR requirements for Ku‑band electrically steered VSATs in 38.101‑5 when communicating with GSO satellites, as shown in the following table. |
| R4-2601748 | MediaTek Inc. |  |
| R4-2601061 | ZTE Corporation, Sanechips |  |
| R4-2601750 | MediaTek (Chengdu) Inc. | Unavailable |

## Open issues summary

### Sub-topic 2-1: Addition of VSAT AMPR

- Proposals:

- Proposal from MediaTek (Chengdu) Inc. in R4-2600777

- CR from MediaTek Inc. in R4-2601748

- Moderator Recommendation:

- What is the applicability of the 3 notes in Tale 9.7.1-3?

- 1.08 MHz is not possible in bands 609/508

- Discussion required

### Sub-topic 2-2: Removal of redundant 60 KHz SCS channel bandwidths

- Proposals:

- CR from ZTE Corporation, Sanechips in R4-2601061

- Moderator Recommendation:

- If this is currently redundant information, is it essential to remove it?

- Discussion required

# Topic #3: SAN RF requirements (agenda 4.2.7.3)

## Companies’ contributions summary

All Tdocs related to the following topics (agenda 4.2.7.3) are listed here:

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2600208 | CATT |  |
| R4-2601062 | ZTE Corporation, Sanechips |  |

## Open issues summary

### Sub-topic 3-1: Corrections for SAN type 1-O operating below 10 GHz

- Proposals:

- CR from CATT in R4-2600208

- Remove redundant statement:

- Moderator Recommendation:

- Approve the first change in CR R4-2600208

### Sub-topic 3-2: Correction for OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz

- Proposals:

- CR from CATT in R4-2600208

- Remove redundant square brackets:

- Moderator Recommendation:

- Approve the second change in CR R4-2600208

### Sub-topic 3-3: Removal of redundant 60 KHz SCS channel bandwidths

- Proposals:

- CR from ZTE Corporation, Sanechips in R4-2601062

- Moderator Recommendation:

- This is the same as sub-topic 2-2 for the UE.

- If this is currently redundant information, is it essential to remove it?

- Discussion required

# Topic #4: RRM requirements (agenda 4.2.7.4)

## Companies’ contributions summary

All Tdocs related to the following topics (agenda 4.2.7.4) are listed here:

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2600657 | MediaTek inc. | Relax the scheduling restriction during a soft satellite switch when 'deriveSSB-IndexFromCell' is not enabled. |
| R4-2601359 | ZTE Corporation, Sanechips | Proposal 1: The re-defined scheduling restriction shall be considered as below: In addition, following scheduling restriction applies during the time period from t-serviceStart to t-service, - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured decided by source SSB configuration and ssb-TimeOffset , and KRX_switch data symbol before and after each consecutive SSB symbols to be measured. - where KRX_switch is for the switch period of UE to steer the downlink spatial domain reception filter toward a target satellite. KRX_switch=1. |
| R4-2601374 | ZTE Corporation, Sanechips |  |
| R4-2601537 | Huawei, HiSilicon | Proposal 1: Define scheduling restriction for soft satellite switch as UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols of the target satellite, and K data symbol before and after each consecutive SSB symbols of the target satellite, where K=1. |
| R4-2601538 | Huawei, HiSilicon |  |

## Open issues summary

### Sub-topic 4-1: Scheduling restriction for NR SAN Satellite switching with re-synchronization

- Proposals:

- CR from MediaTek Inc. in R4-2600657

Relax the scheduling restriction during a soft satellite switch when 'deriveSSB-IndexFromCell' is not enabled.

- Proposal 1 from ZTE Corporation, Sanechips in R4-2601359

The re-defined scheduling restriction shall be considered as below:

In addition, following scheduling restriction applies during the time period from t-serviceStart to t-service,

- The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured decided by source SSB configuration and ssb-TimeOffset , and KRX_switch data symbol before and after each consecutive SSB symbols to be measured.

- where KRX_switch is for the switch period of UE to steer the downlink spatial domain reception filter toward a target satellite. KRX_switch=1.

- CR from ZTE Corporation, Sanechips in R4-2601374

- Proposal 1 from Huawei, HiSilicon in R4-2601537

Proposal 1: Define scheduling restriction for soft satellite switch as

UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols of the target satellite, and K data symbol before and after each consecutive SSB symbols of the target satellite, where K=1.

- CR from Huawei, HiSilicon in R4-2601538

- Moderator Recommendation:

- The three proposals to 6.1C.3.3.3 do not align, discussion required.