3GPP TSG-RAN WG4 Meeting #118	R4-2602087

Gothenburg, Sweden, 9th – 13th February 2026

Agenda item:			6.22

Source:	Moderator (Nokia)

Title:		Topic summary for [118][312] Rel-19 Demodulation_Part3

Document for:	Information

# Introduction

This document summarises the contributions for Rel-19 Demodulation_Part3 (a combination of demodulation aspects from the following NTN Work Items NR_NTN_Ku_bands-Perf, IoT_NTN_Ph3-Perf, IoT_NTN_TDD-Perf) under AI’s  5.7.3, 6.20.4, and 6.21.3 at RAN4#118.

The proposals from the contributions are grouped into the following topics:

- Topic #1: NR NTN Ku Band Demodulation

- Topic #2: IoT NTN Phase 3 Demodulation

- Topic #3: IoT NTN TDD Demodulation

# Topic #1: NR NTN Ku Band Demodulation

## Companies’ contributions summary

| T-doc number | Source | Proposals / Observations |
| --- | --- | --- |
| R4-2600101 | SES | Simulation results |
| R4-2600195 | CATT | Draft CR |
| R4-2600196 | CATT | CR |
| R4-2600485 | MediaTek inc. | Proposal 1: Consider 16 HARQ processes with a maximum of 4 re-transmissions to define requirements. Observation 1: Unlike FR2 mmWave implementations, Ku band implementations at 12–18 GHz generally allow for discrete antenna elements and accessible RF connectors (e.g., SMA or K-type). Observation 2: Introducing the Over-the-Air interface for demodulation tests conflates the modem performance with antenna radiation patterns, making it impossible to debug whether a failure is due to poor demodulation or antenna misalignment. Observation 3: Conductive testing enables cost-effective, massively parallel testing using standard lab equipment, whereas OTA testing imposes a disproportionate CAPEX burden and limits test capacity. Observation 4: Mandating OTA for these tests would create a severe bottleneck, as chamber resources are scarce compared to conductive bench setups. This added complexity and test duration would significantly delay the Time-to-Market for Ku band devices, hindering the rapid commercial deployment that the satellite ecosystem currently demands. Observation 5: By using conductive methods for demodulation, we are simply ensuring that the modem performance is verified with maximum precision, complementary to the OTA RF tests. Proposal 2: Based on Observations 1 to 5, we propose adopting a conducted methodology for Ku-band demodulation tests and capturing the corresponding requirements in Clause 8. |
| R4-2601253 | Huawei, HiSilicon | Simulation results |
| R4-2601283 | Ericsson | Proposal 1 Take Table 2-2 and Table 2-3 for FR1-NTN Ku band PDSCH test cases. Proposal 2 Take updates in Table 2-4 for TS 38.101-5 Table 11.2.1.1.2-1. Proposal 3 Don’t configure “CSI-RS periodicity” for 15 kHz and 30 kHz SCS in “CSI-RS for beam refinement” parameter set. |
| R4-2601284 | Ericsson | Simulation results |
| R4-2601285 | Ericsson | Draft CR |

## Open issues summary

### Sub-Topic 1-1: Ku Band UE Demodulation Requirements

#### Issue 1-1-1: HARQ Processes

Options:

- Option 1: 16 HARQ processes with a maximum of 4 re-transmissions. (MediaTek, Ericsson)

- Option 2: 32 HARQ processes with a maximum of 4 re-transmissions. (Ericsson)

- Option 3: 4 with Feedback Disabled. (Ericsson, SES)

Recommended Way Forward:

#### Issue 1-1-2: Test Methodology

Options:

- Option 1: adopting a conducted methodology for Ku-band demodulation tests and capturing the corresponding requirements in Clause 8.  (MediaTek)

Recommended Way Forward:

- Confirm during the meeting whether conducted methodology will be used, and update any impacted CRs accordingly.

#### Issue 1-1-3: CSI-RS Periodicity configuration

Options:

- Option 1: Don’t configure “CSI-RS periodicity” for 15 kHz and 30 kHz SCS in “CSI-RS for beam refinement” parameter set. (Ericsson)

Recommended Way Forward:

- Discuss during the meeting whether not configuring CSI-RS periodicity is agreeable

#### Issue 1-1-4: Requirements Applicability

Options:

- Option 1: Use the following requirements applicability table (Ericsson)

| UE feature/capability [11] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR2-NTN | PDCCH | Clause 11.2.3.1.1.1 (Test 1-1, Test 1-2) |  |
|  | FR1-NTN | PDCCH | Clause 11.2.3.1.1.1 (Test 2-1, Test 2-2, Test 2-3, Test 2-4) |  |
| NR NTN scenario support (ntn-ScenarioSupport-r17) | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 2-1, Test 2-2, Test 2-3, Test 2-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “gso” |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 4-1, Test 4-2, Test 4-3, Test 4-4, Test 4-5, Test 4-6) |  |
|  | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “ngso” or is not configured. |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 3-1, Test 3-2, Test 3-3, Test 3-4, Test 3-5, Test 3-6) |  |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 1-3, 2-3) |  |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 3-3, 3-4, 4-3, 4-4) |  |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR2-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 1-4, 2-4) |  |
|  | FR1-NTN | PDSCH | Clause 11.2.2.1.1.1 (Test 3-5, 3-6, 4-5, 4-6) |  |

Recommended Way Forward:

- Discuss during the meeting regarding whether the applicability table can be used.

# Topic #2: IoT NTN Phase 3 Demodulation

## Companies’ contributions summary

| T-doc number | Source | Proposals / Observations |
| --- | --- | --- |
| R4-2601234 | ZTE Corporation, Sanechips | Simulation Results |
| R4-2601235 | ZTE Corporation, Sanechips | Simulation Summary |
| R4-2601236 | ZTE Corporation, Sanechips | Big CR |
| R4-2601254 | Huawei, HiSilicon | Simulation Results |
| R4-2601613 | Ericsson | Proposal 1: If the span of the companies’ results is large for NPUSCH format 1 simulation result, RAN4 should discuss how to set the final requirements, for example, 1) removing outlier or 2) adding extra margin. |
| R4-2601614 | Ericsson | Big CR |
| R4-2601634 | Samsung | Draft CR |

## Open issues summary

### Sub-topic 2-1: NPUSCH Aspects

#### Issue 2-1-1: Span

Options:

- Option 1: If the span of the companies’ results is large for NPUSCH format 1 simulation result, RAN4 should discuss how to set the final requirements, for example, 1) removing outlier or 2) adding extra margin. (Ericsson)

Recommended Way Forward:

- Discuss during RAN4#118 based on simulation summary

# Topic #3: IoT NTN TDD Demodulation

## Companies’ contributions summary

| T-doc number | Source | Proposals / Observations |
| --- | --- | --- |
| R4-2600102 | CAMBRIDGE CONSULTANTS LTD | Proposal 1: RAN4 to consider NPRACH demodulation performance requirements for IoT NTN TDD mode for alignment, as follows Proposal 2: Following the example from RAN4#108 [6], RAN4 to apply a 2.0 dB impairment margin to the NPRACH demodulation simulation results for IOT NTN TDD, prior to averaging across companies, followed by an additional 0.5 dB error margin to the averaged value. Proposal 3: RAN4 to consider NPUSCH format 1 demodulation performance for IoT NTN TDD mode to alignment with results from other companies as follows: Proposal 4: RAN4 to apply a 2.0 dB impairment margin to the NPUSCH format 1 demodulation simulation results for IOT NTN TDD, prior to averaging across companies, followed by an additional 0.5 dB error margin to the averaged value if the span is within 2.5 dB. Proposal 5: The existing performance requirement for NPUSCH format 2 transmissions related to the probability of false ACK detection from DTX shall be equal to or less than 1% can also be applied to IoT NTN TDD mode. Proposal 6: RAN4 to consider NPUSCH format 2 missed ACK detection performance Proposal 7: RAN4 to consider impairment margin of 2.0 dB for IOT NTN TDD, in valid NPUSCH format 2 detection performance results from each company before averaging. Consider additional error margin of 0.1 dB on the averaged value. |
| R4-2601255 | Huawei, HiSilicon | Proposal 1: Select the repetition number value that the total TxDuration not larger than 8ms, i.e. do not cross TDD pattern scheduling. (4 for Format 0 and 2 for Format 1) |
| R4-2602044 | THALES, Nordic Semiconductor, Iridium Satellite LLC, Cambridge Consultants LTD | CR |
| R4-2602045 | THALES, Iridium Satellite LLC, Cambridge Consultants LTD, Samsung | CR |
| R4-2602047 | THALES, Iridium Satellite LLC, Cambridge Consultants LTD, Samsung, Nokia | CR |
| R4-2602075 | THALES | Simulation Results |

## Open issues summary

### Sub-topic 3-1: SAN Demodulation – NPRACH

#### Issue 3-1-1: Repetition Number

Options:

- Option 1: Select the repetition number value that the total TxDuration not larger than 8ms, i.e. do not cross TDD pattern scheduling. (4 for Format 0 and 2 for Format 1) (Huawei)

- Option 2: 8 for Format 0 and Format 1 (Cambridge Consultants)

- Option 3: 4 for Format 0 and Format 1 (Cambridge Consultants)

Recommended Way Forward:

- Discuss during the meeting, noting that if alignment cannot be made this is the last meeting, so at a minimum 4 for Format 0 is agreeable to all parties.

### Sub-topic 3-2: Impairment Margin

#### Issue 3-1-2: Impairment Margin

Options:

- Option 1: Add 2 dB Impairment Margin prior to averaging, followed by an additional 0.5 dB error margin for NPRACH (Cambridge Consultants)

- Option 2: Add 2 dB Impairment Margin prior to averaging, followed by an additional 0.5 dB error margin for NPUSCH Format 1 (Cambridge Consultants)

- Option 3: Add 2 dB Impairment Margin prior to averaging, followed by an additional 0.1 dB error margin for NPUSCH Format 2 (Cambridge Consultants)

Recommended Way Forward:

- Discuss during the meeting, but noting that impairment margins are self-declared through simulations typically in demodulation.

# Recommended Disposition of TDocs

| T-doc number | Type | Comments (Optional) |
| --- | --- | --- |
| R4-2600101 | discussion |  |
| R4-2600195 | draftCR |  |
| R4-2600196 | CR |  |
| R4-2600485 | discussion |  |
| R4-2601253 | other |  |
| R4-2601283 | discussion |  |
| R4-2601284 | other |  |
| R4-2601285 | draftCR |  |
| R4-2601234 | discussion |  |
| R4-2601235 | discussion | Reserved only – simulation summary |
| R4-2601236 | CR | Reserved only |
| R4-2601254 | other |  |
| R4-2601613 | discussion |  |
| R4-2601614 | CR |  |
| R4-2601634 | draftCR |  |
| R4-2600102 | discussion |  |
| R4-2601255 | discussion |  |
| R4-2602044 | CR |  |
| R4-2602045 | CR |  |
| R4-2602047 | CR |  |
| R4-2602075 | discussion |  |