3GPP TSG-RAN4 Meeting #118	draft R4-2602868

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.307 | CR | 4514 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.307: introduction of requirements for BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Introduction of requirements for BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of requirements for BOG feature (LTE-based 5G broadcast operation over geosynchronous satellite) would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3A.1, B.2.16, B.4.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 1	Scope

The present document specifies requirements for Rel-19 UEs supporting release independent features like:

-	additional E-UTRA operating frequency bands on top of Rel-19 of TS 36.101 [2], TS 36.102 [6] and TS 36.133 [3];

-	additional E-UTRA CA configurations (intra-band/inter-band) on top of Rel-19 of TS 36.101 [2] and TS 36.133 [3];

-	additional operating bands and/or CA configurations for specific features (like UE category 0, M1, NB1);

-	other release independent features (like 4Rx antenna port, high speed scenario, 8Rx antenna port, NB-IoT, eMTC operation over NTN, or LTE-based 5G Broadcast over Geosynchronous Satellite).

==============Next change==============

## 3A.1	Additional E-UTRA operating bands

Requirements for additional E-UTRA operating bands of TS 36.101 Rel-P [2] are introduced via this clause.

Table 3A.1-1: E-UTRA operating bands and UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands, band number <= 64, Power Class 3 | FDD, TDD | Rel-8 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number > 64, Power Class 3 | FDD, TDD | Rel-9 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, NS-value > 32 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Asymmetric operating bands, Power Class 3 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number <= 64, Power Class 1 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, Power Class 2 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, standalone downlink only | SDO | Rel-17 | Table B.2.1-1 (Clauses 4, 7, 8, and 9 only), Table B.4.1-1 (Clauses 5 and 7 only) |

For example, Band 19 was introduced in the Release 9 specifications. In order to implement a UE conforming to Release 8 but supporting Band 19, it is necessary for the UE to additionally conform to some parts of the Release 9 specifications (see corresponding Annexes of TS 36.307 Rel-9 which will point to the requirements in the Rel-9 of TS 36.101 [2] or TS 36.133 [3] to be fulfilled), such as the radio frequency and radio resource management requirements for the Band 19.

Requirements for additional non-terrestial E-UTRA operating bands of TS 36.102 Rel-P [6] are introduced via this clause.

Table 3A.1-2: Non-terrestial E-UTRA operating bands

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands for LTE-based 5G Broadcast over Geosynchronous Satellite | SDO | Rel-17 | Table B.2.16-1 on RRM requirements, Table B.4.15-1 on UE RF requirements. |

==============Next change==============

## B.2.15	Common RRM requirements for operating bands for UE category 1bis

The requirements and test cases listed in Table B.2.15-1 are specified in TS 36.133 Rel-P [3].

Table B.2.15-1: Common RRM requirements for release independent
operating bands for a UE category 1bis

| Clause | Description |
| --- | --- |
| 8.1.2.5.3, 8.1.2.5.4 | Intra-Frequency OTDOA Measurements for UE Category 1bis |
| 8.1.2.6.5, 8.1.2.6.6, 8.1.2.6.7, 8.1.2.6.8 | Inter-Frequency OTDOA Measurements for UE Category 1bis |
| 9.1.2.7, 9.1.2.8 | Intra-frequency RSRP Accuracy Requirements for UE Category 1bis |
| 9.1.3.1, 9.1.3.4 | Intra-frequency RSRP Accuracy Requirements for UE Category 1bis |
| 9.1.5.5 | Intra-frequency RSRQ Accuracy Requirements for UE Category 1bis |
| 9.1.6.5, 9.1.6.6 | Inter-frequency RSRQ Accuracy Requirements for UE Category 1bis |
| 9.1.10.5, 9.1.10.6 | Reference Signal Time Difference (RSTD) for UE Category 1bis |

## B.2.16	Common RRM requirements for operating bands for LTE-based 5G broadcast operation over geosynchronous satellite

The requirements and test cases listed in Table B.2.16-1 are specified in TS 36.133 Rel-P [3].

Table B.2.16-1: Common RRM requirements for release independent
operating bands for LTE-based 5G broadcast operation over geosynchronous satellite

| Clause | Description |
| --- | --- |
| 4.1 | E-UTRAN RRC_IDLE state mobility requirements: Cell Selection |
| 4.2 | E-UTRAN RRC_IDLE state mobility requirements: Cell Re-selection |
| 4.3 | E-UTRAN RRC_IDLE state mobility requirements: Minimization of Drive Tests (MDT) |
| 4.4 | E-UTRAN RRC_IDLE state mobility requirements: MBSFN Measurements |
| 7.2 | Timing and signalling characteristics requirements: UE timer accuracy |
| 7.6 | Timing and signalling characteristics requirements: Radio Link Monitoring |
| 7.25 | Timing and signalling characteristics requirements: Cell phase synchronization accuracy for MBMS services (FDD) |
| 8.1 | UE Measurements Procedures in RRC_CONNECTED State requirements: General Measurement Requirements |
| 8.2 | UE Measurements Procedures in RRC_CONNECTED State requirements: Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.6.2 | UE Measurements Procedures in RRC_CONNECTED State requirements: Discovery signal measurements: Requirements for CRS based discovery signal measurements |
| 8.9 | UE Measurements Procedures in RRC_CONNECTED State requirements: MBSFN Measurements |
| 9.8 | Measurements performance requirements for UE requirements: MBSFN Measurements |

==============Next change==============

## B.4.14	Common UE RF requirements for operating bands for UE category 1bis

The requirements and test cases listed in Table B.4.14-1 are specified in TS 36.101 Rel-P [2].

Table B.4.14-1: Common UE RF requirements for release independent operating bands for UE category 1bis

| Clause | Description |
| --- | --- |
| 5.5E | Operating bands for UE category 1bis |
| 7.3.1E | Minimum requirements (QPSK) for UE category 1bis |

## B.4.15	UE RF requirements for LTE-based 5G broadcast operation over geosynchronous satellite (BOG)

The requirements and test cases listed in Table B.4.15-1 are specified in TS 36.102 Rel-P [6].

Table B.4.15-1: UE RF requirements for LTE-based 5G broadcast operation over geosynchronous satellite (BOG)

| Clause | Description |
| --- | --- |
| 5.2C | Operating bands |
| 5.3C | Channel bandwidth |
| 5.4C | Channel arrangement |
| 7.3C | Reference sensitivity power level |
| 7.4C | Maximum input level |
| 7.5C | Adjacent Channel Selectivity (ACS) |
| 7.6C | Blocking characteristics |
| 7.7C | Spurious response |
| 7.8C | Intermodulation characteristics |
| 7.9 | RX spurious emissions |

==============End of change==============