3GPP TSG-RAN WG4 Meeting # 118											R4-2602911

Gothenburg, Sweden, Feb 9th ‒ 13th, 2026

Agenda item:			11.1

Source:	Moderator (CATT)

Title:	AH minutes for [118][324] LS_BDaT

Document for:	Information

Introduction

The AH minutes aim at aligning technical understanding in a step-by-step way.

Tentative Agreement:

- Signaling of polarization is needed.

The detailed contents of the signaling is to be decided according to the discussion below.

Issue 1: (Common DL Rx) For mobile VSAT, is it assumed that the receiver is able to detect the linear polarization downlink common channels  (e.g., SSB, SIBs)? (Issue 1-1-1 in the summary 2098)

Common understandings:

- Yes

  - The detected polarization might not be perfectly aligned to common DL Tx

  - The detected polarization would be a reference for UE to perform common UL Tx

    - Co-Pol: means common UL Tx has the same polarization as the detected

    - Cross-Pol: means common UL Tx has the orthogonal polarization to the detected polarization

  - UE has implementation flexibility

    - either by receiving both orthogonal polarization orientation,

    - or by searching the best polarization orientation.

Issue 2: (Common UL Tx) How polarization for common UL Tx is determined by UE?

Common understandings:

- SAN has flexibility

  - Common UL Rx has the same polarization as common DL Tx

  - Common UL Rx has the orthogonal polarization of common DL Tx

- Indication by NW (common signaling for all UEs)

  - Alt. #1: polarization of both common DL and common UL are indicated

  - Alt. #2: only polarization of common UL is indicated

Issue 3: (Dedicated DL Rx) After being connected to NW, how polarization for dedicated DL Rx is determined by UE?

Common understandings:

- SAN has flexibility

  - Dedicated DL Tx has the same polarization as common DL Tx

  - Dedicated DL Tx has the orthogonal polarization of common DL Tx

- Indication by NW (dedicated signaling for the UE)

  - Alt. #1: polarization of both dedicated DL and common DL Tx are indicated

  - Alt. #2: only polarization of dedicated DL Tx is indicated

Issue 4: How polarization for dedicated UL Tx is determined by UE

Common understandings:

- SAN has flexibility

  - Dedicated UL Rx has the same polarization as DL Tx

  - Dedicated UL Rx has the orthogonal polarization of dedicated DL Tx

- Indication by NW (dedicated signaling for the UE)

  - Alt. #1: polarization of both dedicated DL Tx and dedicated UL Rx are indicated

  - Alt. #2: Only polarization of dedicated UL Rx is indicated

Issue 4: How polarization for dedicated UL Tx is determined by UE

Common understandings:

- SAN has flexibility

  - Dedicated UL Rx has the same polarization as DL Tx

  - Dedicated UL Rx has the orthogonal polarization of dedicated DL Tx

- Indication by NW (dedicated signaling for the UE)

  - Alt. #1: polarization of both dedicated DL Tx and dedicated UL Rx are indicated

  - Alt. #2: Only polarization of dedicated UL Rx is indicated

Issue 5: Is skew to be indicated in the signaling for common channels?

Common understandings:

- SAN has flexibility

  - Applying a skew to common channels

- UE can use detected polarization of common DL as a reference

- No need for indication of skew of common channels by NW (common signaling for all UEs)

Issue 6: Is skew to be indicated in the signaling for dedicated channels?

Common understandings:

- SAN has flexibility

  - Applying a skew to dedicated channels enabling the use of multiple antennas

- Skew of dedicated channels for the indicates “polarization deviation” of dedicated channels from common channels

  - Helpful for UE from performance perspective

  - Helpful for SAN to make most use of multiple antennas

  - System performance benefits

- Need for indication of skew of dedicated channels by NW (dedicated signaling for the UE)