3GPP TSG-WG4 Meeting # 118	R4-2600209

# Gothenburg, Sweden, February 9th – 13th , 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.194 | CR | 0007 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (Ambient_IoT_Solutions-Core)CR for 38.194, on framework simplification for co-location/co-existence requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2026-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The agreed CR R4-2523018 proposes to simplify framework for co-location/co-existence requirement in TS 38.104 with new tables, it is reasonable to simplify framework for co-location/co-existence requirement for A-IoT BS in TS 38.194. BS type 1-H is not supported for A-IoT BS, it should be removed from the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add reference for 3GPP TS 25.104. TS 36.104 and TS 38.104. 2) Replace co-existence spurious requirement in Table 6.5.5.2.2-1 with new format table. 3) Replace co-location spurious requirement in Table 6.5.5.2.3-1 with new format table. 4) Remove the BS type 1-H from the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Co-location/co-existence requirement would bring a lot of work every time a new band is introduced or a correction is made, and BS Type 1-H is wrongly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, ,6.5.5.2.2, 6.5.5.2.3, 7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.195 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from R4-2600209 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[3] 	3GPP TS 38.195: "Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing".

[4]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TS 38.211: "Physical channels and modulation".

[6]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception ".

[7]	3GPP TS 38.291: "NR; Ambient IoT Physical layer".

[8]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)"

[9]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[10]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

==============Next change==============

6.5.5.2.2	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.) as listed below.

The spurious emission basic limits are provided in table 6.5.5.2.2 -1 for a BS where requirements for co-existence with the system listed in the first column apply.

Table 6.5.5.2.2-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for A-IoT to co-exist with (Note 3) | Frequency range for co-existence requirement (MHz) (Note 4) | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 | -57 | 100 kHz | Note 1 |
|  | 824 - 849 | -61 |  |  |
| GSM900 | 921 - 960 | -57 |  |  |
|  | 876 - 915 | -61 |  |  |
| DCS1800 | 1805 - 1880 | -47 |  |  |
|  | 1710 - 1785 | -61 |  |  |
| PCS1900 | 1930 - 1990 | -47 |  |  |
|  | 1850 - 1910 | -61 |  |  |
| UTRA, E-UTRA or NR | Frequency range of downlink operating band of the BS to co-exist with | -52 | 1 MHz | Note 1, Note 5 |
|  | Frequency range of uplink operating band of the BS to co-exist with | -49 |  | Note 1 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.5.5.2.2-1 do not apply for the 10MHz frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.5.5.2.2-1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104 [9], table 5.5-1.

NOTE 4:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104 [8] clause 5.2, TS 36.104 [9] clause 5.5 and TS 38.104 [10] clause 5.2, respectively.

NOTE 5:	For TDD bands, -52 dBm/MHz basic limit applies.

Table 6.5.5.2.3-3: Void

6.5.5.2.3	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The basic limits are in table 6.5.5.2.3-1 for a BS where requirements for co-location with a BS type listed in the second column apply, depending on the declared Base Station class.

Table 6.5.5.2.3-1: BS spurious emissions basic limits for BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) (Note 1) |
| --- | --- | --- |
| co-located BS (MHz) (Note 3) |  | MR BS |
| 824 - 849 | GSM850 or CDMA850 | -91 |
| 876 - 915 | GSM900 | -91 |
| 1710 - 1785 | DCS1800 | -91 |
| 1850 - 1910 | PCS1900 | -91 |
| 49, 51/n51, n91, n93 | E-UTRA or NR | N/A |
| 46/n46, 53/n53 | E-UTRA or NR | -91 |
| n100, n101 | NR | N/A |
| n96, n102 | NR | -90 |
| n104 | NR | -90 |
| Other operating band | UTRA, E-UTRA or NR | -91 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.5.5.2.3-1 do not apply for the frequency range extending 10MHz immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4].

NOTE 2:	Table 6.5.5.2.3-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104 [7] clause 5.2, TS 36.104 [9] clause 5.5 and TS 38.104 [10] clause 5.2, respectively.

==============Next change==============

7.4	Out-of-band blocking

### 7.4.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.4.2	Minimum requirement for BS type 1-C

the MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in table 7.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-C is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

Table 7.4.2-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |
| NOTE : PREFSENS depends also on the BS channel bandwidth as specified in Table 7.2.2-1. |  |  |

==============End of change==============