3GPP TSG-RAN WG4 Meeting #118	R4-260xxxx

# Gothenburg, Sweden, February 9th – 13th, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 Introduction of NR SAN handover requirements for RedCap in FR1-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The Rel-19 work item NR_NTN_Ph3 specifies in objective 5 the support of (e)RedCap UEs in FR1-NTN. Thus, RRM performance requirements need to be introduced for NR SAN handover in RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of RRM performance requirements for intra-frequency / inter-frequency / RACH-less NR SAN handover in RRC_CONNECTED for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing NR SAN handover requirements in RRC_CONNECTED for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A20.2.1.X1 (new), A20.2.1.X2 (new), A20.2.1.X3 (new), A20.2.1.X4 (new), A20.2.1.X5 (new), A20.2.1.X6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2601738. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### A.20.2.1.X1	Intra-frequency SAN Handover from FR1 to FR1 for 1Rx RedCap UE

##### A.20.2.1.X1.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN Handover from FR1 to FR1 specified in clause 6.1F.1 for 1Rx RedCap UE.

##### A.20.2.1.X1.2	Test Parameters

Test parameters in clause A.14.2.1.1.2 shall apply except that the supported test configurations are defined in table A.20.2.1.X1.2-1, and NR Cell specific test parameters in Table A.20.2.1.X1.2-2 replace the corresponding parameters in Table A.14.2.1.1.2-3. Other parameters in Table A.14.2.1.1.2-2 and Table A.14.2.1.1.2-3 shall apply to test configuration 1, 2, 3 and 4.

In the test, the target cell is known by the UE and carries only CD-SSB.

Table A.20.2.1.X1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.20.2.1.X1.2-2: Cell specific test parameters for Intra frequency SAN handover test case

| Parameter | Test configuration | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information | Config 1,3 |  | SSC.1 |  |  | NSC.1 |  |  |
|  | Config 2,4 |  | SSC.2 |  |  | NSC.2 |  |  |
| Koffset | Config 1,3 | ms | 239 |  |  | 239 |  |  |
|  | Config 2,4 |  | 4 |  |  | 4 |  |  |
| Antenna Configuration | 1Rx |  | 1x1 |  |  |  |  |  |

##### A.20.2.1.X1.3	Test Requirements

Test requirements in clause A.14.2.1.1.3 shall apply for 1Rx RedCap UEs.

#### A.20.2.1.X2	Intra-frequency SAN Handover from FR1 to FR1 for 2Rx RedCap UE

##### A.20.2.1.X2.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN Handover from FR1 to FR1 specified in clause 6.1F.1 for 2Rx RedCap UE.

##### A.20.2.1.X2.2	Test Parameters

Test parameters in clause A.20.2.1.X1.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.X2.3	Test Requirements

Test requirements in clause A.14.2.1.1.3 shall apply for 2Rx RedCap UEs.

#### A.20.2.1.X3	Inter-frequency SAN Handover from FR1 to FR1 for 1Rx RedCap UE

##### A.20.2.1.X3.1	Test Purpose and Environment

This test is to verify the requirement for Inter-frequency SAN Handover from FR1 to FR1 specified in clause 6.1F.1 for 1Rx RedCap UE.

##### A.20.2.1.X3.2	Test Parameters

Test parameters in clause A.14.2.1.2.2 shall apply except that the supported test configurations are defined in table A.20.2.1.X1.2-1, and NR Cell specific test parameters in Table A.20.2.1.X1.2-2 replace the corresponding parameters in Table A.14.2.1.2.2-3. Other parameters in Table A.14.2.1.2.2-2 and Table A.14.2.1.2.2-3 shall apply to test configuration 1, 2, 3 and 4.

In the test, the target cell is known by the UE and carries only CD-SSB. The antenna configuration for 1Rx RedCap UE is 1x1.

##### A.20.2.1.X3.3	Test Requirements

Test requirements in clause A.14.2.1.2.3 shall apply for 1Rx RedCap UEs.

#### A.20.2.1.X4	Inter-frequency SAN Handover from FR1 to FR1 for 2Rx RedCap UE

##### A.20.2.1.X4.1	Test Purpose and Environment

This test is to verify the requirement for Inter-frequency SAN Handover from FR1 to FR1 specified in clause 6.1F.1 for 2Rx RedCap UE.

##### A.20.2.1.X4.2	Test Parameters

Test parameters in clause A.20.2.1.X3.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.X4.3	Test Requirements

Test requirements in clause A.14.2.1.2.3 shall apply for 2Rx RedCap UEs.

#### A.20.2.1.X5	Intra-frequency SAN RACH-less Handover from FR1 to FR1 for 1Rx RedCap UE

##### A.20.2.1.X5.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN RACH-less Handover from FR1 to FR1 specified in clause 6.1F.1 for 1Rx RedCap UE.

##### A.20.2.1.X5.2	Test Parameters

Test parameters in clause A.14.2.1.8.2 shall apply except that the supported test configurations are defined in table A.20.2.1.X1.2-1, and NR Cell specific test parameters in Table A.20.2.1.X1.2-2 replace the corresponding parameters in Table A.14.2.1.8.2-3. Other parameters in Table A.14.2.1.8.2-2 and Table A.14.2.1.8.2-3 shall apply to test configuration 1, 2, 3 and 4.

In the test, the target cell is known by the UE and carries only CD-SSB. The antenna configuration for 1Rx RedCap UE is 1x1.

##### A.20.2.1.X5.3	Test Requirements

Test requirements in clause A.14.2.1.8.3 shall apply for 1Rx RedCap UE.

#### A.20.2.1.X6	Intra-frequency SAN RACH-less Handover from FR1 to FR1 for 2Rx RedCap UE

##### A.20.2.1.X6.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN RACH-less Handover from FR1 to FR1 specified in clause 6.1F.1 for 2Rx RedCap UE.

##### A.20.2.1.X6.2	Test Parameters

Test parameters in clause A.20.2.1.X5.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.X6.3	Test Requirements

Test requirements in clause A.14.2.1.8.3 shall apply for 2Rx RedCap UE.

--- End of change 1 ---