3GPP TSG-RAN WG4 Meeting #118	R4-260xxxx

# Gothenburg, Sweden, February 9th – 13th, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 Introduction of NR SAN CHO requirements for RedCap in FR1-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The Rel-19 work item NR_NTN_Ph3 specifies in objective 5 the support of (e)RedCap UEs in FR1-NTN. Thus, RRM performance requirements need to be introduced for NR SAN conditional handover in RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of RRM performance requirements for - intra-frequency time based NR SAN conditional handover, - inter-frequency distance-based NR SAN conditional handover, - intra-frequency SAN time-based conditional Handover without L3 measurement criteria, and, - inter-frequency distance-based NR SAN conditional handover without L3 measurement criteria, in RRC_CONNECTED for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing NR SAN conditional handover requirements in RRC_CONNECTED for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A20.2.1.Y1 (new), A20.2.1.Y2 (new), A20.2.Y3 (new), A20.2.Y4 (new), A20.2.1.Y5 (new), A20.2.1.Y6 (new), A20.2.Y7 (new), A20.2.Y8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2601739. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### A.20.2.1.Y1	Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx RedCap UE

##### A.20.2.1.Y1.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN time-based conditional handover from FR1 to FR1 specified in clause 6.1F.2 for 1Rx RedCap UE.

##### A.20.2.1.Y1.2	Test Parameters

Test parameters in clause A.14.2.1.3.2 shall apply except that the supported test configurations are defined in table A.20.2.1.Y1.2-1, and NR Cell specific test parameters in Table A.20.2.1.Y1.2-2 replace the corresponding parameters in Table A.14.2.1.3.2-3. Other parameters in Table A.14.2.1.3.2-2 and Table A.14.2.1.3.2-3 shall apply to test configuration 1, 2, 3 and 4.

In the test, the target cell is known by the UE and carries only CD-SSB.

Table A.20.2.1.Y1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.20.2.1.Y1.2-2: Cell specific test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter | Test configuration | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T1 | T2 |
| Satellite information | Config 1,3 |  | SSC.1 |  |  | NSC.1 |  |
|  | Config 2,4 |  | SSC.2 |  |  | NSC.2 |  |
| Koffset | Config 1,3 | ms | 239 |  |  | 239 |  |
|  | Config 2,4 | ms | 4 |  |  | 4 |  |
| Antenna Configuration | 1Rx |  |  | 1x1 |  |  |  |

##### A.20.2.1.Y1.3	Test Requirements

Test requirements in clause A.14.2.1.3.3 shall apply for 1Rx RedCap UE.

#### A.20.2.1.Y2	Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for 2Rx RedCap UE

##### A.20.2.1.Y2.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN time-based conditional handover from FR1 to FR1 specified in clause 6.1F.2 for 2Rx RedCap UE.

##### A.20.2.1.Y2.2	Test Parameters

Test parameters in clause A.20.2.1.Y1.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.Y2.3	Test Requirements

Test requirements in clause A.14.2.1.3.3 shall apply for 2Rx RedCap UE.

#### A.20.2.1.Y3	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx RedCap UE

##### A.20.2.1.Y3.1	Test Purpose and Environment

This test is to verify the requirement for inter-frequency SAN distance-based conditional handover from FR1 to FR1 specified in clause 6.1F.2 for 1Rx RedCap UE.

##### A.20.2.1.Y3.2	Test Parameters

Test parameters in clause A.14.2.1.6.2 shall apply except that the supported test configurations are defined in table A.20.2.1.Y1.2-1, and NR Cell specific test parameters in Table A.20.2.1.Y1.2-2 replace the corresponding parameters in Table A.14.2.1.6.2-3. Other parameters in Table A.14.2.1.6.2-2 and Table A.14.2.1.6.2-3 shall apply to test configuration 1, 2, 3 and 4.

In the test, the target cell is known by the UE and carries only CD-SSB. The antenna configuration for 1Rx RedCap UE is 1x1.

##### A.20.2.1.Y3.3	Test Requirements

Test requirements in clause A.14.2.1.6.3 shall apply for 1Rx RedCap UE.

#### A.20.2.1.Y4	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 for 2Rx RedCap UE

##### A.20.2.1.Y4.1	Test Purpose and Environment

This test is to verify the requirement for inter-frequency SAN distance-based conditional handover from FR1 to FR1 specified in clause 6.1F.2 for 2Rx RedCap UE.

##### A.20.2.1.Y4.2	Test Parameters

Test parameters in clause A.20.2.1.Y3.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.Y4.3	Test Requirements

Test requirements in clause A.14.2.1.6.3 shall apply for 2Rx RedCap UE.

### A.20.2.1.Y5	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx RedCap UE

#### A.20.2.1.Y5.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN time-based conditional handover without L3 measurement criteria from FR1 to FR1 specified in clause 6.1F.2.3 for 1Rx RedCap UE.

#### A.20.2.1.Y5.2	Test Parameters

Test parameters in clause A.14.2.3.2 shall apply except that the supported test configurations are defined in table A.20.2.1.Y5.2-1. Parameters in Table A.14.2.3.2-2 and Table A.14.2.3.2-3 shall apply to test configuration 1 and 2.

In the test, the target cell is known by the UE and carries only CD-SSB. The antenna configuration for 1Rx RedCap UE is 1x1.

Table A.20.2.1.Y5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

#### A.20.2.1.Y5.3	Test Requirements

Test requirements in clause A.14.2.3.3 shall apply for 1Rx RedCap UE.

### A.20.2.1.Y6	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 2Rx RedCap UE

#### A.20.2.1.Y6.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN time-based conditional handover without L3 measurement criteria from FR1 to FR1 specified in clause 6.1F.2.3 for 2Rx RedCap UE.

#### A.20.2.1.Y6.2	Test Parameters

Test parameters in clause A.20.2.1.Y5.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

#### A.20.2.1.Y6.3	Test Requirements

Test requirements in clause A.14.2.3.3 shall apply for 2Rx RedCap UE.

### A.20.2.1.Y7	Inter-frequency SAN distance-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx RedCap UE

#### A.20.2.1.Y7.1	Test Purpose and Environment

This test is to verify the requirement for inter-frequency SAN distance-based conditional handover without L3 measurement criteria from FR1 to FR1 specified in clause 6.1F.2.3 for 1Rx RedCap UE.

##### A.20.2.1.Y7.2	Test Parameters

Test parameters in clause A.14.2.1.6.2 shall apply except that the supported test configurations are defined in table A.20.2.1.Y5.2-1 and except that general test parameters are defined in A.20.2.1.Y7.2-1; NR Cell specific test parameters in Table A.20.2.1.Y7.2-2 replace the corresponding parameters in Table A.14.2.1.6.2-3. Other parameters in Table A.14.2.1.6.2-3 shall apply to test configuration 1 and 2.

The test scenario comprises of 2 NR FDD carriers and one cell on each carrier. Both handover delay and interruption length are tested. The target cell is known by the UE and carries only CD-SSB.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of Cell 2. During T1, the UE is configured to measure inter-frequency neighbour cell with Gap pattern ID gp0. The RRC message implying distance-based handover to Cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC (10 ms) before the beginning of T2.

Starting T2, Cell 2 becomes detectable and after 11670 ms of T2, location condition event condEventD1-r17 is fulfilled.

Table A.20.2.1.Y7.2-1: General test parameters for Inter-frequency SAN distance-based conditional handover without L3 measurement criteria from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (L,B, H) at T1 start |  |  | [(0, 0, 0)] | Set by any pre-configured means (L,B,H) is Geodetic coordinate, where L is latitude, B is longitude, and H is height. |
| UE moving speed |  | km/h | (108, 0, 0) | Set by any pre-configured means |
| referenceLocation1-r17.condEventD1-r17 |  | m | (-700, 0, 0) | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | (1300, 0, 0) | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | 20 | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | 20 | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  |  | 9 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 12 |  |

Table A.20.2.1.Y7.2-2: Cell specific test parameters for Inter-frequency SAN distance-based conditional handover without L3 measurement criteria from FR1 to FR1

| Parameter | Test configuration | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information | Config 1,2 |  | SSC.2 |  | NSC.2 |  |
| Koffset | Config 1,2 | ms | 4 |  | 4 |  |
| Antenna Configuration | 1Rx |  | 1x1 |  |  |  |

##### A.20.2.1.Y7.3	Test Requirements

Test requirements in clause A.14.2.1.6.3 shall apply for 1Rx RedCap UE.

### A.20.2.1.Y8	Inter-frequency SAN distance-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 2Rx RedCap UE

#### A.20.2.1.Y8.1	Test Purpose and Environment

This test is to verify the requirement for inter-frequency SAN distance-based conditional handover without L3 measurement criteria from FR1 to FR1 specified in clause 6.1F.2.3 for 2Rx RedCap UE.

##### A.20.2.1.Y8.2	Test Parameters

Test parameters in clause A.20.2.1.Y7.2 shall apply. The antenna configuration for 2Rx RedCap UE is 1x2.

##### A.20.2.1.Y8.3	Test Requirements

Test requirements in clause A.14.2.1.6.3 shall apply for 2Rx RedCap UE.

--- End of change 1 ---