3GPP TSG-RAN WG4 Meeting #118	R4-2602853

Göteborg, Sweden, February 9 ‒ February 13, 2026

Title: 	Way Forward for [118][328] Rel-19 Demod_Part3_NTN

Agenda Item:	6.22

Source: 	Ericsson

Document for:	Approval

# Time-varying frequency offset and propagation delay model for NGSO

## Sub-topic 2-1 Support of orbital parameters

Issue 2-1-1: Initial ephemeris information for NGSO satellites in Orbital parameters

Agreement: Set the following values as the initial ephemeris information to support both state vector format and orbital parameter formats.

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| positionX-r17 | -2654549 | semiMajorAxis-r17 | 127437262 |
| positionY-r17 | 4386991 | eccentricity-r17 | 625825 |
| positionZ-r17 | 1594205 | periapsis-r17 | 1514099 |
| velocityVX-r17 | 14581 | longitude-r17 | 88985152 |
| velocityVY-r17 | -34487 | inclination-r17 | 62744502 |
| velocityVZ-r17 | 120182 | meanAnomaly-r17 | 11230448 |

- Add the following note in the specification.

  - Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm.

  - ephemerisInfo in Table G.4.1-2 is derived from Table G.4.1-1 with the assumption x, y, z axis in ECI are aligned with x, y, z axis in ECEF

Issue 2-1-2: Configuration of epochTime

Agreement: When TE updates ephemerisInfo, epochTime-r17 is set such that ephemerisInfo corresponds to the satellite’s position/velocity at the SFN/subframe indicated by epochTime-r17.

Issue 2-1-3: Procedure to support orbital parameters

Agreement: Use the following procedure to set the state vector and orbital parameter formats in ephemerisInfo.

Flow chart for Newton’s method

Flow chart for Runge-Kutta method