3GPP TSG-RAN WG4 Meeting #118	R4-2602849

# Gothenburg, Sweden, 9th -13th, Feb, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0275 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for 38.101-5 for NTN NGSO testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2026-1-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The NGSO time varying Doppler shift and propagation delay channel model was introduced in Rel-19. And the related demodulation requirements for NR NTN and IoT NTN with appling NGSO channel model was specfied. This big CR merges endorsed draft CR to 38.101-5 in RAN4#118 meeting based on endorsed draft big CR R4-2513627 in RAN4#116bis meeting. The reason for change in endorsed draft CR is copied below - R4-2602847: As per the WF R4-2523086, the test applicability rules for NR NTN UE demodulation requirements with NTN dynamic channel model is needed. - R4-2602851: In addition to the contents in the previously endorsed draft CR (R4-2515048), additional descriptions are necessary in Annex G to consider following aspects based on the approved WF in RAN4 #117 (R4-2523086). Extract from WF(R4-2523086) According to TS 38.133 Table A.3.36.3-2, RRM test cases assume ephemeris information is provided in format of orbital parameter in ECI. However, the current draft big CR (R4-2513627) assumes that the Ephemeris information is provided only in format of position and velocity state vector in ECEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - R4-2602847: Addition of applicability rules and removal the square bracket for NR NTN UE demodulation requirements with NTN dynamic channel model based on the endorsed draft big CR R4-2513627. - R4-2602851: Annex G.2 Added a table to show input values for calculation of satellite orbit by Newton-Raphson method (parameters of the orbital elements). Added a table to show output values by Newton-Raphson method. Annex G.2.1.1 Added step 0 to check format of initial ephemeris informationin SIB19. Added a procedure to generate ephemeris information in SIB19 at time t. Annex G.2.2 Added Step 1 to convert initial six Keplerian orbital elements to a state vector. Modified Step 4 to add a procedure to generate ephemeris information in SIB19 at time t. Annex G.4.1 Modified descriptions to show there are two cases to express the initial ephemeris information, state vector format or orbital parameter format. Added a table to describe initial ephemeris information in orbital parameter format. Added a note to clarify that the ephemerisInfo in Table G.4.1-2 is derived from Table G.4.1-1. Added note 2 in Table G.4.1-1 and G.4.1-2 Deleted table for EphemerisInfo field descriptions since it can be referred to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - R4-2602847: The applicability rules for NR NTN UE demodulation requirements with NTN dynamic channel model will be missing. The square bracket will still be kept in the specification. - R4-2602851: Descriptions of the satellite’s orbit calculation method based on the initial ephemeris information with the orbital parameter format will be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) Annex G, 8.2.1.2, 8.2.1.1.3, A.4.1, A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revsion of R4-2601640 |  |  |  |  |  |  |  |  |

==============First change==============

8.2.1.1	Applicability of requirements

<Unchanged part skipped>

8.2.1.1.3	Applicability of different requirements

The applicability rules for different requirements in section 8 are specified in Table 8.2.1.1.3-1.

Table 8.2.1.1.3-1: Applicability of requirements

| If UE has passed |  |  | UE can skip |  |  | Applicabili ty notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FDD | PDSCH | Table 8.2.1.2.2.1.1-4 (Test 2-1, Test 2-2) | FDD | PDSCH | Table 8.2.1.2.2.1.1-3 (Test 1-1, Test 1-2, Test 1-3 and Test 1-4) |  |

==============Next change==============

8.2.1.2	PDSCH demodulation requirements

<Unchanged part skipped>

Table 8.2.1.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Table 8.2.1.2.2.1.1-4: Minimum performance for Rank 1 with time-varying Doppler shift and propagation delay model

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.8 |
| 2-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 8.1 |
| Note1: The time-varying Doppler shift and propagation delay model, as specified in Annex G, is applied. |  |  |  |  |  |  |  |

==============Next change==============

A.4	Testing related to Satellite Access

A.4.1	General

The following test conditions should be maintained for Satellite Access when test equipment emulates the snapshot of the satellite link channel.

-	The same ephemeris info will be maintained during each test.

-	A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 38.508-1 [13].

-	The range of the selected constant delay shift is as follows:

-	For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest value for LEO orbit 1200km).

-	For GSO the range of the one-way delay from UE to satellite is within 119.375ms to 128.79ms.

-	Constant delay value is derived from ephemeris info (SIB19) and UE location associated to zero Doppler or non-zero Doppler value under test.

The following test conditions should be maintained for Satellite Access when test equipment emulates the time varying Doppler shift and propagation delay for NGSO satellite link based on Annex G.

-	The ephemeris info will be updated according to the velocity and position of satellite during each test.

-	The Doppler shift and propagation delay vary due to satellite motion and Earth rotation.

A.4.2	Test condition for transmitter characteristics

All requriements in section 6 for transmitter characteristics, other than frequency error in clause 6.4.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Frequency error requirement in clause 6.4.1 shall be verified for at least two cases: one with zero Doppler condition and the other with a constant Doppler shift where the range of the absolute value of Doppler is greater than zero and up to [0.93] ppm if the IE field ntn-ScenarioSupport-r17 is present and indicated as GSO and up to 24 ppm if the IE field ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present. The delay condition is a constant.

A.4.3	Test condition for receiver characteristics

All requirements in section 7 for receiver characteristics shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

## A.4.4	Test condition for performance requirements

All requirements defined in Table 8.2.1.2.2.1.1-3 of section 8 for performance requirements shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of NGSO satellites.

The one-way delay between UE and satellite for NGSO at an altitude of 600km is 2ms.

The requirements defined in Table 8.2.1.2.2.1.1-4 of section 8 for performance requirements shall be verified when Doppler conditions related to satellite motion for DL in service link are set to time varying and delay conditions are set to time varying for all types of NGSO satellites as defined in Annex G.

==============Next change==============

Annex F (informative): 
Antenna modelling for NTN VSAT

[To be updated]

Annex G (Normative):  
Modelling of time varying Doppler shift and propagation delay for NGSO

G.1	General

This annex specifies the methodologies for time varying Doppler shift and propagation delay modelling for NGSO.

G.2	Satellite position/velocity estimation

As a first step, it is necessary to perform the satellite’s orbital calculations. There are two methodologies applicable to model the time varying Doppler shift and propagation delay by a test equipment, i.e. Newton-Raphson method to solve Keplerian model and fourth-order Runge-Kutta method to solve equation of motion. It is up to the implementation for test equipment on which method to apply to.

Table G.2-1: Input values for calculation of satellite orbit in format of orbital parameters

| Parameters | Unit | Description |
| --- | --- | --- |
| Semi-major axis (a) | m | Half the length of the longest diameter of the elliptical orbit in Earth-centred Inertial frame (ECI). |
| Eccentricity (e) | rad | Shape of the orbit, ranging from 0 to 1. |
| Inclination (i) | rad | Angle between the orbital plane and the reference plane (typically Earth’s equatorial plane) |
| Longitude of ascending node (W) | rad | Angle from a reference direction (i.e. vernal equinox) to the ascending node, where the satellite crosses the equatorial plane from south to north. |
| Argument of periapsis (w) | rad | Angle from the ascending node to the perigee—the point where the satellite is closest to Earth. |
| Mean anomaly (M0) | rad | Satellite’s position along its orbit at time 0, expressed as an angle measured from perigee. |
| t | second | Time to derive the satellite position and velocity.Set to 0 at the initialization. |

Table G.2-2: Input values for calculation of satellite orbit in format of state vectors

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centred earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centred earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity. Set to 0 at the initialization |

Table G.2-3: Output values in format of orbital parameters

| Parameters | Unit | Description |
| --- | --- | --- |
| Semi-major axis (a) | m | Half the length of the longest diameter of the elliptical orbit in Earth-centred Inertial frame (ECI). |
| Eccentricity (e) | rad | Shape of the orbit, ranging from 0 to 1. |
| Inclination (i) | rad | Angle between the orbital plane and the reference plane (typically Earth’s equatorial plane). |
| Longitude of ascending node (W) | rad | Angle from a reference direction (i.e. vernal equinox) to the ascending node, where the satellite crosses the equatorial plane from south to north. |
| Argument of periapsis (w) | rad | Angle from the ascending node to the perigee, the point where the satellite is closest to Earth. |
| Mean anomaly (Mt) | rad | Satellite’s position along its orbit at time t, expressed as an angle measured from perigee. |

Table G.2-4: Output values in format of state vectors

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centred earth-fixed frame (ECEF) |

Table G.2-5: Constant parameters

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

G.2.1	Newton-Raphson method

G.2.1.1 Keplerian model based estimation

The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero. In case e=0, use the method specified in G.2.2.

Step 0	Check format of initial ephemeris information in SIB19

Choose the following step 1 or step 2-1 to proceed depending on the format of provided initial ephemeris information in SIB19. If the provided ephemeris information is described in the format of the state vector, proceed to step 1. Otherwise, if the information is described in the format of the orbital elements, proceed to step 2-1.

Step 1	Derive six orbital elements (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0ECI=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0ECI=v0,xECIv0,yECIv0,zECI=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0ECI=r0,xECI2+r0,yECI2+r0,zECI2

v=v0ECI=v0,xECI2+v0,yECI2+v0,zECI2

vr=v0ECI⋅r0ECIr=r0,xECIv0,xECI+r0,yECIv0,yECI+r0,zECIv0,zECIr

h=hxhyhz=r×v=r0,yECIv0,zECI-r0,zECIv0,yECIr0,zECIv0,xECI-r0,xECIv0,zECIr0,xECIv0,yECI-r0,yECIv0,xECI

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 [8] ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0ECI-r0ECI⋅v0ECIv0ECI=1μv2-μrr0,xECIr0,yECIr0,zECIT-r⋅vrv0,xECIv0,yECIv0,zECIT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0ECIe⋅r=cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr≥02π-cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton-Raphson method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If n≥4, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n :=  n+1.

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

To generate the ephemeris information in SIB19 in orbital parameters, convert the state vector to orbital parameters by procedures from Step 1-0 to Step 2-1 in G.2 at time t.

G.2.2	Fourth-order Runge-Kutta method

G.2.2.1 Equations of motion based estimation

Step 1: Conversion of initial six Keplerian orbital elements to a state vector

When the initial ephemeris information is provided by six Keplerian orbital elements in SIB19, convert the six orbital elements to a state vector by G.2.1.1 step 2-4, where set time t=0.

Step 2: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if updating the satellite position/velocity every one second.

Step 3: Solve the equation of motion with the fourth-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

Step 4: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 2.

To generate the ephemeris information in SIB19 in orbital parameters, convert the state vector to orbital parameters by procedures from Step 1-0 to Step 2-1 in G.2 at time t.

G.3	Varying Doppler shift and propagation delay generation

G.3.1 Determine UE position

This step calculates the UE position according to the earth angular speed. This clause assumes the UE location is given by a format of latitude, longitude, and altitude. This clause specifies how to convert values to ECEF format to align with the satellite position and velocity state vectors.

Table G.3.1-1: Input values for calculation of UE position

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Table G.3.1-2: Output values

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centred earth-fixed frame (ECEF) |

Table G.3.1-3: Constant parameters

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since the UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

G.3.2 Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Table G.3.2-1: Input values for calculation of Doppler shift and propagation delay

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Table G.3.2-2: Output values

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| DelayDL | sec | Downlink propagation delay at time t |
| DelayUL | sec | Uplink propagation delay at time t |

Table G.3.2-3: Constant parameters

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

Distance between satellite and UE is derived as follows:

ρtECEF=ρt,xECEFρt,yECEFρt,zECEF=rt,xECEF-UEt,xECEFrt,yECEF-UEt,yECEFrt,zECEF-UEt,zECEF

ρtECEF=ρt,xECEF2+ρt,yECEF2+ρt,zECEF2

Using the distance above and the following range rate, Doppler shift is also derived as follows:

RangeRate(t)=ρtECEF⋅vtECEFρtECEF=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFρtECEF

Doppler(t)=-RangeRate(t)×fcc

Next, total propagation delay for downlink and uplink case are derived as follows. Note an influence of the receiver movement (i.e. either UE movement due to the Earth’s rotation or Satellite movement) is considered in the following equations.

vtECI=vt,xECIvt,yECIvt,zECI=vtECEF+00ωE×rtECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

DelayDLtR=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

Note tR is the reception time at UE.

DelayULtT=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF

Note tT is the transmission time at UE. It is not mandated that UE follows this formula for UL propagation delay estimation.

Doppler shift and propagation delay for LEO-600, calculated by the Newton-Raphson and fourth-order Runge-Kutta methods, are shown in Figure G.3.2-1 and G.3.2-2. Refer to Annex G.4 for initial conditions.

Figure G.3.2-1: Doppler shift of LEO-600 calculated by Newton-Raphson method and fourth-order Runge-Kutta method

Figure G.3.2-2: Propagation delay of LEO-600 calculated by Newton-Raphson method and fourth-order Runge-Kutta method

G.3.3 Elevation angle and Azimuth angle (for information)

An elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

For example, elevation angle of LEO-600 with maximum positive Doppler scenario is shown in Figure G.3.3-1. Starting the elevation angle from 30 degrees, available connection time between UE and one satellite is approximately 250 seconds.

Figure G.3.3-1: Elevation angle for LEO-600 with maximum positive Doppler scenario

G.4	Initial conditions for numerical solution

G.4.1 Initial ephemeris information for satellites

For LEO-600 scenario, the initial satellite epehemris is specified either by a state vector format in Table G.4.1-1 or by an orbital parameter format in Table G.4.1.-2.

Note ephemerisInfo in Table G.4.1-2 is derived from Table G.4.1-1 with the assumption x, y, z axis in ECI are aligned with x, y, z axis in ECEF.

Table G.4.1-1: Initial ephemeris information for NGSO satellites (LEO-600 with Max Doppler in state vector format)

| Information Element | Value/remark |
| --- | --- |
| SIB19-r17 ::= SEQUENCE { |  |
| ntn-Config-r17 SEQUENCE { |  |
| epochTime-r17 | NOTE2 |
| ephemerisInfo-r17 CHOICE { |  |
| positionVelocity-r17 SEQUENCE { |  |
| positionX-r17 | -2654549 |
| positionY-r17 | 4386991 |
| positionZ-r17 | 1594205 |
| velocityVX-r17 | 14581 |
| velocityVY-r17 | -34487 |
| velocityVZ-r17 | 120182 |
| } |  |
| } |  |
| } |  |
| } |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm NOTE 2: When ephemerisInfo is updated, epochTime-r17 is set such that ephemerisInfo corresponds to the satellite’s position/velocity at the SFN/subframe indicated by epochTime-r17. |  |

Table G.4.1-2: Initial ephemeris information for NGSO satellites (LEO-600 with Max Doppler in orbital parameter format)

| Information Element | Value/remark |
| --- | --- |
| SIB19-r17 ::= SEQUENCE { |  |
| ntn-Config-r17 SEQUENCE { |  |
| epochTime-r17 | NOTE 2 |
| ephemerisInfo-r17 CHOICE { |  |
| orbital-r17 SEQUENCE { |  |
| semiMajorAxis-r17 | 127437262 |
| eccentricity-r17 | 625825 |
| periapsis-r17 | 1514099 |
| longitude-r17 | 88985152 |
| inclination-r17 | 62744502 |
| meanAnomaly-r17 | 11230448 |
| } |  |
| } |  |
| } |  |
| } |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm NOTE 2: When ephemerisInfo is updated, epochTime-r17 is set such that ephemerisInfo corresponds to the satellite’s position/velocity at the SFN/subframe indicated by epochTime-r17. |  |

The position/velocity state vecotors or orbital parameters of the satellite need to be converted based on the definitions in TS 38.331 [8] as EphemerisInfo field descriptions.

The converted satellite coordinate and velocity in ECEF Frame are as follows.

Satellite coordinate: ( -3450913.7 [m], 5703088.3 [m], 2072466.5 [m])

Satellite velocity: (874.86 [m/s], -2069.22 [m/s], 7210.92 [m/s])

The converted orbital elements in ECI frame are as follows.

Semi-major axis (a): 7041480.926238 [m]

Eccentricity (e): 0.00895555575 [rad]

Inclination (i): 1.46884879182 [rad]

Longitude of ascending node (W): 2.08314240832 [rad]

Argument of periapsis (w): 0.03544505759 [rad]

Mean anomaly (M0): 0.26290478768 [rad]

G.4.2 Initial location information for UE

The initial UE position can be referred from TS 38.508-1 [13].

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

Based on the equations described in the annex G.3.1, converted UE location for NGSO satellites in ECEF Frame is as follows.

UE location: (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

G.5	Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory in TE side

The mean of absolute error for Doppler shift should be less than 1 Hz and the mean of absolute error for propagation delay should be less than 0.05µs based on the following error calculation method.

ErrDopplert=11000n=11000Dopplerbaselinet+n1000-Dopplerinterpt+n1000, t=0,1,2,…(s)

ErrDelayt=11000n=11000Delayrbaselinet+n1000-Delayrinterpt+n1000, t=0,1,2,…(s)

where Dopplerbaselinen, Dopplerinterp(n), Delayrbaselinen and Delayinter(n) are calculated once per 1ms, and the average is taken over every 1 second.

Annex H (informative):
Change history

==============End of change==============