3GPP TSG-RAN4 Meeting #118	R4-2602942

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | 1036 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (RInImp9-Rfmulti,TEI17) CR to 37.104: Removal of UTRA TDD from MSR BS specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RInImp9-RFmulti, TEI17 |  |  |  |  |  | Date: |  |  | 2026-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | UTRA TDD has been in the MSR BS specifications since Rel-9. UTRA TDD MSR BS are not anymore produced and there is in general very few, if any, UTRA TDD deployments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR removes co-existence and colocation with UTRA TDD by removing the following: - UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) as “assumed adjacent carrier” for Base Station ACLR in unpaired spectrum with synchronized operation - Transmit intermodulation for colocation with UTRA TDD - Receiver blocking for co-existence with UTRA TDD UTRA TDD (1.28 Mcps) as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge” for Receiver intermodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary requirements for UTRA TDD co-existence would remain in the MSR BS specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 5.3. 6.6.1.4, 6.6.4.1, 6.7.3, 7.4.5, 7.7.1, 7.7.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 37.141, 37.145-1, 37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: References to removed requirements are removed in tables in 4.4 and 5.3. |  |  |  |  |  |  |  |  |

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions. There are additional single-RAT regional requirements that may apply. These are referenced from the present specification, but listed in the specification for the RATs concerned [2][3][4][5][17].

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 4.5 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.2.2 | Base station output power | These requirements apply in Japan for a BS operating in band 34 and Band 41. |
| 6.6.1.1 | Mandatory requirements (spurious emissions) | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [6] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [6] apply. |
| 6.6.1.3 | Additional spurious emissions requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the MSR BS operating band. |
| 6.6.1.3 | Additional spurious emissions requirements | In addition to the requirements in subclauses 6.6.1.1, 6.6.1.2 and 6.6.1.3, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.1.4 | Co-location (spurious emissions) | These requirements may be applied for the protection of other BS receivers when a BS operating in another frequency band is co-located with an MSR BS. |
| 6.6.2.1 | Operating band unwanted emissions | For BS operating in Band 41 in Japan, the operating band unwanted emissions limits shall be applied to the sum of the emission power over all antenna connectors. |
| 6.6.2.4.1 | Additional requirement (Operating band unwanted emissions) | In addition to the requirements in subclauses 6.6.2.1 and 6.6.2.2, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.2.4.2 | Unsynchronized operation for BC3 (Operating band unwanted emissions) | The requirements for unsynchronized TDD co-existence may apply regionally. |
| 6.6.2.4.3 | Protection of DTT (Operating band unwanted emissions) | The requirements for protection of DTT may apply regionally. |
| 6.6.2.4.4 | Co-existence with services in adjacent frequency bands (Operating band unwanted emissions) | This regional requirement may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in clause 4.5, in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.2.4.7 | Additional band 32 unwanted emissions | These requirements may apply in certain regions |
| 6.6.3 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.5.3 | Transmitter spurious emissions | For BS operating in Band 41 in Japan, the spurious emissions limits shall be applied to the sum of the emission power over all antenna connectors |
| 6.7.4 | Additional requirements | These requirements may apply in certain regions. |
| 7.5.2 | Co-location requirement (blocking) | These requirements may be applied for the protection of the BS receiver when a BS operating in another frequency band is co-located with an MSR BS. |

*********************End of change*****************

*********************Next changed section*****************

## 5.3	Band category 3

For all BS operating in bands belonging to Band Category 3, the RF requirements listed in Table 5.3-1 apply for each supported operating band. Requirements apply according to the RAT configuration of the Base Station, as listed in the heading of the table. Some requirements listed in the table may not be mandatory or they may apply only regionally. This is further specified in the clause of each requirement and in Table 4.4-1.

Table 5.3-1: Applicability of requirements for MSR BS operation in Band Category 3

| RF requirement | BS configured for multi-RAT operation | BS configured for single-RAT E-UTRA TDD operation | BS configured for single-RAT UTRA TDD operation | BS configured for single-RAT NR TDD operation | BS configured for single-RAT NB-IoT standalone operation |
| --- | --- | --- | --- | --- | --- |
| Base station output power | 6.2.1 6.2.2 6.2.3 6.2.5 | 6.2.1 6.2.2 6.2.3 | 6.2.1 6.2.5 | 6.2.1 | 6.2.1 6.2.6 |
| Output power dynamics | 6.3.1 6.3.3 6.3.6 | 6.3.1 | 6.3.3 | 6.3.6 | 6.3.5 |
| Transmit ON/OFF power | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |
| Transmitted signal quality |  |  |  |  |  |
| Modulation quality | 6.5.1.1 6.5.1.3 6.5.1.6 | 6.5.1.1 | 6.5.1.3 | 6.5.1.6 | 6.5.1.5 |
| Frequency error | 6.5.2.1 6.5.2.3 6.5.2.6 | 6.5.2.1 | 6.5.2.3 | 6.5.2.6 | 6.5.2.5 |
| Time alignment error | 6.5.3.1 6.5.3.3 6.5.3.5 | 6.5.3.1 | 6.5.3.3 | 6.5.3.5 | 6.5.3.4 |
| Unwanted emissions |  |  |  |  |  |
| Transmitter spurious emissions | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3) |
| Operating band unwanted emissions | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 |
| Occupied bandwidth | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 |
| ACLR | 6.6.4.1 6.6.4.3 6.6.4.6 | 6.6.4.1 | 6.6.4.3 | 6.6.4.6 | 6.6.4.5 |
| Cumulative ACLR | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) |
| Transmitter intermodulation | 6.7.1 | 6.7.1 6.7.4 | 6.7.1 | 6.7.1 | 6.7.1 |
| Reference sensitivity level | 7.2.1 7.2.3 7.2.6 | 7.2.1 | 7.2.3 | 7.2.6 | 7.2.5 |
| Dynamic range | 7.3.1 7.3.3 7.3.6 | 7.3.1 | 7.3.3 | 7.3.6 | 7.3.5 |
| In-band selectivity and blocking |  |  |  |  |  |
| Blocking | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 |
| Narrowband blocking | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 |
| Out-of-band blocking | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 |
| Receiver intermodulation |  |  |  |  |  |
| Intermodulation | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 |
| Narrowband intermodulation | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 |
| In-channel selectivity | 7.8 | 7.8 | - | 7.8.2 | 7.8 |
| Performance requirements | 8.1 8.3 8.5 | 8.1 | 8.3 | 8.5 | 8.6 |
| Note 1: For some requirements in BS configured for multi-RAT operation, there is no general MSR multi-RAT requirement. Instead, the requirement is defined by the respective single-RAT requirement for each RAT supported by the BS as referenced in the table. NOTE 2: The requirement in sub-clause 6.6.4.4 is only applied for BS operating in non-contiguous spectrum. |  |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

#### 6.6.1.4	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, E-UTRA, NB-IoT and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD base stations.

*********************End of change*****************

*********************Next changed section*****************

### 6.6.4	Adjacent Channel Leakage Power Ratio (ACLR)

Adjacent Channel Leakage Power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

#### 6.6.4.1	E-UTRA minimum requirement

For E-UTRA, the minimum requirement for ACLR is specified in Table 6.6.4.1-1 and 6.6.4.1-2 and applies outside the Base Station RF Bandwidth or Radio Bandwidth.

For a BS operating in non-contiguous spectrum, the ACLR also applies for the first adjacent channel inside any sub-block gap with a gap size Wgap ≥ 15MHz. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in subclause 6.6.4.4 applies in sub block gaps for the frequency ranges defined in Table 6.6.4.4-1.

For a BS operating in multiple bands, where multiple bands are mapped onto the same antenna connector, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz.

The CACLR requirement in subclause 6.6.4.4 applies in Inter-RF Bandwidth gaps for the frequency ranges defined in Table 6.6.4.4-1.

The requirements apply during the transmitter ON period.

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For Category A Wide Area BS, either the ACLR limits in the tables below or the absolute limit of -13dBm/MHz shall apply, whichever is less stringent.

For Category B Wide Area BS, either the ACLR limits in the tables below or the absolute limit of -15dBm/MHz shall apply, whichever is less stringent. For Medium Range BS, either the ACLR limits in the tables below or the absolute limit of -25 dBm/MHz shall apply, whichever is less stringent.

For Local Area BS, either the ACLR limits in the tables below or the absolute limit of -32dBm/MHz shall apply, whichever is less stringent.

For operation in paired spectrum, the ACLR shall be higher than the value specified in Table 6.6.4.1-1.

Table 6.6.4.1-1: Base Station ACLR in paired spectrum

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 1.4, 3.0, 5, 10, 15, 20 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 45 dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 45 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

For operation in unpaired spectrum, the ACLR shall be higher than the value specified in Table 6.6.4.1-2.

Table 6.6.4.1-2: Base Station ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 1.4, 3 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
| 5, 10, 15, 20 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 45 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

### 6.7.3	Void

Table 6.7.3-1: Void

*********************End of change*****************

*********************Next changed section*****************

### 7.4.5	Void

-

Table 7.4.5-1: Void

*********************End of change*****************

*********************Next changed section*****************

## 7.7	Receiver intermodulation

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

### 7.7.1	General intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the inter Base Station RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the base station antenna input, using the parameters in Table 7.7.1-1 and 7.7.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.

-	For any UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [3], subclause 7.2.

-	For any GSM/EDGE carrier, the conditions are specified in TS 45.005 [5], Annex P.2.2.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [17], subclause 7.2.

Table 7.7.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48+y (Note 6) | PREFSENS +x dB (Note 2) | See Table 7.7.1-2 |
| Medium Range BS | -44+y (Note 6) | PREFSENS +x dB (Note 3, 5) |  |
| Local Area BS | -38+y (Note 6) | PREFSENS +x dB (Note 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2. NOTE 2: For WA BS supporting GSM and/or UTRA, "x" is equal to 6 in case of NR or E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS supporting GSM and/or UTRA, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of NR or E-UTRA or NB-IoT wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS supporting GSM and/or UTRA, "x" is equal to 12 in case of NR or E-UTRA or NB-IoT wanted signals, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS neither supporting GSM nor UTRA, x is equal to 6 for all BS classes if NR is supported, otherwise x is equal to 6 for WA BS or, 9 for MR BS and 12 for LA BS if NR is not supported. NOTE 6: For a BS supporting NR but neither UTRA nor GSM; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. For all other cases, "y" is equal to zero for all BS classes |  |  |  |

Table 7.7.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2.0 (BC1 and BC3) / ±2.1 (BC2) | CW |
|  | ±4.9 | 1.4MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band 3 MHz | ±4.4 (BC1 and BC3) / ±4.5 (BC2) | CW |
|  | ±10.5 | 3MHz E-UTRA signal |
| UTRA FDD and E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz | ±7.375 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 15 MHz | ±7.25 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz | ±7.125 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| GSM/EDGE/NB-IoT standalone | ±7.575 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 15 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 20 MHz or NR with NB-IoT operation in NR in-band | ±7.395 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 25 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 30 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 35 MHz or NR with NB-IoT operation in NR in-band | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 45 MHz or NR with NB-IoT operation in NR in-band | ±7.37 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz or NR with NB-IoT operation in NR in-band | ±7.35 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20MHz E-UTRA signal |

### 7.7.2	General narrowband intermodulation minimum requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS operating in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in Table 7.7.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in Table 7.7.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the base station antenna input, using the parameters in Table 7.7.2-1 and 7.7.2-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.

-	For any UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [3], subclause 7.2.

-	For any GSM/EDGE carrier, the conditions are specified in TS 45.005 [5], Annex P.2.2.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [17], subclause 7.2.

Table 7.7.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 | PREFSENS +x dB (NOTE 1) | See Table 7.7.2-2 |
| Medium Range BS | -47 |  |  |
| Local Area BS | -44 |  |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2. "x" is equal to 6 in case of NR, NB-IoT, E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. |  |  |  |

Table 7.7.2-1a: Void

Table 7.7.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3.0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±360 (NOTE 3) | CW |
|  | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz (NOTE 2) | ±325 (NOTE 3) | CW |
|  | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 15 MHz (NOTE 2) | ±380 (NOTE 3) | CW |
|  | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz (NOTE 2) | ±345 (NOTE 3) | CW |
|  | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| NB-IoT standalone | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±360 | CW |
|  | ±1420 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±400 | CW |
|  | ±1240 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±370 | CW |
|  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±380 | CW |
|  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±390 | CW |
|  | ±2320 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±325 | CW |
|  | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±335 | CW |
|  | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±345 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±355 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±365 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±375 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz (Note 2) | ±395 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz (Note 2) | ±415 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz (Note 2) | ±435 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz (Note 2) | ±365 | CW |
|  | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz (Note 2) | ±385 | CW |
|  | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 or NR G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band/guard band operation. NOTE 4: If a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the E-UTRA interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |

*********************End of change*****************

*********************Next changed section*****************

# A.2	Void

Table A.2-1: Void