3GPP TSG-RAN4 Meeting #118	R4-2602944

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 1106 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (RInImp9-Rfmulti,TEI17) CR to 37.141: Removal of UTRA TDD from MSR BS specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RInImp9-RFmulti, TEI17 |  |  |  |  |  | Date: |  |  | 2026-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | UTRA TDD has been in the MSR BS specifications since Rel-9. UTRA TDD MSR BS are not anymore produced and there is in general very few, if any, UTRA TDD deployments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR removes co-existence and co-location with UTRA TDD by removing the following: - Co-existence and co-location with UTRA TDD operating bands named a) to f) in BS Spurious emissions - UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) as “assumed adjacent carrier” for Base Station ACLR in unpaired spectrum with synchronized operation - Transmit intermodulation for colocation with UTRA TDD - Receiver blocking for co-existence with UTRA TDD - UTRA TDD (1.28 Mcps) as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge” for Receiver intermodulation - Specific UTRA TDD test signals. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary requirements for UTRA TDD co-existence would remain in the MSR BS specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 4.6.4, 4.6.5, 5.1, 5.2, 5.3, 6.6.1.5.6, 6.6.4.5, 6.6.4.5.1, 6.7.4.2.3, 6.7.5.3, 7.4.4.6, 7.4.5.5, 7.7.5, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.104, 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 37.145-1, 37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: References to removed requirements are removed in 4.6.4, 4.6.5, and in tables in 4.3 and 5.1-5.3. |  |  |  |  |  |  |  |  |

## 4.3	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.3-1lists all requirements in the present specification that may be applied differently in different regions. There are additional single-RAT regional requirements that may apply. These are referenced from the present specification, but listed in the specification for the RATs concerned [3][4][5][6][27].

Table 4.3-1: List of regional requirements

| Clause number | Requirement | Comments |  |
| --- | --- | --- | --- |
| 4.5 | Operating bands and Band Categories | Some bands may be applied regionally. |  |
| 6.2.1.2A | Base station output power | Additional requirements may apply as defined in TS 37.104 [2] clause 6.2.2. |  |
| 6.6.1.5.1 | Spurious emissions (Category A) | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [13] apply. |  |
| 6.6.1.5.2 | Spurious emissions (Category B) | Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [13] apply. |  |
| 6.6.1.5.3 | Additional requirement for BC2 (Category B) | Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [13] apply. |  |
| 6.6.1.5.5 | Additional spurious emissions requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the MSR BS operating band. In addition to the requirements in clauses 6.6.1.5.1, 6.6.1.5.2, 6.6.1.5.3, 6.6.1.5.4 and 6.6.1.5.5, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |  |
| 6.6.1.5.6 | Co-location (spurious emissions) | These requirements may be applied for the protection of other BS receivers when a BS operating in another frequency band is co-located with any BS. |  |
| 6.6.2.5.4.1 | Additional requirements (Operating band unwanted emissions) | In addition to the requirements in clauses 6.6.2.5.1 and 6.6.2.5.2, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |  |
| 6.6.2.5.4.2 | Unsynchronized operation for BC3 (Operating band unwanted emissions) | The requirements for unsynchronized TDD co-existence may apply regionally. |  |
| 6.6.2.5.4.3 | Protection of DTT (Operating band unwanted emissions) | The requirements for protection of DTT may apply regionally. |  |
| 6.6.2.5.4.4 | Co-existence with services in adjacent frequency bands (Operating band unwanted emissions) | This regional requirement may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in clause 4.5, in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |  |
| 6.6.2.5.4.6 | Additional band 32 unwanted emissions | These requirements may apply in certain regions |
| 6.6.3.5 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |  |
| 6.6.4.5.6 | Adjacent Channel Leakage Power Ratio (ACLR) | For Band 41 operation in Japan, absolute ACLR limits shall be applied to the sum of the absolute ACLR power over all antenna connectors. |  |
| 6.7.2A | Additional requirements for Band 41 | These requirements may apply in certain regions for Band 41 |  |
| 6.7.5.4 | Additional test requirements | These requirements may apply in certain regions |  |
| 7.5.2.5 | Co-location requirement (blocking) | These requirements may be applied for the protection of the BS receiver when a BS operating in another frequency band is co-located with any BS. |  |

*********************End of change*****************

*********************Next changed section*****************

### 4.6.4	Co-existence with other systems

The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, E-UTRA, NR and/or PHS operating in another band are deployed. If this is the case, conformance with the applicable test requirement for spurious emissions specified in clause 6.6.1.5.5 shall be demonstrated.

### 4.6.5	Co-location with other Base Stations

The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, E-UTRA and/or NR operating in another band. If this is the case,

-	Conformance with the applicable test requirement for spurious emissions specified in clause 6.6.1.5.6 shall be demonstrated.

-	Conformance with the applicable test requirement for receiver blocking specified in clause 7.5.5.2 shall be demonstrated.

*********************End of change*****************

*********************Next changed section*****************

## 5.1	Multi-RAT capable Base Stations

Table 5.1-1: Test configurations for capability sets (CS 3-7) for Multi-RAT capable BS

| Capability Set | UTRA + E-UTRA NB-IoT in-band***, NB-IoT guard band**** (CS 3) |  |  | GSM+ UTRA (CS 4) | GSM + E-UTRA, NB-IoT in-band***, NB-IoT guard band**** (CS 5) | GSM + UTRA + E-UTRA (CS 6) | GSM+UTRA/ E-UTRA, UTRA+ E-UTRA, NB-IoT in-band***, NB-IoT guard band**** (CS7) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC2 | BC2 | BC2 |
| 6.2 Base Station output power | - | - | - | - | - | - | - |
| Base Station maximum output power | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: TC4a C/NC: TC4a, NTC4a | C: TC4b CNC: TC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: TC4c C/NC: TC4c, NTC4c | C: TC4a*, TC4b, TC3a* CNC: TC4a*, TC4b, TC3a* C/NC: TC4a*, NTC4a*, TC4b, NTC4b, TC3a*, NTC3* NI: TC15,TC16* NG: TC18,TC19* |
| Additional regional requirement (only for band 34) | N/A | N/A | Compliance stated by manufacturer declaration | N/A | N/A | N/A | N/A |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | N/A | N/A | (TS 36.141) | N/A | (TS 36.141) |
| UTRA FDD primary CPICH power | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA FDD secondary CPICH power | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA TDD primary CCPCH power | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| 6.3 Output power dynamics | - | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC4b |
| NB-IoT | (TS 36.141) | (TS 36.141) | N/A | N/A | (TS 36.141) | N/A | (TS 36.141) |
| 6.4 Transmit ON/OFF power | - | - | - | - | - | - | - |
| Transmitter OFF power | N/A | N/A | C: TC3b | N/A | N/A | N/A | N/A |
| Transmitter transient period | N/A | N/A | C: TC3b | N/A | N/A | N/A | N/A |
| 6.5 Transmitted signal quality | - | - | - | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - | - | - | - |
| E-UTRA | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 NI/NG : (Note2) | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 NI/NG : (Note2) | C: TC3b NI/NG : (Note2) | N/A | C: TC4b CNC: TC4b C/NC: TC4b, NTC4b NI/NG: (Note2) | C: TC4c CNC: TC4c C/NC: TC4c, NTC4c | C: TC4b CNC: TC4b C/NC: TC4b, NTC4b NI/NG: (Note2) |
| UTRA FDD | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 | C: TC3a CNC: TC3a C/NC: TC3a, NTC3 | N/A | C: TC4a CNC: TC4a C/NC: TC4a, NTC4a | N/A | C: TC4c CNC: TC4c C/NC: TC4c, NTC4c | C: TC4a* CNC: TC4a* C/NC: TC4a*, NTC4a* |
| UTRA TDD | N/A | N/A | C: TC3b | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | C: TC4a CNC: TC4a C/NC: TC4a, NTC4a | C: TC4b CNC: TC4b C/NC: TC4b, NTC4b | C: TC4c CNC: TC4c C/NC: TC4c, NTC4c | C: TC4b CNC: TC4b C/NC: TC4b, NTC4b |
| NB-IoT | N/A : (Note2) | N/A : (Note2) | N/A: (Note 2) | N/A | N/A: (Note2) | N/A | N/A: (Note2) |
| 6.5.2 Frequency error | - | - | - | - | - | - | - |
| E-UTRA | Same TC as used in 6.5.1 NI/NG: (Note2) | Same TC as used in 6.5.1 NI/NG: (Note2) | Same TC as used in 6.5.1 NI/NG: (Note2) | N/A | Same TC as used in 6.5.1 NI/NG: (Note2) | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 NI/NG: (Note2) |
| UTRA FDD | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | N/A | Same TC as used in 6.5.1 | N/A | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| UTRA TDD | N/A | N/A | Same TC as used in 6.5.1 | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| NB-IoT | N/A: (Note2) | N/A: (Note2) | N/A: (Note2) | N/A | N/A: (Note2) | N/A | N/A: (Note2) |
| 6.5.3 Time alignment error | - | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) NI/NG: (Note2) | (TS 36.141) NI/NG: (Note2) | (TS 36.141) NI/NG: (Note2) | N/A | (TS 36.141) NI/NG: (Note2) | (TS 36.141) | (TS 36.141) NI/NG: (Note2) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| NB-IoT | N/A: (Note2) | N/A: (Note2) | N/A: (Note2) | N/A | N/A: (Note2) | N/A | N/A: (Note2) |
| 6.6 Unwanted emissions | - | - | - | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - | - | - | - |
| (Category A) | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a,TC3a, NTC3)*, TC4b,NTC4b NI: TC15,(TC16)* NG: TC18, (TC19)* |
| (Category B) | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a, TC3a, NTC3)*, TC4b,NTC4b NI: TC15,(TC16)* NG: TC18, (TC19)* |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: TC4a*, TC4b CNC: NTC4a*, NTC4b C/NC: (TC4a, NTC4a)*, TC4b,NTC4b NI: TC15 NG: TC18 |
| Protection of the BS receiver of own or different BS | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a, TC3a, NTC3)*, TC4b, NTC4b NI: TC15,(TC16)* NG: TC18,(TC19)* |
| Additional spurious emissions requirements | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a, TC3a, NTC3)*, TC4b, NTC4b NI: TC15,(TC16)* NG: TC18,(TC19)* |
| Co-location with other Base Stations | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a, TC3a, NTC3)*, TC4b, NTC4b NI: TC15,(TC16)* NG: TC18,(TC19)* |
| 6.6.2 Operating band unwanted emissions | - | - | - | - | - | - | - |
| General requirement for Band Categories 1 and 3 | (TS 25.141) (TS 36.141) C: TC3a CNC: TC3a, NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | N/A | (TS 25.142) (TS 36.141) C: TC3b NI: TC16 NG: TC19 | N/A | N/A | N/A | N/A |
| General requirement for Band Category 2 | N/A | (TS 25.141) (TS 36.141) C: TC3a CNC: TC3a, NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | N/A | (TS 25.141) C: TC4a, TC4d (note1) CNC:TC4a, TC4d(note1), NTC4a C/NC: TC4a, TC4d(note1), NTC4a | (TS 36.141) C: TC4b, TC4e (note1) CNC:TC4b, TC4e(note1), NTC4b C/NC: TC4b, TC4e(note1), NTC4b NI: TC15 NG: TC18 | (TS 25.141) (TS 36.141) C: TC4c, TC4e (note1) CNC: TC4c, TC4e (note1), NTC4c C/NC: TC4c, TC4e (note1), NTC4c | (TS 36.141) (TS 25.141)* C: TC4b, TC3a* CNC:TC4b, NTC4b, TC3a*, NTC3* C/NC: TC4b, NTC4b, TC3a*, NTC3* NI: TC15 NG: TC18 |
| GSM/EDGE single-RAT requirement | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | N/A |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - | - | - | - |
| Minimum requirement | (TS 25.141) (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 25.142) (TS 36.141) | (TS 25.141) | (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 25.141)* (TS 36.141) |
| 6.6.4 Adjacent Channel Leakage power Ratio (ACLR) | - | - | - | - | - | - | - |
| E- UTRA | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 | N/A | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | N/A | (TS 25.141)* |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| NB-IoT | NI: TC16 NG: TC19 | NI: TC16 NG: TC19 | NI: TC16 NG: TC19 | N/A | NI: TC15 NG: TC18 | N/A | NI: TC15,(TC16)* NG: TC18,(TC19)* |
| Cumulative ACLR | CNC: NTC3 C/NC: NTC3 | CNC: NTC3 C/NC: NTC3 |  | CNC: NTC1a C/NC: NTC1a | CNC: NTC2 C/NC: NTC2 | CNC: NTC3 C/NC: NTC3 | CNC: NTC3*, NTC2** C/NC: NTC3*, NTC2** |
| 6.7 Transmitter intermodulation | - | - | - | - | - | - | - |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | CNC: NTC3 C/NC: NTC3 | Same TC as used in 6.6 | N/A | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 Reference sensitivity level | - | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC5b |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | N/A | (TS 36.141) |
| 7.3 Dynamic range | - | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | (TS 25.141) | N/A | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC5b |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | N/A | (TS 36.141) |
| 7.4 In- band selectivity and blocking | - | - | - |  |  | - | - |
| General blocking requirement | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| General narrowband blocking requirement | C: TC3a, TC6b CNC: NTC3, TC6b C/NC: TC3a, NTC3, TC6b NI: TC16 NG: TC19 | C: TC3a, TC6b CNC: NTC3, TC6b C/NC: TC3a, NTC3, TC6b NI: TC16 NG: TC19 | C: TC3b, TC6b NI: TC16 NG: TC19 | C: TC5a, TC6a CNC: NTC5a, TC6a C/NC: TC5a. NTC5a, TC6a | C:TC5b, TC6b CNC: NTC5b, TC6b C/NC: TC5b, NTC5b, TC6b NI: TC15 NG: TC18 | C: TC5b, TC6a CNC: NTC5c, TC6a C/NC: TC5b, NTC5c, TC6a | C: TC5b, TC6a* CNC: NTC5b, TC6a* C/NC: TC5b. NTC5b, TC6a* NI: TC15 NG: TC18 |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC5b |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC5b |
| 7.5 Out-of-band blocking | - | - | - | - | - | - | - |
| General requirement | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c, C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| Co-location requirement | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| 7.6 Receiver spurious emissions | - | - | - | - | - | - | - |
| General requirement | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: (TC4a, TC3a)*, TC4b CNC: (NTC4a, NTC3)*, NTC4b C/NC: (TC4a, NTC4a, TC3a, NTC3)* , TC4b, NTC4b NI: TC15,(TC16)* NG: TC18,(TC19)* |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | C: TC4a CNC: NTC4a C/NC: TC4a, NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b, NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c, NTC4c | C: TC4a*, TC4b CNC: NTC4a*, NTC4b C/NC: (TC4a, NTC4a)*, TC4b, NTC4b NI: TC15 NG: TC18 |
| 7.7 Receiver intermodulation | - | - | - | - | - | - | - |
| General intermodulation requirement | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c C/NC: TC5b, NTC5c | C: TC5b CNC NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| General narrowband intermodulation requirement | C: TC3a, TC6b CNC: NTC3, TC6b C/NC: TC3a, NTC3, TC6b NI: TC16 NG: TC19 | C: TC3a TC6b CNC: NTC3, TC6b C/NC: TC3a, NTC3; TC6b NI: TC16 NG: TC19 | C: TC3b, TC6b NI: TC16 NG: TC19 | C: TC5a, TC6a CNC: NTC5a, TC6a C/NC: TC5a NTC5a, TC6a | C: TC5b, TC6b CNC: NTC5b, TC6b C/NC: TC5b, NTC5b, TC6b NI: TC15 NG: TC18 | C: TC5b, TC6a CNC: NTC5c, TC6a C/NC: TC5b, NTC5c, TC6a | C: TC5b, TC6a* CNC: NTC5b, TC6a* C/NC: TC5b NTC5b, TC6a* NI: TC15,(TC16)* NG: TC18,(TC19)* |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) | (TS 51.021) | TC5b |
| 7.8 In-channel selectivity | - | - | - | - | - | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) | N/A | (TS 36.141) |
| NOTE 1: The TC shall be used for performing tests when the declared Base Station RF Bandwidth for GSM single-RAT operation is not equal to the declared Base Station RF Bandwidth for multi-RAT operations and the frequency range supported by the BS is a subset of the operating band, or when the maximum Base Station RF Bandwidth covers the entire operating band. NOTE 2: There is no specific test with NB-IoT for those requirements, tests could be performed using E-UTRA signal only, without NB-IoT. NOTE *: For Band 3, the test configuration is only applicable if UTRA is declared to be supported in Band 3. For other BC2 bands, the test configurations are always applicable. NOTE **: For Band 3 only, the test configuration is only applicable if UTRA is not declared to be supported in Band 3. NOTE ***: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE****:The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. |  |  |  |  |  |  |  |

Table 5.1-1a: Test configurations for capability sets (CS9-13) for Multi-RAT capable BS

| Capability Set | GSM+NB-IoT standalone (CS 9) | UTRA + NB-IoT standalone (CS 10) |  | E-UTRA + NB-IoT standalone (CS 11) |  |  | GSM+UTRA+NB-IoT standalone (CS 12) | GSM+ E-UTRA+NB-IoT standalone (CS 13) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC2 | BC1 | BC2 | BC1 | BC2 | BC3 | BC1 | BC2 |
| 6.2 Base Station output power | - | - | - | - | - | - | - | - |
| Base Station maximum output power | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| E-UTRA for DL RS power | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA FDD secondary CPICH power | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA TDD primary CCPCH power | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 6.3 Output power dynamics | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| UTRA FDD | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| GSM/EDGE | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.4 Transmit ON/OFF power | - | - | - | - | - | - | - | - |
| Transmitter OFF power | N/A | N/A | N/A | N/A | N/A | TC11 | N/A | N/A |
| Transmitter transient period | N/A | N/A | N/A | N/A | N/A | TC11 | N/A | N/A |
| 6.5 Transmitted signal quality | - | - | - | - | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | TC11 | TC11 | TC11 | N/A | TC13 |
| UTRA FDD | N/A | TC10 | TC10 | N/A | N/A | N/A | TC12 | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| GSM/EDGE | TC9 | N/A | N/A | N/A | N/A | N/A | TC12 | TC13 |
| NB-IoT | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| 6.5.2 Frequency error | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | N/A | Same TC as used in 6.5.1 |
| UTRA FDD | N/A | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | N/A | N/A | N/A | Same TC as used in 6.5.1 | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| GSM/EDGE | Same TC as used in 6.5.1 | N/A | N/A | N/A | N/A | N/A | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| NB-IoT | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| 6.5.3 Time alignment error | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| UTRA FDD | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.6 Unwanted emissions | - | - | - | - | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - | - | - | - | - |
| (Category A) | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| (Category B) | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional requirement for BC2 (Category B) | TC9 | TC10 | TC10 | TC11 | TC11 | N/A | TC12 | TC13 |
| Protection of the BS receiver of own or different BS | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional spurious emissions requirements | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Co-location with other Base Stations | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| 6.6.2 Operating band unwanted emissions | - | - | - | - | - | - | - | - |
| General requirement for Band Categories 1 and 3 | N/A | (TS 36.141) (TS 25.141) TC10 | N/A | (TS 36.141) TC11 | N/A | (TS 36.141) TC11 | N/A | N/A |
| General requirement for Band Category 2 | (TS 36.141) TC9 | N/A | (TS 36.141) (TS 25.141) TC10 | N/A | (TS 36.141) TC11 | N/A | (TS 36.141) (TS 25.141) TC12 | (TS 36.141) TC13 |
| GSM/EDGE single-RAT requirement | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - | - | - | - | - |
| Minimum requirement | (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 36.141) |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) | - | - | - | - | - | - | - | - |
| E- UTRA | N/A | N/A | N/A | TC11 | TC11 | TC11 | N/A | TC13 |
| UTRA FDD | N/A | TC10 | TC10 | N/A | N/A | N/A | TC12 | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Cumulative ACLR | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 6.7 Transmitter intermodulation |  | - |  | - |  |  |  |  |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | N/A | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 Reference sensitivity level | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| UTRA FDD | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| GSM/EDGE | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.3 Dynamic range | - | - | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| UTRA FDD | N/A | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | (TS 25.141) | N/A |
| UTRA TDD | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| GSM/EDGE | (TS 51.021) | (TS 51.021) | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.4 In- band selectivity and blocking | - | - | - | - | - | - | - | - |
| General blocking requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| General narrowband blocking requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional narrowband blocking requirement for GSM/EDGE | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| GSM/EDGE requirements for AM suppression | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| 7.5 Out-of-band blocking | - | - | - | - | - | - | - | - |
| General requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Co-location requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| 7.6 Receiver spurious emissions | - | - | - | - | - | - | - | - |
| General requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional requirement for BC2 (Category B) | TC9 | N/A | TC10 | N/A | TC11 | TC11 | TC12 | TC13 |
| 7.7 Receiver intermodulation | - | - | - | - | - | - | - | - |
| General intermodulation requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| General narrowband intermodulation requirement | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Additional narrowband intermodulation requirement for GSM/EDGE | (TS 51.021) | N/A | N/A | N/A | N/A | N/A | (TS 51.021) | (TS 51.021) |
| 7.8 In-channel selectivity | - | - | - | - | - | - | - | - |
| E-UTRA requirement | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) | N/A | (TS 36.141) |
| NB-IoT | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 5.1-1b: Test configurations for capability sets (CS14-15) for Multi-RAT capable BS

| Capability Set | UTRA + E-UTRA + NB-IoT standalone (CS 14) |  |  | GSM + UTRA + E-UTRA + NB-IoT standalone (CS 15) |  |
| --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 |  |
| 6.2 Base Station output power | - | - | - | - |
| Base Station maximum output power | TC14 | TC14 | TC14 | TC13, TC12* |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A | N/A |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA FDD secondary CPICH power | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD primary CCPCH power | N/A | N/A | N/A | N/A |
| 6.3 Output power dynamics | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | TC13 |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.4 Transmit ON/OFF power | - | - | - | - |
| Transmitter OFF power | N/A | N/A | TC14 | N/A |
| Transmitter transient period | N/A | N/A | TC14 | N/A |
| 6.5 Transmitted signal quality | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - |
| E-UTRA | TC14 | TC14 | TC14 | TC13 |
| UTRA FDD | TC14 | TC14 | TC14 | TC12* |
| UTRA TDD | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | TC13 |
| NB-IoT | TC14 | TC14 | TC14 | TC13 |
| 6.5.2 Frequency error | - | - | - | - |
| E-UTRA | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| UTRA FDD | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| UTRA TDD | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | Same TC as used in 6.5.1 |
| NB-IoT | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| 6.5.3 Time alignment error | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | N/A | N/A |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.6 Unwanted emissions | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - |
| (Category A) | TC14 | TC14 | TC14 | TC13, (TC12, TC13)* |
| (Category B) | TC14 | TC14 | TC14 | TC13, (TC12, TC13)* |
| Additional requirement for BC2 (Category B) | N/A | TC14 | N/A | TC13, (TC12, TC13)* |
| Protection of the BS receiver of own or different BS | TC14 | TC14 | TC14 | TC13, (TC12, TC13)* |
| Additional spurious emissions requirements | TC14 | TC14 | TC14 | TC13, (TC12, TC13)* |
| Co-location with other Base Stations | TC14 | TC14 | TC14 | TC13, (TC12, TC13)* |
| 6.6.2 Operating band unwanted emissions | - | - | - | - |
| General requirement for Band Categories 1 and 3 | (TS 36.141) (TS 25.141) TC14 | N/A | (TS 36.141) (TS 25.141) TC14 | N/A |
| General requirement for Band Category 2 | N/A | (TS 36.141) (TS 25.141) TC14 | N/A | (TS 36.141) (TS 25.141)* TC14, TC12* |
| GSM/EDGE single-RAT requirement | N/A | N/A | N/A | N/A |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - |
| Minimum requirement | (TS 25.141) (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 25.141) (TS 36.141) | (TS 25.141)* (TS 36.141) |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) | - | - | - | - |
| E- UTRA | TC14 | TC14 | TC14 | TC13 |
| UTRA FDD | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | N/A | N/A |
| NB-IoT | TC14 | TC14 | TC14 | TC13 |
| Cumulative ACLR | N/A | N/A | N/A | N/A |
| 6.7 Transmitter intermodulation | - | - | - | - |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | Same TC as used in 6.6 | Same TC as used in 6.6 | N/A | Same TC as used in 6.6 |
| 7.2 Reference sensitivity level | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | TC13 |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.3 Dynamic range | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | (TS 25.141) | (TS 25.141)* |
| UTRA TDD | N/A | N/A | N/A |  |
| GSM/EDGE | N/A | N/A | N/A | TC13 |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.4 In- band selectivity and blocking | - | - | - | - |
| General blocking requirement | TC14 | TC14 | TC14 | TC13 |
| General narrowband blocking requirement | TC14 | TC14 | TC14 | TC13 |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A | TC13 |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A | TC13 |
| 7.5 Out-of-band blocking | - | - | - | - |
| General requirement | TC14 | TC14 | TC14 | TC13 |
| Co-location requirement | TC14 | TC14 | TC14 | TC13 |
| 7.6 Receiver spurious emissions | - | - | - | - |
| General requirement | TC14 | TC14 | TC14 | TC13 |
| Additional requirement for BC2 (Category B) | TC14 | TC14 | TC14 | TC13 |
| 7.7 Receiver intermodulation | - | - | - | - |
| General intermodulation requirement | TC14 | TC14 | TC14 | TC13 |
| General narrowband intermodulation requirement | TC14 | TC14 | TC14 | TC13 |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A | TC13 |
| 7.8 In-channel selectivity | - | - | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | N/A | N/A | N/A | N/A |

Table 5.1-1c: Test configurations for capability sets (CS16-19) for Multi-RAT capable BS

| Capability Set | NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 16) |  | NR + NB-IoT standalone + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 17) |  | GSM + NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 18) | UTRA + NR + E-UTRA NB-IoT in-band (Note 1) NB-IoT guard band (Note 2) (CS 19) |
| --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 and BC2 | BC3 | BC1 and BC2 | BC3 | BC2 | BC1 and BC2 |
| 6.2 Base Station output power | - | - | - | - |  |  |
| Base Station maximum output power | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG:TC22 | C: TC22 NI: TC22 NG:TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| UTRA FDD secondary CPICH power | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| 6.3 Output power dynamics | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC4b | N/A |
| 6.4 Transmit ON/OFF power | - | - | - | - | - | - |
| Transmitter OFF power | N/A | C: TC21 CNC: NTC21 | N/A | C: TC22 | N/A | N/A |
| Transmitter transient period | N/A | C: TC21 CNC: NTC21 | N/A | C: TC22 | N/A | N/A |
| 6.5 Transmitted signal quality | - | - | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - | - | - |
| E-UTRA | C: TC21 NI, NG: (Note 4) CNC: TC21 NCNI, NCNG: (Note 4) C/NC: NTC21, TC21 C/NCNI, C/NCNG: (Note 4) | C: TC21 NI, NG: (Note 4) CNC: TC21 NCNI, NCNG: (Note 4) C/NC: NTC21, TC21 C/NCNI, C/NCNG: (Note 4) | C: TC22 NI, NG: (Note 4) | C: TC22 NI, NG: (Note 4) | C: TC21a NI, NG: (Note 4) CNC: TC21a NCNI, NCNG: (Note 4) C/NC: NTC21a, TC21a C/NCNI, C/NCNG: (Note 4) | C: TC21b NI, NG: (Note 4) CNC: TC21b NCNI, NCNG: (Note 4) C/NC: NTC21b, TC21b C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Standalone: C: TC22 NI, NG: (Note 4) | Standalone C: TC22 NI, NG: (Note 4) | N/A (Note 4) | N/A (Note 4) |
| NR | C: TC21 CNC: TC21 C/NC: NTC21, TC21 | C: TC21 CNC: TC21 C/NC: NTC21, TC21 | C: TC22 | C: TC22 | C: TC21a CNC: TC21a C/NC: NTC21a, TC21a | C: TC21b CNC: TC21b C/NC: NTC21b, TC21b |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | C: TC21b CNC: TC21b C/NC: NTC21b, TC21b |
| GSM/EDGE | N/A | N/A | N/A | N/A | C: TC21a CNC: TC21a C/NC: NTC21a, TC21a | N/A |
| 6.5.2 Frequency error | - | - | - | - | - | - |
| E-UTRA | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Same TC as 6.5.1 | Same TC as 6.5.1 | N/A (Note 4) | N/A (Note 4) |
| NR | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 | Same TC as 6.5.1 |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | Same TC as 6.5.1 |
| GSM/EDGE | N/A | N/A | N/A | N/A | Same TC as 6.5.1 | N/A |
| 6.5.3 Time alignment error | - | - |  |  | - | - |
| E-UTRA | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) | (TS 36.141) NI, NG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | Standalone: (TS 36.141) NI, NG: (Note 4) | Standalone: (TS 36.141) NI, NG: (Note 4) | N/A (Note 4) | N/A (Note 4) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| 6.6 Unwanted emissions | - | - | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - | - | - |
| (Category A) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| (Category B) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | N/A |
| Protection of the BS receiver of own or different BS | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional spurious emissions requirements | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Co-location with other Base Stations | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| 6.6.2 Operating band unwanted emissions | - | - | - | - | - | - |
| General requirement for Band Categories 1 and 3 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | N/A | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b SC: (Note 3) |
| General requirement for Band Category 2 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 SC: (Note 3) | N/A | C: TC22 NI: TC22 NG: TC22 SC: (Note 3) | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a SC: (Note 3) | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b SC: (Note 3) |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - | - | - |
| Minimum requirement | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 36.141) (TS 38.141-1) | (TS 25.141) (TS 36.141) (TS 38.141-1) |
| 6.6.4 Adjacent Channel Leakage power Ratio (ACLR) | - | - | - | - | - | - |
| E- UTRA | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 | C: TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| NB-IoT | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | TC22 | TC22 | NI: TC21 NG: TC21 NCNI: NTC21 NCNG: NTC21 C/NCNI, C/NCNG: NTC21, TC21 | NI: TC21b NG: TC21b NCNI: NTC21b NCNG: NTC21b C/NCNI, C/NCNG: NTC21b, TC21b |
| NR | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21 | C: TC21 | C: TC21 CNC: NTC21 C/NC: NTC21, TC21 | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | C: TC21b CNC: NTC21b C/NC: NTC21b, TC21b |
| Cumulative ACLR | CNC: NTC21 C/NC: NTC21 | CNC: NTC21 C/NC: NTC21 | N/A | N/A | CNC: NTC21 C/NC: NTC21 | CNC: NTC21b C/NC: NTC21b |
| 6.7 Transmitter intermodulation | - | - | - | - |  |  |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | CNC: NTC21 C/NC: NTC21 | N/A | Same TC as used in 6.6 |  | CNC: NTC21a C/NC: NTC21a | CNC: NTC21b C/NC: NTC21b |
| 7.2 Reference sensitivity level | - | - | - | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.3 Dynamic range |  |  |  |  |  |  |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | N/A | N/A | N/A | N/A | (TS 25.141) |
| GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.4 In- band selectivity and blocking | - | - | - | - |  |  |
| General blocking requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| General narrowband blocking requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.5 Out-of-band blocking | - | - | - | - |  |  |
| General requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Co-location requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| 7.6 Receiver spurious emissions | - | - | - | - | - | - |
| General requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional requirement for BC2 (Category B) | N/A | N/A | C: TC22 NI: TC22 NG: TC22 | N/A | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | N/A |
| 7.7 Receiver intermodulation | - | - | - | - |  |  |
| General intermodulation requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| General narrowband intermodulation requirement | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A | N/A | TC5b | N/A |
| 7.8 In-channel selectivity | - | - | - | - | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| NOTE 1: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 2: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. NOTE 3: For Operating band unwanted emissions, NR shall also be tested with SC with widest supported channel bandwidth and highest supported sub-carrier spacing. NOTE 4: There is no specific test with NB-IoT for those requirements, tests could be performed using E-UTRA signal only, without NB-IoT. |  |  |  |  |  |  |  |

## 5.2	Single-RAT Multi-carrier capable Base Stations

This clause includes E-UTRA MC BS with one E-UTRA carrier supporting NB-IoT in-band and/or guard band.

Table 5.2-1: Test configurations for capability sets for Single-RAT capable BS

| Capability Set | UTRA (MC) capable BS (CS1) |  |  | E-UTRA (MC) capable BS NB-IoT in-band*, NB-IoT guard band** (CS2) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 |
| 6.2 Base Station output power | - | - | - | - | - | - |
| Base Station maximum output power | C: TC1a CNC: TC1a C/NC: TC1a, NTC1a | C: TC1a CNC: TC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A | N/A | N/A | Compliance stated by manufacturer declaration |
| E-UTRA DL RS power | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA FDD secondary CPICH power | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD primary CCPCH power | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| 6.3 Output power dynamics | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.4 Transmit ON/OFF power | - | - | - | - | - | - |
| Transmitter OFF power | N/A | N/A | C: TC1b | N/A | N/A | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Transmitter transient period | N/A | N/A | C: TC1b | N/A | N/A | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| 6.5 Transmitted signal quality | - | - | - | - | - | - |
| 6.5.1 Modulation quality | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI/NG: (Note 1) | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI/NG: (Note 1) | C: TC2 CNC: TC2 C/NC: TC2, NTC2 NI/NG: (Note 1) |
| UTRA FDD | C: TC1a CNC: TC1a C/NC: TC1a, NTC1a | C: TC1a CNC: TC1a C/NC: TC1a, NTC1a | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | C: TC1b | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | N/A: (Note 1) | N/A: (Note 1) | N/A: (Note 1) |
| 6.5.2 Frequency error | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | Same TC as used in 6.5.1 NI/NG: (Note 1) | Same TC as used in 6.5.1 NI/NG: (Note 1) | Same TC as used in 6.5.1 NI/NG: (Note 1) |
| UTRA FDD | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | Same TC as used in 6.5.1 | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | N/A: (Note 1) | N/A: (Note 1) | N/A: (Note 1) |
| 6.5.3 Time alignment error | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) NI/NG: (Note 1) | (TS 36.141) NI/NG: (Note 1) | (TS 36.141) NI/NG: (Note 1) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | N/A: (Note 1) | N/A: (Note 1) | N/A: (Note 1) |
| 6.6 Unwanted emissions | - | - | - | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - | - | - | - |
| (Category A) | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| (Category B) | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | N/A | N/A |
| Protection of the BS receiver of own or different BS | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Additional spurious emissions requirements | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Co-location with other Base Stations | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| 6.6.2 Operating band unwanted emissions | - | - | - | - | - | - |
| General requirement for Band Categories 1 and 3 | (TS 25.141) C: TC1a CNC: TC1a, NTC1a C/NC: TC1a, NTC1a | N/A | (TS 25.142) C: TC1b | (TS 36.141) C: TC2 CNC: TC2, NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | N/A | (TS 36.141) C: TC2 CNC: TC2, NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| General requirement for Band Category 2 | N/A | (TS 25.141) C: TC1a CNC: TC1a, NTC1a C/NC: TC1a, NTC1a | N/A | N/A | (TS 36.141) C: TC2 CNC: TC2, NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | N/A |
| GSM/EDGE single-RAT requirements | N/A | N/A | N/A | N/A | N/A | N/A |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - | - | - | - |
| Minimum requirement | (TS 25.141) | (TS 25.141) | (TS 25.142) | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) |  | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC:TC2, NTC2 NI: TC17 NG: TC20 |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | NI: TC17 NG: TC20 | NI: TC17 NG: TC20 | NI: TC17 NG: TC20 |
| Cumulative ACLR | CNC: NTC1a C/NC: NTC1a | CNC: NTC1a C/NC: NTC1a | - | CNC: NTC2 C/NC: NTC2 | CNC: NTC2 C/NC: NTC2 | CNC: NTC2 C/NC: NTC2 |
| 6.7 Transmitter intermodulation | - | - | - | - | - | - |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | CNC: NTC1a C/NC: NTC1a | Same TC as used in 6.6 | N/A | CNC: NTC2 C/NC: NTC2 NI: TC17 NG: TC20 | Same TC as used in 6.6 | N/A |
| 7.2 Reference sensitivity level | - | - | - | - | - | - |
| E-UTRA requirement | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD requirement | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD requirement | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| GSM/EDGE requirement | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT requirement | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.3 Dynamic range | - | - | - | - | - | - |
| E-UTRA | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD | (TS 25.141) | (TS 25.141) | N/A | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | (TS 25.142) | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| NB-IoT | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.4 In-band selectivity and blocking | - | - | - | - | - | - |
| General blocking requirement | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| General narrowband blocking requirement | C: TC1a, TC6a CNC: NTC1a, TC6a C/NC: TC1a, NTC1a, TC6a | C: TC1a, TC6a CNC: NTC1a, TC6a C/NC: TC1a, NTC1a, TC6a | C: TC1b TC6c | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.5 Out-of-band blocking | - | - | - | - | - | - |
| General requirement | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Co-location requirement | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| 7.6 Receiver spurious emissions | - | - | - | - | - | - |
| General requirement | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.7 Receiver intermodulation | - | - | - | - | - | - |
| General intermodulation requirement | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 |
| General narrowband intermodulation requirement | C: TC1a, TC6a CNC: NTC1a, TC6a C/NC: TC1a, NTC1a, TC6a | C: TC1a, TC6a CNC: NTC1a, TC6a C/NC: TC1a, NTC1a, TC6a | C: TC1b, TC6c | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 | C: TC2, TC6b CNC: NTC2, TC6b C/NC: TC2, NTC2, TC6b NI: TC17 NG: TC20 |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.8 In-channel selectivity | - | - | - | - | - | - |
| E-UTRA requirement | N/A | N/A | N/A | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT requirement | N/A | N/A | N/A | (NI: TS 36.141) | (NI: TS 36.141) | (NI: TS 36.141) |

Table 5.2-1a: Test configurations for capability sets for Single-RAT capable BS

| Capability Set | NB-IoT (MC) capable (CS 8) |  |  |
| --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 |
| 6.2 Base Station output power | . | . | - |
| Base Station maximum output power | TC8 | TC8 | TC8 |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A |
| E-UTRA DL RS power | N/A | N/A | N/A |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | N/A | N/A |
| UTRA FDD secondary CPICH power | N/A | N/A | N/A |
| UTRA TDD primary CCPCH power | N/A | N/A | N/A |
| 6.3 Output power dynamics | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.4 Transmit ON/OFF power | - | - | - |
| Transmitter OFF power | N/A | N/A | TC8 |
| Transmitter transient period | N/A | N/A | TC8 |
| 6.5 Transmitted signal quality | - | - | - |
| 6.5.1 Modulation quality | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A |
| NB-IoT | TC8 | TC8 | TC8 |
| 6.5.2 Frequency error | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A |
| NB-IoT | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 | Same TC as used in 6.5.1 |
| 6.5.3 Time alignment error | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.6 Unwanted emissions | - | - | - |
| 6.6.1 Transmitter spurious emissions | - | - | - |
| (Category A) | TC8 | TC8 | TC8 |
| (Category B) | TC8 | TC8 | TC8 |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A |
| Protection of the BS receiver of own or different BS | TC8 | TC8 | TC8 |
| Additional spurious emissions requirements | TC8 | TC8 | TC8 |
| Co-location with other Base Stations | TC8 | TC8 | TC8 |
| 6.6.2 Operating band unwanted emissions | - | - | - |
| General requirement for Band Categories 1 and 3 | (TS 36.141) TC8 | N/A | (TS 36.141) TC8 |
| General requirement for Band Category 2 | N/A | (TS 36.141) TC8 | N/A |
| GSM/EDGE single-RAT requirements | N/A | N/A | N/A |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - | - |
| Minimum requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| NB-IoT | TC8 | TC8 | TC8 |
| Cumulative ACLR | N/A | N/A | N/A |
| 6.7 Transmitter intermodulation | - | - | - |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| Additional requirement (BC1 and BC2) | Same TC as used in 6.6 | Same TC as used in 6.6 | N/A |
| 7.2 Reference sensitivity level | - | - | - |
| E-UTRA requirement | N/A | N/A | N/A |
| UTRA FDD requirement | N/A | N/A | N/A |
| UTRA TDD requirement | N/A | N/A | N/A |
| GSM/EDGE requirement | N/A | N/A | N/A |
| NB-IoT requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.3 Dynamic range | - | - | - |
| E-UTRA | N/A | N/A | N/A |
| UTRA FDD | N/A | N/A | N/A |
| UTRA TDD | N/A | N/A | N/A |
| GSM/EDGE | N/A | N/A | N/A |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| 7.4 In-band selectivity and blocking | - | - | - |
| General blocking requirement | TC8 | TC8 | TC8 |
| General narrowband blocking requirement | TC8 | TC8 | TC8 |
| Additional narrowband blocking requirement for GSM/EDGE | N/A | N/A | N/A |
| GSM/EDGE requirements for AM suppression | N/A | N/A | N/A |
| 7.5 Out-of-band blocking | - | - | - |
| General requirement | TC8 | TC8 | TC8 |
| Co-location requirement | TC8 | TC8 | TC8 |
| 7.6 Receiver spurious emissions | - | - | - |
| General requirement | TC8 | TC8 | TC8 |
| Additional requirement for BC2 (Category B) | N/A | N/A | N/A |
| 7.7 Receiver intermodulation | - | - | - |
| General intermodulation requirement | TC8 | TC8 | TC8 |
| General narrowband intermodulation requirement | TC8 | TC8 | TC8 |
| Additional narrowband intermodulation requirement for GSM/EDGE | N/A | N/A | N/A |
| 7.8 In-channel selectivity | - | - | - |
| E-UTRA requirement | N/A | N/A | N/A |
| NB-IoT requirement | N/A | N/A | N/A |

## 5.3	Multi-band capable Base Stations

Table 5.3-1: Test configurations for Multi-Band capable BS (CS1-CS7, CS16 and CS18-CS19)

| BS test case | Test for Multi-Band capable BS |  | Test configuration for MBT |  |
| --- | --- | --- | --- | --- |
|  | Common antenna connector | Separate antenna connector | BC1/BC2 | BC3 |
| 6.2 Base Station output power | - | - | - | - |
| Base Station maximum output power | SBT, MBT | SBT, MBT | TC7a | TC7a |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A | - |
| E-UTRA for DL RS power | SBT | SBT | - | - |
| UTRA FDD primary CPICH power | SBT | SBT | - | - |
| UTRA TDD primary CCPCH power | SBT | SBT | - | - |
| NB-IoT for DL RS power | SBT | SBT | - | - |
| 6.3 Output power dynamics | - | - |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 6.4 Transmit ON/OFF power |  |  | - | - |
| Transmitter OFF power | MBT, SBT7 | MBT, SBT7 | N/A | TC7a |
| Transmitter transient period | MBT, SBT7 | MBT, SBT7 | N/A | TC7a |
| 6.5 Transmitted signal quality |  |  |  |  |
| 6.5.1 Modulation quality |  |  |  |  |
| E-UTRA | SBT, MBT | SBT, MBT | TC7a | TC7a |
| UTRA FDD | SBT, MBT | SBT, MBT | TC7a | N/A |
| UTRA TDD | SBT, MBT | SBT, MBT | N/A | TC7a |
| GSM/EDGE | SBT, MBT | SBT, MBT | TC7a | N/A |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT | SBT, MBT | TC7a | TC7a |
| 6.5.2 Frequency error |  |  |  |  |
| E-UTRA | SBT, MBT | SBT, MBT | TC7a | TC7a |
| UTRA FDD | SBT, MBT | SBT, MBT | TC7a | N/A |
| UTRA TDD | SBT, MBT | SBT, MBT | N/A | TC7a |
| GSM/EDGE | SBT, MBT | SBT, MBT | TC7a | N/A |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT | SBT, MBT | TC7a | TC7a |
| 6.5.3 Time alignment between transmitter branches |  |  |  |  |
| E-UTRA | SBT, MBT1 | SBT, MBT1 | TC7b | TC7b |
| UTRA FDD | SBT, MBT1 | SBT, MBT1 | TC7b | N/A |
| UTRA TDD | SBT | SBT | N/A | - |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT1 | SBT, MBT1 | TC7b | TC7b |
| 6.6 Unwanted emissions |  |  |  |  |
| 6.6.1 Transmitter spurious emissions |  |  |  |  |
| (Category A) | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| (Category B) | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Additional requirement for BC2 (Category B) | SBT, MBT3 | SBT2, MBT2, 3 | TC7b | N/A |
| Protection of the BS receiver of own or different BS | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Additional spurious emissions requirements | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Co-location with other Base Stations | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| 6.6.2 Operating band unwanted emissions |  |  |  |  |
| General requirement for Band Categories 1 and 3 | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| General requirement for Band Category 2 | SBT, MBT | SBT2, MBT2 | TC7b | N/A |
| GSM/EDGE single-RAT requirement | SBT, MBT6 | SBT2, MBT2,6 | TC7c | N/A |
| Additional requirements | SBT, MBT | SBT2, MBT2 | - | - |
| 6.6.3 Occupied bandwidth |  |  |  |  |
| Minimum requirement | SBT | SBT | - | - |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) |  |  | - | - |
| E- UTRA | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| UTRA FDD | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | N/A |
| UTRA TDD | SBT, MBT4 | SBT2, MBT2, 4 | N/A | TC7b |
| Cumulative ACLR | SBT, MBT4 | SBT2 | TC7b | TC7b |
| NB-IoT | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| NR | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| 6.7 Transmitter intermodulation |  |  |  |  |
| General requirement | SBT | SBT2 | - | - |
| Additional requirement (BC1 and BC2) | SBT | SBT2 | - | N/A |
| 7.2 Reference sensitivity level |  |  |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 7.3 Dynamic range |  |  |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 7.4 In- band selectivity and blocking |  |  |  |  |
| General blocking requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| General narrowband blocking requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Additional narrowband blocking requirement for GSM/EDGE | SBT | SBT | - |  |
| GSM/EDGE requirements for AM suppression | SBT | SBT | - |  |
| 7.5 Out-of-band blocking |  |  |  |  |
| General requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Co-location requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| 7.6 Receiver spurious emissions |  |  |  |  |
| General requirement | SBT, MBT | SBT2 MBT2 | TC7b | TC7b |
| Additional requirement for BC2 (Category B) | SBT, MBT3 | SBT2, MBT2, 3 | TC7b | N/A |
| 7.7 Receiver intermodulation |  |  |  |  |
| General intermodulation requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| General narrowband intermodulation requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Additional narrowband intermodulation requirement for GSM/EDGE | SBT | SBT | - | N/A |
| 7.8 In-channel selectivity |  |  |  |  |
| E-UTRA requirement | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| NOTE 1: MBT is only applicable when DB-DC-HSDPA / inter-band CA is supported. NOTE 2: Single-band requirement apply to each antenna connector for both multi-band operation test and single-band operation test. For single-band operation test, other antenna connector(s) is (are) terminated. NOTE 3: For multi-band operation, this additional requirement for BC2 is applicable only when all supported operating bands belong to BC2 and GSM/EDGE is configured in all operating bands. NOTE 4: For ACLR, MBT shall be applied for the Inter RF Bandwidth gap only. In case of a BS capable of CS4, CS5, CS6, CS7 or CS18, the referenced test configuration shall be the corresponding one for CS1, CS2, CS3 or CS16 respectively, i.e. without GSM/EDGE carriers. NOTE 5: MBT is only applied for multi-band receiver. NOTE 6: MBT is only applicable for multi-band BS supporting CS4, CS5 or CS6 in at least one band. NOTE 7: SBT is only applicable if different Capability Sets are declared for single-band and multi-band operation. NOTE 8: There is no specific test with NB-IoT for those requirements, tests could be performed using E-UTRA signal only, without NB-IoT. |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

##### 6.6.1.5.6	Co-location with other Base Stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The power of any spurious emission shall not exceed the limits of Table 6.6.1.5.6-1 for a BS where requirements for co-location with a BS type listed in the second column apply, depending on the declared Base Station class. For BS capable of multi-band operation, the exclusions and conditions in the Table 6.6.1.5.6-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Table 6.6.1.5.6-1 apply for the operating band supported at that antenna connector.

*********************End of change*****************

*********************Next changed section*****************

#### 6.6.4.5	Test requirements

##### 6.6.4.5.1	E-UTRA test requirement

For E-UTRA, the test requirement is specified in Tables 6.6.4.5.1-1 and 6.6.4.5.1-2, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For a BS operating in non-contiguous spectrum, the ACLR also applies for the first adjacent channel inside any sub-blockgap with a gap size Wgap ≥ 15MHz. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR test requirement in clause 6.6.4.5.4 applies in sub block gaps for the frequency ranges defined in Table 6.6.4.5.4-1.

For a BS operating in multiple bands, where multiple bands are mapped onto the same antenna connector, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in clause 6.6.4.5.4 applies in Inter RF Bandwidth gaps for the frequency ranges defined in Table 6.6.4.5.4-1.

The requirement applies during the transmitter on period.

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centered on the adjacent channel frequency according to the tables below.

For Category A Wide Area BS, either the ACLR limits in the tables below or the absolute limit of -13dBm/MHz shall apply, whichever is less stringent.

For Category B Wide Area BS, either the ACLR limits in the tables below or the absolute limit of -15 dBm/MHz shall apply, whichever is less stringent.

For Medium Range BS, either the ACLR limits in the tables below or the absolute limit of -25 dBm/MHz shall apply, whichever is less stringent.

For Local Area BS, either the ACLR limits in the tables below or the absolute limit of -32dBm/MHz shall apply, whichever is less stringent.

For operation in paired spectrum, the ACLR shall be higher than the value specified in Table 6.6.4.5.1-1.

Table 6.6.4.5.1-1: Base Station ACLR in paired spectrum

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3.0, 5, 10, 15, 20 | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [3], with a chip rate as defined in this table. |  |  |  |  |

For operation in unpaired spectrum, the ACLR shall be higher than the value specified in Table 6.6.4.5.1-2.

Table 6.6.4.5.1-2: Base Station ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lower or above the upper Base StationRF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3 | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 5, 10, 15, 20 |  |  |  |  |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

##### 6.7.4.2.3	Void

1)

*********************End of change*****************

*********************Next changed section*****************

#### 6.7.5.3	Void

*********************End of change*****************

*********************Next changed section*****************

#### 7.4.4.6	Void

*********************End of change*****************

*********************Next changed section*****************

#### 7.4.5.5		Void

Table 7.4.5.5-1: Void

*********************End of change*****************

*********************Next changed section*****************

### 7.7.5	Test requirements

#### 7.7.5.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal, as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in Table 7.7.5.1-1 and 7.7.5.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], clause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], clause 7.2.

-	For any measured NB-IoT carrier (standalone or operating in E-UTRA in-band/guard band),, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured NB-IoT carrier (operating in NR in-band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

Table 7.7.5.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48+y (Note 6) | PREFSENS +x dB (Note 2, 5) |  |
| Medium Range BS | -44+y (Note 6) | PREFSENS +x dB (Note 3, 5) | See Table 7.7.5.1-2 |
| Local Area BS | -38+y (Note 6) | PREFSENS +x dB (Note 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [2]. For E-UTRA channel bandwidths 10, 15 and 20 MHz this requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 2: For WA BS supporting GSM and/or UTRA, "x" is equal to 6 in case of NR or E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS supporting GSM and/or UTRA, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of NR or E-UTRA or NB-IoT wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS supporting GSM and/or UTRA, "x" is equal to 12 in case of NR or E-UTRA or NB-IoT wanted signals, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS neither supporting GSM nor UTRA, x is equal to 6 for all BS classes if NR is supported, otherwise x is equal to 6 for WA BS or, 9 for MR BS and 12 for LA BS if NR is not supported. NOTE 6: For a BS supporting NR but neither UTRA nor GSM; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. For all other cases, "y" is equal to zero for all BS classes. |  |  |  |

Table 7.7.5.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2.0 (BC1 and BC3) / ±2.1 (BC2) | CW |
|  | ±4.9 | 1.4MHz E-UTRA signal |
| E-UTRA 3 MHz or E-UTRA with NB-IoT in-band | ±4.4 (BC1 and BC3) / ±4.5 (BC2) | CW |
|  | ±10.5 | 3MHz E-UTRA signal |
| UTRA FDD and E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz | ±7.375 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band15 MHz | ±7.25 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz | ±7.125 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| GSM/EDGE | ±7.575 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NB-IoT standalone | ±7.575 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 15 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 20 MHz or NR with NB-IoT operation in NR in-band | ±7.395 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 25 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 30 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 35 MHz or NR with NB-IoT operation in NR in-band | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 45 MHz or NR with NB-IoT operation in NR in-band | ±7.37 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz or NR with NB-IoT operation in NR in-band | ±7.35 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20MHz E-UTRA signal |

#### 7.7.5.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal, as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS operating in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in Table 7.7.5.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in Table 7.7.5.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in Table 7.7.5.2-1 and 7.7.5.2-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], clause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], clause 7.2.

-	For any measured GSM/EDGE carrier, the conditions are specified in TS 45.005 [6], Annex P.2.2.

-	For any measured NB-IoT carrier (standalone or operating in E-UTRA in-band/guard band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured NB-IoT carrier (operating in NR in-band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	Maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

Table 7.7.5.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 |  |  |
| Medium Range BS | -47 | PREFSENS +x dB | See Table 7.7.5.2-2 |
| Local Area BS | -44 | (NOTE 1) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104. "x" is equal to 6 in case of NR, NB-IoT,E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. "x" is specified in Table 7.7.5.2-1a for NB-IoT |  |  |  |

Table 7.7.5.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge | Interfering signal centre frequency offset from the Base Station RF Bandwidthedge or sub-block edge inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3.0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±360 (NOTE 3) | CW |
|  | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz | ±325 (NOTE 3) | CW |
| (NOTE 2) | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 15 MHz | ±380 (NOTE 3) | CW |
| (NOTE 2) | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz | ±345 (NOTE 3) | CW |
| (NOTE 2) | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| NB-IoT standalone | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±360 | CW |
|  | ±1420 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±400 | CW |
|  | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±370 | CW |
|  | ±1960 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz or NR with NB-IoT operation in NR in-band | ±380 | CW |
| (Note 2) | ±1960 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz or NR with NB-IoT operation in NR in-band | ±390 | CW |
| (Note 2) | ±2320 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz or NR with NB-IoT operation in NR in-band | ±325 | CW |
| (Note 2) | ±2350 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz or NR with NB-IoT operation in NR in-band | ±335 | CW |
| (Note 2) | ±2350 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±345 | CW |
|  | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz or NR with NB-IoT operation in NR in-band | ±355 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±365 | CW |
|  | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz or NR with NB-IoT operation in NR in-band | ±375 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz | ±395 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz | ±415 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz | ±435 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz | ±365 | CW |
| (Note 2) | ±2530 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz | ±385 | CW |
| (Note 2) | ±2530 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge or sub-block edge inside a gap. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 or NR G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band/guard band operation. NOTE 4: The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band/guard band operation. NOTE 5: If a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the E-UTRA interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |

*********************End of change*****************

*********************Next changed section*****************

# A.2	Void

Table A.2-1: Void