3GPP TSG-RAN4 Meeting #118	R4-2602950

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0414 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AAS_BS_LTE_UTRA,TEI17) CR to 37.145-2: Removal of UTRA TDD from MSR BS specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA, TEI17 |  |  |  |  |  | Date: |  |  | 2026-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | UTRA TDD has been in the AAS BS specifications since Rel-13. UTRA TDD AAS BS are not anymore produced and there is in general very few, if any, UTRA TDD deployments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR removes co-existence and colocation with UTRA TDD by removing the following: - UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) as “assumed adjacent carrier” for Base Station ACLR in unpaired spectrum with synchronized operation - Transmit intermodulation for colocation with UTRA TDD - Receiver blocking for co-existence with UTRA TDD UTRA TDD (1.28 Mcps) as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge” for Receiver intermodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary requirements for UTRA TDD co-existence would remain in the AAS BS specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3.2, 5.4.1, 6.8.5.1.3, 7.5.5.1.3, 7.8.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.104, 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 37.141, 37.145-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: References to removed requirements are removed in tables in 5.2, 5.3.2 and 5.4. |  |  |  |  |  |  |  |  |

## 5.2	Test configurations for AAS BS for operating bands where MSR with more than 1 RAT is supported

Table 5.2-1: Test configuration applicability to requirements
and capability sets for AAS BS supporting MSR operation

| Test case |  | UTRA + E-UTRA (RCSA 3) |  |  | E-UTRA + NR (RCSA 3A) |  |  | UTRA + E-UTRA + NR (RCSA 3B) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 | BC1, BC2 |
| 6.2 | Radiated transmit power | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | C: ATCR3b | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
| 6.3 | OTA Base Station output power | - | - | - | - | - | - | - |
| 6.3.2 | OTA Maximum output power | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
| 6.3.3 | OTA E-UTRA DL RS power | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Subclause 5.3.4 |
| 6.4 | OTA Output power dynamics | - | - | - | - | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A | Subclause 5.3.3 |
|  | NR – RE power control dynamic range | N/A | N/A | N/A | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
|  | NR – total power dynamic range | N/A | N/A | N/A | SC | SC | SC | SC |
| 6.5 | OTA Transmit ON/OFF power | - | - | - | - | - | - | - |
| 6.5.1 | OTA Transmitter OFF power | N/A | N/A | N/A | N/A | N/A | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | N/A |
| 6.5.2 | OTA Transmitter transient period | N/A | N/A | N/A | N/A | N/A | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | N/A |
| 6.6 | Transmitted signal quality | - | - | - | - | - | - | - |
| 6.6.2 | OTA Frequency error | - | - | - | - | - | - | - |
|  | E-UTRA | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in subclause 6.6.4 |
|  | UTRA FDD | Same TC as used in clause 6. 6.4 | Same TC as used in clause 6. 6.4 | N/A | N/A | N/A | N/A | Same TC as used in subclause 6.6.4 |
|  | NR | N/A | N/A |  | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in clause 6.6.4 | Same TC as used in subclause 6.6.4 |
| 6.6.3 | OTA Time alignment error | - | - | - | - | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A | Subclause 5.3.3 |
|  | NR | N/A | N/A | N/A | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
| 6.6.4 | OTA Modulation quality - EVM | - | - | - | - | - | - | - |
|  | E-UTRA | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
|  | UTRA FDD | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3 | N/A | N/A | N/A | N/A | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
|  |  | NR | N/A | N/A | N/A | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
| 6.7 | OTA Unwanted Emissions | - | - | - | - | - | - | - |
| 6.7.2 | OTA Occupied bandwidth | - | - | - | - | - | - | - |
|  | Minimum requirement | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.4 SC, ATCR8b (Note) | Clause 5.3.4 SC, ATCR8b (Note) | Clause 5.3.4 SC, ATCR8b (Note) | Subclause 5.3.3 Subclause 5.3.4 SC |
| 6.7.3 | OTA Adjacent Channel Leakage power Ratio | - | - | - | - | - | - | - |
|  | E-UTRA | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A | Subclause 5.3.3 |
|  | NR | N/A | N/A | N/A | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 | C: ATCR9 CNC: ATCR9 C/NC: ATCR9, ANTCR9 |
|  | Cumulative ACLR | CNC: ANTCR3 C/NC:ANTCR3 | CNC: ANTCR3 C/NC:ANTCR3 |  | CNC: ANTCR7 C/NC:ANTCR7 | CNC: ANTCR7 C/NC:ANTCR7 | CNC: ANTCR7 C/NC:ANTCR7 | CNC: ANTCR9 C/NC: ANTCR9 |
| 6.6.5 | OTA Operating band unwanted emission | - | - | - | - | - | - | - |
|  | General requirement for Band Categories 1 and 3 | Clause 5.3.3 Clause 5.3.4 C: ATCR3a CNC: ATCR3a, ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 | N/A | Clause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 | Subclause 5.3.3 Subclause 5.3.4 |
|  | General requirement for Band Category 2 | N/A | Clause 5.3.3 Clause 5.3.4 C: ATCR3a CNC: ATCR3a, ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | N/A | Clause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 | N/A | BC1: N/A BC2: Subclause 5.3.3 Subclause 5.3.4 C: ATCR9 CNC: ATCR9, ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Additional requirements | (note) | (note) | (note) | (note) | (note) | (note) | (note 1) |
| 6.7.6 | OTA Spurious emission | - | - | - | - | - | - | - |
|  | (Category A) | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | (Category B) | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Protection of the BS receiver of own or different BS | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Additional spurious emissions requirements | C: ATCR3a, CNC: ANTCR3, C/NC: ATCR3a, ATCR3a | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7, CNC: ANTCR7, C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Co-location with other Base Stations | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
| 6.8 | OTA Transmitter intermodulation | - | - | - | - | - | - | - |
|  | General requirement | Same TC as used in clause 6.7 | Same TC as used in clause 6.7 | Same TC as used in clause 6.7 | Same TC as used in clause 6.7 | Same TC as used in clause 6.7 | Same TC as used in clause 6.7 | Same TC as used in subclause 6.7 |
|  | Additional requirement (BC1 and BC2) | CNC: ANTCR3 C/NC:ANTCR3 | Same TC as used in clause 6.7 | N/A | CNC: ANTCR7 C/NC:ANTCR7a | Same TC as used in clause 6.7 | N/A | BC1: CNC: ANTCR9 C/NC:ANTCR9 BC2: Same TC as used in subclause 6.7 |
| 7.2 | OTA sensitivity | - | - | - | - | - | - | - |
|  | E-UTRA requirement | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | subclause 5.3.4 |
|  | UTRA FDD requirement | clause 5.3.3 | clause 5.3.3 | N/A | N/A | N/A | N/A | subclause 5.3.3 |
|  | UTRA TDD requirement | N/A | N/A | clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | NR requirement | N/A | N/A | N/A | ATCR4d | ATRC4d | ATCR4d | ATCR4d |
| 7.3 | OTA reference sensitivity level | - | - | - | - | - | - | - |
|  | E-UTRA requirement | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 | subclause 5.3.4 |
|  | UTRA FDD requirement | clause 5.3.3 | clause 5.3.3 | N/A | N/A | N/A | N/A | subclause 5.3.3 |
|  | NR requirement | N/A | N/A | N/A | ATRC4d | ATCR4d | ATCR4d | ATCR4d |
| 7.4 | OTA Dynamic range | - | - | - | - | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A | Subclause 5.3.3 |
|  | NR | N/A | N/A | N/A | ATCR4d | ATCR4d | ATCR4d | ATCR4d |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking | - | - | - | - | - | - | - |
|  | General blocking requirement | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | General narrowband blocking requirement | C: ATCR3a, ATCR4b CNC:ANTCR3, ATCR4b C/NC: ATCR3a, ANTCR3,ATCR4b | C: ATCR3a, ATCR4b CNC:ANTCR3, ATCR4b C/NC: ATCR3a, ANTCR3,ATCR4b | C: ATCR4b | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7,ATCR4b, ATCR4d | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7,ATCR4b, ATCR4d | C: ATCR7, ATCR4b, ATCR4d CNC: ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d | C: ATCR9, ATCR4a, ATCR4b, ATCR4d CNC: ANTCR9, ATCR4a, ATCR4b, ATCR4d C/NC: ATCR9, ANTCR9, ATCR4a, ATCR4b, ATCR4d |
| 7.6 | OTA Blocking | - | - | - | - | - | - | - |
|  | General requirement | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Co-location requirement | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
| 7.7 | OTA Receiver spurious emissions | - | - | - | - | - | - | - |
|  | General requirement | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.8 | OTA Receiver intermodulation | - | - | - | - | - | - | - |
|  | General intermodulation requirement | C: ATCR3a CNC: ANTCR3 C/NC: ATCR3a, ANTCR3 | C: ATCR3a CNC ANTCR3 C/NC: ATCR3a, ANTCR3 | N/A | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 | C: ATCR9 CNC: ANTCR9 C/NC: ATCR9, ANTCR9 |
|  | General narrowband intermodulation requirement | C: ATCR3a, ATCR4b CNC:ANTCR3, ATCR4b C/NC: ATCR3a, ANTCR3, ATCR4b | C: ATCR3a ATCR4b CNC:ANTCR3,ATCR4b C/NC: ATCR3a, ANTCR3; ATCR4b | C: ATCR4b | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d | C: ATCR7 ATCR4b, ATCR4d CNC:ANTCR7,ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7; ATCR4b, ATCR4d | C: ATCR7, ATCR4b, ATCR4d CNC: ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d | C: ATCR9, ATCR4a, ATCR4b, ATCR4d CNC: ANTCR9, ATCR4a, ATCR4b, ATCR4d C/NC: ATCR9, ANTCR9, ATCR4a, ATCR4b, ATCR4d |
| 7.9 | OTA In-channel selectivity | - | - | - | - | - | - | - |
|  | E-UTRA requirement | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 | Subclause 5.3.4 |
|  | NR requirement | N/A | N/A | N/A | ATCR4d | ATCR4d | ATCR4d | ANTCR9 |
| NOTE 1: ATCR8b is only applicable when contiguous CA is supported NOTE 2: For Operating band unwanted emissions, NR shall also be tested with SC with widest supported channel bandwidth and highest supported sub-carrier spacing. |  |  |  |  |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

### 5.3.2	AAS BS supporting one RAT only MSR in the operating band

This clause contains test configuration applicability to requirements and capability sets for AAS BS supporting one RAT only MSR operation operating with multiple carriers (MC).

Table 5.3.2-1: Test configuration applicability to requirements
and capability sets for operating bands supporting one RAT only MSR operation

| Capability Set |  | UTRA (MC) capable BS (RCSA1) |  |  | E-UTRA (MC) capable BS (RCSA2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case |  | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 |
| 6.2 | Radiated transmit power | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1b | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.3 | OTA Base Station output power | - | - | - | - | - | - |
| 6.3.2 | OTA maximum output power | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.3.3 | OTA E-UTRA DL RS power | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.4 | OTA Output power dynamics | - | - | - | - | - | - |
| 6.4.2 | OTA UTRA Inner loop power control in the downlink | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.4.3 | OTA Power control dynamic range | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.4.4 | OTA Total power dynamic range | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 6.4.5 | OTA IPDL time mask | Clause 5.3.3 | Clause 5.3.3 | N/A |  |  |  |
| 6.4.6 | OTA RE Power control dynamic range | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.5 | OTA Transmit ON/OFF power | - | - | - | - | - | - |
| 6.5.1 | OTA Transmitter OFF power | N/A | N/A | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.5.2 | OTA Transmitter transient period | N/A | N/A | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.6 | OTA signal quality | - | - | - | - | - | - |
| 6.6.2 | OTA Frequency error | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 |
|  | UTRA FDD | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | N/A | N/A | N/A | N/A |
| 6.6.3 | OTA Time alignment error | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.6.4 | OTA Modulation quality - EVM | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2aC/NC: ATCR2a, ANTCR2 |
|  | UTRA FDD | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | N/A | N/A | N/A | N/A |
| 6.7 | OTA Unwanted Emissions | - | - | - | - | - | - |
| 6.7.2 | OTA Occupied bandwidth | - | - | - | - | - | - |
|  | Minimum requirement | Clause 5.3.3 | Clause 5.3.3 | Clause 5.3.3 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.7.3 | OTA Adjacent Channel Leakage power Ratio |  | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | Cumulative ACLR | CNC: ANTCR1a C/NC: ANTCR1a | CNC: ANTCR1a C/NC: ANTCR1a | - | CNC: ANTCR2 C/NC: ANTCR2 | CNC: ANTCR2 C/NC: ANTCR2 | CNC: ANTCR2 C/NC: ANTCR2 |
| 6.7.4 | OTA Spectrum emission mask |  |  |  |  |  |  |
| 6.7.5 | OTA Operating band unwanted emission | - | - | - | - | - | - |
|  | General requirement for Band Categories 1 and 3 | Clause 5.3.3 C: ATCR1a CNC: ATCR1a, ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | Clause 5.3.3 | Clause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 | N/A | Clause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General requirement for Band Category 2 | N/A | Clause 5.3.3 C: ATCR1a CNC: ATCR1a, ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | N/A | Clause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 | N/A |
|  | Additional requirements | (note) | (note) | (note) | (note) | (note) | (note) |
| 6.7.6 | OTA Spurious emission | - | - | - | - | - | - |
|  | (Category A) | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | (Category B) | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Protection of the BS receiver of own or different BS | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Additional spurious emissions requirements | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Co-location with other Base Stations | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2C/NC: ATCR2a, ANTCR2 |
| 6.8 | OTA Transmitter intermodulation | - | - | - | - | - | - |
|  | General requirement | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 |
|  | Additional requirement (BC1 and BC2) | CNC: ANTCR1a C/NC:ANTCR1a | Same TC as used in 6.6 | N/A | CNC: ANTCR2 C/NC: ANTCR2 | Same TC as used in 6.6 | N/A |
| 7.2 | OTA sensitivity | - | - | - | - | - | - |
|  | E-UTRA requirement | N/A | N/A | N/A | clause 5.3.4 | clause 5.3.4 | clause 5.3.4 |
|  | UTRA FDD requirement | clause 5.3.3 | clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD requirement | N/A | N/A | clause 5.3.3 | N/A | N/A | N/A |
| 7.3 | OTA Reference sensitivity level | - | - | - | - | - | - |
|  | E-UTRA requirement | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD requirement | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 7.4 | OTA Dynamic range | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking | - | - | - | - | - | - |
|  | General blocking requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a | C: ATCR1a, NC: ANTCR1a C/NC: ATCR1a , ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General narrowband blocking requirement | C: ATCR1a, ATCR4a CNC: ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | N/A | C: ATCR2a, ATCR4b CNC:ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2 | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b |
| 7.6 | OTA Blocking | - | - | - | - | - | - |
|  | General requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Co-location requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
| 7.7 | OTA Receiver spurious emissions | - | - | - | - | - | - |
|  | General requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
| 7.8 | OTA Receiver intermodulation | - | - | - | - | - | - |
|  | General intermodulation requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General narrowband intermodulation requirement | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | N/A | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b |
| 7.9 | OTA In-channel selectivity | - | - | - | - | - | - |
|  | E-UTRA requirement | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |

*********************End of change*****************

*********************Next changed section*****************

## 5.4	Test configurations for AAS BS operating bands with multi-band dependencies

### 5.4.1	AAS BS operating bands with multi-band dependencies supporting MSR operation

Table 5.4.1-1: Test configuration for AAS BS operating bands containing beams
with multi-band dependencies supporting MSR operation

| test case |  |  | Test for AAS BS operating bands with multi-band dependencies | Test configuration for MBT |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RCSA1,RCSA2, RCSA3, RCSA3A | BC1/BC2 | BC3 |
| 6.2 | Radiated Transmit Power |  | SBT, MBT | ATCR5a | ATCR5a |
| 6.3 |  | OTA Base Station output power | - | - | - |
| 6.3.2 |  | OTA Base Station maximum output power | SBT, MBT | ATCR5a | ATCR5a |
| 6.3.3 |  | OTA E-UTRA DL RS power | E-UTRA for DL RS power | SBT | - |
| 6.4 |  | OTA Output power dynamics | - | - | - |
| 6.4.2 |  | OTA UTRA Inner loop power control in the downlink | SBT | - | - |
| 6.4.3 |  | OTA Power control dynamic range | SBT | - | - |
| 6.4.4 |  | OTA Total power dynamic range | SBT | - | N/A |
| 6.4.5 |  | OTA IPDL time mask | SBT | - | N/A |
| 6.4.6 |  | OTA RE Power control dynamic range | SBT | - | N/A |
| 6.5 |  | OTA Transmit ON/OFF power | - | - | - |
| 6.5.1 |  | OTA Transmitter OFF power | MBT, SBT (note 3) | N/A | ATCR5a |
| 6.5.2 |  | OTA Transmitter transient period | MBT, SBT (note 3) | N/A | ATCR5a |
| 6.6 |  | OTA Transmitted signal quality | - | - | - |
| 6.6.2 |  | OTA Frequency error | - | - | - |
|  |  | E-UTRA | SBT, MBT | ATCR5a | ATCR5a |
|  |  | UTRA FDD | SBT, MBT | ATCR5a | N/A |
|  |  | NR | SBT, MBT | ATCR5a | ATCR5a |
| 6.6.3 |  | OTA Time alignment error | - | - | - |
|  |  | E-UTRA | SBT, MBT (note 1) | ATCR5b | ATCR5b |
|  |  | UTRA FDD | SBT, MBT (note 1) | ATCR5b | N/A |
|  |  | NR | SBT, MBT (note 1) | ATCR5b | ATCR5b |
| 6.6.4 |  | OTA Modulation quality - EVM | - | - | - |
|  |  | E-UTRA | SBT, MBT | ATCR5a | ATCR5a |
|  |  | UTRA FDD | SBT, MBT | ATCR5a | N/A |
|  |  | NR | SBT, MBT | ATCR5a | ATCR5a |
| 6.7 |  | OTA Unwanted Emissions | - | - | - |
| 6.6.2 |  | OTA Occupied bandwidth | - | - | - |
|  |  | Minimum requirement | SBT | - | - |
| 6.7.3 |  | OTA Adjacent Channel Leakage power Ratio | - | - | - |
|  |  | E-UTRA | SBT, MBT (note 2) | ATCR5b | ATCR5b |
|  |  | UTRA FDD | SBT, MBT (note 2) | ATCR5b | N/A |
|  |  | NR | SBT, MBT (note 2) | ATCR5b | ATCR5b |
|  |  | Cumulative ACLR | SBT, MBT (note 2) | ATCR5b | ATCR5b |
| 6.7.4 |  | OTA Spectrum emission mask | - | - | - |
| 6.7.5 |  | OTA Operating band unwanted emission | - | - | - |
|  |  | General requirement for Band Categories 1 and 3 | SBT, MBT | ATCR5b | ATCR5b |
|  |  | General requirement for Band Category 2 | SBT, MBT | ATCR5b | N/A |
|  |  | Additional requirements | SBT, MBT | - | - |
| 6.7.6 |  | OTA Spurious emission | - | - | - |
|  |  | (Category A) | SBT, MBT | ATCR5b | ATCR5b |
|  |  | (Category B) | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Protection of the BS receiver of own or different BS | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Additional spurious emissions requirements | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Co-location with other Base Stations | - | - | - |
| 6.8 |  | OTA Transmitter intermodulation | - | - | - |
|  |  | General requirement | SBT | - | - |
|  |  | Additional requirement (BC1 and BC2) | SBT | - | N/A |
| 7.2 |  | OTA sensitivity | SBT | ATCR4a,ATCR4b | None |
| 7.3 |  | OTA Reference sensitivity level | - | - | - |
|  |  | E-UTRA requirement | SBT | - | - |
|  |  | UTRA FDD requirement | SBT | - | - |
|  |  | NR requirement | SBT | - | - |
| 7.4 |  | OTA Dynamic range | - | - | - |
|  |  | E-UTRA | SBT | - | - |
|  |  | UTRA FDD | SBT | - | - |
|  |  | NR | SBT | - | - |
| 7.5 |  | OTA Adjacent channel selectivity and narrowband blocking | - | - | - |
|  |  | General blocking requirement | MBT, SBT (note 3) | ATCR5b | ATCR5b |
|  |  | General narrowband blocking requirement | MBT, SBT (note 3) | ATCR5b | ATCR5b |
| 7.6 |  | OTA Blocking | - | - | - |
|  |  | General requirement | MBT, SBT(note 3) | ATCR5b | ATCR5b |
|  |  | Co-location requirement | MBT, SBT(note 3) | ATCR5b | ATCR5b |
| 7.7 |  | OTA Receiver spurious emissions | - | - | - |
|  |  | General requirement | SBT, MBT | ATCR5b | ATCR5b |
| 7.8 |  | OTA Receiver intermodulation | - | - | - |
|  |  | General intermodulation requirement | MBT, SBT(note 3) | ATCR5b | ATCR5b |
|  |  | General narrowband intermodulation requirement | MBT, SBT(note 3) | ATCR5b | ATCR5b |
| 7.9 |  | OTA In-channel selectivity | - | - | - |
|  |  | E-UTRA requirement | SBT | - | - |
|  |  | NR requirement | SBT | - | - |
| NOTE 1: MBT is only applicable when DB-DC-HSDPA or inter-band CA is supported. NOTE 2: For ACLR, MBT shall be applied for the Inter RF bandwidth gap only. NOTE 3: SBT is only applicable if different Capability Sets are declared for single-band and multi-band operation. |  |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

##### 6.8.5.1.3	Void

Table 6.8.5.1.3-1: Void

*********************End of change*****************

*********************Next changed section*****************

##### 7.5.5.1.3	Void

Table 7.5.5.1.3-1: Void

*********************End of change*****************

*********************Next changed section*****************

#### 7.8.5.1	MSR operation

##### 7.8.5.1.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA, NR or UTRA signal as specified in TS 37.104 [5], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap at those connectors, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 7.8.5.1.1-1 and 7.8.5.1.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], clause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in TS 25.104 [2], clause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 38.104 [33], clause 7.2.2.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.1.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE 1) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 2, 5) | See table 7.8.5.1.1-2 |
|  | -48 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 2, 5) |  |
| Medium Range BS | -44 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS + x dB (NOTE 3, 5) |  |
|  | -44 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 3, 5) |  |
| Local Area BS | -38 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 4, 5) |  |
|  | -38 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 4, 5) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see clauses 7.3 and 7.2. NOTE 2: For WA BS supporting UTRA, "x" is equal to 6. NOTE 3: For MR BS supporting UTRA, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA . NOTE 4: For LA BS supporting UTRA, "x" is equal to 12 in case of NR or E-UTRA wanted signals, 6 in case of UTRA wanted signal. NOTE 5: For a BS not supporting UTRA, x is equal to 6 for all BS classes if NR is supported, otherwise x is equal to 6 for WA BS or 9 for MR or 12 for LA BS if NR is not supported.NOTE 6: For a BS that supports NR but not UTRA; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. For all other cases, “y” is equal to zero for all BS classes. |  |  |  |

Table 7.8.5.1.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2,0 (BC1 and BC3) / ±2,1 (BC2) | CW |
|  | ±4,9 | 1,4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4,4 (BC1 and BC3) / ±4,5 (BC2) | CW |
|  | ±10,5 | 3 MHz E-UTRA signal |
| UTRA FDD and | ±7,5 | CW |
| E-UTRA 5 MHz | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7,375 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7,25 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7,125 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7,575 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| NR 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 7 MHz | ±7.45 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 10 MHz | ±7.465 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 15 MHz | ±7.43 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 20 MHz | ±7.395 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 25 MHz | ±7.465 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 30 MHz | ±7.43 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 35 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz | ±7.45 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 45 MHz | ±7.37 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz | ±7.35 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20 MHz E-UTRA signal |

##### 7.8.5.1.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal as specified in TS 37.104 [5], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIB supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.8.5.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least as wide as the E-UTRA interfering signal in table 7.8.5.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 7.8.5.1.2-1 and 7.8.5.1.2-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], clause 7.2.5.3.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in TS 25.104 [2], clause 7.2.5.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 38.104 [33], clause 7.2.5.3.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.1.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 - ΔOTAREFSENS | EISREFSENS + 6 dB | See table 7.8.5.1.2-2 |
|  | -52 – ΔminSENS | EISminSENS + 6 dB |  |
| Medium Range BS | -47 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -47 – ΔminSENS | EISminSENS + 6 dB |  |
| Local Area BS | -44 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -44 – ΔminSENS | EISminSENS + 6 dB |  |
| NOTE EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see clauses 10.3 and 10.2. |  |  |  |

Table 7.8.5.1.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or edge of the sub-block | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidthedge or edge of sub-block inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3,0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1 060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz | ±325 | CW |
| (NOTE 2) | ±1 240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz | ±380 | CW |
| (NOTE 2) | ±1 600 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz | ±345 | CW |
| (NOTE 2) | ±1 780 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1 780 (BC1 and BC2) | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz | ±360 | CW |
|  | ±1420 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 7 MHz | ±400 | CW |
|  | ±1240 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz | ±370 | CW |
|  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz | ±380 | CW |
| (Note 2) | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz | ±390 | CW |
| (Note 2) | ±2320 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz | ±325 | CW |
| (Note 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz | ±335 | CW |
| (Note 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz (Note 2) | ±345 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz | ±355 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz (Note 2) | ±365 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz | ±375 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz | ±395 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz | ±415 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz | ±435 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz | ±365 | CW |
| (Note 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz | ±385 | CW |
| (Note 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |