# 3GPP TSG-RAN WG4 Meeting #117	R4-2522652

Dallas, USA, Nov.17-21, 2025

Agenda item:			7.4.1

Source:	Ericsson

Title:	WF on Rel-20 NR_RRM_Ph6

Document for:	Approval

# Topic #1: (e)RedCap UE enhancement requirements (AI 7.4.3.2)

Issue 1-1-1: HD-FDD requirement for 2Rx (e)RedCap UE with LT5

Agreement:

  - The legacy principle of 2Rx Redcap HD-FDD can be directly reused to HD-FDD requirement for 2Rx (e)RedCap with LT5.

Issue 1-1-2: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE

Agreement:

  - Hypothetical PDCCH transmission parameters for RLM out-of-sync evaluation for 1Rx LT5 (e)RedCap UE

    - The number of Control OFDM symbols = 3, Aggregation Level = 8

Issue 1-1-3: Soft combining receiver for 1Rx (e)RedCap UE

Proposals

  - Option 1: Soft combining for 1 Rx UE should be investigated via simulations.

  - Option 2: Not consider soft combining

Issue 2-2-2: Entry and exit criteria, singling indication for RLM/BFD relaxation

Agreement:

  - Re-use the Good Serving cell quality and Low mobility criteria and signaling defined in Rel-17 RLM/BFD relaxation

  - Reuse the condition defined in Rel-17 RLM/BFD relaxation

  - No new criteria and no new signalling on the criteria are expected.

Issue 2-2-1: Relaxed RLM/BFD for 1Rx (e)RedCap UE

RAN4 to further discuss TEvaluate_out_SSB_Relax for RLM/BFD relaxation of 1Rx (e)RedCap

  - Option 1: K1 = 4 for Max(TDRX, TSSB) ≤40ms and K1 = 2 for 40ms<Max(TDRX, TSSB) ≤80ms(legacy 2Rx RLM/BFD relaxation)

  - Option 2: K1 = 2 for Max(TDRX, TSSB) ≤40ms and K1 = 1.5 for 40ms<Max(TDRX, TSSB) ≤80ms

  - Option 3: K1 = 2 for Max(TDRX, TSSB) ≤40ms

  - Option 4: K1 = 4 for Max(TDRX, TSSB) ≤40ms

  - Option 5: No relaxation is defined for 1Rx (e)RedCap

  - Other options are not precluded