3GPP TSG-RAN5 Meeting #110			R5-26XXXX

Malta, Malta, 19th May 2025 - 23rd May 2025

Title:	LS on A-IoT OTA Anechoic chamber method

Response to:

Release:	Rel-19

Work Item:	Ambient_IoT_Solutions_plus_CT1_SA3-ConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	-

Contact Person:

Name:	Dan Song; Flores Fernandez Martos

Tel. Number:

E-mail Address:	songdan@chinamobile.com; flores_fernandez@keysight.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN5 has discussed A-IoT OTA Anechoic chamber method and would like to seek clarifications on the following items:

- For the statement of cl. 8.4.1 in TS 38.191 “Test antenna with two linear orthogonal polarizations supports both CW and Reader, namely CW and Reader share the same antenna with CW and Reader using both polarizations”,

- For the test antenna, it is not clear to RAN5 whether CW and Reader will always be in orthogonal polarizations? Or whether CW and Reader can be configured independently and so there is no need for them to be always in orthogonal polarizations?

- Since CW and Reader share the same test antenna, CW and Reader will share the same direction in testing. However, whether this will be the worst case for A-IoT device testing in procedures listed below is not clear to RAN5 either. Generally, for RAN5 device conformance testing, worst case for the device will be chosen to ensure the effectiveness.

  - Backscattering measurement procedure

  - Sensitivity measurement procedure

  - Unwanted emission measurement procedure

- For Step 4 in cl. 8.4.2 in TS 38.191 “Set the signal generator (i.e., R2D signal) and the CW generator to transmit at the target test frequency with θ-polarization”,

- It is not clear to RAN5 whether A-IoT device will receive signals from both CW generator and signal generator (Reader) simultaneously in time?

2. Actions:

To RAN4 group.

ACTION:

RAN5 respectfully requests RAN4 to respond to the above questions.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#111	 18th – 22nd May 2026	Dalian, China

TSG-RAN5 Meeting#112	 24th – 28th August 2025	Maastricht, Netherlands

4. References: