3GPP TSG-RAN5 Meeting #110	draft R5-26xxxx

# Gothenburg, Sweden, 9th – 13th February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for the capability uplinkTxSwitching-PowerBoosting-r16 and some CA configurations for Rel-16 to Rel-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Nokia, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2026-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The test requirement in the time mask for switching between two uplink carrier test case depends on the capability uplinkTxSwitching-PowerBoosting-r16. A corresponding PICS is missing. Some CA configurations for Rel-16 to Rel-19 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PICS pc_uplinkTxSwitching_PowerBoosting_r16 has been added and tables updated. Some CA configurations for Rel-16 to Rel-19 are added into relevant tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case cannot select the appropriate power requirement. Some CA configurations for Rel-16 to Rel-19 will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2, A.4.3.2A.3.1, A.4.3.2A.4.1, A.4.3.2B.1.0a.1, A.4.3.2C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 3708 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note for spec implementation: Please, replace soft code 37xx in Table A.4.3.2-1 with an appropriate hard code (e.g. 37i) related test case update in CR R5-261046. This CR merges the content of R5-260106, R5-260117, R5-260184, R5-260250, R5-261047. Other work item codes impacted are NR_CADC_NR_LTE_DC_R18-UEConTest, NR_CADC_NR_LTE_DC_R19-UEConTest, NR_FR1_lessthan_5MHz_BW_Ph2-UEConTest, TEI16_Test, NR_RF_FR1-UEConTest |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |
| 36 | Supports pi/2-BPSK modulation scheme for power boosting in FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_powerBoosting_pi2BPSK | No |  |  |
| 37 | Support of dynamic 1Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-16 | pc_ULTxSwitchingBandPair | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37a | Support of 35us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_35us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37b | Support of 140us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_140us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37c | Support of 210us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_210us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37d | Support of DL interruption during 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-18 | pc_uplinkTxSwitching_DL_Interruption_r16 | No |  | Not allowed to be setted to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 37e | Support of switchedUL Option for r16 UL Tx Switching | 38.306 4.2.7.1 | Rel-16 | pc_uplinkTxSwitching_OptionSupport_switchedUL_r16 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport-r16 set to switchtedUL |
| 37f | Support of dualUL Option for r16 UL Tx Switching | 38.306 4.2.7.1 | Rel-16 | pc_uplinkTxSwitching_OptionSupport_dualUL_r16 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport-r16 set to dualUL |
| 37g | Support of switchedUL Option for r17 UL Tx Switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_OptionSupport2T2T_switchedUL_r17 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport2T2T-r17 set to switchtedUL |
| 37h | Support of dualUL Option for r17 UL Tx Switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_OptionSupport2T2T_dualUL_r17 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport2T2T-r17 set to dualUL |
| 37xx | Support of 3dB boosting on the maximum output power for UE transmission under the operation state in which 2-port transmission can be supported on carrier2 in case of inter-band UL CA case where UE supports dynamic UL Tx switching. | 38.306 4.2.7.1 | Rel-16 | pc_uplinkTxSwitching_PowerBoosting_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 and Table A.4.3.2C.2-1. A UE shall only indicate this capability in case the UE supports power class 3 for inter-band UL CA for the band combination. |
| 38 | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 | 38.306 4.2.7.2 | Rel-16 | pc_mpr_PowerBoost_FR2 | No |  |  |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### A.4.3.2A.3	NR Intra-band non-contiguous CA

##### A.4.3.2A.3.1	NR Intra-band non-contiguous CA within FR1

Table A.4.3.2A.3.1-1: Downlink Bandwidth Class capabilities for NR Intra-band non-contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.3.1-3)

| Item | DL NR FR1 Intra-band non-contiguous CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR FR1 Intra-band non-contiguous CA BW Class Combination (2A) | 38.101-1, 5.3A.5 | pc_DL_intra_non_contiguous_CA_NR_FR1_Class_(2A) |  |
| 2 | DL NR FR1 Intra-band non-contiguous CA BW Class Combination (3A) | 38.101-1, 5.3A.5 | pc_DL_intra_non_contiguous_CA_NR_FR1_Class_(3A) |  |
| 3 | DL NR FR1 Intra-band non-contiguous CA BW Class Combination (4A) | 38.101-1, 5.3A.5 | pc_DL_intra_non_contiguous_CA_NR_FR1_Class_(4A) |  |

Table A.4.3.2A.3.1-1a: Downlink Bandwidth Class capabilities for NR mixed Intra-band contiguous and non-contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.3.1-3a)

| Item | DL NR FR1 mixed Intra-band contiguous and non-contiguous CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR FR1 mixed Intra-band contiguous and non-contiguous CA BW Class Combination (A-B) | 38.101-1, 5.3A.5 | pc_DL_intra_contiguous_non_contiguous_CA_NR_FR1_Class_(A-B) |  |
| 2 | DL NR FR1 mixed Intra-band contiguous and non-contiguous CA BW Class Combination (A-C) | 38.101-1, 5.3A.5 | pc_DL_intra_contiguous_non_contiguous_CA_NR_FR1_Class_(A-C) |  |

Table A.4.3.2A.3.1-2: Uplink Bandwidth Class capabilities for NR Intra-band non-contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.3.1-3)

| Item | UL NR FR1 Intra-band non-contiguous CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band non-contiguous CA BW Class Combination (2A) | 38.101-1, 5.3A.5 | pc_UL_intra_non_contiguous_CA_NR_FR1_Class_(2A) |  |

Table A.4.3.2A.3.1-2a: Uplink Bandwidth Class capabilities for NR mixed Intra-band contiguous and non-contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.3.1-3a)

| Item | UL NR FR1 mixed Intra-band contiguous and non-contiguous CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR FR1 mixed Intra-band contiguous and non-contiguous CA BW Class Combination (B) | 38.101-1, 5.3A.5 | pc_UL_intra_contiguous_non_contiguous_CA_NR_FR1_Class_(B) |  |

Table A.4.3.2A.3.1-3: Supported configurations for NR Intra-band non-contiguous CA within FR1

| NR FR1 Intra-band non-contiguous CA configuration / Item (Note 5) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 3) | Supported Bandwidth Combination Set(s) (Note 1) |
| --- | --- | --- | --- | --- |
| CA_n2(2A) | Rel-17 |  |  |  |
| CA_n25(2A) | Rel-16 |  |  |  |
| CA_n41(2A) | Rel-16 |  |  |  |
| CA_n48(2A) | Rel-16 |  |  |  |
| CA_n66(2A) (Note 4) | Rel-16 |  |  |  |
| CA_n66(3A) | Rel-17 |  |  |  |
| CA_n71(2A) | Rel-17 |  |  |  |
| CA_n77(2A) | Rel-17 |  |  |  |
| CA_n77(3A) | Rel-18 |  |  |  |
| CA_n78(2A) | Rel-17 |  |  |  |
| Note 1: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.2-1. Note 2: Void. Note 3: See UL(table_index) in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 4: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 5: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |

< Unchanged sections omitted >

##### A.4.3.2A.4.1	NR Inter-band CA within FR1 (two bands)

< Unchanged tables omitted >

Table A.4.3.2A.4.1-3: Supported configurations for NR Inter-band CA within FR1 and two bands

| NR FR1 Inter-band CA configuration / Item (Note 1, 9) | Release | Supported | Supported power class for single uplink carrier (Note 12) | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) | Supported 1Tx-2Tx ULTxSwitch-ing Band Pair (Note 7, 8) | Supported 3 dB Boosting in UL Tx Switching (Note 7, 8) | Supported 2Tx-2Tx ULTxSwitching Band Pair (Note 7) | Supported uplinkTx Switching-DL-Interruption-r16 (Note 10) | Supported simultaneousRxTx (Note 11) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n5A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n1(2A)-n3A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n1(2A)-n5A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n1(2A)-n8A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n26A | Rel-18 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n41A | Rel-16 |  |  |  |  |  |  |  |  | Yes |
| CA_n1A-n71A | Rel-19 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n1A-n78A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n1(2A)-n78A | Rel-17 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n1A-n78(2A) | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n1A-n78C | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n1A-n79A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n2A-n5A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n5A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n14A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n14A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n30A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n30A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48B | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66(3A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n5A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n3(2A)-n5A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n3(2A)-n8A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n40A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n3A-n41A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n3A-n77A | Rel-15 |  |  |  |  |  |  |  | Not supported for UEs up to Rel-18 | CA_n3A-n77A for UEs up to Rel-18 |
| CA_n3A-n77(2A) | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n3A-n78A | Rel-15 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n3A-n79A | Rel-15 |  |  |  |  |  |  |  | Not supported |  |
| CA_n5A-n28A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48(2A) | Rel-17 |  |  |  |  |  |  |  |  | CA_n5A-n48(2A) |
| CA_n5A-n48B | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n71A | Rel-19 |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78(2A) | Rel-17 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n3(2A)-n78A | Rel-17 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n5A-n25A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n29A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66(3A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78(2A) | Rel-17 |  |  |  |  |  |  |  |  | Yes |
| CA_n5A-n7A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n30A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77A | Rel-16 |  |  |  |  |  |  |  |  | Yes |
| CA_n5A-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n5A-n79A | Rel-16 |  |  |  |  |  |  |  | Not supported |  |
| CA_n5B-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n5B-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n7A-n78A | Rel-16 |  |  |  |  |  |  |  |  | Yes |
| CA_n8A-n77A | Rel-18 |  |  |  |  |  |  |  |  |  |
| CA_n8A-n78A | Rel-15 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n8A-n77(2A) | Rel-19 |  |  |  |  |  |  |  |  |  |
| CA_n8A-n78(2A) | Rel-17 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n14A-n30A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n14A-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n14A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n14A-n66(3A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n14A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n14A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n20A-n78A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48B | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n24A-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n41A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66(2A) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n78A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n25A-n78(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n26A-n28A | Rel-18 |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n26A-n70A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n26A-n78A | Rel-18 |  |  |  |  |  |  |  |  |  |
| CA_n28A-n40A | Rel-16 |  |  |  |  |  |  |  |  | Yes |
| CA_n28A-n41A | Rel-16 |  |  |  |  |  |  |  |  | Yes |
| CA_n28A-n41C | Rel-17 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n28A-n71A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A | Rel-16 |  |  |  |  |  |  |  | Not supported |  |
| CA_n28A-n77(2A) | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n28A-n78A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n28A-n78(2A) | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n28A-n79A | Rel-17 |  |  |  |  |  |  |  |  | Yes |
| CA_n29A-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66B | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66(2A) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n29A-n70A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n29A-n71A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n39A-n41A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n30A-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n30A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n30A-n66(3A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n30A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n30A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n40A-n71A | Rel-19 |  |  |  |  |  |  |  |  |  |
| CA_n40A-n77A | Rel-17 |  |  |  |  |  |  |  |  | Yes |
| CA_n41A-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n41C-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71A | Rel-16 |  |  |  |  |  |  |  |  | Yes for UEs up to Rel-18 |
| CA_n41A-n71B | Rel-16 |  |  |  |  |  |  |  |  | Yes for UEs up to Rel-18 |
| CA_n41C-n71A | Rel-16 |  |  |  |  |  |  |  |  | Yes for UEs up to Rel-18 |
| CA_n41(2A)-n71A | Rel-16 |  |  |  |  |  |  |  |  | Yes for UEs up to Rel-18 |
| CA_n41A-n77A | Rel-17 |  |  |  |  |  |  |  |  | Yes |
| CA_n41A-n78A | Rel-15 |  |  |  |  |  |  |  |  | Yes |
| CA_n41A-n79A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n41A-n79C | Rel-18 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n41C-n79A | Rel-16 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n41C-n79C | Rel-18 |  |  |  |  |  |  |  | Not supported | Yes |
| CA_n48A-n66A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n70A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n71A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n71(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48A-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48B-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48B-n70A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48B-n71A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48B-n77A | Rel-17 |  |  |  |  |  |  |  |  | CA_n48B-n77A |
| CA_n48(2A)-n66A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n66(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n70A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n71A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n71(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66B-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77(3A) | Rel-18 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77C | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66(3A)-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n78A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n78(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71A | Rel-16 |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n71A-n77A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n71A-n77(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n71A-n78A | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n71A-n78(2A) | Rel-17 |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A | Rel-15 |  |  |  |  |  |  |  |  |  |
| CA_n100A-n101A | Rel-19 |  |  |  |  |  |  |  |  |  |
| Note 1: Notation used for inter-band CA Bands is according to TS 38.101-1 [23] Table 5.5A.3.1-1, e.g. ‘CA_n1A-n78C’ indicates CA operation on NR band n1 and n78 with DL CA Bandwidth Class A and C respectively. Note 2: The UL CA capabilities as per Table A.4.3.2A.4.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.3.1-1. For this release of specification valid choices are ’N’, ‘nXA-nYA’, ‘nX(2A)’, ‘nXB’ and ‘nXC’, where both nX and nY are the NR bands. For example, for CA_n1A-n77A, ‘N’ would mean only DL CA, ‘n1A-n77A’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3.1-1. Note 4: Void. Note 5: See UL(table_index) in in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: The ULTxSwitching capability can be reported on inter-band CA band combinations. The UE supplier shall indicate inter-band CA band pairs on which it supports 1Tx-2Tx or 2Tx-2Tx ULTxSwitching or 3dB boosting. The 3dB boosting is on the maximum output power under the operation state in which 2-port transmission can be supported on carrier2. The boosting shall only be indicated in case the UE supports power class 3 for the band combination. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for CA_n1A-n77A, ‘N’ would mean not supporting ULTxSwitching, ‘n1-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at least one inter-band UL CA configuration on the same band pair in the column “Supported CA Bandwidth Class(es) in UL”. The ULTxSwitching is only tested with 2 UL or 3 UL CCs, so UE is allowed to report ‘N’ by default for CA configuration with > 3 component carriers. Note 8: See ULTxSwitching(table_index) and 2Tx_ULTxSwitching(table_index) in Note 6 of Table 4.0-3 in TS 38.522 [9]. Note 9: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. Note 10: A UE that supports ULTxSwitching on a band pair might report the uplinkTxSwitching-DL-Interruption-r16 capability on the same band pair. If UE doesn’t report this capability, no DL interruption is allowed during UL Tx switching. For certain band configurations DL interruption is not allowed according to Note 8 in Table 5.2A.2.1-1 of TS 38.101-1 [23], therefore the corresponding entry is prefilled by ‘Not Supported’. Note 11: For configurations with Note 1 in Table 5.2A.2.1-1 of TS 38.521-1 [5], UE capability simultaneousRxTxInterBandCA is mandatory, therefore the corresponding entry is prefilled with ‘Yes’. Note 12: The UE supplier shall indicate the supported single uplink carrier with power class other than PC3, as per TS 38.101-1 [23] Table 5.5A.3.1-1. For this release of specification valid choices are ’-’, ‘nX PC2’, ‘nY PC2’, where both nX and nY are the NR bands. For example, for CA_n1A-n78A, ‘-’ would mean only supports PC3 single uplink carrier, ‘n1 PC2’ would mean supports single carrier PC2 on band n1, ‘n78 PC2’ would mean supports single carrier PC2 on band n78. |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### A.4.3.2B	NR-DC, EN-DC and NE-DC Physical Layer Baseline Implementation Capabilities

NOTE:	See Annex B for status of completed NR-DC, EN-DC and NE-DC configurations and power classes in this version of 3GPP UE conformance test specifications.

#### A.4.3.2B.1	NR-DC Physical Layer Baseline Implementation Capabilities

#### A.4.3.2B.1.0	General NR-DC capabilities

Table A.4.3.2B.1.0-1: Downlink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_2CC |  |
| 2 | DL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_3CC |  |
| 3 | DL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_4CC |  |
| 4 | DL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_5CC |  |
| 5 | DL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_6CC |  |
| 6 | DL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_7CC |  |
| 7 | DL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_8CC |  |
| 8 | DL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_9CC |  |
| 9 | DL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_10CC |  |

Table A.4.3.2B.1.0-2: Uplink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_2CC |  |
| 2 | UL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_3CC |  |
| 3 | UL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_4CC |  |
| 4 | UL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_5CC |  |
| 5 | UL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_6CC |  |
| 6 | UL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_7CC |  |
| 7 | UL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_8CC |  |
| 8 | UL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_9CC |  |
| 9 | UL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_10CC |  |

#### A.4.3.2B.1.0a	NR-DC within FR1

##### A.4.3.2B.1.0a.1	NR-DC within FR1 (two bands)

Table A.4.3.2B.1.0a.1-1: Downlink NR-DC Bandwidth Class Combination capabilities within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.1.0a.1-3)

| Item | DL NR-DC FR1 Bandwidth Class (two bands) | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR-DC FR1 BW Class Combination A-A (two bands) | 38.101-1, 5.5B | pc_DL_NR_DC_FR1_2B_Class_A-A |  |

Table A.4.3.2B.1.0a.1-2: Uplink NR-DC Bandwidth Class Combination capabilities within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.1.0a.1-3)

| Item | UL NR-DC FR1 Bandwidth Class (two bands) | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR-DC FR1 BW Class Combination A-A (two bands) | 38.101-1, 5.5B | pc_UL_NR_DC_FR1_2B_Class_A-A |  |

Table A.4.3.2B.1.0a.1-3: Supported NR-DC configurations within FR1 (two bands)

| NR FR1 Inter-band NR-DC configuration / Item (Note 1, 6) | Release | Supported | Supported NR-DC Bandwidth Class(es) in UL (Note 2,3) | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 4, 5) |
| --- | --- | --- | --- | --- | --- |
| DC_n48A_n70A | Rel-17 |  |  |  |  |
| DC_n100A_n101A | Rel-19 |  |  |  |  |
| Note 1: Notation used for inter-band NR-DC Bands is according to TS 38.101-1 [23] Table 5.5B.1-1, e.g. ‘DC_n2A-n48C’ indicates NR-DC operation on NR band n2 and n48 with DL CA Bandwidth Class A and C respectively. Note 2: The UL NR-DC capabilities as per Table A.4.3.2B.1.0a.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL NR-DC Bandwidth Class(es), in uplink of the supported NR-DC Band(s), as per TS 38.101-1 [23] Table 5.5B.1-1. For this release of specification valid choices are ’N’, ‘nXA-nYA’, ‘nX(2A)’, ‘nXB’ and ‘nXC’, where both nX and nY are the NR bands. For example, for DC_n48A-n70A, ‘N’ would mean only DL NR_DC, ‘n48A-n70A’ would mean both DL and UL NR-DC. Note 3: See UL(table_index) in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 4: The ULTxSwitching capability can be reported on inter-band NR-DC band combinations. The UE supplier shall indicate inter-band NR-DC band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for DC_n48A-n70A, ‘N’ would mean not supporting ULTxSwitching, ‘n48-n70’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at least one inter-band UL NR-DC configuration on the same band pair in the column “Supported NR-DC Bandwidth Class(es) in UL”. The ULTxSwitching is only tested with 2 UL CCs, so UE is allowed to report ‘N’ by default for NR-DC configuration with > 2 component carriers. Note 5: See ULTxSwitching(table_index) Note 6 of Table 4.0-3 in TS 38.522 [9]. Note 6: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |  |

< Unchanged tables omitted >

#### A.4.3.2C.2	SUL band combinations without CA

Table A.4.3.2C.2-1: Supported SUL configurations without CA

| SUL configuration / Item (Note 1) | Release | Supported | Supported Bandwidth Combination Set(s) | Supported 1Tx-2Tx ULTxSwitching Band Pair (Note 2, 3) | Supported 3 dB Boosting in UL Tx Switching (Note 2) | Supported 2Tx-2Tx ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n83A | Rel-17 |  |  |  |  |  |
| SUL_n78A-n80A | Rel-15 |  |  |  |  |  |
| SUL_n78A-n81A | Rel-15 |  |  |  |  |  |
| SUL_n78A-n84A | Rel-15 |  |  |  |  |  |
| SUL_n79A-n83A | Rel-17 |  |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-1, e.g. ‘SUL_n78A-n80A’ indicates SUL operation on NR bands n78 and n80 with UL CA Bandwidth Class A on both bands. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports 1Tx-2Tx or 2Tx-2Tx ULTxSwitching or 3dB boosting. The 3dB boosting is on the maximum output power under the operation state in which 2-port transmission can be supported on carrier2. The boosting shall only be indicated in case the UE supports power class 3 for the band combination.. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. The ULTxSwitching is only tested with 2 UL CCs, so UE is allowed to report ‘N’ by default for SUL configuration with > 2 component carriers. Note 3: See ULTxSwitching(table_index) and 2Tx_ULTxSwitching(table_index) in Note 6 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |  |  |

< End of changes >