3GPP TSG-RAN5 Meeting #109	R5-255966r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0115 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 252 into RX TCs for category M1 and NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The RX reference sensitivity level and in/out-of band blocking requirement for category M1 and NB-IoT NTN band 252 is missing and needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RX reference sensitivity level and In-/Out-of band blocking requirement for category M1 and NB-IoT NTN band 252 have been added into clause 7.3A, 7.3B, 7.6A.2, 7.6A.3 and 7.6B.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 252 RX requirement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A, 7.3B, 7.6A.2, 7.6A.3, 7.6B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | WIC belongs to Rel-19 that will trigger the specific upgrade to Rel-19 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correction to the CR number in coversheet to align with 3GU |  |  |  |  |  |  |  |  |

==============First change==============

7.3A	Reference sensitivity power level for UE category M1

7.3A.1	Test purpose

To verify the category M1 UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area of an e-NodeB.

7.3A.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category M1 that support satellite access operation.

7.3A.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A.3-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |
| 253 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A.3-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |
| 253 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3A.3-1/Table 7.3A.3-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A.3-3.

NOTE:	Table 7.3A.3-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G] (informative).

Table 7.3A.3-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 252 | 61 | FDD and HD-FDD |
| 253 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.3A.1.

==============Next change==============

7.3A.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 with parameters specified in Table 7.3A.5-1 and Table 7.3A.5-2.

Table 7.3A.5-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -101.5 | FDD |
| 253 | -102.0 | FDD |
| 254 | -101.5 | FDD |
| 255 | -102.0 | FDD |
| 256 | -101.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A.5-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.3 | HD-FDD |
| 253 | -102.8 | HD-FDD |
| 254 | -102.4 | HD-FDD |
| 255 | -102.8 | HD-FDD |
| 256 | -102.3 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

==============Next change==============

7.3B	Reference sensitivity power level for UE category NB1 and NB2

7.3B.1	Test purpose

To verify the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.3B.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 of TS 36.101 [7] with received signal level as specified in Table 7.3B.3-1. Requirement in Table 7.3B.3-1 applies for any uplink configuration and with default TX-RX carrier center frequency separation except for cases specified in Table 7.3B.3-2.

Table 7.3B.3-1: Reference sensitivity for category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Table 7.3B.3-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 0.2 MHz | 160.2 MHz, 219.8 MHz |
| 255 | 0.2 MHz | -67.7 MHz, -135.3 MHz |
| 254 | 0.2 MHz | 857.2 MHz, 889.8 MHz |
| 253 | 0.2 MHz | -143.2 MHz, -156.8 MHz |
| 252 | 0.2MHz | 160.2 MHz, 199.8 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 7.3B.

==============Next change==============

7.6A.2	In-band blocking for category M1

7.6A.2.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.6A.2.3	Minimum Conformance Requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.2.3-1 and 7.6A.2.3-2. For operating bands with an unpaired DL part (as noted in Table 5.2A-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.2.3-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.3-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.6A.2.

==============Next change==============

7.6A.2.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.6A.2.5-1 and 7.6A.2.5-2.

Table 7.6A.2.5-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.5-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

==============Next change==============

7.6A.3	Out-of-band blocking for category M1

7.6A.3.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the category M1 UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5A and sub-clause 7.6A.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.6A.3.3	Minimum Conformance Requirements

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5A and subclause 7.6A.2 shall be applied.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.3-1 and 7.6A.3-2.

Table 7.6A.3-1: Out-of-band blocking parameters for category M1 UE

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 1.4 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [7] with PCMAX_L as defined in subclause 6.2.5. |  |  |

Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 253, 2542, 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 2: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

For Table 7.6A.3-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration. For these exceptions the requirements of subclause 7.7A spurious response are applicable.

The normative reference for this requirement is TS 36.102 [11] clause 7.6A.3.

==============Next change==============

7.6A.3.5	Test Requirement

Except for the spurious response frequencies recorded at the final step of test procedure, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.6A.3.5-1 and 7.6A.3.5-2.

For frequency range 1, 2, and 3, the number of spurious response frequencies recorded in the final step of test procedure shall not exceed in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7A Spurious Response are applicable.

Table 7.6A.3.5-1: Out-of-band blocking parameters for category M1 UE

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 1.4 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [7] with PCMAX_L as defined in subclause 6.2.5. |  |  |

Table 7.6A.3.5-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 253, 2542,255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 2: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz |  |  |  |  |  |

==============Next change==============

7.6B.3	Out-of-band blocking for category NB1 and NB2

7.6B.3.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5B and sub-clause 7.6B.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.6B.3.3	Minimum Conformance Requirements

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5B and subclause 7.6B.2 shall be applied.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.3.2 with parameters specified in Table 7.6B.3.3-1.

For Table 7.6B.3.3-1 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B spurious response are applicable.

Table 7.6B.3.3-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 253, 2545, 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Void. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 5: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.6B.3.

==============Next change==============

7.6B.3.5	Test Requirement

Except for the spurious response frequencies recorded at the final step of test procedure, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in clause A.3.2 with parameters specified in Tables 7.6B.3.5-1.

For Table 7.6B.3.5-1 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B spurious response are applicable.

Table 7.6B.3.5-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 253, 2545,255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Void. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 5: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

==============End of change==============