3GPP TSG-RAN5 Meeting #110	R5-260822r1

Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5312 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 10.6.1.2 Testcase- MA PDU Session Establishment UE Establishes PDN Connection over Untrusted Non-3GPP Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Message content of MA PDU session establishment / ATSSS / UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session test case has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message content for test case has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---- Start of change 1 ----

#### 10.6.1.2	UE-requested MA PDU session establishment / ATSSS / UE establishing a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session

10.6.1.2.1	Test Purpose (TP)

(1)

with { UE is registered over 3GPP access to 5GC, 5GMM idle mode over 3GPP access, no coverage of untrusted non-3GPP connected to EPC }

ensure that {

when { UE is configured to request a new MA PDU session }

then { UE sends a PDU Session Establishment Request over 5GC access and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message and in the Extended protocol configuration options IE of the PDU SESSION ESTABLISHMENT REQUEST message to indicate its support of establishing a PDN connection  over untrusted non-3GPP access network as the user plane resource of an MA PDU session }

(2)

with { UE is registered over 3GPP access to 5GC, and an MA PDU session has been established successfully on this access }

ensure that {

when { UE detects a good non-3gpp coverage connected to EPC }

then { UE sends a IKE_AUTH request message including an indication about Attach Type indicating handover Attach and ATSSS_REQUEST Notify payload including PDU session ID of the already established MA PDU session to the ePDG }

10.6.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clause 6.4.1.2, TS 24.193, clause 5.3a.3. Unless otherwise stated these are Rel-18 requirements.

[TS 24.501, clause 6.4.1.2]

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

…

If the UE is registered to a network which supports ATSSS, the UE may request to establish an MA PDU session. If the UE requests to establish an MA PDU session, the UE shall set the request type to "MA PDU request" in the UL NAS TRANSPORT message. If the UE is registered to a network which does not support ATSSS, the UE shall not request to establish an MA PDU session.

…

If the UE requests to establish a new MA PDU session or if the UE requests to establish a new PDU session and the UE allows the network to upgrade the requested PDU session to an MA PDU session:

a)	if the UE supports ATSSS Low-Layer functionality with any steering mode (i.e., any steering mode allowed for ATSSS Low-Layer functionality) as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

NOTE 8:	The ATSSS Low-Layer functionality cannot be used together with the redundant steering mode. When the UE indicates that it is capable of supporting the ATSSS Low-Layer functionality with any steering mode, it implies that the UE supports the ATSSS Low-Layer functionality with any steering mode except the redundant steering mode.

b)	if the UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

c)	if the UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS-LL functionality) as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

d)	if a performance measurement function in the UE can perform access performance measurements using the QoS flow of the non-default QoS rule as specified in subclause 5.32.5 of 3GPP TS 23.501 [8], the UE shall set the APMQF bit to "Access performance measurements per QoS flow supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

e)	if the UE supports MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

f)	if the UE supports MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS-LL functionality) as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message;

g)	if the UE supports MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message; and

h)	if the UE supports MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS-LL functionality) as specified in subclause 5.32.6 of 3GPP TS 23.501 [8], the UE shall set the ATSSS-ST bits to "MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported" in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message.

…

If the UE requests to establish a new MA PDU session and the UE supports to establish a PDN connection as the user plane resource of an MA PDU session, the UE shall include the ATSSS request parameter in the Extended protocol configuration options IE of the PDU SESSION ESTABLISHMENT REQUEST message.

…

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID of the PDU session being established, being handed over, being transferred, or been established as an MA PDU session;

c)	if the request type is set to:

1)	"initial request" or "MA PDU request" and the UE determined to establish a new PDU session or an MA PDU session based on either a URSP rule including one or more S-NSSAIs in the URSP (see subclause 6.2.9) or UE local configuration, according to subclause 4.2.2 of 3GPP TS 24.526 [19]:

i)	if the UE is in the HPLMN or the subscribed SNPN, an S-NSSAI in the allowed NSSAI which corresponds to one of the S-NSSAI(s) in the matching URSP rule, if any, or else to the S-NSSAI(s) in the UE local configuration or in the default URSP rule, if any, according to the conditions given in subclause 4.2.2 of 3GPP TS 24.526 [19];

ii)	if the UE is in a non-subscribed SNPN, the UE determined according to the conditions given in subclause 4.2.2 of 3GPP TS 24.526 [19] to establish a new PDU session or an MA PDU session based on a URSP rule including one or more S-NSSAIs, and the URSP rule is a part of a non-subscribed SNPN signalled URSP (see 3GPP TS 24.526 [19]):

A)	an S-NSSAI in the allowed NSSAI, which is one of the S-NSSAI(s) in the URSP rule; and

B)	a mapped S-NSSAI associated with the S-NSSAI in A); or

iii)	otherwise:

A)	one of the mapped S-NSSAI(s) which is equal to one of the S-NSSAI(s) in the matching URSP rule, if any, or else to the S-NSSAI(s) in the UE local configuration or in the default URSP rule, if any, according to the conditions given in subclause 4.2.2 of 3GPP TS 24.526 [19]; and

B)	the S-NSSAI in the allowed NSSAI associated with the S-NSSAI in A); or

…

d)	if the request type is set to:

1)	"initial request" or "MA PDU request" and the UE determined to establish a new PDU session or an MA PDU session based on either a URSP rule including one or more DNNs in the URSP (see subclause 6.2.9) or UE local configuration, according to subclause 4.2.2 of 3GPP TS 24.526 [19], a DNN which corresponds to one of the DNN(s) in the matching URSP rule, if any, or else to the DNN(s) in the UE local configuration or in the default URSP rule, if any, according to the conditions given in subclause 4.2.2 of 3GPP TS 24.526 [19];

…

e)	the request type which is set to:

…

5)	"MA PDU request", if:

i)	the UE requests to establish an MA PDU session;

ii)	the UE requests to establish user plane resources over other access of an MA PDU session established over one access only; or

iii)	the UE performs inter-system change from S1 mode to N1 mode according to subclause 4.8.2.3.1 and requests transfer of a PDN connection which is a user plane resource of an MA PDU session; and

f)	the old PDU session ID which is the PDU session ID of the existing PDU session, if the UE initiates the UE-requested PDU session establishment procedure upon receiving the PDU SESSION MODIFICATION COMMAND messages with the 5GSM cause IE set to #39 "reactivation requested";

NOTE 13:	If the PDU SESSION MODIFICATION COMMAND message included alternative S-NSSAI, the UE includes alternative S-NSSAI in the PDU SESSION ESTABLISHMENT REQUEST message for PDU session re-establishment.

using the NAS transport procedure as specified in subclause 5.4.5, and the UE shall start timer T3580 (see example in figure 6.4.1.2.1).

For bullet c) 1), if the matching URSP rule does not have an associated S-NSSAI, or if the UE does not have any matching URSP rule and there is no S-NSSAI in the UE local configuration or in the default URSP rule, the UE shall not provide any S-NSSAI in a PDU session establishment procedure.

…

For bullet d) 1),

-	If the matching non-default URSP rule does not have an associated DNN, then the UE shall not provide any DNN in a PDU session establishment procedure;

-	If the UE does not have any matching non-default URSP rule, the UE requests a connectivity that requires PAP/CHAP and the UE is configured with the default DNN for the S-NSSAI in the UE local configuration corresponding to the request, then the UE should provide such DNN in a PDU session establishment procedure;

-	If the UE does not have any matching non-default URSP rule, the UE requests a connectivity that requires PAP/CHAP, the UE is not configured with the default DNN for the S-NSSAI in the UE local configuration corresponding to the request, and the application provides the DNN, then the UE shall use such DNN in a PDU session establishment procedure;

-	If the UE does not have any matching non-default URSP rule, the UE requests a connectivity that does not require PAP/CHAP, the UE is not configured with the DNN for the S-NSSAI in the UE local configuration corresponding to the request, and the application provides the DNN, then the UE shall use such DNN in a PDU session establishment procedure;

-	If the UE does not have any matching non-default URSP rule, the UE requests a connectivity that requires PAP/CHAP, the UE is not configured with the default DNN for the S-NSSAI in the UE local configuration corresponding to the request, the application does not provide the DNN and there is no DNN in the default URSP rule, then the UE shall not provide any DNN in a PDU session establishment procedure; or

-	If the UE does not have any matching non-default URSP rule, the UE requests a connectivity that does not require PAP/CHAP, the UE is not configured with the DNN for the S-NSSAI in the UE local configuration corresponding to the request, the application does not provide the DNN and there is no DNN in the default URSP rule, then the UE shall not provide any DNN in a PDU session establishment procedure.

…

[TS 24.193, clause 5.3a.3]

In order to establish a PDN connection over untrusted non-3GPP access network as a user-plane resource of an already established MA PDU session, the UE shall initiate the IPsec tunnel establishment procedure using the IKEv2 protocol according to 3GPP TS 24.302 [17].

In the IKE_AUTH request message to the ePDG:

a)	the CFG_REQUEST configuration payload shall include the INTERNAL_IP4_ADDRESS attribute, the INTERNAL_IP6_ADDRESS attribute or both, indicating the PDU session address of the established MA PDU session;

b)	the IDr payload shall be set to the APN mapped from the DNN of the established MA PDU session;

c)	the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload shall be set to the PDU session identity of the establish MA PDU session;

d)	the S-NSSAI Value field of the N1_MODE INFORMATION Notify payload shall be set to the S-NSSAI of the established MA PDU session; and

e)	the ATSSS_REQUEST Notify payload shall be included as specified in clause 8.2.9.20 of 3GPP TS 24.302 [17].

Upon receipt of the IKE_AUTH response as specified in clause 7.2.2.1 of 3GPP TS 24.302 [17], the UE shall consider that the PDN connection over untrusted non-3GPP access network is established as a user-plane resource of the MA PDU session.

Upon receipt of an IKE_AUTH response message including a Notify payload with a Private Notify Message Type as specified in clause 7.2.2.2 of 3GPP TS 24.302 [17], the UE shall consider that the PDN connection over non-3GPP access network is not established as a user-plane resource of the MA PDU session.

10.6.1.2.3	Test description

10.6.1.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to TS 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	NR Cell 1 is set to '' Serving cell'' and WLAN Cell is set to “Off Cell”.

UE:

-	UE is registered over 3GPP access to the 5GC.

Preamble:

-	The UE is brought to state 3N-A on NR Cell 1 with UE test loop mode B active according to TS 38.508-1 [4].

10.6.1.2.3.2	Test procedure sequence

Table 10.6.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request to establish an MA PDU session by MMI or AT command on NR Cell 1. | - | - | - | - |
| 2 | Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message and sets the ATSSS-ST bits in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message according to the support of establishing a PDN connection over untrusted non-3GPP access network on NR Cell 1? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 3 | The SS transmits an PDU SESSION ESTABLISHMENT ACCEPT on NR Cell 1 | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 4 | WLAN Cell 27 is set to “Serving cell”. | - | - | - | - |
| 5 | Check: Does the UE provide following information: INTERNAL_IP4_ADDRESS attribute, INTERNAL_IP6_ADDRESS attribute or both, PDU Session ID field, ATSSS_REQUEST Notify while establishing an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1 | - | - | 2 | P |
| 6 | The SS transmits one IP Packet on MA PDU session established on WLAN Cell 27. | <-- | IP packet | - | - |
| 7 | Check: Does the UE loop back the IP Packet? | --> | - | 2 | P |
| Note 1: The request to establish a MA PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

10.6.1.2.3.3	Specific message contents

-	TBD

Table 10.6.1.2.3.3-1: PDU SESSION ESTABLISHMENT REQUEST (step 2, Table 10.6.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GSM capability |  |  |  |
| All octets with the exception of octet 3, bits 4 to 7 | Any allowed value |  |  |
| ATSSS-ST(octet 3, bits 4 to 7) | Any value between ‘0001’B, ‘0010’B, ‘0011’B | Supported ATSSS steering functionality and steering mode set by UE |  |

Table 10.6.1.2.3.3-2: UL NAS Transport (step 2, Table 10.6.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘110’B | MA PDU request | INITIAL_PDU_REQUEST |

Table 10.6.1.2.3.3-3: PDU SESSION ESTABLISHMENT ACCEPT (step 3, Table 10.6.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ATSSS container |  |  |  |  |
| Length of ATSSS container contents |  | ‘00000110’B |  |  |
| ATSSS rule ID |  | ‘00000001’B | Remains subject to network configuration. |  |
| ATSSS rule operation |  | ‘00000001’B | Remains subject to network configuration. |  |
| Precedence value of ATSSS rule |  | ‘00001010’B | Remains subject to network configuration. |  |
| Traffic descriptor |  | ‘00000001’B | Remains subject to network configuration. |  |
| Steering functionality |  | ‘00000011’B | Remains subject to network configuration. |  |
| Steering mode |  | ‘00000001’B | Remains subject to network configuration. |  |
| Steering mode information |  | ‘00000100’B | Remains subject to network configuration. |  |

Table 10.6.1.2.3.3-4: IKE_AUTH Request (step 5, Table 10.6.1.2.3.2-1)

| Derivation path: TS 36.508 [18], Table 4.7G-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE_AUTH |  | IKE_AUTH request header. |  |
| Initiator SPI | Not checked |  |  |
| Responder SPI | Not checked |  |  |
| Next Payload | Present but content not checked | UE may send other payloads preceding CP or Notify payloads, such as SK, AUTH, etc. payloads, which are not checked. |  |
| Major Version | Not checked |  |  |
| Minor Version | Not checked |  |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Flags | Not checked |  |  |
| Message ID | Not checked |  |  |
| Length | Not checked |  |  |
| CP |  | CFG_REQUEST configuration payload |  |
| Next Payload | Present but content not checked | Order of the payloads is not strict. |  |
| Critical | Not checked |  |  |
| Payload Length | Not checked |  |  |
| CFG Type | ‘00000001’B | CFG_REQUEST |  |
| Attributes |  |  |  |
| INTERNAL_IP4_ADDR ESS | Same as address configured for NR Cell1 | UE MA PDU session IPv4 address | If IPv4 |
| INTERNAL_IP6_ADDRESS | Same as address configured for NR Cell1 | UE MA PDU session IPv6 address | If IPv6 |
| IDr |  | Identification – Responder payload |  |
| Next Payload | Present but content not checked | Order of the payloads is not strict. |  |
| Critical | Not checked |  |  |
| Payload Length | Not checked |  |  |
| ID Type | ‘00000010’B | ID_FQDN |  |
| Identification Data | APN/DNN set during MA PDU session creation |  |  |
| N |  | N1_MODE_CAPABILITY notify payload |  |
| Next Payload | Present but content not checked | Order of the payloads is not strict. |  |
| Critical | Not checked |  |  |
| Payload Length | Not checked |  |  |
| Protocol ID | Not checked | IKE |  |
| SPI Size | Not checked | No SPI |  |
| Notify Message Type | ‘1100011110100000’B | N1_MODE_CAPABILITY |  |
| PDU Session ID | The value used in the PDU Session request message on NR Cell 1 | Identifies MA PDU session |  |
| N |  | N1_MODE_INFORMATION notify payload | If S-NSSAI |
| Next Payload | Present but content not checked |  |  |
| Critical | Not checked |  |  |
| Payload Length | Not checked |  |  |
| Protocol ID | Not checked |  |  |
| SPI Size | Not checked |  |  |
| Notify Message Type | ‘1100011110100001’B | N1_MODE_INFORMATION |  |
| S-NSSAI Value | S-NSSAI value given in PDU session request message on NR Cell 1 |  |  |
| N |  | ATSSS_REQUEST Notify payload |  |
| Next Payload | Present but content not checked | Order of the payloads is not strict. |  |
| Critical | Not checked |  |  |
| Payload Length | Not checked |  |  |
| Protocol ID | Not checked |  |  |
| SPI Size | Not checked |  |  |
| Notify Message Type | ‘1100 0111 1010 0100’B | ATSSS_REQUEST |  |
| ATSSS Container |  | ATSSS mode indicator |  |
| ATSSS Steering Functionality | ‘00000011’B |  |  |
| ATSSS Steering Mode | ‘00000001’B |  |  |

---- End of change 1 ----