3GPP TSG-SA2 Meeting #171	 S2-2509466

Wuhan, China, October 13-17 2025	(revision of S2-2509420)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6341 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on exposure of energy consumption threshold by EIF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The EIF can be provided with a threshold-based reporting when periodic reporting is requested. It is not clear what happens if the threshold is not crossed in a period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that if the consumption threshold is not crossed, the EIF reports no consumption information for the period. Text is also made a bit clearer for the reader on what “future” means. NF is replaced with AF/NEF as these are the expected consumers in rel-19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification as one could also conceive a combined periodic and threshold reporting where the reporting happens in event-based manner in the period in addition to the regular reporting at the end of period. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF subscribes the EIF for Energy consumption information of required granularities (UE, S-NSSAI, PDU session and/or Service Data Flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For S-NSSAI of the UE level exposure, the consumer NF provides UE ID (SUPI/GPSI), S-NSSAI.

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For Service Data Flow level exposure (e.g. per UE per application), the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and application information, e.g. Application Identifier, or Flow description(s).

The consumer AF/NEF may also subscribe the above information exposure with providing reporting period, or time window. An AF/NEF may also subscribe the Energy consumption information exposure of required granularity and additionally provide a reporting threshold for the required granularity and the reporting period . If threshold-based reporting is configured for the reporting period, the consumer AF/NEF does not receive consumption information other than whether the threshold on energy consumption has been crossed in a reporting period. The information that the threshold has been crossed is sent to the AF/NEF as soon the threshold crossing is detected in a reporting period.

NOTE 1:	For the threshold-based reporting, the EIF sends the Energy consumption information of the required granularity to the NEF/AF once in the reporting period when the Energy consumption information of the required granularity matches or exceeds the threshold during the reporting period. If the threshold is not crossed in the reporting period, the EIF does not send any consumption information for the reporting period unless the periodic energy consumption reporting is separately requested without specifying the threshold for the reporting period.

NOTE 2:	In this Release of the specification, requesting energy-related information by the consumer is supported only for reporting periods and time windows occurring after the subscription time.

End of changes