3GPP TSG-SA WG2 Meeting #171	S2-2509423

# Wuhan, China, October 13 – 17, 2025 												   (revision or 9371 of 8750)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6446 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Energy Consumption Estimation Formula |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current energy consumption formula documented in Annex T of TS 23.501, estimates the energy consumption in finer granularities using total energy consumption of UPF and RAN node and data volume of the request granularity level. It leads to different estimations of energy consumption for the same amount of transmitted data because of dependency on the total load of the UPF/RAN node (even if the UE location and radio condition remain unchanged). This behaviour of the EIF may cause question/confusion for the consumer. This CR adds a note to explicitly clarify that this behaviour is the ingredient characteristic of the estimation formula (not an implementation/deployment issue/bug). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a NOTE to clarify that the EC estimation formula may provide different estimations for the same volume of transmitted data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of EIF behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | T.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

# T.2	Calculation of Energy Consumption parameters

The energy consumed at a UPF (EUPF) or a gNB (EgNB) is known over a time interval T. And the EIF is aware of the data volume (DV) consumed at a UPF by a UE (DVUE,UPF), UE S-NSSAI (DVS-NSSAI,UPF), UE PDU session (DVSession,UPF) and UE QoS flow (DVFlow,UPF) over the time interval T. The EIF is aware of the overall data volume at a gNB (DVgNB), as well as at a UPF  (DVUPF) which serving the UE. Then based on the below example formulas, the required granularities Energy consumption information can be obtained.

To derive the of energy consumed at a gNB over a time interval T by a UE, S-NSSAI, PDU session, QoS flow, the formulas are:

EUE,gNB=EgNBDVUE,gNBDVgNB

ES-NSSAI,gNB=EgNBDVS-NSSAI,gNBDVgNB

ESession,gNB=EgNBDVSession,gNBDVgNB

EFlow,gNB=EgNBDVFlow,gNBDVgNB

To derive the energy consumed at a UPF over a time interval T by a UE, S-NSSAI, PDU session, QoS flow the formulas are:

EUE,UPF=EUPFDVUE,UPFDVUPF

ES-NSSAI,UPF=EUPFDVS-NSSAI,UPFDVUPF

ESession, UPF=EUPFDVSession,UPFDVUPF

EFlow,UPF=EUPFDVFlow,UPFDVUPF

It should be noted that the values EgNBDVgNB and  EUPFDVUPF are coefficients that are common across all granularities, within the time interval T, for the respective network entities and are values that related to the energy consumption per bit of the gNB, and, respectively, the UPF.

To derive the energy consumed in the network in the time interval T by a UE (EUE), S-NSSAI (ES-NSSAI), PDU Session (ESession), QoS flow (EFlow), the formula is:

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi

where gNBUET are all gNBs used by the UE in an interval T, UPFUET are all UPFs used by the UE in an interval T.

ES-NSSAI=gNBi∈gNBS-NSSAITES-NSSAI, gNBi+ UPFi∈UPFS-NSSAITES-NSSAI, UPFi

where gNBS-NSSAIT are all gNBs used by the S-NSSAI in an interval T, UPFS-NSSAIT are all UPFs used by the S-NSSAI in an interval T.

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi

where gNBSessionT are all gNBs used by the PDU Session in an interval T, UPFSessionT are all UPFs used by the PDU Session in an interval T.

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

where gNBFlowT are all gNBs used by the QoS Flow in an interval T, UPFFlowT are all UPFs used by the QoS Flow in an interval T.

EUE can also be obtained as the sum of ESession for the given UE:

EUE=Sessioni∈SessionUEESessioni

NOTE: 		The energy consumption calculation formulas are sensitive to DVUPF and DVgNB. Therefore, for the same data volume DVUE, DVS-NSSAI, DVsession, and DVFlow in two time windows, the calculated values of EUE, ES-NSSAI, Esession, and EFlow may be different.

********** End of Changes **********