3GPP TSG SA WG 1 Meeting #113 	S1-261047

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.14-3 Mission critical services

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254410. This table was discussed during SA1 #112. Yellow highlight refers to CPRs still under discussion, green highlight refers to consensus achieved during discussion.

Based on the outcome of discussion in SA1 #112 Ad hoc meeting, the following wording is changed:

User consent – subscriber permission

Based on operator policy – subject to operator’s policy

This pCR proposes to update Table 14.1.14-3 (Mission critical services) in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change (All New CPRs) * * * *

Table 14.1.14-3 – Mission critical services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.14-3-1 | Subject to regulatory requirements and operator’s policy, the 6G system shall support MCX service continuity during UE mobility across PLMNs, as well as between PLMN and PSBN, (e.g. based on agreed QoS, priority and/orpre-emption settings). NOTE 1: PSBN is a network generally owned and operated by a public safety network operator and specially designed for public safety users (e.g. police, ambulance, fire service), having their own PLMN identity. | PR 11.14.6-1 |  |
| CPR 14.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access networks (e.g. terrestrial access and satellite access), and/or when MCX service switches between on-network and off-network operations with or without UE relay. | PR 11.14.6-2 |  |
| Alternative CPR 14.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switches between on-network and off-network operations, with or without UE relay. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are capable of maintaining connectivity in infrastructure-less environments. |  | Mobility should be a separate requirement |
| CPR 14.1.14-3-3 | The 6G system shall enable an MCX UE to determine its position using multiple positioning methods, including network-assisted GNSS via satellite access, and to convey its position information to the MCX server, triggered by an event, or periodically. | PR 11.14.6-3 | Duplicated in ubiquitous positioning 1112 CATT suggested to keep it here. |
| CPR 14.1.14-3-4 | The 6G system shall enable the MCPTT Service to interwork with non-3GPP PTT Systems that are compliant with the PDT standards, in alignment with P25 and TETRA. | PR 11.14.6-4 |  |
| CPR 14.1.14-3-5 | The 6G system shall support interworking between the MCData SDS and PDT Short Data Service, in alignment with TETRA. | PR 11.14.6-5 |  |