3GPP TSG-SA WG1 Meeting #113 	draft S1-261108

9-13 February 2026, Goa, India	(revision of S1-261116, S1-260113)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.10-1 (ISAC) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.4

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This pCR provides the 14.1.10-1 for continued consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-260113 was the final end of discussion status during SA1 #112 Ad Hoc-2. It is provided here for continued discussions.

- Green indicates there was consensus in SA1 #112 to include the CPR for inclusion into the TR.

- Yellow indicates that there the CPR was discussed, and some additional work is needed.

- Red indicates that this proposal was discussed, and it was decided to not pursue.

NOTE: below the proposed consolidation table, is the table of the removed CPRs (with reasons). It is provided here for information & convenience.

From S1-250113, original PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Moved PR 7.5.6-1 from Table 14.1.5-3: Data Collection and Consumption

- Removed PR 7.12.6-2 as it is in Table 14.1.2-1: Security and Privacy and will be addressed there.

- Added proposed PR (Interdigital pCR S1-261036 change 2/S1-261118) – pending agreement on EN removal.

- Corrected PR # that was agreed in CPR 14.1.10-1-3 (from PR 7.5.6-3 to PR 7.7.6-1).

- Added Ericsson update from reflector email on 3 Feb.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.10-1: ISAC

| CPR # | Consolidated Potential Requirement | Original PR # |  | Comment |
| --- | --- | --- | --- | --- |
| CPR 14.1.10-1-1 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network should support suitable means to collect non-3GPP sensing data from authorized third party. | PR 7.16.6-1 |  | Non-3GPP sensing data collection Considering the sensing data collection is specific, it is suggested to remain it in 14.1.10 QC: it looks like the only difference is “3rd party” and non-3GPP sensors in 5G requirements. ZTE sees difference. From 22.137: Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network. E/// agrees with Lola. Gordon sees slight difference. Erik sees broader scope with this CPR. |
| Alt CPR 14.1.10-1-3 (Futurewei) | Futurewei proposal Subject to operator policies, the 6G Network shall provide mechanisms for configuring a sensing operation including sensing transmitter/sensing receiver with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). Note: the sensing transmitter/Sensing receiver is defined in TS 22.137. (Potentially to include the configuration of third party) Huawei Proposal Subject to operator policies, the 6G Network shall provide mechanisms for configuring sensing transmitter/sensing receiver with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). Note: the sensing transmitter/Sensing receiver is defined in TS 22.137. | PR 7.14.6-1 |  | Sensing modes configuration [BMWE: In this CPR, the 6G Network shall provide mechanisms for configuring a sensing operation etc. Along similar lines, we are not in support to change from 6G Network to 6G System to broaden the CPR and propose to keep to the original CPR. Interdigital prefers FW version. [Huawei: the wording is modified slightly] [Ericsson]: Huawei proposal better, still I cannot find any new requirement in this compared to 22.137. Which sensing modes to be used is up to later stages and not SA1. Xiaomi2: since multistatic is a sensing configuration related to a sensing operation and not a transmitter/receiver configuration persay some clarification is necessary, or keep the original wording |
| CPR 14.1.10-1-4 | Subject to operator’s policy and regulatory requirements, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. Ericsson proposal Subject to operator’s policy and regulatory requirements, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given the requested sensing characteristics per target, which include localization accuracy and sensing service latency [11]. Futurewei proposal is to delete this CPR. InterDigital proposal Subject to operator’s policy and regulatory requirements, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a configured sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.5.6-3 This CPR needs to be reconsidered without merging w/CPR 7.7.6-1 as it was already approved as CPR 14.1.10-1-3. A “clean” (original) PR 7.5.6-3 was added below (as CPR 14.1.10-1-New) for consolidation discussions. |  | Sensing Service Capacity Multiple targets [ZTE]: Sensing QoS: capacity/density/latency upper-bound [Ericsson: Sensing QoS is not the correct term to use and can easily be mixed up with QoS for communication services] [Ericsson]: The last part of the sentence i.e. sending sensing result to a UE is not covered by PR 7.5.6-3 or PR 7.7.6-1 and should be removed Lets continue the discussion and try to clarify the novelty between the KPI table and the wording. Futurewei: Theis QoS is covered by KPI table, not needed in this requirement. [ZTE: "sensing system capacity" is not in the KPI table while sensing target density is in. And based on the PRs related, the senisng system capacity and sensing target density are requested by the 3rd party for a given sensing service. So, they can't be removed.] [InterDigital] proposed rewording, similar to another row] |
| Existing CPR 14.1.10-1-3 | CPR 14.1.10-1-3: Subject to operator’s policy and regulatory requirements, the 6G system shall support mechanisms to provide sensing service with a given sensing target density. | NOTE: PR 7.7.6-1 is AGREED as CPR 14.1.10-1-3 |  | Provided for info |
| CPR 14.1.10-1-New | The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. | PR 7.5.6-3 |  | Covered by Existing CPR 14.1.10-1-3 Correct PR that was mistakenly marked as approved as CPR 14.1.10-1-3. This is the orig text for the PR for consolidation discussions). |
| CPR 14.1.10-1-new | Huawei’s proposal Subject to operator’s policy and regulatory requirements, the 6G system shall be able to provide a means to consider a latency upper-bound, requested by trusted third party, when providing sensing results to UEs (e.g. AMRs). LGE Proposal The 6G network shall be able to provide a means to ensure an upper-bound of delivery time window, requested by trusted third party, when providing sensing results to nearby UEs (e.g. AMRs) about environment situation. NOTE 2: The delivery time window depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.8.6-3 |  | [Huawei]: split PR 7.8.6.3 from 14.1.10.-1-4 Regarding PR 7.8.6-3, the wording is suggested to be modified slightly. Needs to be revised as part of SA1 112 Ad Hoc-e Ericsson] This is a request for sensing with a “max sensing service latency”. The result is sent to a application on a UE.This potential requirement can be covered by the requirement we have in 22.137. (and 22.261, i.e. QoS to a Ue) |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data. NOTE: The processing of sensing data is compliance with applicable regulations, and other pre-defined service criteria. | PR 7.12.6-2 |  | Provided for info Move to Table 7.1.2-1 Security & Privacy? |
| Alt CPR 14.1.10-1-6 (Huawei) | Based on operator’s policy and regulatory requirements, the 6G network in the area of the disaster shall provide secure mechanisms to collect sensing results with a requested level of accuracy that can be used to generate real time maps. | PR 7.2.6-1 |  | Accuracy Target Density Latency 3rd Party Support [Futurewei: is this already covered by KPI?] [Huawei]: What’s the corresponding KPI, refreshing rate? Ericsson: Agree this should be covered in KPI table, if not it needs to be rewritten as a way for the 3rd party to request this sensing target density. It will always be up to the network to decide if it can fulfill the request or not.] |
| Alt CPR 14.1.10-1-7 (Philips Int B.V.) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s). | PR 7.10.6-1 |  | Prediction [Huawei]: Confusing. PR 7.10.6-1 appears serval times such as 14.1.10-1-7 , 14.1.10-1-8 and 14.1.10-1-17. [Philips Int BV: The sensing target could be a human, or a car used by the subsciber of the sensing service or a related user (e.g. family member), so the subscriber should give permission to allow prediction of the location or speed.] |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s), while maintaining the privacy of the sensing target(s). | PR 7.10.6-1 |  | Provided for info |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-2 |  | Provided for info |
| Alt CPR 14.1.10-1-8 | Futurewei Proposal Subject to operator’s policy and regulatory requirements, the 6G network shall provide a target prediction capability to provide predicted target characteristics (e.g. size, shape, location, velocity) at certain future time, while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. Huawei Proposal Subject to operator’s policy and regulatory requirements, the 6G core network shall provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. ZTE Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. Philips Int BV Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide a target prediction capability to derive predicted target characteristics (e.g. size, shape, location, velocity), while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-2 |  | Prediction Exposure Privacy Third Party support [Huawei}: Suggest to split PR 7.10.6-1 and PR 7.10.6-2 since former is about predictive capability and the latter is about exposure. ZTE: As discussed in Dallas meeting, it was suggested to seperate PR7.10.6-1(prediction) and PR7.10.6-2(exposure). So, suggestion revision. ZTE2: It is suggested to separate the PR7.10.6-1 and PR7.10.6-2, because that PR7.10.6-1 is for prediction provision, PR7.10.6-2 is for exposure. ZTE:They are separated and already covered by CPR-14.1.10-1-7 and 14.1.10-1-8. [Philips Int B.V: - - Similar to 14.1.10-1-8, this requirement is subject to subscriber permission, especially when it comes to exposing the sensing results. Not clear if this is covered by 14.1.10-1-17 which is just below. Is that an alternative proposal for 14.1.10-1.8 or separate proposal. It is confusing that that one has a different number than 14.1.10-1-8.] |
| Alt CPR 14.1.10-1-9 | ZTE Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network shall support to derive sensing result based on stored sensing data and ensure that only authorised entities are able to access sensing results. InterDigital Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored sensing data and results during storage and processing. | PR 7.11.6-1 PR 7.12.6-1 |  | Sensing Data Storage Usage & Security [ZTE]: what is the meaning of "the authorized entities"? And in my understanding, only sensing ressult could be exposed. [InterDigital] re-wording to include 7.12.6-2] |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network shall support the use of stored sensing data to provide a sensing service. | PR 7.11.6-1 |  | Provided for info |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall ensure that only authorised entities are able to access the stored collected sensing data and results. | PR 7.12.6-1 |  | Provided for info |
| Alt CPR 14.1.10-1-10 | InterDigital Proposal The 6G system shall be able to apply policies for prioritising communication, sensing and positioning services relative to each other. Ericsson Proposal The 6G system shall be able to prioritize communication service, sensing service and positioning service when request by the operator. The purpose is to allow users of e.g. UAVs, cars, etc. to get awareness of the target area. Ericsson Proposal 2 (Replace with?) The 6G system shall be able to jointly prioritize, communication service, sensing service and positioning service for an application on a UE when used in combination. | PR 7.19.6-2 |  | Prioritization Futurewei: Not clear this for per user or whole system level prioritization. If it’s per system, this is more implementation.] [Huawei]: Intended to say communication should be prioritized when all three are all in use in the network? Unclear which will be prioritized. ZTE: more clarification is needed. What will be prioritized? What does the network assisted smart transportation mean? [InterDigital] wording proposal for clarification Xiaomi2: the original PR (seems?) to confirm the 3 services should be jointly prioritized (over any other active service) for this network assisted smart transportation Ericsson: As indicated by Xiaomi the intention is to prioritize the 3 services together (sensing, positioning , communication) to a application on a UE |
| Alt CPR 14.1.10-1-11 | Huawei Proposal Subject to operator’s policy and subscriber permission, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to a UE (AMR). NOTE 1: the exposed sensing results can be used for prediction in a given sensing area. Philips Int BV Proposal Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest at the request of a trusted third party. LGE Proposal Subject to operator’s policy and subscriber permission, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to nearby UEs that are used for prediction in a given sensing area of interest at a particular time of interest at the request of a trusted third party. Xiaomi Proposal (based on LGE proposal) Subject to operator’s policy and subscriber permission, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to nearby UEs that are used for prediction of detected objects in a given sensing area of interest at a particular time of interest at the request of a trusted third party. NOTE 1: the detected object is detected by one or more active sensing UEs for the same sensing service. NOTE 2: nearby UEs are UEs registered for the sensing service with the third party in the sensing target area. | PR 7.8.6-1 |  | Exposure Third party support Delivery synchronization [Ericsson: First of all, should it not be “an application on a UE” ? the AMR is not a UE, there is a UE on the AMR. If my understanding is correct this is about exposure sensing result by a request from a trusted third party. The second part of the sentence is not needed, it is about what the application should use the sensing result? As a reminder we have two requirements in 22.137 about exposure: Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met. NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area. Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.] [Huawei]: Huawei: the wording is modified slightly 6G network -> 6G core network Is UE also included in this PR? Is subscriber permission needed? [LGE:proposed changes considering the comments received.] [Philips Int BV: I am ok with your proposal. My main point was to improve the readability. Your proposal address that.] Mario: This CPR really made various modifications from initial PR wording to now and got a bit clearer. And as discussed in the past, as BMWE, we have a concern with respect to some security consideration in using “6G system” for authorization/exposure and our preference would be to avoid it. So we propose a small change here from 6G System to 6G Network. [Xiaomi: Then this CPR requires the 3rd party to indicate to the 6G system sensing service to activate or deactivate the exposure of the sensing results to nearby UEs for predictive purposes? And this predictive use by nearby UEs is specific, but can this be guaranteed by the 3rd party/6G system? Regarding “nearby UEs” then this means UEs in the sensing target area (not necessarily UEs engaged in the sensing service?) or another area? Is it a feature of this exposure indication (by the 3rd party) to indicate which UEs these are, or something for the 6G system to work out, based on the “sensing target area”? How are these UEs authorised to be able to receive these results, are they a subscriber to the sensing service at the third party? (maybe this addresses the concern raised by Mario in the other thread? [SA1#112-ad-hoc], [S1-260016], [ Table 14.1.10-1・ ISAC]) I’m assuming here that the UE supports an app to perform the prediction locally (i.e. acts on its own volition) and based on the predicted output slows the AMR down or takes evasive action and alters course/stops/sounds an alarm etc. Presumably this is the prediction of detected objects e.g. humans, other UEs/AMRs. If there is no detected object, is there a need by the nearby UEs to perform prediction? Or is it still preferable to send as a nearby UE can confirm prediction that no object is detected? I assume it is useful regardless of whether or not an object is detected. But is it also intended to report all object types and label/classify them in the sensing results so that the nearby UE prediction can make a reasonable prediction regarding the object future location/movement? - Maybe covered by QC CPR elsewhere but could be useful to confirm? [Xiaomi – based on the CPR provided by LGE in revision of table r4. Exposure of sensing results are based on 3rd party to UEs in the sensing target area, to enable the nearby UEs to predict detected objects in order to carryout further actions based on the predicted result] |
| CPR 14.1.10-1-12 | The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer. | PR 7.14.6-2 |  | Sensing result exposure sync.with other traffic [Huawei]: What is synchronised manner? How to synchronise a sensing result of sensing target and the associated other traffic data? [Ericsson] Agree with Huawei comment |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to expose sensing results to UE which is authorised by the network operator to use the sensing results for a specific service (e.g. communication service). NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimise communication service | PR 7.20.6-1 |  | Provided for info |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall be able to provide a mechanism for a network operator to authorise a UE or an application on the UE to obtain sensing result as a consumer for a specific service. | PR 7.21.6-1 |  | Provided for info |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide sensing results to a UE for a specific service, where the UE is authorised by mobile network operator providing sensing service. | PR 7.5.6-4 |  | Provided for info |
| Alt CPR 14.1.10-1-13 | China Unicom proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to authorise UE and provide sensing results to an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. Qualcomm proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to authorise UE and expose sensing results (using suitable APIs) to an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. ZTE Proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide sensing results to a UE or an application on a UE, while the UE is authorised by the network operator to use the sensing result for a specific service (e.g. optimise communication service, provide sensing service by the UE, etc.). Telefonica proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to expose sensing results to an application on the UE for a specific service, where the provision of such sensing results is authorised at subscription or service level by the mobile network operator.. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 |  | Exposure to UE [Ericsson: A question for clarification, what does authorize a UE means in this context? Is it an application on the UE that should be authorized to receive sensing result? The note is not needed and seems not fully aligned with the requirement. In the note it is the UE (modem ) that is using sensing result to optimize communication whereas the requirement is exposure sensing result to an application on the UE.] [China Unicom]: I struggled to understand the difference between these two sentences (-13 and -14), but I couldn't find, so I suggest retaining only one of them. If it's the first one, some modifications could be made [QC]: the difference between the two requirements is the following: 14.1.10-1-13 targets network exposure requirement (e.g. suitable APIs) for a UE application. 14.1.10-1-14 is targeting the requirements for network to generate/derive and offer the sensing results to the UE. [QC: if keeping -13 and -14] [Huawei: Huawei comments: Suggest to retain CPR-13 and CPR-14 by highlighting the differences between them. The difference of CPR-13 and CPR-14 is about who will receive/consume the sensing result and how to provide, such as using suitable APIs. We can use Qualcomm’s revision for the CPR-13, which adds “using suitable APIs”] ZTE: The intention of sourced PRs is to obtain the sensing result from 6G system to UE, thus the UE could e.g. adjust beam direction for communication service or provide sensing service or combine with non-3GPP sensing data. [Ericsson: A question for clarification, what does authorize a UE means in this context? Is it an application on the UE that should be authorized to receive sensing result? The note is not needed and seems not fully aligned with the requirement. In the note it is the UE (modem ) that is using sensing result to optimize communication whereas the requirement is exposure sensing result to an application on the UE.] |
| CPR 14.1.10-1-14 | Original proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. Qualcomm proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide (including deriving and exposing using suitable APIs) sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. ZTE proposal is to delete this as they believe it is already covered by CPR-14.1.10-1-13 Telefonica proposal Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to provide sensing results for a specific service to a UE. | PR 7.5.6-4 |  | [QC: retain this if keeping -13 and -14] Ericsson: What does it mean “the UE is authorized by mobile network operator providing sensing service.” And how does that relate to provide sensing result to a specific application on the UE?] Huawei comments: Prefer the original wording of CPR-14 QC: delete CPR -13, the inserted text could also be included in a NOTE] [China Unicom]: this one is OK for me. The first one has been covered by the second one after the changes [ZTE: supports the merger of -13 and -14 and merged from QC looks fine.] |
| Alt CPR 14.1.10-1-16 | Huawei proposal Subject to network operator’s policy, regulatory requirements and subscriber permission, the 6G network shall be able to use the 6G sensing service to enable gesture recognition. Ericsson proposal is to delete this PR as it is included in the KPI tables (so a requirement is not needed) Philips Int BV proposal Subject to regulatory requirements and subscriber permission, the 6G network shall be able to use the 6G sensing service to monitor and recognize human gestures. | PR 7.24.6-1 |  | Gesture Recognition [Huawei]: Huawei: the wording is modified slightly [Siemens prefers to keep this requirement] Xoaomi2: note that TS 22.137 captures gesture recognition in KPI table as a supported service. Updated KPIs maybe sufficient assuming normative work is updated accordingly |
| CPR 14.1.10-1-18 | The 6G system shall be able to support energy-efficient sensing operations. | PR 7.5.6-2 | Energy-efficient Sensing Operations [LGE]: the context of "energy efficiency" and that of "efficient (something)" look similar, and that is often true. However, more benefit might come if we look closer. [PR 7.8.6-2] is intended to keep the "sensing service" offered to customers (human or robots) "stable" while minimizing resource waste. We hope this context is not lost through Xiaomi2: support to keep the focus of this CPR to provision of sensing services |  |
| CPR 14.10-1-new | The 6G system shall be able to provide a means to enable efficient use of sensing resources for stable sensing operation. | PR 7.8.6-2 | Provided separate CPR since no support to merge w/ CPR 14.1.10-1-18 |  |
| Alt CPR 14.1.10-1-19 | Huawei proposal Subject to operator’s policy and regulatory requirements, when offering sensing service, if the assistance information (e.g. the actual sensing target characteristics) from a trusted 3rd party is available, the 6G core network shall provide means for a mobile network operator to monitor and validate the sensing result (e.g. by comparing the sensing results with the actual sensing target characteristics etc.). Philips Int BV proposal [PR.7.26.6-1] Subject to operator’s policy, regulatory requirements and subscriber permission, when offering sensing service, if the assistance information (e.g. the actual sensing target characteristics) from a trusted 3rd party is available, the 6G network shall provide means for a mobile network operator to monitor and validate the sensing result against the assistance information (e.g. by comparing the sensing results with the actual sensing target characteristics etc.). | PR.7.26.6-1 | Assistance Info from trusted 3rd party [Huawei]: Huawei: 6G network -> 6G core network [Philips: it mentions validation, but it does not state against what the system validates the sensing result] [Philips Int BV: - I would feel safer if in Alt CPR 14.1.10-1-19 (Philips Int BV) also “subscriber permission” is added, because the 3rd party providing the assistance data may be trusted by the operator, but not necessarily by the subscriber, e.g. if Google is trusted by the operator and asks if a sensing result matches the description of a certain human or certain house or car of a subscriber, then the sensing service of the operator should not just answer that question e.g. with “yes” to that 3rd party without the subscriber giving permission to do so.] |  |
| CPR 14.1.10-1-20 | Subject to network operator’s policy, the 6G network shall be able to provide secure means to an authorized 3rd party for providing dynamic re-configuration requests of communication services in order to ensure continuous reliable connectivity to mobile UEs with the required Quality of Service as provided in Table 7.27.6-2 in a changing environment. | PR 7.27.6-1 | Dynamic reconfiguration requests for reliability [Huawei]: Any relationship with sensing service? [Ericsson: This requirement seems to be unclear without the table, by reading the requirement it is not clear that it is about sensing] [Siemens: This requirement should indeed be moved to a suitable other section. (Huawei made a similar comment, too.] |  |
| NEW CPR Was 14.1.1-2-1 | Subject to operator’s policy or regulatory requirements, the 6G network shall support sharing of radio access network with sensing capability among operators. Alternative proposal from Qualcomm, CATT and China Unicom: Subject to operator’s policy or regulatory requirements, the 6G network shall allow mobile operators to provide sensing services using a shared RAN (supporting 3GPP (e.g. 6G) wireless sensing) | PR 5.7.10.6-1 | Network Sharing Xioami2: should we confirm the intention(?) to maintain or achieve the required sensing service performance? Does this relate to all sensing services, including as requested by 3rd party to the (original) 6G network? |  |
| NEW CPR | Subject to operator’s policy and regulatory requirements, the 6G system shall provide a mechanism to predict characteristics of the environment and/or objects (e.g. shape and size). | PR 7.6.6-2 | Xioami2: Is environment prediction correct or is it objects or features within the environment? The UC 7.6 describes in table buildings and vehicles |  |
| Alt New CPR Moved from Table 14.1.5-3: Data Collection and Consumption | ZTE Proposal Subject to operator’s policy, the 6G network shall enable the base station to send sensing data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from base stations. Futurewei Proposal Subject to operator’s policy, the 6G network shall enable the sensing entity to send sensing measurement data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from sensing entity. | PR 7.5.6-1 | Sensing Data Processing FW: Also, this should be rephase to make it more general about collect sensing data from sensing entity (not only base station) Ericsson: This has been discussed there, this is making quite some architectural assumptions and should not be part of SA1 |  |
| New CPR Table 14.1.10-1-NEW EN | Subject to operator policy for operating non-3GPP sensors (e.g. WiFi), the 6G Network should be able to provide configuration information for the non-3GPP sensing operation under 3GPP operator control. NOTE: This requirement is applicable only to 6G Networks where the 3GPP operator operates non-3GPP sensors in addition to 3GPP sensors. | PR 7.25.6-1 | Pending agreement of S1-261036/1118 (clearing of EN) |  |

* * * End of Changes* * *

For information & convenience, here is a table of the removed CPRs (with reasons)

| Old (initial) CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.10-1-2 | Subject to operator policies, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics to be counted. | PR 7.23.6-1 | Approved in S1-260104 as 14.1.10-1-1 |
| 14.1.10-1-15 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | Approved in S1-260104 as 14.1.10-1-2 |
| Alt 14.1.10-1-4 Part 1 | Subject to regulation and operator’s policy, the 6G system shall support mechanisms to provide sensing service with a given sensing target density. | PR 7.7.6-1 | Approved in S1-260104 as 14.1.10-1-3 |