3GPP TSG-SA WG1 Meeting #113 	draft S1-261115

9-13 February 2026, Goa, India	(revision of S1-261091)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.12-1 (Immersive communication) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.12-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254296, Merged proposed changes on Clause Y.1.12 (Immersive) (SA1 #112)

- S1-254190 (ZTE Corporation)

- S1-254300r1 (Huawei)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up Table numbering (to align w/TR) and CPR numbering

- Added comments from S1-251081 (Qualcomm)

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.12-1: Immersive communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.12-1-1 | Subject to operator’s policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed throughput (current and/or predicted, within the boundary of the 6G system) of the associated ongoing session. Qualcomm proposal (S1-261082) Subject to operator’s policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed QoS (e.g. current and/or predicted throughput within the boundary of the 6G system) for the associated ongoing session. | PR 9.3.6-3 | Third-party Support QoS (throughput) availability QC: proposes some reword |
| Alt CPR 14.1.12-1-2 (Huawei) | Subject to operator’s policy, the 6G network (e.g. core network) shall support hosting of e.g. an AI/ML model in the Service Hosting Environment (excluding RAN) based on latency, traffic load or data privacy requirements. | PR 9.5.6-3 | Hosting AI/ML model in Service Hosting Environment [Huawei] to add ‘(excluding RAN)’ for better clarity [ZTE]: proposes to move this to 14.1.9 computing QC: Agrees to move to 14.1.8-5 (AI model and inferencing) |
| CPR 14.1.12-1-3 | The 6G system shall support service continuity for XR services between indoor and outdoor, and between Dense Urban and Rural deployments. | PR 9.9.6-1 | Service Continuity Nokia: is “mixed reality gaming” a defined service which justifies a dedicated requirement? Seems more related to KPIs |
| CPR 14.1.12-1-4 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide means to allow a trusted third-party to provide assistance information in order to identify and deliver the error tolerant data traffic. NOTE 1: The assistance information can be IP-tuple or something in existing header field that is used to mark the data for error tolerant data traffic. Qualcomm proposal (S1-261082) Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide means to allow a authorized third-party application (e.g. AI application) to provide assistance information for identification and delivery of the error tolerant data traffic associated with that third-party application. NOTE 1: The assistance information is used to identify which data is error tolerant . | PR 9.11.6-1 | Third party Support Assistance info to ID and deliver error tolerant data Nokia: NOTE is very technical, suggest to remove QC: Propose rewording but still have the following questions , Who is the trusted third party? How is the trusted party related to the traffic. The context is missing. NOTE also points towards a solution and requirement |
| CPR 14.1.12-1-5 | Subject to operator’s policy, the 6G system shall be able to transmit error tolerant data traffic with adaptive QoS . NOTE: The QoS (e.g. reliability and latency) can be adjusted automatically based on SLA with authorized third party application. | PR 9.11.6-2 | QoS Error tolerant data transmission |
| CPR 14.1.12-1-6 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a suitable means for error-tolerant communication. | PR 9.11.6-4 | QoS Error tolerant data evaluation |
| CPR 14.1.12-1-7 | Subject to operator’s policy and privacy considerations, the 6G system shall support a mechanism, including enabling one or more authorized third party(ies) to adapt XR session QoS dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the fluctuations of the 6G connection conditions and QoE policies. Qualcomm proposal (S1-261082) Subject to operator’s policy and privacy considerations, the 6G system shall support a mechanism, including enabling one or more authorized third party(ies), to adapt the QoS of an XR session dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the change of the 6G connectivity conditions and QoE policies. | PR 9.12.6-3 | Exposure QoS adaptation QoE Does “privacy considerations” need alignment? QC: Propose rewording |
| CPR 14.1.12-1-8 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a means to expose to a 3rd party application the status of the communication link specific to a user upon request from 3rd party. NOTE 3: The communication link status can be used by the application to better recover the lossy content. Qualcomm proposal (S1-261082) Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide a means to expose to an authorized third-party application the status of the communication link specific to a user upon request from the authorizedthird-party (e.g. the communication link status can be used by the application to better recover lossy content during communication). | PR 9.11.6-3 | Exposure Communication link status QC: Propose to move note to requirement. |
| CPR 14.1.12-1-9 | Subject to operator’s policy and privacy considerations, the 6G system shall provide means (e.g. association of XR content with location and relevant positioning information) to ensure an optimal placement in viewer relative to the real environment (e.g. to ensure safety or optimal interactions between participants, etc.). | PR 9.12.6-1 | Check if covered by R19 metaverse Placement of content in real environment Does “privacy considerations” need alignment? |
| CPR 14.1.12-1-10 | Subject to operator’s policy and privacy considerations, the 6G system shall provide a means to synchronize media/flow and data for XR rendering. | PR 9.12.6-2 | Synchronization of media flow and data for rendering Does “privacy considerations” need alignment? Added NOTE from v1.1.0 QC: Are these examples components of the XR space of associated with the flows that need synchronization.? It is not clear what e.g. list are. |
| CPR 14.1.12-1-11 | Subject to operator’s policy and privacy considerations, the 6G system shall provide a means to synchronize heterogeneous data flows from/to one or more UEs (e.g. phone, glasses, tethered ring) associated with a single user and between UEs associated with multiple users in a multimodal communication. | PR 9.12.6-4 | Synchronization of data flows across UEs Does “privacy considerations” need alignment? |
| Alt CPR 14.1.12-1-12 (Nokia) | Subject to operator’s policy and subscriber permission, the 6G system shall provide a means to keep heterogeneous media flows synchronized within an acceptable threshold whether rendered on a single or multiple UEs, for each user in a multimodal communication. | PR 9.12.6-5 | Synchronization of media flows on one or more UEs alignment? QC: Is there a definition for multimodal communication? Otherwise “for each user communicating multimodal data” |
| Alt CPR 14.1.12-1-11 merged with -12 (Nokia) | Subject to operator’s policy and subscriber permission, the 6G system shall provide a means to synchronise, heterogeneous data flows (including media flows) from/to one or more UEs associated with a single user, and also among UEs associated with multiple users in a multimodal communication. | PR 9.12.6-4 PR 9.12.6-5 | Synchronization of data flows across UEs |
| CPR 14.1.12-1-13 | Subject to operator’s policy, the 6G network shall support QoS adjustment based on network status to ensure user experience. | PR 9.13.6-1 | Adaptive QoS is covered by other CPR. Adjustment of QoS based on network status QC: The NOTE is a solution instead of requirement so we propose to remove |
| Alt CPR 14.1.12-1-14 (Nokia) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network may support exposure of predicted network status based on context information (e.g. user location, mobility path, and environmental knowledge to authorised 3rd parties. | PR 9.13.6-2 | Prediction of network status Nokia: shall & may in same requirement should be avoided. Prediction per se is not used unless for exposure based on the wording |
| Alt CPR 14.1.12-1-15 | ZTE Proposal Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide - means to receive information about the target user service experience and information related to the application from authorized UE or 3rd party applications (such as real time multimedia services) and, - support mechanisms to leverage information related to the application to efficiently deliver communication service and meeting the application’s performance requirements and meet target user service experience. NOTE: Example of information related to the application can include previous, current and/or expected user application characteristics. When provided by the UE, the network can verify the information. Qualcomm proposal (S1-261082) Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system (may include IMS) shall provide means to receive information about an application (e.g. previous, current or expected traffic characteristics) or information about target user service experience (e.g. for real-time multimedia applications) from authorized UE or 3rd party and leverage the information to deliver efficient - communication service and meet the application’s performance requirements - . | PR 9.14.6-1 PR 9.14.6-2 PR 9.17.6-1 PR 9.17.6-2 | Third Party support User experience |
| Orig PR | Based on operator’s policy, regulatory requirements and subscriber permission, the 6G system shall provide means to receive information about the target user service experience from authorized 3rd party applications (such as real time multimedia services.) | PR 9.14.6-1 | Provided for info |
| Orig PR | Based on operator’s policy, regulatory requirements and subscriber permission, the 6G network shall support mechanisms to efficiently meet target user service experience. | PR 9.14.6-2 | Provided for info |
| Orig PR | Subject to operator’s policy, 6G network shall provide means to receive information related to the application from an authorized UE or 3rd party application. NOTE: Example of information related to the application can include previous, current and/or expected user application characteristics. When provided by the UE, the network can verify the information. | PR 9.17.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy, the 6G network shall support mechanisms to leverage information related to the application in delivering efficient communication service and meeting the application’s performance requirements. | PR 9.17.6-2 | Provided for info |
| New CPR (moved from Table 14.1.1-2: Enhancements to legacy services and capabilities) | Subject to operator’s policy, the 6G system shall support the dynamic changes in QoS, when the traffic characteristic dynamically changes or is predicted to change, upon request by an authorised third party. | PR 6.61.6-1 | Duplicated in AI traffic table Dynamic QoS ZTE: It is suggested to merge PR 6.26.6-3, PR 6.52.6-3 with PR 6.61.6-1, which is proposed in AI traffic characteristics part. [Nokia-NEW]: Predicted by whom, NW or UE? AI might be able to predict something, but this is implementation specific. Point is that QoS can be changed under certain conditions and this is the requirement on network side. |
| New CPR (moved from Table 14.1.1-2: Enhancements to legacy services and capabilities) | Subject to operator’s policy, the 6G system shall support mechanisms to adapt QoS of the media traffic in a interactive XR communication (e.g. holographic) to achieve consistent user experience under varying network conditions. | PR 9.8.6-2 | Adaptive QoS Holographic Cooms |
| New CPR (moved from Table 14.1.1-2: Enhancements to legacy services and capabilities) | Based on operator’s policy, the 6G system shall enable a UE to indicate to the network coordination information (e.g. influencing policies for QoS handling) for transmitting the set of data flows for a multimodal communication session associated with an application. NOTE: One example of influencing policies can be alternative sets of data flows with different QoS requirements for the multimodal communication session, so that the network can dynamically determine optimal QoS handling, e.g. based on network resource availability. | PR 9.19.6-1 | UE input to NW for QoS Multimodal comms. |

* * * End of Changes * * *

Note (provided for information): the following initial table entries in S1-254020 were moved to the KPI discussions.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.12-1-16 | Table 9.2.6-1: Key Performance Indicator (KPI) for Immersive Gaming | Clause 9.2.6.1 (no PR #) | KPI |
| 14.1.12-1-17 | Subject to user consent, the 6G system shall support burst type QoS meeting the KPI requirements from Table 9.3.6-1: KPIs for multi-media services with deterministic experience via collaborative processing among UE-network-cloud | PR 9.3.6-1 | KPI [Huawei]: deterministic experience Need to reflect the PR together with the KPI table [Huawei] ok to replace ‘deterministic experience’ to ‘consistent user experience’ |
| 14.1.12-1-18 | The 6G system shall be able to provide deterministic user experience for multi-party call meeting the KPI requirements from Table 9.3.6-2: KPIs for deterministic user experience for multi-party call | PR 9.3.6-4 | KPI [Huawei]: deterministic experience Need to reflect the PR together with the KPI table [Huawei] ok to replace ‘deterministic experience’ to ‘consistent user experience’ |
| 14.1.12-1-19 | 6G system shall be able to support immersive media content production meeting the KPIs from Table 9.6.6-1: Performance requirements for immersive media content production via wireless link | PR 9.6.6-3 | KPI |
| 14.1.12-1-20 | Table 9.7.2-1: KPIs for media synchronization for multiple applications | Clause 9.7.2 (no PR #) | KPI [Huawei]: media synchronization for multiple applications |
| 14.1.12-1-21 | Dense Urban, Urban and Rural indoor/outdoor scenario should meet the KPIs in Table 9.9.6-1: Performance of Mixed Reality Gaming | PR 9.9.6-2 | KPI [Huawei]: KPI media synchronization for single application |
| 14.1.12-1-22 | Use case 9.12.6 (interactive immersive guided tour) needs KPIs from Table 9.12.6-1: KPIs for personalized interactive immersive use-cases | Clause 9.12.6 (no PR #) | KPI |