3GPP TSG SA WG 1 Meeting #113 	S1-261263

9-13 February 2026, Goa, India	(revision of S1-261029)

Source:	Nokia (ISAC KPI moderator)

pCR Title:	Pseudo-CR on consolidated ISAC KPI table

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.4

Document for:	Approval

Contact:	hideaki dot takahashi at nokia dot com

Abstract: This pCR provides the updated ISAC KPI table to reflect the email discussion outcome.

1. Introduction

An email discussion took place before SA1 #113 meeting to categorize ISAC KPI tables, based on Rel-19 ISAC table captured in TS 22.137.

2. Reason for Change

To update the consolidated ISAC KPI table by reflecting KPI categorization discussed by email in S1-261028.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.1.0.

* * * First Change * * * *

### 14.2.2	ISAC

The 6G system shall be able to provide sensing with the following performance requirements:

NOTE 1: The definitions of the terms used in the following KPI tables are defined in [6].

NOTE 2:	The references and notes for the sensing service categories of 1 to 7 are described in [6].

Table 14.2.2-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 (use cases 7.3, 7.5 UAV detect), 7.9 ship detection | 95 | 10 | 10 | N/A | N/A | 10 | 5 | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV,ship) |
|  | 2 (use cases 7.19) | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 (use cases 7.5 UAV tracking), 7.9, 7.19) | 95 | 1 | 1 | 1 | 1 | 1 | 1 x 1 | 100, or 1000; 5000 for detection in highway/ship in near water/inland waterway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, ship) |
|  | 4 (use cases 7.6 vehicle) | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
|  | 8 (use case 7.23 | 95 | 1 | 1 | 1 | 1 | 0.3 – 1 | 1 | N/A | ≤1000 | ≤1 | 2 | Outdoor UAV detection, classification and counting |
|  | 9 UC7.3 | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤ [500] | ≤ [0.1] | ≤ [5] | ≤ [5] |  |
| Environment monitoring | 5 | 95 | 10 | 0.2 | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
|  | 9 (use case 7.13) | 99.9 | ≤0.5 | 0.5 | 0.5 | N/A | 0.1- 1 | 0.5 | ≤100 | 0.1 - 1 | 1 | 1 | Enhanced XR user navigation |
|  | 10 (use case 7.24) | 99 | 0.1 | 0.1 | 0.3 for head gesture, n/a otherwise | 0.3 for head gesture, n/a otherwise | 0.01 to 0.1 | 0.3 for head gesture, n/a otherwise | ≤ 20 | ≤ 0.02 | ≤ 1 | ≤ 10 | Gesture recognition in industrial environments |
|  | 11 (use cases 7.16, 7.18) | 99.99 | 0.1 | 0.1 | 1 | 1 | 0.5 | 0.5 | 100 - 500 | 0.1 - 1 | 0.1 | 1 | Collaborative sensing |
| Digital Twin of Environment | 12 (use case 7.27 path way) | 99 | 0.25 | 0.25 | 0.1 | n/a | 0.75 | 1.5 | ≤100 | 0.1 | < 10% | < 10% | Indoor digital twinning for static objects |
|  | 13 (use cases 7.14 robot, 7.27 surrounding) | 99 | 0.1 | 0.1 | 0.1 | n/a | 0.5 to 1 | 1.5 | ≤100 | 0.1 | < 10% | < 10% | Indoor digital twinning for moving objects |
|  | 14 (use cases 7.6 building, 7.14 robot, 7.15) | 95 | 4 | N/A | N/A | N/A | 4 | N/A | TBD | 1 | 1~2 | 1~2 | Outdoor digital twinning for static objects |
|  | Use case 7.7 | 95 | 3 | N/A | 3 | N/A | 1 | 1 | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 | Sensing target moving speed: up to 140 km/h Sensing target density is required |
|  |  | Use case 7.7 |  | 95 |  | 0.5 |  | N/A |  | 0.5 |  | N/A |  | 0.2 |  | 0.2 |  | 1~5 |  | ≤ 0.1 |  | ≤ 5 |  | ≤ 5 |  | Sensing target moving speed: Static/moving, up to 5 km/h; Sensing target density is required |  |
|  | Use case 7.22 | 95 | 0.1 | 0.1 | N/A | N/A | N/A | N/A | 5000 | 60 | 5 | 5 | Outdoor for static micro objects |
|  | 15 (use cases 7.4 vehicle, 7.6 vehicle, 7.14 robot) | 95 | 0.10 | 0.10 | 1.5 | N/A | 0.4 | 0.5 | 50 | ≤ 0.2 | ≤5 | <1 | Outdoor digital twinning for moving objects |
| NOTE 1: ... |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 14.2.2-2: Additional performance requirements for sensing service category 12 and 13 in digital twin of environment.

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS availability: target value (%) | CS reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area | Remarks |
| 99.999 | 4 h | < 20 ms | Sensing Result: 1 Mbit/s Sensing Data: 2.5 Gbit/s | 15 kByte 50 kByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Sensing Information: Robot third party application |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per video camera stream | 4 MByte | 33 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Video Sensing: (Robot third party application) |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per stream | 1 MByte to 5 MByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Application data derived from aggregated sensing results: (third party application Robots) |
| NOTE: References are for communication between robot and third party application [373] and for video sensing [372]. |  |  |  |  |  |  |  |  |  |  |

Table 14.2.2-3: Performance requirements for  combined communication, sensing and positioning services

|  | Communication KPIs |  |  |  | Positioning KPIs | Sensing KPI |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | User experienced data rate | E2E latency | Communication Service availability | Connection density | Location accuracy | Sensing accuracy |
| Combined communication, sensing and positioning services (UC 7.x Network assisted smart transportation) | [1-10 Mb/s] | [20 ms] | 99.99% (note 1) | 104 devices/km2 | [1m] * [1m] * [1m] with 90% probability (note 2) | category 2 or 3 (notes 2, 3) |
| NOTE 1: within service volume NOTE 2: within 99% of the service volume NOTE 3: Category 2 or 3 in Table 6.2-1 in TS 22.137 [6] |  |  |  |  |  |  |