3GPP TSG-SA WG1 Meeting #113 	S1-261387

9-13 February 2026, Goa, India	(revision of S1-261339, S1-261318, S1-261312, S1-261238, S1-261099)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.4-1 (Energy-related Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.4-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254160 (ZTE)

- S1-254250 (Qualcomm)

- November 2025 reflector comments

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Removed PR 5.8.7.6-1 since it in Table 14.1.7-1: NDT

- Removed PR 7.5.6-2 since it is in Table 14.1.10-1: ISAC

- Removed PR 6.16.6-1 since it is in the TR as approved CPR 14.1.8-1-3

This revision captures the discussions in the intial CPR discussions. Changes from the draft version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

This revision adds a CPR that was discussed and agreed in another session.

This revision captures the (to date) unagreed CPRs for this table. CPRs that were “all greem” (aka agreeable) were moved to S1-261353.

This document is the current status of the consolidation work on this table with proposed changes from KPN.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.4-1: Energy-related Aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Orig PR | Subject to operator’s policy and regulatory requirements, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time), with the respective information on predicted energy consumption and carbon equivalent emissions per scenario. NOTE: The following network energy information about energy consumption and carbon equivalent emissions can be calculated and/or estimated. | PR 5.8.6.6-1 | Provided for info |
| Orig PR | Subject to regulatory requirements, the 6G system shall be able to optimise the network energy saving considering the current traffic conditions of the network. | PR 5.8.4.6-1 | Provided for info Huawei: suggest to keep the original wording. |
| CPR 14.1.4-1-10 | Subject to operator’s policy and regulatory requirements, the 6G network shall provide suitable network energy saving methods enabling the operator to select among different service delivery options (e.g. when the requested service is delivered) based on - respective information regarding predicted energy consumption - carbon equivalent emissions per service delivery option based on information received from the energy provider, if applicable - and/or the current network traffic conditions. NOTE: This requirement describes service delivery options where the user experience could be impacted. | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving |
| Orig PR | Subject to operator’s policy and regulatory requirementsregulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related with to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | PR 6.37.6-1 | Provided for Info |
| Orig PR | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party | PR 6.37.6-2 | Provided for Info |
| CPR 14.1.4-1-11 | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to expose to an authorised third-party the network energy consumption information and carbon equivalent emissions based on information received from the energy provider (e.g. derived through monitoring and/or estimation) related to 3GPP services, over a specific time period (e.g. month). | PR 5.8.6.6-2 PR 6.37.6-1 PR 6.37.6-2 | Network (Sensing, AI, computing) related Energy Consumption Huawei: not clear how to expose the energy consumption for service. Need clarify. Nokia: prefer to avoid the word monitor, which may not always be achievable (prefer original CPR) |
| CPR 14.1.4-1-13 | The 6G network shall be able to support mechanisms to select computing resources in the Service Hosting Environment (excluding RAN) considering energy related information from authorized 3rd party (e.g. energy management system). | PR 6.24.6-3 | According to the use case of 6.24, revise it as: Huawei: clarify what is the energy supplier, and what is the relationship to 3GPP. |
| CPR 14.1.4-1-14 | Subject to operator’s policy, the 6G network shall provide 3GPP services (e.g. sensing, AI, computing) considering energy consumption information and carbon equivalent emissions based on information received from the energy provider (e.g. related to network resources). | PR 5.8.6.6-3 |  |

* * * End of Changes * * *