3GPP TSG SA WG 1 Meeting #113 	S1-261049

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.14-5 Other

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254298. This table was not discussed during SA1 #112 but addressed the comments from SA1 #112 from companies’ emails and draft on wording proposals.

Based on the outcome of discussion in SA1 #112 Ad hoc meeting, the following wording is changed:

User consent – subscriber permission

Based on operator policy – subject to operator’s policy

This pCR proposes to update Table 14.1.14-5 (Other) in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change (All New CPRs) * * * *

Table 14.1.14-5 – Other

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.14-5-1 | The 6G network with AI capabilities should be able to collaborate with the AI capabilities in the authorized third-party (e.g.: split inference) to assist Real Time Digital Twin in meeting the required service performance. | PR 11.4.6-2 | Digital Twin [ZTE] move to AI CPRs: AI Model training and inferencing |
| QC: CPR 14.1.14-5-1 | The 6G network with AI capabilities should be able to collaborate with an authorized third-party. e.g. for split inference to assist Digital Twin applications in meeting the required service performance. |  |  |
| CPR 4.1.14-5-2 | Subject to operator’s policy, the 6G network shall be able to expose information related to a network slice (e.g. current or predicted latency and reliability) to an authorized third party. | PR 11.21.6-1 | Exposure |
| CPR 14.1.14-5-3 | Subject to regulatory requirements and operator’s policy, the 6G system shall support a means for an MNO to improve availability for regulated services (e.g. PWS [62], emergency call [58]) despite a lack of energy supply, through exchange of information with a DSO. | PR 11.18.6-1 | [ZTE] move to resilience CPRs |
| CPR 14.1.14-5-4 | The 6G System shall ensure that IoT communication services cause minimal impact on other services (e.g. regular voice, video and data). | PR 11.25.6-1 | IoT |
| CPR 14.1.14-5-5 | Subject to the operator’s policies and control, the 6G system shall provide means to support extended coverage. | PR 11.25.6-2 | IoT [ZTE] what is the meaning of “extended coverage”? QC: This seems not IOT specific |
| CPR 14.1.14-5-6 | The 6G System shall support diverse device types with long lifetime (e.g. 10 to 20 years). NOTE: 3GPP should consider support of device lifetimes longer than the lifetime of the supporting 6G core network. | PR 11.25.6-3 | IoT [ZTE]It is covered by the KPI table 11.25.6-1 Propose to delete |
| CPR 14.1.14-5-7 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall be able to support means to identify the candidate UEs based on their location and relative distance between them and provide communication service among these UEs on demand. | PR 11.27.6-1 |  |
| CPR 14.1.14-5-8 | Subject to regulatory requirements and operator’s policy, the 6G system shall be able to provide secure and continuous communication with agreed QoS between a group of designated critical-infrastructure UEs in a limited service area in case of service disruptions in the mobile network. NOTE 1: An example for such a group of designated critical-infrastructure UEs are UEs that manage LV grid flexibility—such as load shifting or energy storage utilization—based on real-time electricity demand and LV grid conditions (controllers DS of the LV-cells, smart grid devices (e.g. renewable energy sources, energy storage) in the LV-cells). NOTE 2: New additional devices will not join the group of designated critical-infrastructure UEs. NOTE 3: Service area is up to 20 km x 30 km. | PR 11.29.6-1 |  |
| CPR 14.1.14-5-9 | The 6G system shall provide suitable means at a designated critical-infrastructure UE to provide TX (sending) timestamps with accuracy of ±4 µs to an authorised third party application at the UE. NOTE 4: The KPI of this requirement is taken from [414]. | PR 11.29.6-2 | Exposure |