13GPP TSG-SA WG1 Meeting #113 	S1-261311

9-13 February 2026, Goa, India	(revision of S1-261239, S1-26110)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.3-1 (Resilience) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.3-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254092 (China Telecom)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

- S1-254159 (NTT DOCOMO)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

Differences from the latest draft version:

- Removed initial CPRs if alternative(s) were proposed

- Removed CPRs if company proposing them requested them to be removed/withdrawn.

- Removed comments no longer needed (Table moved, alignment notes)

- Cleaned up CPR numbering

- Added PRs 5.6.1.2-1 and 5.6.3.6-1 from Table 14.1.7-2: OAM

This revision captures the discussions in the intial CPR discussions. Changes from the draft version include:

- Revising the CPR numbers, removing change marks and comments from “green” CPRs (e.g., preparing them to final form for agreement). Removing “gray” (orig PRs provided in Table for info).

- With the “greening” of PRs 5.9.4.2-1, 5.9.5.6-1 and 5.9.5.6-2, removed proposals that included them to merge. That left PR 5.6.2.6-1 to be discussed/consolidate as a separate CPR.

- Removed PRs 5.6.2.6-1 and PR 5.6.2.6-2. They will be addressed in Table 14.1.14-2 – Localized network.

This revision adds in CPRs from Table Table 14.1.14-5 Ind/Vertical) to be discussed/consolidated into this Table.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.3-1: Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.3-1-1 | Subject to operator’s policy and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience. | PR 5.9.5.6-1 PR 5.9.5.6-2 |  |
| CPR 14.1.3-1-2 | The 6G system shall be able to support means to simplify network operation and service delivery compared to 5G system, e.g. in order to reduce signalling, minimise connection setup time. NOTE: This simplification should not mean reducing service quality provided | PR 5.9.4.2-1 |  |
| CPR 14.1.3-1-3 | Subject to operator’s policy and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected/not affected parts of a single or multiple network(s) serving an area impacted by a specific type of disaster. NOTE: In case of multiple networks serving the disaster area, it is assumed that under disaster conditions, the remaining available parts of the networks in that area can serve the UEs of other networks. | PR 5.6.4.6-1 |  |
| CPR 14.1.3-1-4 | The 6G network shall provide a mechanism to minimize the signalling arising from recovery of network failure. NOTE: Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element. | PR 5.6.5.6-1 |  |
| CPR 14.1.3-1-5 | Subject to operator’s policy and regulatory requirements, the 6G system shall support mechanisms to prolong the operation time of the system when in a situation of a power-grid outage or power shortage. NOTE: During a power-grid outage situation, the network may run on generators with a limited energy lifetime. | PR 5.6.6.2-1 |  |
| CPR 14.1.3-1-6 | Based on operator’s policy, regulatory requirements and agreements with spectrum licensees, in response of disaster situation the 6G network may obtain spectrum scanning results of surrounding 3GPP networks to determine spectrum availability in a geographical area. NOTE 1: The collected spectrum scanning results can be used in the establishment of a temporary private 6G network and to enable spectrum coexistence between networks. NOTE 2: This requirement is to enable first responder and disaster relief private networks to perform spectrum scanning and rapidly provide service only to first responder UEs. | PR 12.5.6-1 |  |
| CPR 14.1.3-1-7 | Subject to operator’s policy and regulatory requirements, the 6G network (OAM) should support autonomous detection, isolation, and recovery from network or service failure with minimum human intervention, to ensure continuous operation. | PR 5.6.1.2-1 PR 5.6.3.6-1 |  |
| CPR 14.1.3-1-8 | Subject to operator’s policy and regulatory requirements, 6G network should support suitable means to minimize the degradation of services in case of abnormal transmission over the user plane path (e.g.between the public network and edge network). | PR 5.6.1.2-2 | Edge resilience From S1-254250 [QC] clarify “stablity”…change to resilience? |
| CPR 14.1.3-1-9 | Subject to operator’s policy and regulatory requirements, the 6G system shall support a means for an MNO to exchange information with a DSO to improve availability of regulated services (e.g. PWS [62], emergency call [58]), e.g. in case of power outage. | PR 11.18.6-1 | Was CPR 14.1.14-5-3 (Moved from Table 14.1.14-5 Ind/Vertical) |
| CPR 14.1.3-1-10 | Subject to operator’s policy and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. Comment: the 3rd party is unclear | PR 5.6.2.6-1 | Localised Network? provision network services on-demand |
| CPR 14.1.3-1-11 | Subject to operator’s policy and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand, but is available from the PLMN of the same operator | PR 5.6.2.6-2 | Localised Network? authorize a local UE accessing service in PLMN |

* * * End of Changes * * *