8.3GPP TSG-SA WG1 Meeting #113 	draft S1-261xxx

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.11-1 (Satellite-based communication) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.11-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting. As agreed in SA1 #112 Ad Hoc-e, Table14.1.11-1 will include CPRs about the communication with satellite access and/or satellite backhaul.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254412, the output of the initial consolidation discussions for Ubiquitous Connectivity (SA1 #112)

- S1-254120  and S1-254121 (CATT)

- S1-254163 (ZTE Corporation, CSCN)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

There are some CPRs/PRs that are included in both Table 14.1.11-1 (Satellite-based comms) and Table 14.1.11-3 (Other aspects). This needs to be resolved, and I tried to mark those entries.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite. | PR 8.3.6-1 PR 8.3.6-3 | LEO Coverage Flex configs [QC} this PR may need some further rewording / clarification. Our stage-2 folks cannot understand the meaning of it. |
| Alt CPR 14.1.11-1-1 (CATT) | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| Orig PR | The 6G system with satellite access shall optimize the coverage per LEO satellite. | PR 8.3.6-1 | Provided for info |
| Orig PR | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation. | PR 8.3.6-3 | Provided for info |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity |
| Alt CPR 14.1.11-1-2 (CATT) | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity ZTE & Qualcomm proposals were marked to be not pursued in S1-254412. |
| CPR 14.1.11-1-3 | The 6G system with satellite access shall support service continuity within NTN, and between TN and NTN, minimising the interruption to the UE during the transition | PR 8.4.6-1 PR 8.8.6-1 | Service continuity TN-NTN |
| Alt CPR 14.1.11-1-3 (Qualcomm) |  | PR 8.4.6-1 PR 8.8.6-1 | Service continuity TN-NTN CATT & ZTE/CSCN proposals were marked to be not pursued in S1-254412. [QC]: already part of existing 5GS requirements. |
| Orig PR | The 6G system shall be able to ensure service continuity with minimum interruption for UEs during the transition between terrestrial and satellite access and vice versa. | PR 8.4.6-1 | Provided for info |
| Orig PR | The 6G system with satellite access shall support service continuity within NTN, and between TN and NTN. | PR 8.8.6-1 | Provided for info |
| CPR 14.1.11-1-5 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS [62] for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | PWS Text Messaging Adverse propagation conditions |
| Alt CPR 14.1.11-1-5 (CATT) | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | PWS Text Messaging ZTE/CSCN proposal was marked to be not pursued in S1-254412. |
| Orig PR | Subject to regulatory requirements, the 6G system with satellite access, shall be able to support PWS [62] for broadcasting warning notifications to UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 | Provided for info |
| Orig PR | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation. | PR 8.3.6-3 | Provided for info |
| CPR 14.1.11-1-8 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-UE direct communications |
| Alt CPR 14.1.11-1-8 (CATT) | The 6G system shall support communication between UEs which connect to the6G network (including base station) onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | Other non-terrestrial access] UE-HAPS-UE communications ZTE/CSCN and Qualcomm proposals were marked to be not pursued in S1-254412. [FW] base station along can’t route traffic between UEs. |
| CPR 14.1.11-1-9 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching [QC] should be moved out of this table (about SAT access), e.g. can go in “others” CATT & ZTE/CSCN and proposals were marked to be not pursued in S1-254412. FW: agree with QC |
|  |  |  | Time synchronization Moved to Table 14.1.11-3 |
|  |  |  | Moved to Table 14.1.11-3. |
| CPR 14.1.11-1-12 Resolve to which table these PRs/CPRs belong | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) in the satellite and mechanisms to - , - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.15.6-1 PR 8.9.6-1 PR 8.9.6-3 PR 8.9.6-4 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing [Huawei]: propose to move this to Table 14.1.11-3 (Other aspects) CATT proposed to move PR 8.9.6-2 to be moved to Table 14.1.11-3: Other aspects [FW] not clear “minimise bandwidth” means as bandwidth is relative fixed. Or need rephase. |
| Alt CPR 14.1.11-1-12 (ZTE/CSCN) Resolve to which table these PRs/CPRs belong | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) to minimise the necessary bandwidth of the inter-satellite and feeder links, and provide a computing service via to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing [ZTE] It is suggested to seperate which are targeting different aspects. [FW] agree to separate CPRs |
| New CPR 14.1.11-1-NEWa Resolve to which table these PRs/CPRs belong | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV), provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 | Revised as part of SA1 112 Ad Hoc-e CATT proposed to move to Table 14.1.11-3: Other aspects FW: suggest to move this data collection and consumption clause |
| New CPR 14.1.11-1-NEWb Resolve to which table these PRs/CPRs belong | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 |  |
| Orig PR | The 6G network using satellite access shall be able to provide the service hosting environment onboard satellite minimizing the necessary bandwidth of the inter satellite links and feeder links. | PR 8.15.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 | Provided for info |
| CPR 14.1.11-1-13 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions. | PR 8.18.6-1 | QoS Improvement [ZTE] remove the examples which are solution oriented. CATT proposal was marked to be not pursued in S1-254412. |
| NEW CPR | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services to/from UEs in adverse propagation conditions e.g. indoor or outdoor conditions. | PR 8.6.6-3 | Text Msg Service Not included in previous table. Could go into enh legacy? |

* * * End of Changes * * *

Note (provided for information): the following initial table entries in S1-254020 were moved to the KPI discussions.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.11-1-4 | The 6G system with satellite access, shall support SMS delivery to a high density of UEs i.e. up to [1000] UE per km2. | PR 8.4.6-2 | KPI (Y.2) high density # of UEs SMS CATT, QC, ZTE/CSCN all proposed to move to KPI |
| 14.1.11-1-6 | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | eMBB [ZTE] It is suggested to merge it with KPI table 8.6.6-1 |
| 14.1.11-1-7 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes [QC]: These values are already part of the merged KPI table. |
| 14.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB [ZTE]: This is part of KPI table. |
| 14.1.11-1-15 | The KPIs in Table 8.8.6-1: KPIs for global mobile video should be met for global mobile video service. | PR 8.8.6-2 | KPI Global mobile video service |